## Work on Availability Objectives from ANM Committee

## Excerpt from ANM2 report:

## Review / Develop a Recommendation on Availability Objectives of Aids to Navigation (Task 8, wording amended)

It was agreed by the WG that the Draft Recommendation on Availability Objectives for Tradition Aids to Navigation (prepared by the ENG Committee, Dated October 26, 2001), which was submitted to the Council in December 2001, should be revised. There was a consensus that this document represents a good basis for development of the final recommendation to the Council.  The WG prepared a draft recommendation (ANM2/WG2/WP1) to be brought forward to ANM3 for further review.

Action Item

G. Prosser, A. Châteauvert and L. Mendes were asked to advance the draft recommendation on Availability Objectives (ANM2/WG2/WP1) and submit the resulting document to ANM3.

(resulting document is attached for the information of the EEP Committee)

Draft - Recommendation on

Availability Objectives

of Aids to Navigation Services

THE COUNCIL,

RECOGNISING that it is important for member authorities to state management objectives for operational performance levels of Aids to Navigation services provided to mariners;

RECOGNISING FURTHER the need to provide guidance on suitable and realistic levels of operational performance for members to adopt;

RECALLING that IALA recommends (E-105) that “purchasing authorities include reliability and quality requirements in their specifications” when procuring Aids to Navigation equipment;

NOTING that it is possible to ensure the required level of availability during the design phase of Aids to Navigation services since the theoretical relationship between individual component reliability and system availability is known;

NOTING ALSO that many Aids to Navigation Authorities already have adopted the suggested availability objectives in their management of Aids to Navigation systems;

HAVING CONSIDERED the proposal of the EEP, RNAV and ANM Committees;

RECOMMENDS that all members providing Aids to Navigation should:

- Categorise each individual or group of aids to navigation as per the categories defined in the Annex to this Recommendation, utilising an appropriate risk assessment methodology (IALA Guidelines on Risk Management, December 2000) and, preferably involving a form of consultations with the mariners;

- Adopt the availability objectives set out in the annex to this recommendation in accordance to the level of service factors (IALA Guidelines on Level of Service, October 1997);

- Define the failure state(s) for each type of Aids to Navigation service provided (see note 4);

- Establish formal procedures for collecting, processing and recording availability data.

# ANNEX

Draft - RECOMMENDED AVAILABILITY OBJECTIVES OF

AIDS TO NAVIGATION SERVICES

Introduction

Availability Objectives of aids to Navigation Services have traditionally been linked to the size and complexity of the Aid to Navigation or system concerned, for example, major lighthouses rated as Category 1 and light buoys as Category 3.  This correlation although relevant for past Aids to Navigation and systems, may not always be applicable as changes occur in navigational safety requirements.

The adoption of contemporary risk management practices enables the aids to navigation management authority to reappraise define, preferably in consultations with mariners, the availability requirements for the Aid to Navigation or system concerned and assess its future categorisation based upon its criticality to the mariner and to the protection of the marine environment.  The resulting categorisation of the Aid to Navigation or system may result in some traditional higher category aids being downgraded and alternatively the potential for lower categorised aids to be increased.

There are many examples around the world, where authorities and organisations are employing modern risk management practices in ‘safety critical’ operations to ensure that resources are utilised in the most efficient manner while contributing to safe navigation and to the protection of the marine environment.

Definitions / Acronyms

Availability:	The probability that an Aid to Navigation or a mix of aids to navigation, referred as aids system, is performing its specified function at any randomly chosen time, this is expressed as a percentage of total time that an Aid to Navigation or system is performing its specified function.

Considerations

- The Availability Objectives will depend on a Risk Assessment methodology taking into consideration factors such as:

| - Significance of the waterway - Environmentally sensitive areas - Nature and type of cargo & navigation - Traffic density | - Aids to Navigation coverage - Climatology - Political considerations |
| --- | --- |

- The Availability Objectives may also depend on aspects such as:

| - Existing technology - Logistics - Redundancy | - Accessibility - Other navigational services available to the mariner including, Pilotage, VTS, GNSS services etc. |
| --- | --- |

- The Availability Objectives should preferably be defined or confirmed in consultations with mariners of the particular aids to navigation or aids system.

Categories

| CATEGORY | AVAILABILITY OBJECTIVE | CALCULATION |
| --- | --- | --- |
| 1 | 99.8% | Availability Objectives are calculated over a three-year continuous period, unless otherwise specified |
| 2 | 99.0% |  |
| 3 | 97.0% |  |

See Note 2

Category 1: 	An Aid to Navigation (AtoN) or mix of AtoN that is considered by the Competent Authority to be of primary navigational significance. For example, lighted aids to navigation and racons that are considered essential for marking landfalls, primary routes, channels, waterways or new dangers.

Category 2: 	An AtoN or mix of AtoN that is considered by the Competent Authority to be of navigational significance. For example, it may include any lighted aids to navigation and racons that mark secondary routes and those used to supplement the marking of primary routes.

Category 3: 	An AtoN or mix of AtoN that is considered by the Competent Authority to be of less navigational significance than Category 1 and 2.

Notes

- The absolute minimum level of availability for individual Aids to Navigation is 95%, calculated over a three-year period. When the minimum availability of 95% cannot be achieved, alternative system design or aids to navigation configuration should be considered;

- IALA Recommendations related to specific types of AtoN should be consulted;

- The availability of an Aid to Navigation may be calculated using the following equation, using the most accurate time available:

Availability= Total Time−Down TimeTotal Time

The IALA Navguide should be consulted for further information on the method of calculating the availability;

- A failure state is the malfunction of an aid to navigation or mix of AtoN to display its correct characteristics or to be on its assigned position for its proper use by the mariners. As such, a failure of a technical function is not necessary considered an aid to navigation discrepancy. For example, if the main power supply has failed but the light continues to function at normal intensity on standby power, this is not considered a failure or discrepancy since the aids to navigation continue to provide its correct characteristics to the mariners.

Related Documentation

IALA Aids to Navigation Guide (Navguide)

Draft IALA Guidelines on Risk Management, December 2000