Summary of Responses to Ambient Light Level Questionnaire

At EEP1 a questionnaire on Ambient Light Levels at which AtoN lights should be switched on and off was prepared and subsequently circulated to IALA members. The results where made available to the committee at EEP2 where undersigned offered to prepare a report summarising the responses received.

A total of 8 responses where received. These came from Finland, France, Germany, Sweden, Denmark, Holland, England and Hong Kong. The responses have been collated in enclosure 1.

The following observations have been made:

- Most of the respondents have established formal policies defining at what ambient light level their AtoN’s turn on and off.

- One respondent (Germany) seems to have no ambient light sensors at their lighthouses, but controls ON and OFF switching on the basis of the time of sunset – 1 hour and sunrise + 1 hour at a reference location. This method seems to discard any effects from varying meteorological conditions.

- The ON switching level for lighthouses varies in the range 20-100 lux.

- The ON switching level for buoys varies in the range 15-200 lux.

- The OFF switching level for lighthouses varies in the range 40-200 lux.

- The OFF switching level for buoys varies in the range 40-200 lux.

- According to one respondent it is important to ensure that a system of AtoNs turn ON and OFF at approximately the same time, in particular on leading lines where lights preferably should switch simultaneously.

- Most of the respondents have a switching time delay (hysteresis) to avoid switching oscillation but the switching delay time varies in the range 15-600 seconds.

- In many cases the switching level can be adjusted, also in the field.

- Both resistor (LDR) and semiconductor type sensors are used.

- At lighthouses, most respondents face the sensors away from the sun.

- During manufacturing process, switching levels may or may not have been tested against ambient light.

- In many cases lighthouse switching levels are tested during commissioning and in some cases monitored afterwards.

- Buoy lanterns are mostly not tested during commissioning, but in some cases monitored afterwards.

- The switching level adjustments may either be done by flasher programming or by mechanical means including adjustable optical filters.

Ómar Frits Eriksson

Head of Technical Division

Royal Danish Administration of
Navigation and Hydrography

Enclosure 1 to “Summary of Responses to Ambient Light Level Questionnaire”

|  | Question | Comments (Lighthouse) | Comments (Buoy) |
| --- | --- | --- | --- |
| 1 | Does your organisation have standards/policy for switching of light levels of AtoN’s? | NO = 1, YES = 4 “ON switching level is set” “We depend upon lantern suppliers standard for their equipment and is normally one hour before dark and one hour after dawn” “There are no switching sensors in lighthouses” | NO = 1, YES = 5 “ON switching level is set” “as supplied by lantern manufacturer” “We depend upon lantern suppliers standard for their equipment and is normally one hour before dark and one hour after dawn” |
| 2 | ON switching level (lux)? | 20, 50, 100, 100, 50-100 “Depends on the sunset. Light switches ON one hour before sunset in the city of Cuxhaven.” | 15, 20, 50-00, 100, 100, 100, 200 |
| 3 | OFF switching level (lux)? | 40, 100, close to 100, 150, 200 “Depends on the sunset. Light switches ON one hour before sunset in the city of Cuxhaven.” | 40, 60, 100, 100-150, 150, 200, 200 |
| 4 | Acceptable tolerance levels? | +30 and -0, ON 10-40 and OFF 10-60, Varies from 70-350 “Switching levels are field adjusted according to amount of trees and other obstructions around AtoN to have AtoN switch on and off times approximately equal between AtoN’s. Adjustment is especially important and difficult on leading lines because lights should switch on and off simultaneously.” | ±20 10% ON 10-40 and OFF 10-60 Varies from 70-350 Not defined |
| 5 | Is there a switching time delay? | NO = 2 YES = 4 | NO = 2 YES = 6 |
| 6.1 | Switching delay time? | 60, 15-60, 15-240, 600 seconds | 15-60, 15-240, 30, 60, 60, 600 seconds |
| 6.2 | Can the switching level be adjusted? | NO = 2 YES = 4 | NO = 1 YES = 5 “Only in some lanterns” |
| 7 | What is the adjustment range of switching level | 2 lux 10-1000 lux ±50% | 2 lux 15-60 lux, 10-1000 lux ±50% |
| 8 | Can the switching level adjustment be factory or field set? | YES = 2 “Factory and field” “Field set” “Factory setting is usually 100 lux” | YES = 2 “Factory and field” “Field set”, “Field” “Factory setting is usually 100 lux” “Only factory/workshop set” |
| 9 | What type of light switching sensor is used? | LDR = 3 “Cadmium Sulphate Photocell” “Photodiode” “Photoconductive cell” | LDR = 3 “Cadium Sulphate Photocell” “Photodiode”, “Photoconductive cell” “Phototransistor BP 103/4 |
| 10 | Which direction does the sensor face (vertical, facing the sun or away from the sun)? | “Facing North”, “Vertical facing North” “Vertical away from the sun” “Away from the sun” “To the most open direction and away from the forest” “Fitted in base of a light tube which collects light all around (360 degrees)” | “Typically horizontal” “Vertical, sensor is facing the sun” |
| 11.1 | Are switching levels tested against ambient light levels during manufacture? | YES = 2 NO = 3 | YES = 3 NO = 2 |
| 11.2 | Are switching levels tested against ambient light during commissioning (user)? | YES = 3 NO = 2 “On and Off times are monitored after commissioning” | YES = 1 NO = 4 “On and Off times are monitored after commissioning” |
| 11.3 | Please provide details of switching level adjustment procedures. | “With old flashers adjustment is made by rotation optical filter. With new programmable flashers adjustment can be made either by filter or programming” “Mechanical Adjustment” “to cover switching resistor” “Resistor value” | “DIP Switch setting according to manual” “Programmable flasher” “LMT lux measurements lamp + daylight. “to cover switching resistor” |