DRAFT

Guide on Sector lights and Sector Projector Lights

# TABLE OF CONTENTS

TABLE OF CONTENTS	2

1.	INTRODUCTION	3

1.1.	The application of Sector Lights	3

1.2.	Types of Sector Lights	4

1.3.	Historical evolution of Sector Lights	5

2.	BASIC PRINCIPLES	6

2.1.	Definitions	6

2.2.	General considerations for Sector Lights	7

2.3.	Methods for creating sectors	8

2.4.	Sector boundaries	9

2.5.	Distribution between white and a coloured light seen in sector boundary.	10

2.6.	Minimum width of a sector	11

2.7.	Variation in luminous intensity between adjacent sectors	11

2.8.	Total Subtense?	11

3.	TRADITIONAL SECTOR LIGHTS	12

3.1.	Principle of operation	12

3.2.	Level of performance	12

3.3.	Technical considerations	12

3.4.	Operational considerations	12

4.	LASER SECTOR LIGHTS	13

4.1.	Principle of operation	13

4.2.	Level of performance	13

4.3.	Technical considerations	13

4.4.	Operational considerations	13

5.	PORT ENTRY SECTOR LIGHTS (PEL)	14

5.1.	Principle of operation	14

5.2.	Level of performance	14

5.3.	Technical considerations	14

5.4.	Operational considerations	14

6.	SLOT SECTOR LIGHTS	15

6.1.	Principle of operation	15

6.2.	Level of performance	15

6.3.	Technical considerations	15

6.4.	Operational considerations	15

7.	LED SECTOR LIGHTS	16

7.1.	Principle of operation	16

7.2.	Level of performance	16

7.3.	Technical considerations	16

7.4.	Operational considerations	16

8.	FURTHER CONSIDERATIONS	17

8.1.	Support structures	17

8.2.	Installation considerations - reflections	17

8.3.	Integration with other light system	17

8.4.	Maintenance	17

8.5.	Sector checking	17

8.6.	Notice to Mariner	17

# INTRODUCTION

## The application of Sector Lights

A sector light is an aid to navigation that displays different colours and/or rhythms over designated arcs. Sector lights are used to mark safe channels and danger areas. Figure 1 shows an example of the use of sector lights.

Figure 1  Several applications for sector lights.

Light 1 is a coastal white light with a red sector indicating a danger.

Light 2 is a sector light obscured over the shore, with two white sectors indicating a safe channel.  When sailing towards the sector light it shows red to port and green to starboard following the IALA Maritime Buoyage System colour convention for Region A.  The boundary between the red and the green sector also indicates the position of a buoy.

Light 3 is a sector light with a red light and 4 white sectors indicating four anchorage positions.  It is obscured over the shore.

Light 4 is a sector light with a white sector indicating a safe channel.

## Types of Sector Lights

There are 5 (3?) basic types of sector lights:

- Traditional Sector Lights	(RADIAL)

- Laser Sector Light 	(RADIAL)

- Port Entry Sector Lights 	(PEL)

- Slot Sector Lights 	(MULTISOURCE)

- Light Emitting Diode Sector Lights 	(MULTISOURCE)

Traditional Sector Lights generally take the form of a coloured; adjustable glass is used as a filter for the light from the lamp. Indoor Sector Screening, Light houses and Beacons with lantern house, have the Sector Screening placed longer from the lamp and will hence obtain sharper boundaries in comparison with so called external Sector Screening.

Laser Sector Lights  (As far as I know it doesn’t exist any plain laser sector light system. What can probably closely be called a sector light is a system that has been tested as a laser range light by Russia and Canada. The system consist, briefly described, of two stationary laser beams which produces flashing coloured light on each side of the of the channel and is bright enough to be in use at day time.)

Port Entry Sector Lights (PEL) is a specialised form of projection light that can generate sharply defined sector boundaries. This feature is particularly useful for applications that require one or several narrow sectors.

Slot Sector Light consists of a sealed beam lamps used as a light source which can be oriented to different focal apertures. From each lamp the light beam are oriented to the correspondingly focal apertures in the lantern house. The luminous intensity in different sectors can alter depending on the number of lamps orientated to that sector. The lamps are connected in several separate groups, which are switch on in different order, this will give a stable power load and a character like a rotating light. With a Slot Sector Light it is possible to create very distinct boundaries and also sectors if the lamp stands are put in varying distance to the focal apertures.

LED-Sector Lights  (Here we need a few lines on LED Sector lights)

## Historical evolution of Sector Lights

### Some word about the sector lights in the history………..

The Slot Sector Light system was invented by the lighthouse engineer Bengt Holm, Swedish Maritime Adm. In the end of 1940. The Swedish lighthouse Grundkallen was the first lighthouse with this system (1958).

# BASIC PRINCIPLES

## Definitions

### Sector Light

A Sector Light is a single light that shows different colours when viewed from different directions. The colour of the light provides directional information to the mariner.

A light presenting different characters (usually different colours) over various parts of the horizon of interest to marine navigation.

A sector light is an aid to navigation that displays different colours and/or rhythms over designated arcs.

### Sector Screen /Cut Screen

An opaque screen so placed as to provide a sharp cut-off to a beam in sector light, and to reduce the angel of uncertainty.

### Angle of uncertainty

The horizontal angle of the region of indefinite character near the boundaries of a sector of a sector light.

## General considerations for Sector Lights

Where a single sector light defines a navigable channel:

- There is no reference of the vessel’s lateral position within the channel until a sector boundary is reached.  This may cause a problem in channels subject to a strong cross current.  For vessels with local knowledge, the zones defined by the angle of uncertainty can sometimes provide a useful guide to the vessel’s proximity to a sector boundary;

- Where practicable, there should be a margin of safety between the sector boundary and adjacent hazards.  If an appropriate safety margin cannot be achieved within the sector boundary, the hazards could be marked separately.

- Zones defined by the angle of uncertainty should be considered an additional margin of safety over the actual sector boundary;

- The design process for a sector light needs to consider the speed and manoeuvrability of vessels likely to be negotiating the sector, how quickly they can respond once they cross a sector boundary and the situations that may develop when other vessels are in the vicinity;

- A sector design should take account of the spectral distribution of the light source and the proportion of this light transmitted through the filter material.  The process should also check for potential for glare problems;

- The period of the light flash should be selected to provide ample time for a mariner to recognise the transitional phases that occur at the sector boundary;

- A white light is normally the first preference for a lighthouse of beacon.  If a single coloured sector is added, the preferred colour for the sector is red.

- If a white sector light is used to mark a navigation channel, coloured sectors may be used either side of the white to indicate the lateral limits.  In such cases it is common practice to use red and green sectors that follow the convention of the IALA Maritime Buoyage System;

- Multiple sectors can be used to provide a better indication of a vessel’s lateral position within the channel but at the expense of complexity for both the system designer and navigators.

## Methods for creating sectors

A common means of creating a sector is to fit a coloured filter in front of the main light but a sector can also be created by a subsidiary light on the same structure.

The subsidiary light can take any of the following forms:

- Range light;

- Beacon with a coloured lens, masked to achieve the sector angle;

- Beacon fitted with internal or external filter panels;

- Precision Direction Light.

The limits or boundaries of a sector are not always precisely cut off due to the characteristics of the light source and fading of colours or changing rhythms between adjacent sectors.

The transition zone is defined by an "angle of uncertainty"

## Sector boundaries

The most important feature of a sector light is the boundary between two colours.

When crossing the boundary, the mariner has good indication of a bearing to the sector light.

Within a sector of a certain colour the mariner only knows that the ship is somewhere within the sector.

The boundary resolution determines the sharpness of the colour chance from one boundary to the other.

The angle of uncertainty is the angle between two colours where the light does not fall within the specified colour coordinates of the IALA recommendation.

Due to the horizontal dimension of the light source the boundary between two coloured sectors will not be distinct; a horizontal boundary with indefinable light will be obtained, a so-called “Angle of uncertainty”.

Coloured filter

D



d

d = Horizontal dimension of light source.
D = Distance from filter to light source.

Angel of uncertainty  = d/D * 57 (degrees)

## Distribution between white and a coloured light seen in sector boundary.

Depending on the factor of transmission on the coloured glass, the glass needs to be increased in to the white sector = X.

Coloured filter

Filament

Controlled sektor boundary

Observer

Consequences that as follows could arise, if the share of a coloured light is cut down due to snow or the montage of aerial masts etc. the sector boundary can then unintentional move to the coloured sector. A wider white sector will be the result.



WHITE

X

Controlled sektor boundary

Widh  Horizontal dimension of light source

With multisource sector lights it is possible to measure the intensity/angle curve of different colours separately but to define the angle of uncertainty of other types of sector lights a colour meter has to bee used. Figure 2 illustrates a typical colour change on the boundary.

## Minimum width of a sector

To avoid mixed colour in a sector the sector should not be narrower then the horizontal dimension of light source.

(A very thin white sector can be arranged if a red and a green glass is glued together with a minimal seam, the boundary that is of mixed light, experiences then as white.)

## Variation in luminous intensity between adjacent sectors

Due to its low transmission factor coloured filter made of ordinary glass should be avoided, only acrylic glass with high transmission factor should be used.

## Total Subtense?

Total subtense is the total projected angle in the horizontal plane.

Vertical divergence

If we talk about PEL sector light the basic light beam is circular in section.

.

Total sub tense       	      Fig.3.1.

Vertical divergence is the total projected   angle in the vertical plane.

# TRADITIONAL SECTOR LIGHTS

## Principle of operation (radial)

In general a coloured, adjustable screening glass is used as a filter for the light from the lamp.

Traditional Sector Lights can be found in two different variations:

Indoor Sector Screening. Lighthouses and Beacons with a lantern house where the Sector Screening is placed inside the lantern house glass, alternative the lantern house glass is replaced with a Sector Screen. This system have normally the coloured filter placed longer from the lamp and will hence obtain sharper boundaries in comparison with so called external Sector Screening.

(Figure)

External Sector Screening. For use on Beacons that consist of only a lantern. The Sector Screening is mounted in a rack in front of the lantern.

(Figure)

## Level of performance

Achievable range and minimum angular uncertainty?

the observed angle of uncertainty is generally less than the geometric angle due to the relative intensities of sector colours (ie. colour mixing) as the observer passes through the transition zone;

if space on the aid to navigation structure is not a limiting factor, it is usually possible to achieve an angle of uncertainty of around 0.25 with this type of sector arrangement;

the angle of uncertainty can be reduced by decreasing the physical size of the light source or by increasing the radial distance to the coloured filter;

in situations where the main light has a large projected area, such as a rotating lens or reflector array, it is generally preferable to use a separate sector light rather than installing a coloured filter in front of the main light.

## Technical considerations

Light source should be of incandescent type so the wanted colour would be in the spectrum.

If large coloured sectors (>20o) is wanted, they would preferably be created by putting a Error: Reference source not foundSector Screen inside the lantern.

The area of the Sector Screen glass in the actual Sector Screen rack will in this way be minimised.

This will facilitate cleaning of the windows in Lantern House and in the case of only a screened Lantern, it will reduce the surface exposed to the wind.

Reliability?

# LASER SECTOR LIGHTS

## Principle of operation

(As far as I know it doesn’t exist any plain laser sector light system. What can probably closely be called a sector light is a system that has been tested as a laser range light by Russia and Canada. The system consist, briefly described, of two stationary laser beams which produces flashing coloured light on each side of the of the channel and is bright enough to be in use at day time.)

## Level of performance

(Roger Beauchesne has earlier in the ENG committee meeting presented laser light system tested in Canada. Laser light has also been reported from the IALA LITES workshop in Koblenz 2001 and in IALA XVth Conference by JCG, CCG and Russia.)

## Technical considerations

# PORT ENTRY SECTOR LIGHTS (PEL)

## Principle of operation

A PEL Sector Light is a projection light which shows a different colour when viewed from different angles at sea.

(Figure)

## Level of performance

PEL Sector Lights are so precise that a complete colour change at a sector boundary occurs over an angle of less than 1 minute (0.02°) in most models.

This corresponds to a lateral distance of just 1 metre at a viewing distance of 3.5 km.

In addition the intensity is maintained right to the edge of the beam, and does not reduce the further the observer is away from the axis.

The Sector Light is bright enough to use by day.

## Technical considerations

Light source?

# SLOT SECTOR LIGHTS

## Principle of operation

Coloured sectors are arranged with coloured glass direct mounted in channels in front of the lamp. The boundaries arranges with baffle plates that are some millimetres wider than the focal apertures in the lantern house, that gives a impairment of the light exactly in the boundary crossing. The baffle plates are mounted in the same way as the filter glass.

(Figure)

With a Slot Sector Light it is possible to create very distinct boundaries and also sectors if the lamp stands are put in varying distance to the focal apertures.

Systems with up to 2 x 96 lamps have been build, but smaller systems also exist with only a couple of boundaries and 3-6 lamps.

## Level of performance

## Technical considerations

Sealed beam lamps, PAR 56 200W 30V, mounted in simple stands are used as a light source. The light system is in general divided in two systems, main and standby system.

The lamps are normally mounted in two horizontal rows. From a lamp stand, lamps can be oriented to different focal apertures. From each lamp the light beam are oriented to the correspondingly focal apertures in the lantern house. Normally it is 8 focal apertures, symmetrical distributed, around the lantern house. The focal apertures are about 250 mm wide and 1400 mm high, the lantern house is about 5 m in diameter, and with this construction a stabile foundation for a helicopter platform is obtained.

The luminous intensity in different sectors can altered depending on the number of lamps orientated to that sector. It exist system with up to 2 x 96 lamps. The lamps are connected in several separate groups, which are switch on in different order, this will give a stable power load and a character like a rotating light. A rotating induction regulator controlled earlier system. It rotate in time with the character and switch in proper order the lamp groups, this was a excellent system to give a soft lightning of the lamps and they proved to last a very long time.

In later system thyristors are used, to give the lamp a soft lightning but, this system still consumes more lamps.

A burnt-out lamps will not lead to a total loss of light in boundaries or sectors, only a impairment (weakening) in that part of sector due to that the boundary are lighted by several overlapping lights.

# LED SECTOR LIGHTS

## Principle of operation

## Level of performance

## Technical considerations

# FURTHER CONSIDERATIONS

## Support structures

## Installation considerations - reflections

## Integration with other light system

## Maintenance

## Sector checking

## Notice to Mariner

At EEP1 Timo Kaartinen suggested the following basic categories of Sector Lights: Radial, PEL and Multisource. EEP3 needs to decide what type of categorization should be used.

Vega Industries brochure

IALA Dictionary 2-5-215

(Where does this 3rd definition come from?)

IALA Dictionary 2-4-125

IALA Dictionary 2-5-230

See also IALA Recommendation E 110, The Rhythmic Character of Lights on Aids to Navigation, Paragraph 2.4.