# IALA EEP3 WG4 – Actions from EEP2

ACTION:  C Baker was requested to research the background for the changes in the CIE standard S004/E-2001 and summarise the findings as an input to EEP3.

The CIE recommendations for signal colours were first adopted in 1951 based upon work carried out in the 1930s and 1940s and they were revised in 1975 as CIE Publication 2.2 – 1975 Colours of Light Signals. In 1987 the CIE saw a need to revise the publication based upon further experimental work and changing needs and technology in visual signalling. Published and unpublished reports of experiments on the recognition of signal colours were compiled and analysed. The data from those reports were brought on to a common base for dim signals, moderately bright signals and bright signals under conditions simulating both night and day observation. Some of the experiments included subjects with defective colour vision. Consideration of the data by the CIE and its expert advisors has lead to a number of recommendations for changes to the internationally accepted colour domains for signal colours, notably

(1) 	restriction of the yellow boundary of red, abandonment of the restricted purple boundary of red and re-definition of the recommended red in signal systems used by persons with defective colour vision,

(2) 	substantial re-definition of the colour domain for yellow including the definition of special domains for yellow signals of high and low illuminance,

(3) 	restriction of the boundaries of the colour domain for green,

(4) 	amendment of the colour domain for white including the definition of a preferred domain and a secondary domain that can be used when there is little chance of confusion with white lights,

(5) 	restricting the green and white boundaries of blue to enhance recognition as well as defining a more desaturated blue that can be used when there is little chance of confusion with white and a special domain to be used for low illuminance blue signals, and

(6) 	recommending the abandonment of violet as a signal colour.

The resulting report CIE Technical Report 107 ‘A review of the Official Recommendations of the CIE for the Colours of Signal Lights’ was publish in 1994. Based on this report CIE published the International Standard on "Colours of Signal Lights" CIE S 004/E-2001.

A comparison of the CIE standard chromaticity areas with those of the IALA 1977 ‘Recommendation for the colours of light signals on marine aids to navigation’ is attached

ACTION:  C. Baker was requested to make enquiries as to whether the CIE were studying the effect of colour in relation to the modelling of human perception of pulsed light.

Currently work is proceeding on modelling the effective intensity of pulsed light by studying the various models proposed including:

Allard

Blondel-Rey

Blondel-Rey Douglas

Schmidt-Clausen (Form factor)

Convolution

At the moment the effects of human perception of colour within the effective intensity model would introduce too many difficulties and is therefore not being considered.

Schmidt-Clausen included experiments with coloured lights in his original work of 1968. However, for the sake of simplicity, IALA decided not to include any reference to colour in its 1981 recommendations on effective intensity calculations.

Many scientists believe that a model of effective intensity would be valid only at the threshold of visual perception. At threshold, effective luminance is too low to reliably recognise colour.