REPORT on the 49th session of the IMO Sub-Committee on ## Safety of Navigation (NAV)

Contents

# 1.	Introduction

# 2.	Places of refuge

# 3	Performance standards for navigational equipment

# 3.1	Simplified Voyage Data Recorders for existing cargo ships

# 3.2	Performance standards for radar equipment

# 3.3	Performance standards for radar reflectors

4.	Shipborne Automatic Identification System

4.1	Display and integration of AIS target information

4.2	Maritime security issues – Long-range identification and tracking of ships

4.3	Review of AIS binary messages

# 4.4	IMO/IALA Seminar on AIS

# 5.	ITU matters, including Radiocommunication Study Group 8

5.1	Compatibility of radionavigation and radiolocation services operating in the bands 9 000 – 9 200 MHz and 9 300 – 9 500 MHz

# 6.	Ships’ routeing and related matters

## 6.1	Traffic Separation Schemes (TSS) and other routeing measures

6.2	Mandatory ship reporting system

7.	Other items

7.1 	World-wide radionavigation system

7.2	Safety measures for ships navigating in narrow waterways and/or areas of dense traffic

## 8.	Recommended actions for IALA

8.1		the Radionavigation Committee

8.2 		the VTS Committee

8.3		the AIS Committee

8.4		the Aids to Navigation Management Committee

# 8.5   	the Engineering Committee

# 9.	Date of the next session.

### A N N E X  1

Sub-Committee on Safety of Navigation (NAV) - 50th session - Agenda proposed to the Maritime Safety Committee

A N N E X  2

Draft Resolution on Performance Standards for Radar Reflectors

A N N E X  3

Draft SN/Circ. on Guidance on the application of AIS Binary Messages

*  *  *

1.	Introduction

The 49th session of the IMO Sub-Committee on Safety of Navigation was held at the headquarters of the organization in London during the period 30 June to 4 July, 2002. It was attended by representatives from 62 Member States and 29 Observer Organizations. The Secretary General and J-C. Leclair represented the Association.

The two most important items for the maritime industry discussed during the session were on the Places of Refuge and on the phased-in carriage requirement for simplified shipborne Voyage Data Recorders on existing cargo ships. On both subjects, draft resolutions were prepared and approved for adoption by the next Assembly, for the places of refuge, or by the 78th MSC, for VDR. The agenda for this session also contained some items of most interest to IALA members. In particular, the implementation of the Automatic Identification System (AIS) was still discussed during the meeting, under different aspects such as the representation of its information, its use for maritime security purposes (long-range identification and tracking) and the adoption of a list of seven binary messages for a trial period of four years. Decisions were also taken regarding radar reflectors, performance standards for radar equipment, world-wide radionavigation system, in particular Galileo, and  risk analysis for confined waterways.

The present report describes these outcomes as well as the measures adopted regarding routeing of ships and ship reporting systems.

2. 	Places of refuge

This issue had been considered by IMO for some considerable time. It had come to prominence in the incidents involving the tankers Castor, Erika and Prestige. At MSC 76, the Committee had approved, in principle, the two draft Assembly resolutions on Guidelines for ships in need of places of refuge and on Maritime Assistance Services (MAS), which had been prepared by NAV 48. At its 77th session, the Maritime Safety Committee requested the NAV Sub-Committee to complete the draft texts for direct submission to the 23rd Assembly (November 2003) for adoption. The Sub-Committee succeeded to do so and approved the two Guidelines and their associated Resolutions inviting the Assembly to adopt them.

The Guidelines on places of refuge for ships in need of assistance include developments on action required from masters in need of places of refuge, action expected of coastal States and evaluation of risks associated with the provisions of places of refuge. The Guidelines on Maritime Assistance Services include a definition of a MAS that is a center responsible for receiving reports in the event of incidents and serving as the point of contact between the shipmaster and the authorities of the coastal State in the event of an incident. They also stated that the establishment of a MAS should not entail the setting up of a new organization : insofar as the guidelines are observed, the functions of the MAS could be discharged by an existing organization, such as an MRCC, a harbour master’s office, a coast guard operations center (if one exists) or other body.

3.	Performance standards for navigational equipment

3.1	Simplified Voyage Data Recorders for existing cargo ships

The Sub-Committee finalized the feasibility study on mandatory carriage of Voyage Data Recorders on existing cargo ships and its consideration on draft performance standard and on a phased in carriage requirement for simplified shipborne Voyage Data Recorders (S-VDRs). It approved proposed draft amendments to regulation 20 to SOLAS Chapter V with a view for submission to MSC 78 and adoption at MSC 79. It also adopted, subject to comments by the COMSAR Sub-Committee at its 8th session (February 2004), the draft MSC Resolution on performance standards for shipborne S-VDRs.

3.2	Performance standards for radar equipment

At its 47th session, the Sub-Committee had considered the envisaged ITU requirements in the frequency bands 2.9 – 3.1 Ghz and 9.2 – 9.5 Ghz and the threat to current maritime safety radionavigation services. It expressed its view that there should be extreme caution both over impositions of unwanted emission limits on a safety service within an unrealistic timescale and with regard to the sharing of exclusive radiodetermination frequency bands, in which safety services operate, with other services.

NAV 47 had agreed that consideration should be given to the review of the requirements for radars in the light of their current performance requirements contained in the relevant IMO resolutions and that, as a minimum, the aspects of the performance standards for radar needed to be studied on the following basis:

- minimum range and range discrimination

- detection of SART’s and RACON’s

- target detection including performance under anomalous propagation and clutter conditions

- probability of detection and false alarm rate

- hazard and acceptable risk of interference to maritime radar

- the provision of hazard warning of fixed and floating objects

- maximum range.

Due to heavy workload, NAV 48 did not discuss the issue and referred it to NAV 49. At this session, the Sub-Committee decided to establish a correspondence group, co-ordinated by Norway, and agreed that issues to be addressed by this group should include:

- confirmation of the Radar Cross Section (RCS) of relevant targets

- consideration of the harmonization of design and certain important operational controls

- further studies of the relative virtues of sea and ground stabilization of the radar display and display modes

- the use made of RACONs operating at S-band

- assessment of costs associated with enhanced performance.

The Sub-Committee also noted that work is ongoing in IEC TC 80 WG 1 to develop revised testing standards for radars. The next meeting of WG 1 is in September 2003.

3.3	Performance standards for radar reflectors

The Sub-Committee approved the draft MSC resolution on performance standards for radar reflector for adoption by the Maritime Safety Committee at its 78th session. It should replace the resolution A.384(X). The draft text is annexed to the present report (annex 2).

4.	Shipborne Automatic Identification System

4.1	Display and integration of AIS target information

At its 48th session, the Sub-Committee expressed its concern over the uncoordinated display of some essential navigational information in current bridge equipment, including lack of appropriate symbology. It also received information on the work developed by the IEC Technical Committee 80 Working Group 13, which was preparing an International standard dealing with displays for the presentation of navigation related information for the use on ships.

A progress report was presented to the Sub-Committee at this session by the IEC delegation providing an outline for proposed performance standards for the presentation of navigation related information. The IEC delegation also provided a report on the development of the harmonization of terms and symbols used to present navigation-related information.  The Sub-Committee agreed to wait the results of the work from IEC on these matters in view of adopting an SN circular on display of navigation related information at its 50th session.

4.2	Maritime security issues – Long-range identification and tracking

In December 2002, MSC 76 agreed, in principle, to consider Inmarsat-C polling as a system for long-range ships identification and tracking, subject to further study by the NAV and COMSAR Sub-Committees. At its 7th session, COMSAR started to draft a recommendation on functional requirements on long-range ship identification and tracking and MSC 77 decided to create a correspondence group on the matter and requested NAV 49 to give advice.

On this issue, IALA had submitted a document to the Sub-Committee recommending that long-range ship’s identification, position reporting and monitoring should be flexible in terms of the technology used and be standardized in terms of the data elements communicated and the data format. In particular, it was suggested that the information contained in the AIS equipment should also be used for long-range identification to avoid that the officer of the watch needs to input the required information several times in different systems.

IALA also pointed out that AIS contained more detail information that those provided by the present Inmarsat-C polling system. The Sub-Committee agreed that the ability to obtain more detail about a ship might be useful in case of doubt on the ship’s identity and that such a long-range identification and tracking system that included an ability of interrogating for further information would therefore be advantageous. It requested the correspondence group to take this observation into consideration.

4.3	Review of AIS binary messages

The MSC Circular 1062 confirmed the willingness to transfer the responsibility for the development, maintenance and administration of the AIS binary messages from IALA to IMO and requested the NAV Sub-Committee to make an inventory of the messages in use.

IALA, Germany, Sweden and Finland submitted documents to this session of the Sub-Committee which decided to issue an SN circular on guidance on the application of AIS binary messages, annexed to the present report (annex 3), describing the introducing process step by step of the binary messages, explaining, in simple terms, the purpose, the scope and the use of binary messages (appendix 1), and describing the detailed format of the seven messages selected at this stage (appendix 2).

Taking into account the lack of experience of using this AIS binary messages worldwide and on a large scale, the Sub-Committee finally decided to select only seven messages to be used for a trial period of 4 years with no change except the introduction, during this period, of two new additional messages to test the process of introducing new binary messages to users, manufacturers and IMO.

The Sub-Committee, having noted that some of the recommended AIS binary messages may directly impact shore side interest (e.g. VTS and aids to navigation), agreed that this information be made available to IALA and IEC.

4.4	IMO/IALA Seminar on AIS

The Sub-Committee was informed by the IMO Secretariat of the success of the IMO/IALA Seminar on AIS, held at IMO Headquarters, London, on the 15th and 16th July 2002, following NAV 48. The objective of the seminar was to provide potential users of the AIS system with a wider understanding of its capabilities including its limitation. The Seminar was also intended to provide information on the system to encourage VTS Centres, Aids to Navigation Authorities and other appropriate shore authorities to fit AIS equipment.

The IALA delegation took this opportunity to again express its appreciation to IMO for the excellent co-operation in organizing this joint and successful AIS Seminar.

5.	ITU matters, including Radiocommunication Study Group 8

5.1	Compatibility of radionavigation and radiolocation services operating in the bands 9 000 – 9 200 MHz and 9 300 – 9 500 MHz

The Sub-Committee was informed that the ITU Radiocommunication Assembly in May 2003 adopted a new question concerning compatibility of radionavigation and radiolocation services operating in the bands 9 000 – 9 200 MHz and 9 300 – 9 500 MHz and assigned it to Study Group 8 for finalization by 2006. Taking into account the importance of the matter for the safety of life at sea since the band 9 300 – 9 500 MHz is used by the X-band marine radar and radar transponders, Member Governments and other interested organizations were invited to actively participate in the study to be carried out in ITU and also submit their comments and proposals to NAV 50 for consideration.

6.	Ships’ routeing and related matters

6.1	Traffic Separation Schemes (TSS) and other routeing measures

The Sub-Committee approved and invited the Maritime Safety Committee to adopt:

- the new TSS off Ra’s al Kuh (Iran)

- the new TSS for the approaches to the Port of Ra’s al Khafji (Saudi Arabia)

- the new recommended TSS in the Adriatic Sea

- the amendment to the existing Cape Finisterre TSS (Spain) in the Gulf of Finland

- the amendment to the existing TSS “Between Korsoer and Sprogoe” (Denmark)

- the amendment to the existing TSS in the Singapore Strait

- the mandatory area to be avoided off the north east coast of the North Island of New Zealand

- the amendment of the existing charting measure in the Great North East Channel of the Torres Strait, off the north east coast of Australia

- the recommended routes, a precautionary area and an area to be avoided in the                      Adriatic Sea

- the area to be avoided in the Paracas National Reserve

All these routeing measures will be implemented at a date six months after the adoption by the Maritime Safety Committee or, for the Cape Finisterre TSS, by the Assembly.

6.2	Mandatory ship reporting system

The Sub-Committee approved and invited the Maritime Safety Committee to adopt:

- the amendment of the existing mandatory ship reporting system in the Torres Strait and Inner Route of the Great Barrier Reef off the North East coast of Australia (REEFREP)

- the amendment of the existing mandatory ship reporting system Off Cape Finisterre.

7.	Other items

7.1	World-wide radionavigation system

The Sub-Committee considered a submission by the European Commission providing a preliminary assessment of the Galileo navigation service requirements and confirming that analyses performed so far in the Galileo programme indicated that it met all the requirements for oceanic, coastal, port approach and restricted waters operations as given in the revised resolution A.915(22).

The Sub-Committee further noted that, in addition, the Galileo Mission Requirement document was currently being revised and it had been recently decided to add performance requirements directly applicable to the system design. This would further ensure that Galileo delivers a service fully in line with the expectations of the international maritime community.

7.2	Safety measures for ships navigating in narrow waterways and/or areas of dense traffic

Following the “Erika” accident, the Maritime Safety Committee and the Marine Environement Protection Committee had prepared a list of proposals to improve oil tanker safety. Among these proposals, measure 17 requested the NAV Sub-Committee to :

17 – Consider what additional safety measures may be necessary for ships navigating in narrow waterways and/or areas of dense traffic.”

After being informed by the IALA delegation at its 47th session on the current work undertaken by the Association on operational procedures, risk analysis, pilotage, VTS and AIS issues for confined waterways, the Sub-Committee expressed the opinion that the outcome of IALA’s work could form the basis for some future work, and invited it to keep the Sub-Committee informed of the work.

In that context, the IALA delegation provided the Sub-Committee with information on the more recent developments of a Risk Management Tool that will assist in measuring the risk level of waterways by the use of co-ordinated and compatible models. The delegation added that it is expected that the result of the study will enable an IALA Recommendation to be adopted by the IALA Council in 2004 and a proposal could be addressed to NAV 51.

8.	Recommended actions for IALA

It is recommended that:

8.1	the Radionavigation Committee

- note the development on performance standards for radar equipment (item 3.2)

- note the development on long-range ships identification and tracking system (item 4.2)

- note the ITU works on the X-band for marine radars and radar transponders (item 5.1)

- note the information provided on Galileo (item 7.1)

8.2 	the VTS Committee

- note the draft resolutions on places of refuge (item 2)

- note the development on performance standards for radar equipment (item 3.2)

- note the works on display and integration of AIS target information (item 4.1)

- note the draft SN circular on review of AIS binary messages (item 4.3)

8.3	the AIS Committee

- note the works on display and integration of AIS target information (item 4.1)

- note the development on long-range ships identification and tracking system (item 4.2)

- note the draft SN circular on review of AIS binary messages. (Regarding procedural matter, if the Committee, with the Council agreement, wishes to propose one or both of the additional messages that may be introduced during the next four years trial period, the proposal should be co-sponsored  by an IMO Member State and be sent to the Maritime Safety Committee) (item 4.3)

8.4	the Aids to Navigation Management Committee

- note the development on performance standards for radar equipment (item 3.2)

- note the works on display and integration of AIS target information (item 4.1)

- note the information provided by IALA on ships navigating in confined waterways (item 7.2)

8.5     the Engineering Committee

- note the draft  MSC resolution on radar reflector (item 3.3)

9.	Date of the next session.

The 50th session of the NAV Sub-Committee is tentatively scheduled to be held from 5 to 9 July 2004.

The IMO Secretariat should receive documents intended for consideration by the Sub-Committee approximately as follows:

(a)	those containing more than 6 pages of  text (bulky documents) and those which 		require	action or decision by the 31st of March 2004;

(b)	other documents by the 30th of April 2004.

The proposed agenda for the session is attached as annex 1.

* * *

J.Ch. Leclair

IALA Representative to IMO

12 July 2003.

### A N N E X   1

Sub-Committee on Safety of Navigation (NAV) - 50th session - Agenda proposed to the Maritime Safety Committee

Opening of the session

1	Adoption of the agenda

2	Decisions of other IMO bodies

3 	Routeing of ships, ship reporting and related matters

4	Requirements for the display and use of AIS information on shipborne navigational displays

5	Review of the 2000 HSC Code and amendments to the DSC Code and the 1994 HSC         Code

6	Anchoring, mooring and towing equipment

7	Revision of the fishing vessel safety code and voluntary guidelines

8	Recommendations on high-risk oceanic crossing by adventure craft

9	Review of performance standards for radar equipment

10	ITU matters, including Radiocommunication ITU-R Study Group 8 matters

11	Large passenger ship safety: effective voyage planning for large passenger ships

12	Measures to enhance maritime security

13	World-wide radionavigation system (WWRNS)

14	Casualty analysis

15	Work program and agenda for NAV 51

16	Election of Chairman and Vice-Chairman for 2005

17	Any other business

18	Report to the Maritime Safety Committee

__________

ANNEX 2

DRAFT RECOMMENDATION ON PERFORMANCE STANDARDS

FOR RADAR REFLECTORS

1 	INTRODUCTION

1.1  Radar reflectors carried under SOLAS chapters V and X should comply with the minimum performance requirements as specified in this Recommendation.

1.2   In  the  following  paragraphs,  radar  cross  sections  are  specified  for  the frequencies  of  3  GHz  (S-band)  and  9  GHz  (X-band)  whose  wavelengths  are  10 cm and  3 cm respectively.

1.3   The performance requirements stated  should  apply to either passive or active radar reflectors in both 9 Ghz and 3 Ghz bands. (Active radar reflectors are also known as radar target enhancers).

2 	APPLICATION

2.1  All  ships  required to be fitted with  a  radar  reflector, if practicable,  to  enable detection by ships navigating by radar at both 9 GHz and 3 GHz.

3 	PERFORMANCE

3.1  The radar reflector should have a “Stated Performance Level” measured in square meters radar cross section (m² RCS) of at least 7.5 m² in X-band and 0.5 m² in S-band mounted at a minimum height of 4 m above water level.

3.2  Required minimum level for reflector performance – the Stated Performance Level should:

.1 be maintained over a total of at least 280° azimuth;

.2 not remain below this level over any single angle of more than 10° - a null; and

.3 not have distances between nulls of less than 20°.

3.3 For power driven vessels and sailing vessels designated to operate with little heel (catamaran/trimaran), this performance should be maintained through angles of (athwartships) heel 10° either side of vertical. For other sailing vessels, the reflector should maintain this performance over 20° either side of vertical.

3.4  Active  reflectors  should  conform  to  Recommendation  ITU-R  M.1176.

4 	CONSTRUCTION

4.1 The reflector  should  be  capable  of  maintaining  its  reflection  performance  under  the conditions  of  sea  states,  vibration,  humidity  and  change  of  temperature  likely  to  be experienced in the marine environment as defined by resolution A.694(17).

5	INSTALLATION

5.1  Fixing arrangements should be provided so that the reflector can be fitted either on a rigid mount or suspended in the rigging.

5.2 The recommended mounting height of 4 m and any preferred orientation should be permanently and clearly marked on the reflector.

5.3  The  reflector should  be clearly and permanently marked if it will meet the performance requirement to + 20° inclination (heel).

5.4 For small-craft, the maximum mass for mounting at 4m is 5kg.  Reflectors designed for mounting at a greater height should be of weight calculated as equivalent to, or less than 4m/5kg. Physical sizes should be minimized and should not exceed 0.05 m3.

_____________

ANNEX  3

DRAFT SN CIRCULAR ON GUIDANCE ON THE APPLICATION OF AIS BINARY MESSAGES

1	Automatic Identification System (AIS) is a working system for ship identification and tracking that has the capability of the service of binary messages.  The concept, functional requirements and technical constraints are described in appendix 1.

2	The Sub-Committee on Safety of Navigation, at its forty-ninth session (30 June to 4 July 2003) selected seven (7) binary messages as shown in appendix 2 to this annex to be used as a trial set of messages.  The idea is to use this set of 7 messages for a trial period of 4 years with no change.  It should be noted that 4 additional system-related messages identified in Recommendation ITU-R M.1371 are needed for the operation of the system.

3	The criteria for selecting the 7 trial messages were:

.1	demonstrated operational need;

.2	a cross-section of users, including ships, VTS, pilots, and port authorities;  and

.3	messages already developed for format and content.

4	In addition, messages were limited to a maximum number of 3 slots to reduce the potential for overloading the AIS frequencies designated for IMO.

5	In addition to these 7 messages and 4 system-related messages, the Sub-Committee agreed to allow 2 additional messages in the 4-year trial period to test the process of introducing new binary messages to users, manufacturers and the Organization.

6	By the end of the trial period, all SOLAS ships and a large number of non-SOLAS vessels, are expected to be equipped with AIS, allowing IMO to evaluate the benefit and practicability of AIS binary messages, as well as the loading of AIS frequencies.

7	If the evaluation is positive, the use of binary messages could be extended.  Should a Member Government see the need to propose a new binary message, it should address to the Sub-Committee a demonstrated operational need and provide the proposed format and content of the message.  Then, the Organization may accept, assign an identifier and publish it in an updated SN circular for the benefit of the maritime industry.

8	Member Governments are invited to bring the annexed information to the attention of all concerned.

APPENDIX 1

DRAFT GUIDANCE ON AIS BINARY MESSAGES

1	This document provides an overview of the purpose and scope of AIS Binary Messages and their applications.

System Requirements

2	Binary Messages may be transmitted and received by shipborne mobile AIS devices and AIS base stations that are equipped to process these messages.  Shore-based stations may receive ships’ Binary Messages and distribute them to other ships and/or users.

3	The display capability of AIS binary messages is not part of the mandatory functions of the MKD (Minimum Keyboard and Display). The display of the information content of binary messages may require hardware additional to the AIS and dedicated software.

Purpose and scope of AIS Binary Messages

4	Automatic Identification System (AIS) was originally and is primarily a means for positive identification and tracking of vessels, e.g. by transmitting and receiving static, dynamic and voyage-related data of ships, as well as short safety related messages. In addition, AIS will be beneficial to the safety of navigation and protection of the environment by monitoring the maritime traffic and by providing various basic services.  In particular, AIS may use Binary Messages as a means for certain types of limited communication. These messages will be dedicated to specific applications, which must be approved by IMO.

5	Binary Messages may be either Addressed Binary Messages or Broadcast Binary Messages. Recommendation ITU-R M.1371 specifies the technical characteristic and the structure of the binary messages. The content is tailored to different applications.  IMO defines this content. To avoid system overload, the number of binary messages should be limited. Therefore, Binary messages should be approved only if there is a high operational need for them.  These messages have to be distinguished from Addressed Safety Related Messages and Broadcast Safety Related Messages both of which allow the exchange of format-free ASCII-text.

6	Binary Messages may provide a variety of capabilities for pre-defined information packages.   For example, they may permit:

- ships to report information to other ships and shore stations,

- shore stations to report navigation information, conditions and warnings,

- ship reporting to be simplified.

7	Moreover, binary messages may reduce verbal communications and enhance reliable information exchange and reduce operator’s workload.  Binary Messages are not intended to replace standard services such as GMDSS and SAR.

Use of AIS Binary Messages

8	The use of Binary Messages is optional.  Binary Messages may be generated manually or automatically.  Pre-defined forms for each binary message type may be used to easily generate the message.

9	Since the use of binary messages places an additional load on the VHF data link, care must be taken not to impair the main functions of AIS for ship identification and tracking.  In this regard, longer binary messages may adversely impact the VHF data link and should be avoided.

APPENDIX 2

APPLICATION 1

Message “METEOROLOGICAL AND HYDROLOGICAL DATA”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8; always 8 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | MMSI number of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | DAC = 001; FI = 11 |
| Latitude | 24 | Measuring position, 0 to + /- 90 degrees, 1/1000th minute |
| Longitude | 25 | Measuring position, 0 to + /- 180 degrees, 1/1000th minute |
| Date and time | 16 | Time of transmission, Day, hour, minute, (ddhhmm in UTC) |
| Average wind speed | 7 | Average of wind speed values for the last 10 minutes. 0-120 kts, 1 kt |
| Wind gust | 7 | Wind gust is the maximum wind speed value reading during the last 10 minutes, 0 - 120 kts, 1 kt |
| Wind direction | 9 | 0 - 359 degrees ,1 degree |
| Wind gust direction | 9 | 0 – 359 degrees, 1 degree |
| Air temperature | 11 | Dry bulb temperature - 60.0 to + 60.0 degrees Celsius 0.1 of a degree |
| Relative humidity | 7 | 0 – 100%, 1% |
| Dew point | 10 | - 20.0 - + 50.0 degrees, 0.1 degree |
| Air pressure | 9 | 800 – 1200 hPa, 1 hPa |
| Air pressure tendency | 2 | 0 = steady, 1 = decreasing, 2 = increasing |
| Horizontal visibility | 8 | 0.0 – 25.0 NM, 0.1 NM |
| Water level (incl. tide) | 9 | Deviation from local chart datum, . –10.0 to + 30.0 m 0.1 m |
| Water level trend | 2 | 0 = steady, 1 = decreasing, 2 = increasing |
| Surface current speed (incl. tide) | 8 | 0.0 – 25.0 kts 0.1 kt |
| Surface current direction | 9 | 0 – 359 degrees, 1 degree |
| Current speed, #2 | 8 | Current measured at a chosen level below the sea surface, 0.0 – 25.0 kts, 0.1 kt |
| Current direction, #2 | 9 | 0 – 359 degrees, 1 degree |
| Current measuring level, #2 | 5 | Measuring level in m below sea surface , . 0 –30 m 1 m |
| Current speed, #3 | 8 | 0.0 – 25.0 knots, 0.1 knot |
| Current direction, #3 | 9 | 0 – 359 degrees, 1 degree |
| Current measuring level, #3 | 5 | Measuring level in m below sea surface, 0 – 30 m 1 m |
| Significant wave height | 8 | 0.0 – 25.0 m, 0.1 m |
| Wave period | 6 | Period in seconds, 0 – 60 s, 1 s |
| Wave direction | 9 | 0 – 359 degrees, 1 degree |
| Swell height | 8 | 0.0 – 25.0 m, 0.1 m |
| Swell period | 6 | Period in seconds, 0 – 60 s, 1 s |
| Swell direction | 9 | 0 – 359 degrees, 1 degree |
| Sea state | 4 | According to Beaufort scale (manual input?), 0 to 12, 1 |
| Water temperature | 10 | -10.0 - + 50.0 degrees, 0.1 degree |
| Precipitation (type) | 3 | According to WMO |
| Salinity | 9 | 0.0 – 50.0 ‰, 0.1‰ |
| Ice | 2 | Yes/No |
| ###### Spare | 6 |  |
| Total Number of bits | 352 | ###### Occupies 2 slots |

Purpose

This message allows the distribution of meteorological and hydrological information. Should there be no positional information or time of measurement, this message should not be transmitted. If there is no data available, default value to be transmitted is the highest available binary value for that particular data field.  It is to be displayed as ‘not available’ (not 9999 or zero or similar).  This message takes 2 slots.  Not all the information specified in the tables will be available at all stations.  The interval between the broadcasting of this message should not exceed 12 minutes. Attribute of message: broadcast, shore station transmitting, no acknowledgement required.

APPLICATION 2

Message “DANGEROUS CARGO INDICATION”

| Parameter | No. bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6, always 6 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. Refer to § 4.6.1; 0 - 3; default = 0; 3 = do not repeat any more |
| Source ID | 30 | MMSI number of source station |
| Sequence Number | 2 | 0 - 3; refer to § 5.3.1 |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit Flag | 1 | Retransmit Flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted. |
| Spare | 1 | Not used. Should be zero |
| IAI | 16 | DAC = 001; FI =12 |
| Last Port of call | 30 | UN Locode 5 characters 6 bit ASCII "@@@@@" = not available = default |
| ATD from Last Port of Call | 20 | Actual Time of Departure; MMDDHHMM UTC |
|  |  | Bits 19 – 16: month; 1 - 12; 0 = not available = default; |
|  |  | Bits 15 – 11: day; 1 - 31; 0 = not available = default; |
|  |  | Bits 10 - 6: hour; 0 - 23; 24 = not available = default; |
|  |  | Bits 5 - 0: minute; 0 - 59; 60 = not available = default |
| Next Port of call | 30 | UN Locode 5 characters 6 bit ASCII "@@@@@" = not available = default |
| ETA at Next Port of Call | 20 | Estimated Time of Arrival; MMDDHHMM UTC |
|  |  | Bits 19 – 16: month; 1 - 12; 0 = not available = default; |
|  |  | Bits 15 – 11: day; 1 - 31; 0 = not available = default; |
|  |  | Bits 10 - 6: hour; 0 - 23; 24 = not available = default; |
|  |  | Bits 5 - 0: minute; 0 - 59; 60 = not available = default |
| Main Dangerous Good | 120 | Maximum 20 characters 6 bit ASCII "@@@@@@@@@@@@@@@@@@@@" = not available = default |
| IMD category of Main Dangerous Good | 24 | Maximum 4 characters 6 bit ASCII "@@@@@" = not available = default |
| UN Number of Main Dangerous Good | 13 | 1 - 3363 UN Number 3364- 8191 should not be used 0 = not available = default |
| Value of Quantity of Main Dangerous Good | 10 | 0 = not available = default; 1 - 1023 = value of quantity |
| Unit of Quantity of Main Dangerous Good | 2 | 0 = not available = default 1 = in kg 2 = in tons (10E3 kg) 3 = in 1.000 tons (10 E 6 kg) |
| Spare | 3 | Not used. Should be set to zero |
| Total Number of bits | 360 | Occupies 2 slots |

Purpose:

This message should be used as a respond on a request for Dangerous Cargo Indication from a competent authority. The message content is essential to identify that harbour where the necessary documents for the dangerous goods cargo can be found, e. g. last and next port of call. The indication of main dangerous goods and its quantity gives at least an estimation of a potential danger.   Intended Application: The data are for use of a competent authority only. Attributes of message: addressed, ship transmitting, no acknowledgement.

APPLICATION 3

Message “FAIRWAY CLOSED”

| Parameter | No. of Bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8; always 8 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | Name of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | DAC = 001; FI = 13 |
| Reason for closing | 120 | Maximum 20 characters 6-bit ASCII; „@@@@@@@@@@@@@@@@@@@@“ = not available = default |
| Location of closing from | 120 | Maximum 20 characters 6-bit ASCII; „@@@@@@@@@@@@@@@@@@@@“ = not available = default |
| Location of closing To | 120 | Maximum 20 characters 6-bit ASCII; „@@@@@@@@@@@@@@@@@@@@“ = not available = default |
| Extension of closed area (radius) | 10 | extension; (valid range 0-1000, 1001 = not available = default) |
| Unit of extension value | 2 | 0=[m], 1=[km], 2=[nm], 3=[cbl] |
| Closing from day | 5 | 1-31; 0 = day not available = default |
| Closing from month | 4 | 1-12; 0 = month unavailable = default;13-15 unused |
| From LT hour (appr) | 5 | 0-23; 24 = LT hour not available = default; 25-31 not used |
| From LT minute (appr) | 6 | 0-59; 60 = LT minute not available = default; 61-63 not used |
| To day | 5 | 1-31; 0 = day not available = default |
| To month | 4 | 1-12; 0 = month unavailable = default;13-15 unused |
| To LT hour (appr) | 5 | 0-23; 24 = LT hour not available = default; 25-31 not used |
| To LT minute (appr) | 6 | 0-59; 60 = LT minute not available = default; 61-63 not used |
| ###### Spare | 4 |  |
| Total number of bits | 472 | occupies 3 slots |

Purpose

This message should be used to inform ships, in particular to give guidance to large vessels about temporary closed fairways or sections in ports. Attributes: broadcast, shore station transmitting, no acknowledgement.

APPLICATION 4

Message “TIDAL WINDOW”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 6; always 6 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0-3; 0 = default; 3 = do not repeat anymore |
| Source ID | 30 | MMSI number of source station |
| Sequence Number | 2 | 0-3. |
| Destination ID | 30 | MMSI number of destination station |
| Retransmit Flag | 1 | Retransmit Flag should be set upon retransmission: 0 = no retransmission = default; 1 = retransmitted. |
| Spare | 1 | Not used. Should be zero. |
| IAI | 16 | DAC = 001; FI = 14 |
| UTC month | 4 | 1-12; 0 = UTC month not available = default; 13-15 not used |
| UTC day | 5 | 1-31; 0 = UTC day not available = default |
| Position #1 Lat | 27 | 1/10 000 min (90 degrees, North = positive, South = negative; 91 degrees = not available = default). |
| Position #1 Lon | 28 | 1/10 000 min (180 degrees, East = positive, West = negative; 181 degrees = not available = default). |
| From UTC hour | 5 | 0-23; 24 = UTC hour not available = default; 25-31not used |
| From UTC minute | 6 | 0-59; 60 = UTC minute not available = default; 61-63 unused |
| To UTC hour | 5 | 0-23; 24 = UTC hour not available = default; 25-31not used |
| To UTC minute | 6 | 0-59; 60 = UTC minute not available = default; 61-63 unused |
| Current direction predicted #1 | 9 | Current direction in degrees. ( valid range 0-359, 360 = not available = default). |
| Current speed predicted #1 | 7 | Current speed in 0,1 knots. (valid range 0-126; 127 = not available = default). |
| Position #2 Lat | 27 | 1/10 000 min (90 degrees, North = positive, South = negative; 91 degrees = not available = default). |
| Position #2 Lon | 28 | 1/10 000 min (180 degrees, East = positive, West = negative; 181 degrees = not available = default). |
| From UTC hour | 5 | 0-23; 24 = UTC hour not available = default; 25-31not used |
| From UTC minute | 6 | 0-59; 60 = UTC minute not available = default; 61-63 unused |
| To UTC hour | 5 | 0-23; 24 = UTC hour not available = default; 25-31not used |
| To UTC minute | 6 | 0-59; 60 = UTC minute not available = default; 61-63 unused |
| Current direction predicted #2 | 9 | Current direction in degrees. ( valid range 0-359, 360 = not available = default). |
| Current speed predicted #2 | 7 | Current speed in 0,1 knots. (valid range 0-126; 127 = not available = default). |
| Position #3 Lat | 27 | 1/10 000 min (90 degrees, North = positive, South = negative; 91 degrees = not available = default). |
| Position #3 Lon | 28 | 1/10 000 min (180 degrees, East = positive, West = negative; 181 degrees = not available = default). |
| From UTC hour | 5 | 0-23; 24 = UTC hour not available = default; 25-31not used |
| From UTC minute | 6 | 0-59; 60 = UTC minute not available = default; 61-63 unused |
| To UTC hour | 5 | 0-23; 24 = UTC hour not available = default; 25-31not used |
| To UTC minute | 6 | 0-59; 60 = UTC minute not available = default; 61-63 unused |
| Current direction predicted #3 | 9 | Current direction in degrees. ( valid range 0-359, 360 = not available = default). |
| Current speed predicted #3 | 7 | Current speed in 0,1 knots. (valid range 0-126; 127 = not available = default). |
| Total number of bits | 376 | occupies 3 slots |

Purpose

This message should be used to inform vessels about tidal windows which allow a vessel the safe passage of a fairway. The message includes predictions of current speed and current direction. In this example, three points of tidal information are given. Attributes of message: addressed, shore station transmitting, acknowledgement required.

APPLICATION 5

Message “EXTENDED SHIP STATIC AND VOYAGE RELATED DATA”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8; always 8 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | Name of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | DAC =001; FI =15 |
| Air Draught | 11 | in 1/10 m, 2047 = height over keel 204,.7 m or greater, 0 = not available = default |
| Spare | 5 | Not used. Should be set to zero. |
| Total Number of Bits | 72 | This message uses one slot |

Purpose

This message should be used by a ship to report the height over keel.

Attributes:  broadcast, ship transmitting, no acknowledgement.

APPLICATION 6

Message “NUMBER OF PERSONS ON BOARD”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 86; always 86 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | Name of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | DAC = 001; FI =16 |
| Number of Persons | 13 | Current number of persons on-board, including crew members: 0 – 8191; default = 0 = not available; 8191 = 8191 or more |
| Spare | 3 | Not used. Should be set to zero. |
| Total Number of Bits | 72 | This message uses one slot |

Purpose

This message should be used by a ship to report the number of persons on board, e.g. on request by a competent authority.  Attributes:  addressed, acknowledgement required.

APPLICATION 7

Message “PSEUDO-AIS TARGETS”

| Parameter | No. of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 8; always 8 |
| Repeat Indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| Source ID | 30 | Name of source station |
| Spare | 2 | Not used. Should be set to zero. |
| IAI | 16 | DAC = 001; FI =17 |
| VTS Target 1 | 120 | Refer to table below; occupies 2 slots |
| VTS Target 2 | 120 | Optional; refer to table below; occupies 2 slots |
| VTS Target 3 | 120 | Optional; refer to table below; occupies 3 slots |
| VTS Target 4 | 120 | Optional; refer to table below; occupies 3 slots |
| Total Number of bits | Max 536 | 3 slots |

Each VTS Target should be structured as follows:

| Parameter | No. Of Bits | Description |
| --- | --- | --- |
| Type of Target Identifier | 2 | Identifier Type: 0 = The target identifier should be the MMSI number. 1 = The target identifier should be the IMO number. 2 = The target identifier should be the call sign. 3 = Other (default). |
| Target ID | 42 | Target Identifier. The Target ID should depend on Type of Target Identifier above. When call sign is used, it should be inserted using 6-bit ASCII. If Target Identifier is unknown, this field should be set to zero. When MMSI or IMO number is used, the least significant bit should equal bit zero of the Target ID. |
| Spare | 4 | Spare. Should be set to zero. |
| Latitude | 24 | Latitude in 1/1000 of a minute. |
| Longitude | 25 | Longitude in 1/1000 of a minute. |
| COG | 9 | Course over ground in degrees (0-359); 360 = not available = default. |
| Time Stamp | 6 | UTC second when the report was generated (0-59, or 60 if time stamp is not available, which should also be the default value) |
| SOG | 8 | Speed over ground in knots; 0-254; 255 = not available = default. |
| Total Number of bits | 120 |  |

A VTS target should only be used, when the position of the target is known. However, the target identity and/or course and/or time stamp and/or speed over ground may be unknown.

Purpose

This message should be used to transmit VTS targets. This message should be variable in length, based on the amount of VTS targets. The maximum of VTS Targets transmitted in one International FM 16 should be seven (7). Because of the resulting effects of VDL channel loading, the transmission of International FM 16 should be no more than necessary to provide the necessary level of safety.

Attributes:  broadcast, VTS transmitting, no acknowledgement.

***