## Aids to Navigation Lights

### Light Sources

#### Brief History

Until the first application of electricity to lighting late in the nineteenth century, all artificial light was produced by fire.  Illuminants progressed from pyres of wood (used up until the 1700’s), to oil wick lamps, vaporised oil and gas burners, electric arc and tungsten filament lamps.  Optical devices matched these developments, first with reflector systems and later with lenses.

It is interesting to note that the efforts to understand human perception of light and to improve the efficiency and effectiveness of aids to navigation illuminants and optical apparatus were for many years at the forefront of scientific endeavours.

The glass lens design pioneered by Fresnel around 1760 remains a principal element of the modern aid to navigation light.  Present day lenses are often made of plastic rather than glass.

A few countries still have gas lights that burn acetylene or propane; however, the majority of aids to navigation lights use electric lamps of various types.  Increasingly, these lamps draw their power from renewable energy sources such as solar, wind and wave power.

Electric lamps have been specifically designed for aids to navigation applications.  However, lamps selected from the enormous range of commercial products have also been used or adapted for aids to navigation.

Light emitting diode (LED) technology is emerging as an alternative to filament lamps.

#### Gas Lights

Acetylene

The acetylene light has a special place in the history of aids to navigation, primarily for being the first reliable means of automating lighthouses, buoys and beacons during the earlier part of the 20th century.  The predominant acetylene lighting systems carry the AGA brand, and these originate from the inventions of Gustaf Dalen.

Acetylene lighting technology was further enhanced by the development of the Dalen “mixer” that allowed gas and air to be drawn into a chamber and then consumed in an incandescent mantle to produce a brighter light source than the open flame type.  The incandescent mantle can be operated as a flashing source inside a fixed lens or as a continuous source inside a rotating lens.  Related developments included a gas-operated mechanism for rotating a lens and a clockwork powered automatic mantle changing device.  Refer to the IALA Technical Note on the Safe Handling of Gases.

Propane and Butane

Propane and butane gases have been used as fuels for gas lighting lighting systems.  The lighting equipment has to use an incandescent mantle burner.

#### Electric Lights

##### 3.4.1.3.1  Incandescent filament

Tungsten filament

Description of operation:  [include starter]

Typical use:  [type of ATON]

Technical data:

- Power:

- Efficiency:

- Operating life:

Advantages:

Disadvantages:

Safety:

### Tungsten halogen

Description of operation:  [include starter]

Typical use:  [type of ATON]

Technical data:

- Power:

- Efficiency:

- Operating life:

Advantages:

Disadvantages:

Safety:

##### 3.4.1.3.2 Discharge lamps

Fluorescent

Description of operation:  [include starter]

Typical use:  [type of ATON]

Technical data:

- Power:

- Efficiency:

- Operating life:

Advantages:

Disadvantages:

Safety:

Metal halide

Description of operation:  [include starter]

Typical use:  [type of ATON]

Technical data:

- Power:

- Efficiency:

- Operating life:

Advantages:

Disadvantages:

Safety:

Xenon

Description of operation:  [include starter]

Typical use:  [type of ATON]

Technical data:

- Power:

- Efficiency:

- Operating life:

Advantages:

Disadvantages:

Safety:

##### 3.4.1.3.3 Light Emitting Diode (LED)

Colored LEDs

Description of operation:  [include starter]

Typical use:  [type of ATON]

Technical data:

- Power:

- Efficiency:

- Operating life:

Advantages:

Disadvantages:

Safety:

White LEDs

Description of operation:  [include starter]

Typical use:  [type of ATON]

Technical data:

- Power:

- Efficiency:

- Operating life:

Advantages:

Disadvantages:

Safety:

##### 3.4.1.3.4 Lasers

Description of operation:  [include starter]

Typical use:  [type of ATON]

Technical data:

- Power:

- Efficiency:

- Operating life:

Advantages:

Disadvantages:

Safety: