# chapter 3	          AIDS TO NAVIGATION

## 3.1 Definition of Aids to Navigation

A marine Aid to Navigation is a device or system external to vessels that is designed and operated to enhance the safe and efficient navigation of vessels and/or vessel traffic.

## Scope

This chapter describes the major types of aids to navigation in current use and provides comments on the application and performance of the technology.  Pilotage and Vessel Traffic Services (VTS) are covered in the chapter, since these services can also satisfy the definition of an aid to navigation.

## Visual Marks

### Types

Visual marks for navigation can be either natural or man-made objects. They include structures specifically designed to assist navigation, as well as conspicuous features such as headlands, mountain-tops, rocks, trees, church-towers, minarets, monuments, chimneys, etc.

Visual marks can be provided with a light if navigation at night is required, or left unlit if daytime navigation is sufficient.

Navigation at night is possible, to a limited extent, if the unlit aids are provided with:

- a radar reflector and the vessel has a radar, or;

- retro-reflecting material and the vessel has a searchlight.  This approach would generally only be acceptable for small boats operating in safe waterways and with the advantage of local knowledge.

### Visual Aids to Navigation

#### Description

Visual aids to navigation are purpose-built facilities that communicate information to a trained observer on a vessel for the purpose of assisting the task of navigation. The communication process is referred to as Marine Signalling.

Common examples of visual aids to navigation include lighthouses, beacons, leading (range) lines, lightvessels, buoys (lit or unlit), daymarks (dayboards) and traffic signals.

The effectiveness of a visual aid to navigation is determined by factors such as:

- type and characteristics of the aid provided;

- location of the aid relative to typical routes taken by vessels;

- distance (range) of the aid from the observer;

- atmospheric conditions;

- contrast relative to background conditions (conspicuity);

- the reliability and availability of the aid.

Refer to IALA publications: - The IALA Guide to the Availability and Reliability of Aids to Navigation; - Chapter 11 of this Guide.

#### Distinguishing Features

Visual aids to navigation are distinguished by:

- type;

  - fixed structure;

  - floating platform;

- location;

  - inclusion of subsidiary aids

  - relationship to other aids to navigation and observable features

- characteristics;

  - shape;

  - size;

  - elevation;

  - colour;

  - lit/unlit;

  - signal character;

  - light intensity;

  - sectors;

  - construction materials;

  - retro-reflective features;

  - names, letters and numbers.

### Signal Colours

IALA has made recommendations on colours for lit (lighted) aids to navigation and for surface colours for visual signals on aids to navigation.

- Lights use a four-colour system comprised of white, red, green and yellow that conforms to the International Commission on Illumination (CIE) publication No.2.2 (1975) “Colours of Signal Lights”;

- Recommended surface colours for visual signals on aids to navigation are as follows:

  - Ordinary colours should be limited to white, black, red, green, yellow and blue.

  - Orange and fluorescent red, yellow, green and orange may be used for special purposes requiring high conspicuity.

Refer to IALA publications: - Recommendation for the colours for light signals on aids to navigation, December 1977 ; - Recommendation for the use of retro-reflecting material on aids to navigation marks within the IALA Maritime Buoyage System (E106), May 1998; - Recommendation for the surface colours used as visual signals on aids to navigation (E108), May 1980.

The CIE standard on the measurement of colours (colorimetry) is based on three reference colours (ie. a tri-stimulus system) that in varying combination can generate the spectrum of colours.  A particular colour is described by the symbols; x, y and z that represent the proportions of the reference colours.

Using ratios of the tri-stimulus values, such that: x + y + z = 1, colours can be defined in terms of chromaticity using just the x and y values.  The advantage of this arrangement is that colours can be mapped on a two-dimensional chromaticity diagram.

CIE colour standards for marine signalling can be depicted as areas on the chromaticity diagram.  These areas are defined by boundaries expressed as functions of x and y (equations).

If the chromaticity co-ordinates of a coloured light, filter material or a paint product are known, its acceptability for marine signalling applications can easily be determined.

Fig 3.1 Illustrates the colour zones on the 1931 CIE chromaticity diagram.

(Note that the colour rendering is indicative only).

The CIE standard for marine signalling colours has recently been revised, with some adjustments to the boundaries of signal colours.  An extract from the CIE illustration of the colour boundaries is shown in Fig 3.2.  For further details on the chromaticity coordinates and equations of the boundaries, please refer to

CIE S 004/E-2001 Colours of Light Signals

Fig 3.2 IALA allowed chromaticity areas for red, orange, yellow, green, blue, white and black of Ordinary Surface Colours plotted on the 1983 CIE chromaticity diagram (Courtesy of CIE)

### Visibility of a Mark

The visibility of a mark is affected by one or more of the following factors:

- observing distance (range);

- curvature of the Earth;

- atmospheric refraction;

- atmospheric transmissivity (meteorological visibility);

- height of the aid above sea level;

- observer's visual perception;

- observer's height of eye;

- observing conditions (day or night);

- conspicuity of the mark (shape, size, colour, reflectance, and  the properties of any retro-reflecting material);

- contrast (background lighting);

- mark lit or unlit;

- intensity and character;

### Observational Factors

#### Meteorological Visibility

Meteorological visibility (V) is defined as the greatest distance at which a black object of suitable dimensions can be seen and recognised by day against the horizon sky, or, in the case of night observations, could be seen if the general illumination were raised to the normal daytime level.  It is usually expressed in kilometres or nautical miles.

#### Atmospheric Transmissivity

The atmospheric transmissivity (T) is defined as the transmittance or proportion of light from a source that remains after passing through a specified distance through the atmosphere, at sea level.

Since the atmosphere is not uniform over the observing distances of most visual aids, a representative value is used:

- typically, the atmospheric transmissivity is taken as T = 0.74 over one nautical mile;

- a figure of T = 0.84 is occasionally used in regions where the atmosphere is very clear.

A number of countries collect data on atmospheric transmissivity for different parts of their coastline.  This enables the luminous range of lights to be:

- calculated more precisely, and;

- better matched to local conditions and user requirements.

#### Atmospheric Refraction

This phenomenon results from the normal decrease in atmospheric density from the Earths’ surface to the stratosphere.  This causes light rays that are directed obliquely through the atmosphere to be refracted (or bent) towards the Earth.

#### Contrast

The ability to detect differences in luminance between an object and an otherwise uniform background is a basic visual requirement and is used to define the term contrast.  It is represented by the equation:

C = (Lo −LB)LB

where:

C   =       contrast

| LB = | luminance of background (cd/m2) |
| --- | --- |
| Lo = | luminance of object (cd/m2) |

The contrast at which an object can be detected against a given background 50% of the time is called the threshold contrast.  For meteorological observations, a higher threshold must be used to ensure that the object is recognised.

A contrast value of 0.05 has been adopted as the basis for the measurement of meteorological optical range.

#### Use of Binoculars

While it is generally assumed that observations will be made with the naked eye, mariners will quite often use binoculars.  This can allow:

- a light being observed, or the characteristics resolved, at a greater luminous range than with the naked eye;

- a limited improvement in the sensitivity of leading lights;

  - about a 30% improvement in the detectable difference from a given bearing;

- the identification of a light operating against background lighting conditions.

Generally, the most suitable binoculars for use at sea are considered to be:

- 7 x 50 type at night, and;

- 10 x 50 binoculars by day.

### Range of a Visual Mark

The range of an aid to navigation can broadly be defined as the distance at which the observer’s receiver can detect and resolve the signal.  In the case of visual marks the observer’s receivers are his/her eyes.  This broad definition of range leads to a number of more specific definitions that are described below.

#### Geographical Range

This is the greatest distance at which an object or a light source could be seen under conditions of perfect visibility, as limited only by the curvature of the earth, by refraction of the atmosphere, and by the elevation of the observer and the object or light. (IALA International Dictionary of Aids to Marine Navigation).  A Geographical Range table is shown in Table 3.1.

Table 3.1 IALA Geographical Range Table.

| Geographical Range in Nautical Miles |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Observer eye height metres | Elevation of Mark metres |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 10 | 50 | 100 | 200 | 300 |
| 1 | 2.0 | 4.1 | 4.9 | 5.5 | 6.1 | 6.6 | 8.5 | 16.4 | 22.3 | 30.8 | 37.2 |
| 2 | 2.9 | 4.9 | 5.7 | 6.4 | 6.9 | 7.4 | 9.3 | 17.2 | 23.2 | 31.6 | 38.1 |
| 5 | 4.5 | 6.6 | 7.4 | 8.1 | 8.6 | 9.1 | 11.0 | 18.9 | 26.9 | 33.3 | 39.7 |
| 10 | 6.4 | 8.5 | 9.3 | 9.9 | 10.5 | 11.0 | 12.8 | 20.8 | 26.7 | 35.1 | 41.6 |
| 20 | 9.1 | 11.1 | 12.0 | 12.6 | 13.1 | 13.6 | 15.5 | 23.4 | 29.4 | 37.8 | 44.2 |
| 30 | 11.1 | 13.2 | 14.0 | 14.6 | 15.2 | 15.7 | 17.5 | 25.5 | 31.4 | 39.8 | 46.3 |

The values in Table 3.1 are derived from the formula:

Rg=2.03×(ho+ Hm)

where:

| Rg = | geographical range (nautical miles) |
| --- | --- |
| ho = | elevation of observer’s eye (metres) |
| Hm = | elevation of the mark (metres) |

Note

The factor 2.03 accounts for refraction in the atmosphere.

Climatic variations around the world may lead to different factors being recommended.   Typical range of factors is 2.03 to 2.12.

#### Meteorological Optical Range

This is the distance through the atmosphere that is required for 95% attenuation in the luminous flux of a collimated beam of light using a source colour temperature of 2700ºK.

The meteorological optical range is related to the atmospheric transmissivity by the formula:

V=dlog0.05logT		or		T=0.05dV

Where:

V  =	meteorological optical range (nautical miles)

d  =	distance (nautical miles)

T  =	atmospheric transmissivity

It is often convenient to simplify the above expression by giving the distance term a  value of one, such that:

T=0.051V		or		TV=0.05

#### Visual Range

This is the maximum distance at which the contrast of the object against its background is reduced by the atmosphere to the contrast threshold of the observer.  The visual range can be enhanced if the observer uses binoculars, although the effectiveness depends on the stability of the observer’s platform.

#### Luminous Range

This is the maximum distance at which a given light signal can be seen by the eye of the observer at a given time, as determined by the meteorological visibility prevailing at that time. It takes no account of elevation, observer's height of eye or the curvature of the Earth.

#### Nominal Range

Nominal range is the luminous range when the meteorological visibility is 10 nautical miles, which is equivalent to a transmission factor of T = 0.74.  It is generally the figure used in official documentation such as charts, Lists of Lights, etc.

Nominal range assumes that the light is observed against a dark background, free of background lighting.

## 3.4  Aids to Navigation Lights

### Light Sources

#### Brief History

Until the first application of electricity to lighting late in the nineteenth century, all artificial light was produced by fire.  Illuminants progressed from pyres of wood (used up until the 1800’s), to oil wick lamps, vaporised oil and gas burners and then electric arc and tungsten filament lamps.  Optical devices matched these developments, first with reflector systems and later with lenses.

It is interesting to note that the efforts to understand human perception of light and to improve the efficiency and effectiveness of aids to navigation illuminants and optical apparatus were for many years at the forefront of scientific endeavours.

The glass lens design pioneered by Fresnel around 1820 remains a principal element of the modern aid to navigation light.  Present day lenses are often made of plastic rather than glass.

A few countries still have gas lights that burn acetylene or propane. However, the majority of aids to navigation lights use electric lamps of various types.  Increasingly, these lamps draw their power from renewable energy sources such as solar, wind or wave power.

Electric lamps have been specifically designed for aids to navigation applications.  However, lamps selected from the enormous range of commercial products have also been used or adapted for aids to navigation.

Light emitting diode (LED) technology is rapidly emerging as an alternative to filament lamps.

#### Gas Lights

Acetylene

The acetylene light has a special place in the history of aids to navigation, primarily for being the first reliable means of automating lighthouses, buoys and beacons during the earlier part of the 20th century.  Acetylene lighting systems originated from the inventions of Gustaf Dalen and were made by a number of suppliers. Acetylene gas has the unusual property of burning with a white flame when correctly mixed with air. This enabled the development of exceptionally reliable open flame lanterns.

Acetylene lighting technology was further enhanced by the development of the Dalen “mixer” that allowed gas and air to be drawn into a chamber and then consumed in an incandescent mantle to produce a brighter light source than the open flame type.  The incandescent mantle can be operated as a flashing source inside a fixed lens or as a continuous source inside a rotating lens.  Related developments included a gas-operated mechanism for rotating a lens and a clockwork powered automatic mantle changing device.

Refer to IALA publication: - Practical Note for the Safe Handling of Gases, October 1993.

Propane and Butane

Propane and butane gases have been used as fuels for gas lighting systems.  The lighting equipment has to use an incandescent mantle burner as both gasses burn with yellow/orange flames when an open flame burner is used.

Gas lighting installations continue to be used in some countries, where they are chosen for their robustness and simplicity of operation. They have been replaced by solar/electric installations in many countries generally resulting in considerable savings in operating costs.

#### Electric Lights

##### 3.4.1.3.1 Incandescent filament

Tungsten filament

Description of operation:

Can be operated directly from an appropriate electrical supply.

Nominal voltage 6 to 240 V both AC and DC

These light sources have been used in most countries from at least the early 1900s. Many special lamp designs have been used over the years, as filament size, shape, and location must be well matched to the lens system.

Typical use:

All types of lighted beacons  (leading lights, sector lights, 360° lights, lanterns on light buoys...)

Some countries and manufacturers have adopted standard designs, with reference codes, for lamps designed specially for lighthouse applications. These designs typically include filament supports to maintain filament shape and ensure an even output over 360 degrees azimuth.

Technical data:

- Power:

From 2  to 1000 Watt (3 500 W lamps have been used in the past)

- Efficiency:

From 9 to 19 lumens/ W

- Operating life:

From 300 to1500 hours

Advantages:

Coding can be easily achieved by interrupting the electrical supply.

Filament geometry can be designed to match the optic.

Diffusing envelope (e.g. pearl or etched) can improve lens illumination when used in older optics, but at the expense of reduced intensity.

Broad colour spectrum provides good performance with most coloured filters.

Output reasonably stable over lifetime, but lamp envelope may start to blacken before lamp failure.

Remote monitoring by current sensing is simple.

Disadvantages:

,Relatively short lifetime.

Specialist AtoN lamps are expensive.

Colour rendering is not in the preferred white region (tends towards yellow).

Efficiency poor.

Safety:

High envelope temperature when in use.

Operating voltage may be hazardous.

Possible risk of arcing

General glass hazard

Disposal:

Safe disposal appropriate for metal and glass waste.

### Tungsten halogen

Description of operation:

The tungsten halogen lamp encloses a small amount of halide with inert gas, and the tungsten evaporating from the hot filament combines with the halogen to diffuse around the envelope wall. Due to careful design the envelope operates at high temperature, and this prevents deposition of tungsten on the glass. The tungsten-halide is then carried by convection towards the filament where it is decomposed and the tungsten metal deposited back onto the filament.

Can be operated directly from an appropriate electrical supply.

Nominal voltage 12 to 240 V both AC and DC.

Typical use:

All types of lighted beacons, but several lamps could be used in a cluster in large rotating optics, to produce a large light source similar to the original non-electric light source.

Technical data:

- Power:

From 5 to 1000 W, exceptionally 1500 and 2000 W

- Efficiency:

20 to 25 lumen/W

- Operating life:

300 to 4000 hours.         Note- some very bright lamps have short working life.

Advantages:

Coding can be easily achieved by interrupting the electrical supply, but see below.

Higher luminance.than tungsten lamps.

Output very stable over lifetime.

Colour rendering good in white preferred region.

High performance general-purpose lamps available at low cost.

Small lamp sizes (10 to 100W) are very mechanically robust.

Envelope sizes are typically smaller than tungsten lamps, and may allow smaller optic system sizes.

Disadvantages:

The filament size is usually small hence the geometry is poor when modernising old lens systems.

Generally low operating voltage results in high current requiring careful design of lamp holder and associated wiring.

A cluster of several lamps will be required to match these small lamps to existing large optics.

Lamps are not made specifically for AtoN use and specifications may be changed without notice.

The flashing of tungsten halogen lamps may lead to the interruption of the halogen cycle with consequent blackening of the envelope and premature failure. Practical trials are recommended with the proposed operating voltage and duty cycle or consultation with other IALA members.

Lamps should not be touched with bare hands due to consequential reduction in lamp life.

Safety:

Operating voltage may be hazardous.

General glass hazards.

Very high surface temperatures because of the small envelope size.

Possible high UV radiation risk (dependant on lamp type).

Risk of explosion with high-pressure lamp types.

Disposal:

Local and national waste disposal regulations should be consulted regarding tungsten halogen lamps.

##### 3.4.1.3.2 Discharge lamps

Fluorescent

Description of operation:

110 to 240 V system voltage with control circuitry to provide a high starting voltage.

Typical use:

Direction arrows, signs and light tubes or bars used for leading lights. Applications where large areas of illumination are an advantage.

Technical data:

- Power:

8 to 100 W

- Efficiency:

80 to 100 lumen/W

- Operating life:

Up to 20000 hours

Advantages:

High luminous efficiency.

Large illuminated area. In suitable applications no optical elements are needed thus providing a very low cost AtoN.

Very wide range of commercially available products at low prices.

Many colours available (no additional colour filters needed).

Disadvantages:

Low luminance.

Difficult to use with lens systems due to source size.

Light output falls considerably over lifetime.

Requires control circuitry matching lamp and supply voltage.

Not normally coded as complex circuitry needed to flash.

Possible EMC problems.

Safety:

Mains voltage.

General glass hazard.

Internal tube coatings may be hazardous if exposed and they contain traces of gaseous mercury.

High voltage due to starting equipment.

Disposal:

Tube coatings may be hazardous and contain traces of mercury. Consult local and national disposal regulations.

Low-pressure sodium vapour lamps

Description of operation:

110 and 240 V AC with associated control circuitry.

NOTE: Only available as yellow colour

Typical use:

Flood lighting and external illumination of structures, towers, locks etc.

Technical data:

- Power:

20 to 180 W

- Efficiency:

180 lumen/W

- Operating life:

10 000 hour

Advantages:

Long life.

High luminous efficiency.

Mercury free.

Low envelope surface temperature.

Can be used to provide yellow signal colour.

Minimum attraction for insects.

Disadvantages:

Only produces yellow light.

Low luminance.

Not practical to flash (code).

Limited operating positions.

Safety:

General glass hazards.

High AC voltage.

Chemical hazard due to sodium content.

Disposal:

Consult local and national regulations regarding sodium disposal.

High Pressure Sodium Vapour Lamps

Description of operation:

110 or 240 V AC with associated control circuitry.

Typical Use:

White lamps may be used as AtoN light source.

Technical Data:

- Power

50 to 400W

- Efficiency

90 lumens/W

- Operating Life

10000 hour

Advantages:

Long life.

Mercury free.

High efficiency.

Available in white.

Disadvantages:

Cannot be coded (flashed).

Only practical as white Low red content makes colour filtering impractical.

High strike voltage to start.

Complex lamp changer required due to long warm-up period and  cool-down necessary before restart.

Arc tube geometry is poor for most optics.

Light output falls over life and white colour degrades to yellow.

Safety:

General glass hazards.

High voltages.

Chemical hazards giving rise to disposal or health problems.

Disposal:

Consult national and local regulations regarding sodium disposal.

Metal halide

Description of operation:

The metal halide lamp is one of a family of HID [High Intensity Discharge] lamps, and its arc tube is made of silica glass. The principle of emission is as follows. 1) High voltage from the ballast initiates current flow between the electrodes. 2) As the lamp temperature rises, metals in the lamp evaporate, and light emission occurs.

110 and 240 V    Working with control circuitry allows input voltages 12V to 240V

Typical use:

Used as a fixed light source in rotating optics, fixed lenses with rotating screens and general floodlighting.

Technical data:

- Power:

10 to 2000 W

- Efficiency:

80 to110 lumen/W

- Operating life:

6 000  to 20 000 h

Note life is very dependant on number of times that lamp is switched on

Advantages:

High luminous efficiency.

Clear envelope types have high illuminance.

Coated envelope types have good geometry for traditional lenses.

Long life.

Many commercial lamp types available.

Colour rendering is good, within IALA white preferred region.

Absence of a filament means good resistance to vibration and shock.

Disadvantages:

Not practical to code (flash).

Initial warm-up is slow.

Cooling time needed before re-strike hence complex lamp changer design.

Difficult to remote monitor by simple current sensing.

Light output falls with life.

Red spectrum is limited so red filtering is possible but poor, green is good.

Safety:

High voltage hazards.

High UV radiation.

Possible EMC problems.

Possible explosion hazard.

General glass hazard.

May contain hazardous metal.

Disposal:

Consult local and national regulations for these lamps, as there is some mercury content.

Xenon Lamps

Description of operation:

Xenon lamps are discharge lamps, with the xenon gas enclosed in a silica tube at high pressure. An electrical discharge through the xenon gas generates a high intensity white light. [Xenon discharge is commonly used in camera flash guns.]

110 and 240V typically.

Charging DC supply requires complex control circuitry.

Available as pulsed or continuous discharge types.

Typical use:

A specialised light source used where high intensity is of paramount importance. Can be used in fixed or rotating optics.

Technical data:

- Power:

150 to 2000 W

- Efficiency:

35 lumens/W

- Operating life:

2000 h

Advantages:

High luminance enabling high intensities to be achieved with suitable optics.

Broad white colour spectrum allowing good colour filtering.

Disadvantages:

Electrical control system is complicated.

Lamp changer design is very complex.

Short lamp life.

Electronic control components have short life.

Relatively expensive.

Power consumption is variable due to the charging/discharging cycle of the system, resulting in varying loads on the power supply system.

Safety:

High voltages.

Possible danger of explosion as the pressure in the lamp is high.

High UV radiation

High surface temperature

General glass hazards

Disposal:

Check local and national regulations for the lamp type. There may be some mercury content.

##### 3.4.1.3.3 Light Emitting Diode (LED)

Description of operation:

Coloured LEDs

LEDs are electronic semiconductor devices that produce near monochromatic light. The semiconductor junction is encapsulated in a clear plastic housing that usually incorporates a lens. Several LEDs may be grouped together in a cluster or an array to provide a light source of the required size and intensity with lamp redundancy. New high power LEDs allow short range lanterns using a single LED.

LEDs operate from a low voltage DC supply. Correct operation depends on accurate control of the supply current.

White LEDsA semiconductor junction emitting blue/violet light is encapsulated with an integral phosphor such that both blue and broad band yellow light are emitted together to form a near white light.  Research is in progress on combining red and green LED lights to produce a white lights within the IALA chromaticity specification.

LED marine lanterns are sometimes reported as having intense colours and ranges longer than the current IALA calculation method would suggest. Current work by IALA is investigating this.

Typical use:

Lighted beacons on buoys and other short range AtoN, but longer range LED lanterns are appearing on the market.

Range lights consisting of flat arrays of LEDs or single high power LEDs

Signs and signals formed by arrays of LEDs in the shape of letters, numerals, signs etc.

Technical data:

- Power

Single LED  1mW to 5W

LED cluster in lantern 1W to 60W or even higher.

- Efficiency:

This depends on colour:

  - Red and green – 25 to 30 lumen/W

  - Yellow	-20 to 30 lumen/W

  - White –20 to 30 lumen/W

  - Blue – 10 lumen/W

NOTE- Extensive research in LED technology is continually providing more efficient products

- Operating life:

Depends on the LED junction operating temperature and operating    environment

Possible values in excess of 100,000 hours for a single diode

Advantages:

Very long life (so long as input power and temperature is carefully controlled) and hence low whole life costs.

Life is so long that lampchangers are not considered necessary.

High luminous efficiency in red and green

Light produced in saturated signal colours therefore coloured filters not needed

Mechanically robust when compared with conventional lamps

Rise and fall times are very fast.

Relatively cool operation.

Disadvantages:

Complex electronic control needed to achieve long life and high performance.

Generally difficult to match to existing optics.

Luminous efficiency decreases slowly with life.

White LEDs will be outside the new CIE (2001) white colour region.

White LEDs will be very inefficient with red and green filters.

Lamp life can be severely reduced if input power and temperature are not carefully controlled.

LEDs are not yet suitable for long range lights.

Safety:

No special hazard.

Disposal:

Consult National and local regulations regarding disposal of semiconductors.

##### 3.4.1.3.4 Lasers

A laser is a device that produces a coherent collimated beam of monochromatic light.

Their use has not been established in AtoN light systems, despite the work over some decades. However there are two areas of development.

(1) High power lasers can be used to provide a line of light in the sky where particles of dust, water etc. are illuminated by the laser beam to provide a leading line. These devices require considerable electrical power (several kW). Detailed safety procedures are required for operation and servicing.

(2) Low power lasers are being trialled by the Canadian Coastguard where the laser is aimed directly at the mariner. Different coloured lasers are used to identify areas of navigational importance. The laser light is visible at useful range in daylight.

Power requirements are low (tens of Watts). The laser projectors are expensive and require complex control systems. Detailed procedures are required for safe servicing.

Laser trials are also taking place in Russia and Japan.

Advantages

Single wavelength

Strong directivity

Simple optic design

Disadvantages

High power requirements for high power lasers

System complexity may be a problem at some locations

Safety

Possible eye damage under certain conditions, if appropriate ASTM safety standards are not applied.

Refer to IALA publication: - Guidelines on new light sources and associated power supplies, December 2001.

### Photometry of Marine Aids to Navigation Signal Lights

#### Measurement of Light

The behaviour of light, in physics, is normally seen in the context of either a form of electromagnetic radiation or particle motion.  The latter includes the concept of “rays” of light that are used in analysing the interactions of light and lenses.

The units of interest for  electromagnetic applications of light are generally metres (wavelength) and watts (power).

The study of photometry and the use of lights for signal application has necessitated a parallel set of units to be developed to  account for the physiological aspects of how the human eye evaluates a light source.

The spectral sensitivity of the human eye (or the response of the eye to different coloured light) has been evaluated in tests of large numbers of people.  The results have been presented as a standard spectral sensitivity distribution or V-lambda curve.

Fig 3.6 Spectral sensitivity distributions or V-lambda curves

for the human eye also showing the difference between day and night vision.

#### Units of Measurement

Table 3.2 Photometric units of measurement.

| Term | Description | Unit | Abbreviation |
| --- | --- | --- | --- |
| Luminous flux | This is the total light emitted from the source (ie. lamp) The peak sensitivity of the human eye occurs at about 555 nanometres, a wavelength that corresponds to green. At this wavelength, the photometric equivalent of one watt is defined as 680 lumens. | lumens | lm |
| Luminous intensity | This is the part of the luminous flux in a particular direction. Also expressed as the luminous flux per solid angle (or steradian) | candela | cd |
| Luminance (Brilliance) | This is the portion of the luminous flux emitted in a specific direction by a surface element of the luminous body. This variable is an important term for rating the brightness impression of light sources and illuminated objects. Luminance is the parameter that causes the response in the eye. | candelas per square meter and also as candelas per square centimetre | cd/m2 cd/cm2 |
| Illuminance | This is the density of the luminous flux incident on a surface. It is the quotient of the luminous flux by the area of the surface when the surface is uniformly illuminated | lux (lumens/square metre) | lx |
| Luminous efficacy | This is the efficacy with which electrical power is converted to visible radiation. | lumens per watt of electrical power consumed |  |
| Colour temperature | This is a measure of how yellow or blue-white the light appears to the eye. It is related to the temperature of a tungsten filament. | Kelvin | ºK |
| Colour rendering index CRI | characterises the colour rendering quality of the light from a lamp. It is the same for all incandescent lamps by definition and equal to the maximum value of 100. |  |  |

#### Threshold of Illuminance

In physical terms, the threshold of illuminance is the lowest level of illuminance from a point source of light, against a given background level of luminance, that causes a visual response at the eye.

For visual signalling applications, the threshold of illuminance (E) is taken to be 0.2 micro lux.

In the case of leading lights of limited range and with a high level of shore illumination, the above figures may be found too low.

It is recommended that to observe the relative position of the lights easily and to derive the maximum possible accuracy from leading lights, it is generally necessary to have a minimum illuminance of 1 micro lux at the eye of the observer. Refer to IALA publication: - Recommendation for Leading Lights (E112), May 1998.

This condition is to be met at the outer limits of the useful segment for the minimum meteorological visibility under which the leading lights are to be used.

#### Luminous intensity

The luminous intensity of a navigation light is directly proportional to the luminance of the light.  The dimension of the light source is also important because the luminous intensity of any given fixed lens is proportional to the area of the light source.

#### Inverse Square Law

Light emitted from a source radiates out in all directions. For a point source, the wave fronts of light can be imagined to generate a series of spherical surfaces.  As shown in Fig 3.7, the further the light travels from the source, the greater is the surface area of the sphere and consequently, the lower the illuminance.  Since illuminance is measured in lumens/metre2 and the surface area of a sphere increases in proportion to the square of the radius, the illuminance decreases in proportion to the square of the distance from the source.  The decline in illuminance with distance is described as an inverse-square law.

.

Fig 3.7 Illustration of the Inverse-Square-Law concept.

#### Allards Law

The illuminance of a light source reaching an observer’s eye determines whether the light is seen.  The relationship between the illuminance produced at the observer’s eye, the luminous intensity of the light source and the atmospheric transmissivity is known as Allards Law:

E = I×Tdd2

where:

| E = | illuminance at the observer’s eye (lm/m2) |
| --- | --- |
| I = | effective intensity of the light source (cd) |
| T = | atmospheric transmissivity |
| d = | distance between the light source and the observer. [Because T is measured per nautical mile, d in the numerator must also be in nautical miles. In the denominator, d is in metres] |

Note

Allards law applies only when the luminance of the background is small compared to the average illuminance of the light.

Where average background luminance is large, as typically happens during the daytime, the equation becomes:

E = [I − (L − L')A] × Tdd2

where:

| L = | luminance of the background of the light in candelas per unit and is measured in the direction of the line of sight from a position near the light (cd/m2) |
| --- | --- |
| L' = | average luminance of the unlighted projector in candelas per unit and is measured in the direction of the line of sight from a position near the light (cd/m2) |
| A = | area of the entire projector projected on a plane normal to the line of sight (m2) |
| (L - L' )A= | intensity required of the light to make its average luminance equal to that of the background (cd) |

### Rhythms / Character

IALA has produced a recommendation on the characters for light on aids to navigation

Refer to IALA publication: - Recommendation for the rhythmic characters of lights on aids to navigation (E110), May 1998.

The tables of classifications and specifications of aid to navigation characters from Recommendation E110 are reproduced below.

(EDITORIAL NOTE: WG 4 HAS BEEN REQUESTED TO REVIEW THE NEED FOR GRAPHICAL REPRESENTATIONS OF THE CHARACTERS TO ASSIST WITH INTERPRITATION OF THE REQUIREMENTS)

| CLASSIFICATION OF THE RHYTHMIC CHARACTERS OF LIGHTS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Class | Abbreviation | General description | IALA Specification | Particular use in the IALA Maritime Buoyage System |
| 1 | FIXED LIGHT | F | A light showing continuously and steadily. | A single fixed light should be used with care because it may not be recognized as an aid to navigation light. | A single fixed light shall not be used. |
| 2 | OCCULTING LIGHT |  | A light in which the total duration of light in a period is longer than the total duration of darkness and the intervals of darkness (eclipses) are usually of equal duration. | A light in which the total duration of light in a period is clearly longer than the total duration of darkness and all the eclipses are of equal duration. |  |
| 2.1 | Single-occulting light | Oc | An occulting light in which an eclipse is regularly repeated | The duration of an appearance of light should not be less than three times the duration of an eclipse. The period should not be less than 2 s | A single-occulting White light indicates a safe-water mark. |
| 2.2 | Group-occulting light | Oc(#) eg. Oc(2) | An occulting light in which a group of eclipses, specified in number, is regularly repeated. | The appearances of light between the eclipses in a group are of equal duration, and this duration is clearly shorter than the duration of the appearance of light between successive groups. The number of eclipses in a group should not be greater than four in general, and should be five only as an exception. The duration of an appearance of light within a group should not be less than the duration of an eclipse. The duration of an appearance of light between groups should not be less than three times the duration of an appearance of light within a group. In a group of two eclipses, the duration of an eclipse together with the duration of the appearance of light within a, group should not be less than 1 s. In a group of three or more eclipses, the duration of an eclipse together with the duration of an appearance of light within the group should not be less than 2 s. | A group-occulting Yellow light indicates a special mark. |
| 2.3 | Composite group-occulting light | Oc(2 + 1) | A light similar to a group-occulting light except that successive groups in a period have different numbers of eclipses. | This class of light character is not recommended because it is difficult to recognize. |  |
| 3 | ISOPHASE LIGHT | Iso | A light in which all the durations of light and darkness are clearly equal. | The period should never be less than 2 s, but preferably it should not be less than 4 s in order to reduce the risk of confusion with occulting or flashing lights of similar periods. | An isophase White light indicates a safe-water mark. |
| 4 | FLASHING LIGHT |  | A light in which the total duration of light in a period is shorter than the total duration of darkness and the appearances of light (flashes) are usually of equal duration. | A light in which the total duration of light in a period is clearly shorter than the total duration of darkness and all the flashes are of equal duration. |  |
| 4.1 | Single-flashing light | Fl | A flashing light in which a flash is regularly repeated (at a rate of less than 50 flashes per minute). | The duration of the interval of darkness (eclipse) between two successive flashes should not be less than three times the duration of a flash. The period should not be less than 2 s (or not less than 2.5 s in those countries where a quick rate of 50 flashes per minute is used). | A single-flashing Yellow light indicates a special mark. |
| 4.2 | Long-flashing light | LFI | A single-flashing light in which an appearance of light of not less than 2 s duration (long flash) is regularly repeated. |  | A long-flashing White light with a period of 10 s indicates a safe-water mark. |
| 4.3 | Group-flashing light | Fl(#) eg. Fl(3) | A flashing light in which a group of flashes, specified in number, is regularly repeated. | The eclipses between the flashes in a group are of equal duration, and this duration is clearly shorter than the duration of the eclipse between successive groups. The number of flashes in a group should not be greater than five in general, and should be six only as an exception. The duration of an eclipse within a group should not be less than the duration of a flash. The duration of an eclipse between groups should not be less than three times the duration of an eclipse within a group. In a group of two flashes, the duration of a flash together with the duration of the eclipse within the group should not be less than 1 s. In a group of three or more flashes, the duration of a flash together with the duration of an eclipse within a group should not be less than 2 s (or not less than 2.5 s in those countries where a quick rate of 50 flashes per minute is used). | A group-flashing White light with a group of two flashes, in a period of 5 s or 10 s, indicates an isolated-danger mark. A group-flashing Yellow light with a group of four, five or (exceptionally) six flashes indicates a special mark |
| 4.4 | Composite group-flashing light | eg Fl(2 + 1) | A light similar to a group-flashing light except that successive groups in a period have different numbers of flashes. | Light characters should be restricted to (2 + 1) flashes in general, and should be (3 + 1) flashes only as an exception. | A composite group-flashing Red or Green light with a group of (2 + 1) flashes indicates a modified lateral (preferred-channel) mark. A composite group-flashing Yellow light indicates a special mark. |
| 5 | QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 50 flashes per minute but less than 80 flashes per minute. | A light in which identical flashes are repeated at the rate of 60 (or 50) flashes per minute. The higher rate of flashing is preferred. |  |
| 5.1 | Continuous quick light | Q | A quick light in which a flash is regularly repeated. |  | A continuous quick White light indicates a north cardinal mark. |
| 5.2 | Group quick light | Q(3) Q(9) IQ(6) + LFl | A quick light in which a specified group of flashes is regularly repeated. | The number of flashes in a group should be three or nine. An exceptional light character is reserved for use in the IALA Maritime Buoyage System to indicate a south cardinal mark. | A group quick While light with a group of three flashes, in a period of 10 s, indicates an east cardinal mark. A group quick White light with a group of nine flashes, in a period of 15 s, indicates a west cardinal mark. A group quick White light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 15 s, indicates a south cardinal mark. |
| 5.3 | Interrupted quick light | IQ | A quick light in which the sequence of flashes is interrupted by regularly repeated eclipses of constant and long duration. | The number of flashes in a period should be at least eight. The duration of the long eclipse should not be less than 3 s. | An interrupted quick light should not be used. |
| 6 | VERY QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 80 flashes per minute but less than 160 flashes per minute. | A light in which identical flashes are repeated at the rate of 120 (or 100) flashes per minute. The higher rate of flashing is preferred. |  |
| 6.1 | Continuous very quick light | VQ | A very quick light in which a flash is regularly repeated. |  | A continuous very quick White light indicates a north cardinal mark. |
| 6.2 | Group very quick light | VQ(3) VQ(9) VQ(6) + LFl | A very quick light in which a specified group of flashes is regularly repeated. | The number of flashes in a group should be three or nine. An exceptional light character is reserved for use in the IALA Maritime Buoyage System to indicate a south cardinal mark. | A group very quick White light with a group of three flashes, in a period of 5 s, indicates an east cardinal mark. A group very quick White light with a group of nine flashes, in a period of 10 s, indicates a west cardinal mark. A group very quick White light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 10 s, indicates a south cardinal mark. |
| 6.3 | Interrupted very quick light | IVQ | A very quick light in which the sequence of flashes is interrupted by regularly repeated eclipses of constant and long duration. | The number of flashes in a period should be at least eight. The duration of the long eclipse should not be less than 3 s. | An interrupted very quick light should not be used. |
| 7 | ULTRA QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 160 flashes per minute. | A light in which flashes are repeated at a rate of not less than 240 flashes per minute and not more than 300 flashes per minute. |  |
| 7.1 | Continuous ultra quick light | UQ | An ultra quick light in which a flash is regularly repeated. |  |  |
| 7.2 | Interrupted ultra quick light | IUQ | An ultra quick light in which the sequence of flashes is interrupted by eclipses of long duration. | An ultra quick light in which the sequence of flashes is interrupted by regularly repeated eclipses of constant and long duration. The approximate duration of the sequence of flashes may be specified. number of flashes in a period should be at least 25. The duration of the long eclipse should not be less than 3 s. |  |
| 8 | MORSE CODE LIGHT | Mo(#) eg. Mo(L) | A light in which appearances of light of two clearly different durations are grouped to represent a character or characters in the Morse Code. | Light characters should be restricted to a single letter in the Morse Code in general, and should be two letters only as an exception. The duration of a "dot" should be about 0.5 s, and the duration of a "dash" should not be less than three times the duration of a "dot". | A Morse Code White light with the single character "A" indicates a safe-water mark. A Morse Code Yellow light, but not with either of the single characters "A" or "U"*, indicates a special mark. |
| 9 | FIXED AND FLASHING LIGHT | FFl | A light in which a fixed light is combined with a flashing light of' higher luminous intensity. | This class of light character should be used with care because the fixed component of the light may not be visible at all times over the same distance as the rhythmic component. |  |
| 10 | ALTERNATING LIGHT | AlWR | A light showing different colours alternately. | This class of light character should be used with care, and efforts should be made to ensure that the different colours appear equally visible to an observer. |  |

| APPENDIX RHYTHMIC CHARACTERS OF THE LIGHTS IN THE IALA MARITIME BUOYAGE SYSTEM |  |  |  |
| --- | --- | --- | --- |
|  | Mark | Rhythmic character of the light | Remarks and further recommendations. |
|  | LATERAL | All recommended classes of rhythmic character , but a composite group flashing light with a group of (2 + 1) flashes is solely assigned to modified lateral marks that indicate preferred channels. | Only the colours Red and Green are used. |
|  | Modified lateral (preferred channel) | Composite group flashing light with a group of (2 + 1) flashes, in a period of not more than 16 s | The duration of the eclipse after the single flash should not be less than three times the duration of the eclipse after the group of two flashes. |
|  | CARDINAL |  | Only the colour White is used. |
|  | North cardinal | (a) Continuous very quick light. (b) Continuous quick light. |  |
|  | East cardinal | (a) Group very quick light with a group of three flashes, in a period of 5 s. (b) Group quick light with a group of three flashes, in a period of 10 s. |  |
|  | South cardinal | (a) Group very quick light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 10 s. (b) Group quick light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 15 s. | The duration of the eclipse immediately preceding a long flash should be equal to the duration of the eclipses between the flashes at the very quick rate. The duration of a long flash should not be greater than the duration of the eclipse immediately following the long flash. The duration of the eclipse immediately preceding a long flash should be equal to the duration of the eclipses between the flashes at the quick rate. The duration of a long flash should not be greater than the duration of the eclipse immediately following the long flash. |
|  | West cardinal | (a) Group very quick light with a group of nine flashes, in a period of 10 s. (b) Group quick light with a group of nine flashes, in a period of 15 s. |  |
|  | ISOLATED DANGER | (a) Group-flashing light with a group of two flashes, in a period of 5 s. (b) Group-flashing light with a group of two flashes, in a period of 10 s. | Only the colour White is used. The duration of a flash together with the duration of the eclipse within the group should be not less than 1 s and not more than 1.5 s. The duration of a flash together with the duration of the eclipse within the group should be not less than 2 s and not more than 3 s. |
|  | SAFE-WATER | (a) Long-flashing light with a period of 10 s. (b) Isophase light. (c) Single-occulting light. (d) Morse Code light with the single character "A". | Only the colour White is used. |
|  | SPECIAL | (a) Group-occulting light. (b) Single-flashing light, but not a long-flashing light with a period of 10 s. (c) Group-flashing light with a group of four, five or (exceptionally) six flashes. (d) Composite group-flashing light. (e) Morse Code light, but not with either of the single characters "A" or “U”`. | Only the colour Yellow is used. A group-flashing light with a group of five flashes at a rate of 30 flashes per minute, in a period of 20 s, is assigned to Ocean Data Acquisition Systems (ODAS) buoys. |

#### Maximum Periods for Light Characters

Refer to IALA publications: - Recommendation for the rhythmic characters of lights on aids to navigation (E110), May 1998; - Recommendation for the calculation of the effective intensity of a rhythmic light, November 1980.

Table 3.3 is an extract of the recommended maximum periods for rhythmic characters of lights.

Table 3.3 Maximum period for rhythmic characters

of aids to navigation lights.

| Character Class | Maximum period (seconds) |
| --- | --- |
| Isophase light | 12 |
| Single-occulting light | 15 |
| Single-flashing light | 15 |
| Group very quick light | 15 |
| Interrupted very quick light | 15 |
| Interrupted ultra quick light | 15 |
| Group-occulting light | 20 |
| Long-flashing light | 20 |
| Group-flashing lights of two flashes | 20 |
| Group-quick light | 20 |
| Interrupted quick light | 20 |
| Group-occulting light of three or more eclipses | 30 |
| Group-flashing light of three or more flashes | 30 |
| Composite group-flashing light | 30 |
| Morse code light | 30 |

### Timing of Astronomical Events

The description of a lighted Aid to Navigation emphasises the night time operations but daytime role is often as important.  The astronomical events that define the transitions from day to night are shown in Table 3.4..

Table 3.4 Timing of Astronomical Events.

| Event | Condition | Typical Illumination Lux | Comment (Assuming the absence of moonlight, artificial lighting or adverse atmospheric conditions) |
| --- | --- | --- | --- |
| Sunset/Sunrise | Upper edge of the sun’s disc is coincident with the horizon. | 600 |  |
| Civil Twilight (beginning / ending) | Centre of the sun is at a depression angle of six (6) degrees below the horizon. | 6 | Illumination is sufficient for large objects to be seen but no detail is discernible. The brightest stars and planets can be seen For navigation at sea, the sea horizon is clearly defined. |
| Nautical Twilight (beginning / ending) | Centre of the sun is at a depression angle of twelve (12) degrees below the horizon. | 0.06 | It is dark for normal practical purposes. For navigation at sea, the sea horizon is not normally visible. |
| Astronomical Twilight (beginning / ending) | Centre of the sun is at a depression angle of eighteen (18) degrees below the horizon. | 0.0006 | Illumination due to scattered light from the sun is less than that from starlight and other natural light sources in the sky. |

### Night Operations

Refer to IALA publication: - Recommendation for the notation of luminous intensity and range of lights, November 1966.

#### Nominal Range and Luminous Intensity

Table 3.5 is an extract of the IALA recommendation for the notation of luminous intensity and range of lights and provides a conversion between nominal range and luminous intensity.

Table 3.5 IALA conversion table for Luminous Intensity and Nominal Range for night observations .  This assumes an atmospheric transmissivity of T=0.74 and a threshold of illumination of 0.2 microlux.

| Nominal Range (nautical miles) | Luminous Intensity (candela) | Nominal Range (nautical miles) | Luminous Intensity (candela) |
| --- | --- | --- | --- |
| 1 | 0.9 | 12 | 3600 |
| 1.5 | 2.4 | 13 | 5700 |
| 2 | 5 | 14 | 8900 |
| 2.5 | 9 | 15 | 14000 |
| 3 | 15 | 16 | 21000 |
| 3.5 | 24 | 17 | 32000 |
| 4 | 36 | 18 | 49000 |
| 4.5 | 53 | 19 | 73000 |
| 5 | 77 | 20 | 110000 |
| 6 | 150 | 21 | 160000 |
| 7 | 270 | 22 | 240000 |
| 8 | 480 | 23 | 360000 |
| 9 | 820 | 24 | 520000 |
| 10 | 1400 | 25 | 770000 |
| 11 | 2200 | 26 | 1100000 |

#### Switch-on / switch off times

For lit (lighted) aids to navigation that only operate at night, the switch-on / switch-off times can be regulated by either time switches or photo-sensitive devices that are calibrated to correspond to a nominated light intensity.

IALA is reviewing switch-on/switch-off times with a view to issuing a Recommendation on the topic. (EDITORIAL NOTE: DETAILS OF THE RECOMMENDATION WILL BE AVAILABLE BEFORE THE NAVGUIDE IS RE-ISSUED)

#### Background Lighting

Nominal range at night is calculated with no allowance for glare from background lighting.  Excessive background lighting, from street lights, neon signs etc., frequently makes an aid to navigation light less effective and, in some cases, it becomes completely lost in the general background clutter.

The light can be made more conspicuous by increasing its intensity, changing its colour or by varying its rhythm.

#### Glare

Glare can be caused by bright lights emitted from the shore, such as car headlights, or from another vessel indiscreetly using a search-light.  An aid to navigation light can also cause glare if it is too bright for the shortest viewing distance, especially when the focal plane of the light and the observer's eye are at the same height.  This situation can arise with two station leading lines.

For aids to navigation lights it is generally accepted that the illuminance at the eye of the navigator from the light: - should not exceed 0.1 lux, and; - should be reduced to 0.01 lux if the background is very dark . Refer to IALA publications: - Recommendation for leading lights (E112), May 1998; - Guidelines for the design of leading lines, December 2001.

In situations where glare is a problem, one or more of the following alterations may lead to a satisfactory result:

- raise the focal plane of the light so that the mariner uses the loom of the light or a less intense part of the vertical distribution of the light;

- reduce the intensity of the light by;

  - reducing the illuminance of the light source;

  - reducing the size of the optic;

  - masking the optic with, for example, perforated metal sheet;

- screen unnecessary arcs of the light;

- use two or more lower intensity lights instead of one higher intensity light.

Whatever methods are used, it will be necessary to measure or calculate the intensity and distribution of the modified light or lighting system.

#### Intensity Losses through Glazing

Some lighting equipment has to be installed inside a protective lantern housing.  Unless it is practicable to measure the luminous intensity of the complete installation, it is normal practice to apply a de-rating factor to the intensity of the lighting equipment to allow for the reflection and transmission losses at the lantern glazing.  Generally referred to as the glazing loss factor.

IALA recommends that, in the absence of more definitive information, the glazing loss factor be taken as 0.85 for a system in clean condition.

#### Service Conditions Factor

Under normal operating conditions the luminous intensity of a light is likely to degrade between service (maintenance) intervals.  There are several components to this degradation:

- meteorological conditions (which may only be temporary);

- dirt and salt deposition (which can be minimised by an efficient regular programme of cleaning of the optical system and housing), and;

- progressive deterioration of the light source over the service interval.

It is clearly impossible to represent such a complex array of factors in any simple way, and a proper assessment of the various effects could only be made by measurements on site at regular intervals.  However, in order to give a more realistic figure for the performance of the light under normal operating conditions than when the luminous intensity is measured in a laboratory or on a photometric range, it may be appropriate to apply a service conditions factor to the measured intensity.

IALA recommends that, in the absence of more definitive information, the service condition factor be taken as 0.75.10

### Day Operations

#### Trends

A number of authorities have established daytime lighted leading  lines in major ports and waterways to achieve a more consistent performance than is possible with dayboards.

#### Nominal Daytime Range and Luminous Intensity

IALA has published a Recommendation for a definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day, April 1974.

Table 3.6 is an extract of this  Recommendation and provides a conversion between nominal daytime range and luminous intensity.

Table 3.6 IALA conversion table for Luminous Intensity and Nominal Daytime Range.

| Nominal Daytime Range (nautical miles) | Luminous Intensity (candela) | Nominal Daytime Range (nautical miles) | Luminous Intensity (candela) |
| --- | --- | --- | --- |
| 1 | 4600 | 11 | 11000000 |
| 2 | 25000 | 12 | 18000000 |
| 3 | 75000 | 13 | 28000000 |
| 4 | 182000 | 14 | 45000000 |
| 5 | 383000 | 15 | 69000000 |
| 6 | 745000 | 16 | 105000000 |
| 7 | 1400000 | 17 | 161000000 |
| 8 | 2400000 | 18 | 244000000 |
| 9 | 4100000 | 19 | 367000000 |
| 10 | 6900000 | 20 | 549000000 |

#### Daymarks (Dayboards)

The size of a dayboard should be determined for the maximum useful viewing distance and minimum visibility conditions.  Daymarks used on leading lines are typically rectangular with the long side vertical.  The aspect ratio for the rectangle is commonly 2:1.

The typical operational range of daymarks under different visibility conditions is shown in Table 3.7.

Table 3.7 Typical operational range of daymarks.

| Operational Range of Daymarks (Nautical Miles) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Daymark height (metres). Aspect ratio h=2w |  |  |  |  |
| Minimum visibility (Nautical Miles) | 1.8 | 2.4 | 3.7 | 4.9 | 7.3 |
| 1 | 0.5 | 0.7 | 0.9 | 1.0 | 1.1 |
| 2 | 0.6 | 0.9 | 1.2 | 1.4 | 1.5 |
| 3 | 0.6 | 1.1 | 1.5 | 1.9 | 2.1 |
| 4 | 0.7 | 1.3 | 1.8 | 2.3 | 2.7 |
| 5 | 0.8 | 1.5 | 2.1 | 2.7 | 3.3 |
| 6 | 0.8 | 1.6 | 2.3 | 2.9 | 3.6 |
| 7 | 0.9 | 1.7 | 2.4 | 3.3 | 4.0 |
| 8 | 0.9 | 1.7 | 2.6 | 3.5 | 4.2 |
| 9 | 0.9 | 1.9 | 2.8 | 3.8 | 4.5 |
| 10 | 1.0 | 2.0 | 3.0 | 4.0 | 5.0 |

### Luminous Range Diagram

The Luminous Range Diagram, shown in Fig. 3.8, enables the mariner to determine the approximate range at which a light may be sighted, by night or by day, in the meteorological conditions prevailing at the time, and for various levels of background lighting or sky luminance, respectively.

This Luminous Range Diagram draws on information contained in:

- IALA Recommendation for the notation of luminous intensity and range of lights, November 1966;

- IALA Recommendation for a definition of nominal daytime range of maritime signal lights Intended for the guidance of shipping by day, April 1974.

Fig 3.8 Luminous Range Diagram The top scale on the graph gives the equivalent intensity values in candelas for both day and night operation, and should be used when only the intensity of a light is known. It must be remembered that : - The atmosphere transmissivity is not necessarily consistent between the observer and the light. - No allowance is made for range limitations imposed by the height-of-eye of the observer or the elevation of the light. A separate check should be made using the geographic range table to confirm that the light will be visible at the estimated luminous range. - Glare from background lighting will reduce considerably the range at which lights are sighted at night.

### Using the Luminous Range Diagram

Example 1

There are some countries that publish the range of lights to a different meteorological visibility than the 10 nautical miles meteorological visibility used in the definition of nominal range.  If the meteorological visibility value is known, the quoted range of a light can be converted to a nominal range using the graph as follows:

- Locate the quoted range on the left hand axis;

- Run a horizontal line across to the intercept with the curve for the stated meteorological visibility;

- Then move vertically down to the bottom scale to read the equivalent nominal range;

- For example:  A light with a published range of 24 nautical miles with a meteorological visibility of 20 nautical miles has a nominal range of 15 nautical miles.

Example 2

To determine the luminous range of a light after making an estimate of the prevailing meteorological conditions:

- Locate the nominal range on the bottom axis;

- Then move vertically to the intercept with the curve for the estimated meteorological visibility;

- Read off the luminous range on the left hand scale;

- For example:  A light with a nominal range of 9 nautical miles will have a range of 6 nautical miles when the visibility is 5 nautical miles.

Example 3

To determine the luminous range at night when the light is affected by glare from background lighting.

- Estimate the level of background lighting between none and substantial;

- Select the adjustment interval from the right hand auxiliary scale on the graph;

- Subtract this interval from the nominal range of the light;

- Then move vertically to the intercept with the curve for the estimated meteorological visibility;

- Read off the luminous range on the left hand scale;

- For example: If there is substantial background lighting from a port facility or large urban development, an 18 mile light must first be corrected to 8 miles before the luminous range can be determined for the prevailing meteorological conditions.

Example 4

To determine the luminous range of a light intended for day-time use under prevailing day-time luminance conditions.

- Estimate the day sky luminance between glaring cloud and dark overcast. The nominal condition for day-time use is bright cloud or a clear sky (looking away from the sun);

- Select the adjustment interval from the nominal condition from the left hand auxiliary scale on the graph;

- Add or Subtract this interval from the nominal range of the light;

- Then move vertically to the intercept with the curve for the estimated meteorological visibility;

- Read off the luminous range on the left hand scale;

- For example: When a light with a 5 nautical mile nominal range by day is viewed against an overcast sky, the nominal range can be increased to around 8.8 nautical miles before determining the luminous range for the prevailing meteorological conditions;

- If a light with a 5 nautical mile nominal range by day is viewed against very bright glaring cloud the nominal range would be reduced to 3 nautical miles before determining the luminous range for the prevailing meteorological conditions.

Example 5

The graph can also be used to show:

- Using the top and bottom scales that a light with an intensity of 100,000 candelas has a nominal range of a little over 20 nautical miles;

- If the same light was sighted at 12 nautical miles it would imply that the meteorological visibility was about 5 nautical miles;

- The same light would have a nominal day-time range of a little over 3 nautical miles.

## FIXED AIDS TO NAVIGATION

### Description

The IALA  International Dictionary of Aids to Marine Navigation defines a beacon as “a fixed artificial navigation mark” that can be recognised by its shape, colour, pattern, topmark or light character, or a combination of these. While this functional definition includes lighthouses and other fixed aids to navigation, the terms lighthouse and beacon are used more specifically to indicate importance and size.

#### Lighthouses

A lighthouse is generally considered to be:

- A large conspicuous structure (visual mark) on land, close to the shoreline or in the water;

  - that acts as a daymark, and;

  - provides a platform generally for higher range marine signalling lights.

  - other aids to navigation or audible signals on or near the lighthouse;

- A manned or automated facility.

  - the former is becoming less common;

  - an automated lighthouse will often be remotely monitored and in some cases remotely controlled.

#### Beacons

A beacon is usually considered to be a small fixed visual mark on land or in the water.  Visual characteristics are often defined by daymarks, topmarks, and by numbers.  A marine signalling light, if fitted, would generally  be of a lower range than lighthouses.

In navigable channels a pile beacon may be used as an alternative to a buoy.

#### Purpose of Lighthouses and Beacons

A lighthouse or beacon may perform one or more of the following navigational functions:

- mark a landfall position;

- mark an obstruction or a danger;

- indicate the lateral limits of a channel or navigable waterway;

- indicate a turning point or a junction in a waterway;

- mark the entrance of a Traffic Separation Scheme (TSS);

- form part of a leading (range) line;

- mark an area;

- provide a reference for mariners to take a bearing or line of position (LOP).

However it is not uncommon for lighthouses, in particular, to be used for other purposes that can include:

    base for AIS equipment;

- coastwatch or coastguard functions;

- VTS functions;

- base for audible (fog) signals;

- collection of meteorological and oceanographic data;

- radio and telecommunication facilities;

- tourist facilities.

### Performance Criteria for Lighthouses and Beacons

The availability of a light is the principal measure of performance determined by IALA.  For lighthouses and beacons IALA recommends the following availability targets:

| Type of Aid | Availability Target |  |
| --- | --- | --- |
| Lighthouses and beacons considered to be of primary navigational significance | Category 1 | at least 99.8% |
| Lighthouses and beacons considered to be of navigational significance | Category 2 | at least 99% |
| Lighthouses and beacons considered to have less navigational significance than either Categories 1 or 2 | Category 3 | at least 97% |
| Daymarks and Topmarks (where fitted) |  | at least 97% |

Note

- The importance of a visual aid to navigation may well change over time.

## Floating Aids to Navigation

### Description

A floating aid to navigation serves a similar purpose to a beacon or lighthouse.  However the floating aid to navigation is normally associated with locations where:

- it would be impractical due to water depth, seabed conditions or cost to establish a fixed aid;

- the hazard shifts over time (eg. sand banks, an unstable wreck, etc.);

- where the aid is at high risk of damage or loss from ice flows or ship impacts and as a consequence is treated as expendable;

- a temporary mark is required.

      - BUOYS

Buoys are defined as minor floating aids and whilst it is normal that they are lit there are instances where no light is installed.

These types of aids to navigation:

- are specifically covered by the IALA Maritime Buoyage System;

- tend to have circular hull forms in range of 1 to 3 m diameter;

- where lights are exhibited they are usually solar or single cycle battery powered, however gas buoys are still in operation;

- due to limited power on the buoy light ranges are restricted, to typically 2 to 5 nautical miles although much higher ranges are used in some applications;

- due to limited power on the buoy additional services are restricted but Racon and AIS units are sometimes deployed in additional to a light.

      - LIGHTVESELS, LIGHTFLOATS AND LARGE NAVIGATIONAL BUOYS (LANBYS)

Lightvessels, lightfloats and lanbys (or LNB) are defined as major floating aids and may carry a racon, sound signal, and in some cases, a radio beacon in addition to the aid to navigation light. A lightvessel may also display a white riding light to signify a vessel at anchor.

These types of aids to navigation:

- generally have high operating costs;

- are only deployed at critical locations;

- are often assigned an availability target that is higher than for a buoy;

- are not specifically covered by the IALA Maritime Buoyage System.

Some lightvessels continue to be manned, but the trend is towards making them operate automatically, often with remote monitoring and control.

Refer also to IALA Recommendation for off-station signals for major floating aids (O104), November 1989.

### IALA Maritime Buoyage System (MBS)

The Maritime Buoyage System represents one of IALA’s major contributions to enhancing the safety of navigation.  As recently as 1976 there were more than thirty buoyage systems in use world wide and conflicting sets of rules applied.  In 1980 Lighthouse Authorities from fifty countries and representatives from nine international organisations reached agreement on the rules for a single system.

#### General Principles and Rules of the MBS System

The content of the General Principles and Rules of the IALA Maritime Buoyage System can be found at Annex A.

### Performance Criteria for Floating Aids

The availability of a floating aid is the principal measure of performance determined by IALA.  The recommended availability targets are as follows:

| Type of Aid (examples only) | Availability Target |  |
| --- | --- | --- |
| Floating aids to navigation that are considered to be of primary navigational significance | Category 1 | at least 99.8% |
| Floating aids to navigation that are considered to be of navigational significance | Category 2 | at least 99% |
| Floating aids to navigation that are considered to be of less navigational significance than Category 1 or 2. | Category 3 | at least 97% |

Note

The availability objective assigned to floating aids to navigation conforming to the IALA Maritime Buoyage System should also apply to the topmark.

### Technical Considerations for Floating Aids to Navigation

#### Cost:

The cost of establishing a floating aid at a given location will generally be less than for a fixed structure.  The cost difference increases with increasing water depth and exposure to wind and wave loads.

In contrast, the maintenance cost of floating aids to navigation tends to be high relative to the capital value.  This has caused many authorities to critically examine the potential for savings through design changes, use of alternative materials and amending maintenance practices, generally with the aim of extending maintenance intervals.

Where an authority operates large numbers of floating aids, it may become practicable to operate a dedicated buoy tender vessel with specialised equipment to minimise buoy change-out times and to improve occupational safety.

#### Floating Aid Design

The process of designing a buoy to meet specific requirements is a specialised task.  It involves, but is not limited to:

- defining the operational performance characteristics;

- defining the equipment, power requirements and power source(s);

- defining the type and capabilities of the vessels that will be used to service the buoy;

- selecting the initial type proportions and mooring for the buoy  (See section 3.6.4.3);

- integration of equipment and power supply;

- consideration of the maintenance requirements;

  - deployment and recovery techniques;

  - protecting equipment from damage;

  - ability to rectify faults without having to lift the buoy;

- determining the buoy response to the wave, wind and current conditions at the site(s);

- design optimisation.

Refer also to: - IALA Maritime Buoyage System; - IALA Maritime Buoyage System Guidelines; - IALA Guidelines on Plastic Buoys; - Report on the IALA Workshop on Large Navigation Buoys and Automatic Lightvessels Floataid 84.

#### Mooring Design

3.6.4.3.1 General

The mooring system for a floating aid to navigation is the sum of the components that keep the aid within a nominated area.  These components have to withstand the forces of wind, wave and current on the floating aid and drag on the mooring line.  Methods for determining the forces are covered in the IALA Recommendation E107.  The basic assumptions made are:

- the chain is tangential to the sea bed under all conditions of current and wind at the site;

- the buoy axis is vertical under the most common conditions of current and wind;

- the ratio of the breaking stress of the chain to the calculated stress is not less than 5 under the most unfavourable conditions of current and wind;

- the reserve buoyancy of the fully equipped floating aid is greater than the combined loads of current and wind under the most unfavourable conditions.

#### 3.6.4.3.2  Swing Radius

The IALA Recommendation on the design of normal moorings (E-107), May 1998 indicates that the maximum swing radius (watch circle radius) of a floating aid is:

rm  = L2 − H2

where:

| rm = | Maximum swing radius (m) |
| --- | --- |
| L = | Length of mooring line (m) |
| H= | Depth of water (m) (This is defined as the maximum depth of water and includes the highest tide level and half the maximum wave height at the particular site.) |

The recommended minimum length of mooring line is:

- Lmin = 2H for depths less than 50 metres;

- Lmin = 1.5H for depths greater than 50 metres;

### IALA References on Floating Aids AND Moorings

#### IALA Publications

- IALA Recommendation on the design of normal moorings (E-107), May 1998; - IALA Practical Notes on the use of mooring chains for floating aids to navigation, June 1989.

#### Conference References

- Development of a 5 year buoy, Canadian Coast Guard, IALA Conference 1998;

- Computer aided buoy and mooring design, US Coast Guard, IALA Conference 1994;

- A large solar buoy to replace light vessels and lanby buoys, Trinity House UK, IALA Conference 1994;

- Conversion of acetylene powered light float to solar operation, Trinity House UK, IALA Conference 1994;

- Evaluation of the visibility of buoys and topmarks, US Coast Guard, IALA Conference 1994;

- Environmental aspects relating to buoys, Seezeichenversuchsfeld, Germany, IALA Conference 1994.

### Positioning of Floating Aids

The charted position of a floating aid defines the nominal (or true) position for the anchor.

With most floating aids there is potential for the mooring anchor to be moved off-station during storms or for positional errors to occur while laying the anchors.

Anchors have traditionally been laid while taking cross bearings and/or horizontal sextant angles from fixed visual marks.  When out of sight of land the process may have relied on radionavigation or radio-positioning aids.  While some authorities may still use these procedures, the use of DGPS position fixing is increasingly seen as the preferred method because of its convenience, accuracy and repeatability.

A buoy tender using DGPS can generally be brought to within 10 metres of the nominal buoy position at the time of releasing the anchor.  If the anchor is allowed to free-fall, its final resting position will depend on the prevailing current, water depth, shape of the anchor and the nature of the seabed.  Controlling the decent of the anchor may serve to improve the positional accuracy of the buoy.

### Markings and Topmarks

#### Markings

Floating aids to navigation are often identified by names, abbreviations of names, letters and/or numbers.  Authorities should ensure that the actual marking is identical to the List of Lights reference and the charted marking.

#### Topmarks

The type, colour and arrangement of topmarks on a buoy are shown in the IALA Maritime Buoyage System, extracts of which are shown in Annex A.

3.6.7.3  Dimensions of Topmarks

- Conical topmarks (for lateral and cardinal marks):

  - The vertical height of a cone from base to apex should be about 90% of the base diameter;

  - For cardinal marks, the separation distance between cones should be about 50% of the base diameter of the cone;

  - The vertical clear space between the lowest point of the topmark and all other parts of the mark should be at least 35% of the base diameter of the cone;

  - In the case of a buoy, the base diameter should be 25%-30% of the diameter of the buoy at waterline.

- Cylindrical (CAN) topmarks (for lateral marks):

  - The vertical height of a cylinder should be 1 to 1.5 times the base diameter;

  - The vertical clear space between the lowest part of the cylinder and all other parts of the mark should be at least 35% of the diameter of the cylinder;

  - In the case of a buoy, the base diameter of the cylinder should be 25%-30% of the diameter of the buoy at the waterline.

- Spherical topmarks (for isolated danger and safe water marks):

  - In the case of a buoy, the diameter of the sphere(s) should be at least 20% of the diameter of the buoy at the waterline;

  - For isolated danger marks the separation distance between spheres should be about 50% of their diameter;

  - The vertical space between the lowest part of the sphere(s) and all other parts of the mark should be at least 35% of the diameter of the sphere(s).

- 'X' (Diagonal Cross) topmarks (for special marks):

  - In the case of a buoy, the arms of the 'X' should be diagonally contained within a square with length of side of about 33% of the buoy diameter at the waterline. The width of the arms of the 'X' should be about 15% of the length of side of the square.

## Sector Lights and Leading (Range) Lines

Note: Bearings, directions of leading (range) lines and limits of sectors should always be stated in terms of the bearings that would be seen by the mariner. Bearings may carry a suffix ‘TBS” or True Bearing from Seaward as confirmation.

### Sector Lights

A sector light is an aid to navigation that displays different colours and/or rhythms over designated arcs.

A Precision Direction Light  (PDL)is a specialised form of sector light that can generate

sharply defined sector boundaries.  This feature is particularly useful for applications that

require one or several narrow sectors. The PDL may use a white light source with

coloured filter, but newer designs are utilising LED or laser lights.

A common means of creating a sector is to fit a coloured filter in front of the main light.  A sector can also be produced by a subsidiary light on the same structure.  The subsidiary light can take any of the following forms:

- Range (directional) light;

- Beacon with a coloured lens, masked to achieve the sector angle;

- Beacon fitted with internal or external filter panels;

- Precision Direction Light.

The limits or boundaries of a sector are not always precisely cut off due to the characteristics of the light source and fading of colours or changing rhythms between adjacent sectors.

For a beacon fitted with coloured filter panels, the reason for the lack of a precise transition at the sector boundary is readily apparent from Fig 3.10 which shows the light source, lens and filter geometry.  The transition zone is defined by an "angle of uncertainty" (Refer IALA Dictionary 2-6-305).

Fig 3.10 Angle of Uncertainty

It can also be noted that:

- the observed angle of uncertainty is generally less than the geometric angle due to the relative intensities of sector colours (ie. colour mixing) as the observer passes through the transition zone;

- if space on the aid to navigation structure is not a limiting factor, it is usually possible to achieve an angle of uncertainty of around 0.25 with this type of sector arrangement;

- the angle of uncertainty can be reduced by decreasing the physical size of the light source or by increasing the radial distance to the coloured filter;

- in situations where the main light has a large projected area, such as a rotating lens or reflector array, it is generally preferable to use a separate sector light rather than installing a coloured filter in front of the main light.

From time to time specialised sector lights have been developed to exhibit different rhythms over different sector bearings.  This capability is found in some Precision Direction Lights.

PDL Sector Lights are so precise that a complete colour change at a sector boundary occurs over an angle of less than 1 minute of arc in most models.

#### Applications

The design of sector lights can be a complex task.  The process should be carried out with reference to a good quality chart of the area.  In some cases good local knowledge is also required.

A sector light may indicate one or more of the following:

- boundaries of a navigable waterway;

- change of course position;

- shoals, banks, etc.;

- an area or position (eg. an anchorage);

- the deepest part of a waterway;

- position checks for floating aids.

A Precision Direction Light (PDL) allows for further applications that include the ability to:

- produce narrow sectors with an angles of uncertainty down to around one minute of arc;

- define the central zone of a channel;

- accurately mark one side of a straight channel.  (A pair of PDLs can cover the permutations of converging, diverging and parallel channels);

- define different rhythms over adjacent sectors.

#### Examples

Some examples of sector lights applications are illustrated in Figs 3.11 and 3.12.

Figure 3.11

This illustration follows the IALA Maritime Buoyage System colour convention for Region A (‘red to port when approaching the aid from seaward’).  The white sector should, if possible, be wide enough to provide a margin of safety for a vessel that inadvertently leaves the white sector. Curves C and D indicate depth contours or limiting dangers that dictate the boundaries of sectors.

Figure 3.12  Shows several applications for sector lights.

The function of each light is described below:

Light 1 is a coastal white light with a red sector indicating a danger.

Light 2 is a sector light obscured over the shore, with two white sectors indicating a safe channel.  When sailing towards the sector light it shows red to port and green to starboard following the IALA Maritime Buoyage System colour convention for Region A.  The boundary between the red and the green sector also indicates the position of a buoy.

Light 3 is a sector light with a red light and 4 white sectors indicating four anchorage positions.  It is obscured over the shore.

#### Light 4 is a sector light with a white sector indicating a safe channel.

#### Design Considerations for Sector Lights

Where a single sector light defines a navigable channel:

- There is no reference of the vessel’s lateral position within the channel until a sector boundary is reached.  This may cause a problem in channels subject to a strong cross current.  For vessels with local knowledge, the zones defined by the angle of uncertainty can sometimes provide a useful guide to the vessel’s proximity to a sector boundary;

- Where practicable, there should be a margin of safety between the sector boundary and adjacent hazards.  If an appropriate safety margin cannot be achieved within the sector boundary, the hazards could be marked separately.

- Zones defined by the angle of uncertainty should be considered an additional margin of safety over the actual sector boundary;

- The design process for a sector light needs to consider the draught and manoeuvrability of the largest vessels likely to utilise the sector, how quickly they can respond once they cross a sector boundary and the situations that may develop when other vessels are in the vicinity;

- A sector design should take account of the spectral distribution of the light source and the proportion of this light transmitted through the filter material.  The process should also check for potential for glare problems;

- The period of the light flash should be selected to provide ample time for a mariner to recognise the transitional phases that occur at the sector boundary;

- A white light is normally the first preference for a lighthouse or beacon.  If a single coloured sector is added, the preferred colour for the sector is red.

- If a white sector light is used to mark a navigation channel, coloured sectors may be used either side of the white to indicate the lateral limits.  In such cases it is common practice to use red and green sectors that follow the convention of the IALA Maritime Buoyage System;

- Multiple sectors can be used to provide a better indication of a vessel’s lateral position within the channel but at the expense of complexity for both the system designer and navigators.

Refer to IALA publication: - Guideline on sector lights, to be issued in 2004/2005.

### Leading (Range) Lines

A leading line is an aid to navigation system that comprises two separated structures with marks or lights that are aligned when viewed from the centreline or deepest route along a straight section of channel.

In a two station leading line, the structures lie along an extension of the centreline of the nominated channel.  The rear structure must have a greater elevation than the front structure to enable both marks or lights to be viewed simultaneously.

A leading line provides a vessel with a heading reference and a visual indication of the size and direction of any cross track error.

#### Purposes of Leading Lines

A leading line may be used to:

- indicate the centreline of a straight section of a navigable channel;

- indicate to deep draught vessels the deepest part of the waterway;

- indicate the navigable channel where fixed and floating aids to navigation are not available or do not satisfy the accuracy requirements for safe navigation;

- define a safe approach bearing to a harbour or river entrance, particularly where there are cross currents;

- separate two-way traffic (ie. when passing a bridge)

#### Design Considerations for Leading Lines

A well-designed leading line will enable vessels of the type and size that commonly use the channel to :

- Identify the marks or lights when the ship is at the inner and outer sections of the channel and readily detect cross track position errors from the centreline of the channel;

- Detect cross track position errors with sufficient sensitivity that the channel can be utilised without abrupt changes to the vessels heading and speed;

- Observe both lights together, by selection of leading light character rhythms that appropriately overlap in their free running condition. In some situations it may be preferable to provide additional equipment to synchronise the light characters.

- Observe the lights in all ambient conditions for which they are designed to be used without glare, if lights are to be used for both day and night operations light intensities will need to be varied.

- The characters of rhythmic leading lights should be selected so that the front and rear lights, in their free running states, can generally be observed together.  In some situations it may be preferable to provide additional equipment to synchronise the light characters;

- If lights are to be used both day and night, the light intensities should be adapted for each situation to avoid glare at night.

Leading Lines (or lights) should conform to the following IALA Recommendations: - Recommendation for leading lights (E112), May 1998; - Guidelines for the design of leading lines, December 2001; - Recommendation for a definition of nominal daytime range of maritime signal lights intended for guidance of shipping by day, April 1974.

ANNEX A – IALA MARITIME BUOYAGE SYSTEM

Within the IALA Buoyage System there are 5 types of marks that may be used in combination.  The mariner can easily distinguish between these marks by readily identifiable characteristics.

Lateral marks differ between Buoyage Regions A and B, as described below, whereas the other four (4) types of marks are common to both regions.

LATERAL MARKS

Following the sense of a “conventional direction of buoyage”, Lateral marks in Region A utilize red and green colours by day and night to denote the port and starboard sides of channels respectively.  However, in Region B these colours are reversed with red to starboard and green to port.

A modified Lateral mark may be used at the point where a channel divides to distinguish the preferred channel, that is to say the primary route or channel that is so designated by an Authority.

CARDINAL MARKS

Cardinal marks indicate that the deepest water in the area lies to the named side of the mark.  This convention is necessary even though for example, a North mark may have navigable water not only to the North but also East and West of it.  The mariner will know he is safe to the North, but must consult his chart for further guidance.

Cardinal marks do not have a distinctive shape but are normally pillar or spar.  They are always painted in yellow and black horizontal bands and their distinctive double cone topmarks are always black.

An aide-memoire to their colouring is provided by regarding the topmarks as pointers to the positions of the black band(s):

- Topmarks pointing upward: black band above yellow band

- Topmarks pointing downward: black band below yellow band

- Topmarks pointing away from each other: black bands above and below a yellow band

- Topmarks pointing towards each other: black band with yellow bands above and below.

Cardinal marks also have a special system of flashing white lights.  The rhythms are basically all “very quick” (VQ) or “quick” (Q) flashing but broken into varying lengths of the flashing phase.

- “very quick flashing” is defined as a light flashing at a rate of either 120 or 100 flashes per minute,

- “quick flashing” is a light flashing at either 60 or 50 flashes per minute.

The characters used for Cardinal marks will be seen to be as follows:

- North: Continuous very quick flashing or quick flashing;

- East: Three “very quick” or “quick” flashes followed by darkness;

- South: Six “very quick” or “quick” flashes followed immediately by a long flash, then darkness;

- West: Nine “very quick” or “quick” flashes followed by darkness.

The concept of three, six, nine is easily remembered when one associates it with a clock face.  The long flash, defined as a light appearance of not less than 2 seconds, is merely a device to ensure that three or nine “very quick” or “quick” flashes cannot be mistaken for six.

It will be observed that two other marks use white lights.  Each has a distinctive light rhythm that cannot be confused with the very quick or quick flashing light of the Cardinal marks.

ISOLATED DANGER MARK

The Isolated Danger mark is placed on a danger of small area that has navigable water all around it.  Distinctive double black spherical topmarks and Group flashing (2) white lights, serve to associate Isolated Danger marks with Cardinal marks.

SAFE WATER MARKS

The Safe Water mark has navigable water all around it, but does not mark a danger.  Safe Water marks can be used, for example, as mid-channel or landfall marks.

Safe Water marks have an appearance quite different from danger marking buoys. They are spherical, or alternatively pillar or spar with a single red spherical topmark.  They are the only type of mark to have vertical stripes (red and white).  Their lights, if any, are white using isophase, occulting, one long flash or Morse “A” rhythms.

SPECIAL MARKS

Special marks are not primarily intended to assist navigation but are used to indicate a special area or feature whose nature may be apparent from reference to a chart or other nautical document.

Special marks are yellow.  They may carry a yellow “X” topmark, and any light used is also yellow.  To avoid the possibility of confusion between yellow and white in poor visibility, the yellow lights of Special marks do not have any of the rhythms used for white lights.  Their shape will not conflict with that of navigational marks, this means, for example, that a special buoy located on the port hand side of a channel may be cylindrical, but will not be conical.  Special marks may also be lettered or numbered to indicate their purpose.

NEW DANGERS

It should be specially noted that a “new danger” which is one not yet shown in nautical documents may be indicated by exactly duplicating the normal mark until the information is sufficiently promulgated.

A “new danger” mark may carry a Racon coded Morse “D”.

RULES OF THE MARITIME BUOYAGE SYSTEM (MBS)

- General

  - Scope

The Maritime Buoyage System provides rules that apply to all fixed and floating marks (other than lighthouses, sector lights, leading lights and marks, lightships and large navigational buoys) serving to indicate:

    - The lateral limits of navigable channels.

    - Natural dangers and other obstructions such as wrecks.

    - Other areas or features of importance to the mariner.

    - New dangers.

  - Types of marks

The Maritime Buoyage System provides five types of marks that may be used in combination:

    - Lateral marks, used in conjunction with a “conventional direction of buoyage”, generally used for well defined channels.  These marks indicate the port and starboard sides of the route to be followed.  Where a channel divides, a modified lateral mark may be used to indicate the preferred route.  Lateral marks differ between Buoyage Regions A and B as described in MBS Sections 2 and 8.

    - Cardinal marks, used in conjunction with the mariner's compass, to indicate where the mariner may find navigable water.

    - Isolated Danger marks to indicate isolated dangers of limited size that have navigable water all around them.

    - Safe Water marks to indicate that there is navigable water all around their position, e.g. mid-channel marks.

    - Special marks, not primarily intended to assist navigation but to indicate an area or feature referred to in nautical documents.

  - Method of characterising marks

The significance of the mark depends upon one or more of the following features:

    - By night, colour and rhythm of light.

    - By day, colour, shape, topmark.

- LATERAL MARKS

  - Definition of “conventional direction of buoyage”

The “conventional direction of buoyage”, which must be indicated in appropriate nautical documents, may be either:

    - The general direction taken by the mariner when approaching a harbour, river, estuary or other waterway from seaward, or

    - The direction determined by the proper authority in consultation, where appropriate, with neighbouring countries.  In principle it should follow a clockwise direction around land masses.

  - Buoyage Regions

There are two international Buoyage Regions A and B where lateral marks differ. These buoyage regions are indicated in Section 8.

  - Description of Lateral Marks used in Region A

|  | 2.3.1. Port hand Marks | 2.3.2. Starboard hand Marks |
| --- | --- | --- |
| Colour | Red | Green |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single red cylinder (can) | Single green cone, point upward |
| Light (when fitted) |  |  |
| Colour | Red | Green |
| Rhythm | Any, other than that described in section 2.3.3. | Any, other than that described in section 2.3.3. |

    - At the point where a channel divides, when proceeding in the “conventional direction of buoyage”, a preferred channel may be indicated by a modified Port or Starboard lateral mark as follows:

|  | 2.3.3.1. Preferred channel to Starboard | 2.3.3.2. Preferred channel to Port |
| --- | --- | --- |
| Colour | Red with one broad green horizontal | Green with one broad red horizontal |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single red cylinder (can) | Single green cone, point upward |
| Light (when fitted) |  |  |
| Colour | Red | Green |
| Rhythm | Composite group flashing (2 + 1) | Composite group flashing (2 + 1) |

  - Description of Lateral Marks used in Region B

|  | 2.4.1. Port hand Marks | 2.4.2. Starboard hand Marks |
| --- | --- | --- |
| Colour | Green | Red |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single green cylinder (can) | Single red cone, point upward |
| Light (when fitted) |  |  |
| Colour | Green | Red |
| Rhythm | Any, other than that described in section 2.4.3. | Any, other than that described in section 2.4.3. |

    - At the point where a channel divides, when proceeding in the “conventional direction of buoyage”, a preferred channel may be indicated by a modified Port or Starboard lateral mark as follows:

|  | 2.4.3.1. Preferred channel to Starboard | 2.4.3.2. Preferred channel to Port |
| --- | --- | --- |
| Colour | Green with one broad red horizontal | Red with one broad green horizontal |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single green cylinder (can) | Single red cone, point upward |
| Light (when fitted) |  |  |
| Colour | Green | Red |
| Rhythm | Composite group flashing (2 + 1) | Composite group flashing (2 + 1) |

  - General Rules for Lateral Marks

    - Shapes

Where lateral marks do not rely upon cylindrical (can) or conical buoy shapes for identification they should, where practicable, carry the appropriate topmark.

    - Numbering or lettering

If marks at the sides of a channel are numbered or lettered, the numbering or lettering shall follow the “conventional direction of buoyage”.

- CARDINAL MARKS

  - Definition of Cardinal quadrants and marks

    - The four quadrants (North, East, South and West) are bounded by the true bearings NW-NE, NE-SE, SE-SW, SW-NW, taken from the point of interest.

    - A Cardinal mark is named after the quadrant in which it is placed.

    - The name of a Cardinal mark indicates that it should be passed to the named side of the mark.

  - Use of Cardinal Marks

A Cardinal mark may be used, for example:

    - To indicate that the deepest water in that area is on the named side of the mark.

    - To indicate the safe side on which to pass a danger.

    - To draw attention to a feature in a channel such as a bend, a junction, a bifurcation or the end of a shoal.

  - Description of Cardinal Marks

|  | 3.3.1. North Cardinal Mark | 3.3.2. East Cardinal Mark |
| --- | --- | --- |
| Topmark (a) | 2 black cones, one above the other, points upward | 2 black cones, one above the other, base to base |
| Colour | Black above yellow | Black with a single broad horizontal yellow band |
| Shape (Buoys) | Pillar or spar | Pillar or spar |
| Light (when fitted) |  |  |
| Colour | White | White |
| Rhythm | VQ or Q | VQ(3) every 5s or Q(3) every 10s |

|  | 3.3.3. South Cardinal Mark | 3.3.4. West Cardinal Mark |
| --- | --- | --- |
| Topmark (a) | 2 black cones, one above the other, points downward | 2 black cones, one above the other, point to point |
| Colour | Yellow above black | Yellow with a single broad horizontal black band |
| Shape (Buoys) | Pillar or spar | Pillar or spar |
| Light (when fitted) |  |  |
| Colour | White | White |
| Rhythm | VQ(6) + Long flash every 10s or Q(6) + Long flash every 15s | VQ(9) every 10s or Q(9) every 15s |

Note: (a) The double cone topmark is a very important feature of every Cardinal mark by day, and should be used wherever practicable and be as large as possible with a clear separation between the cones.

- ISOLATED DANGER MARKS

  - Definition of Isolated Danger Marks

An Isolated Danger mark is a mark erected on, or moored on or above, an isolated danger that has navigable water all around it.

  - Description of Isolated Danger Marks

| Topmark (b) | 2 black spheres, one above the other |
| --- | --- |
| Colour | Black with one or more broad horizontal red bands |
| Shape (Buoys) | Optional, but not conflicting with lateral marks; pillar or spar preferred |
| Light (when fitted) |  |
| Colour | White |
| Rhythm | Group flashing (2) |

Note: (b) The double sphere topmark is a very important feature of every Isolated Danger mark by day, and should be used wherever practicable and be as large as possible with a clear separation between the spheres.

- SAFE WATER MARKS

  - Definition of Safe Water Marks

Safe Water marks serve to indicate that there is navigable water all round the mark; these include centre line marks and mid-channel marks. Such a mark may also be used as an alternative to a Cardinal or a Lateral mark to indicate a landfall.

  - Description of Safe Water Marks

| Colour | Red and white vertical stripes |
| --- | --- |
| Shape (Buoys) | Spherical; pillar or spar with spherical topmark |
| Topmark (if any) | Single red sphere |
| Light (when fitted) |  |
| Colour | White |
| Rhythm | Isophase, occulting, one long flash every 10s or Morse “A” |

- SPECIAL MARKS

  - Definition of Special Marks

Marks not primarily intended to assist navigation but which indicate a special area or feature referred to in appropriate nautical documents, for example:

    - Ocean Data Acquisition Systems (ODAS) marks.

    - Traffic separation marks where use of conventional channel marking may cause confusion.

    - Spoil Ground marks.

    - Military exercise zone marks.

    - Cable or pipeline marks.

    - Recreation zone marks.

  - Description of Special Marks

| Colour | Yellow |
| --- | --- |
| Shape (Buoys) | Optional, but not conflicting with navigational marks |
| Topmark (if any) | Single yellow “X” shape |
| Light (when fitted) |  |
| Colour | Yellow |
| Rhythm | Any, other than those described in sections 3, 4 or 5 |

  - Additional Special Marks

Special marks other than those listed in paragraph 6.1 and described in paragraph 6.2 may be established by the responsible administration to meet exceptional circumstances. These additional marks shall not conflict with navigational marks and shall be promulgated in appropriate nautical documents and the International Association of Lighthouse Authorities notified as soon as practicable.

- NEW DANGERS

  - Definition of New Dangers

The term “New Danger” is used to describe newly discovered hazards not yet indicated in nautical documents. “New Dangers” include naturally occurring obstructions such as sandbanks or rocks or man made dangers such as wrecks.

  - Marking of New Dangers

    - “New Dangers” shall be marked in accordance with these rules. If the appropriate Authority considers the danger to be especially grave at least one of the marks shall be duplicated as soon as practicable.

    - Any lighted mark used for this purpose shall have an appropriate Cardinal or Lateral VQ or Q light character.

    - Any duplicate mark shall be identical to its partner in all respects.

    - A “New Danger” may be marked by a racon, coded Morse “D” showing a signal length of 1 nautical mile on the radar display.

    - The duplicate mark may be removed when the appropriate Authority is satisfied that information concerning the “New Danger” has been sufficiently promulgated.

- INTERNATIONAL BUOYAGE REGIONS A AND B

There are two international Buoyage Regions A and B where Lateral marks differ as described in Section 2. The current geographical divisions of these two Regions are shown on the following world map.

# Power Supplies

## Types

A wide range of power systems and energy sources have been used or contemplated for operating lighthouses and floating aids.  Everything from clockwork to radio-active isotopes have been used.  Some of the more common types are listed in Table 7.1.

Table 7.1 Power sources for operating lighted aids to navigation.

| Electric Power Systems | Non-Electric Energy Sources |
| --- | --- |
| - Primary cells | - Acetylene |
| - Mains (utility) electricity supply | - Propane |
| - Diesel and Petrol engine driven generators | - Butane |
| - Photovoltaic Solar modules |  |
| - Wind generators |  |
| - Wave activated generators |  |

### IALA Survey Data on Power Supplies

#### IALA Annual Questionnaire

IALA carry out a questionnaire survey of all IALA Members annually. [insert here what is done with this information and where the information can be accessed]

There is a general trend away from gas, using mains utility electricity where available and photovoltaic solar power where mains is not available.

## Non-Electric

### Acetylene

Acetylene (C2H2) has been used to operate lights on buoys and unattended aids to navigation for many years.  Acetylene can explode if compressed directly, but can be safely contained under low pressure in special cylinders when dissolved in acetone.

The manufacture of acetylene, standards for the cylinders and the filling process are usually controlled by government regulations.

Acetylene has been a convenient and reliable energy source for aids to navigation. However appropriate attention should be given to:

- safe handling of cylinders;

- the broad range of explosive mixtures with air (between 3 and 82% acetylene).

- the purity off the gas, and;

- minimizing leaks in pipe work and fittings;

### Propane

Propane gas (CH3 CH2 CH3) has been used as an alternative fuel to acetylene, particularly in buoys.  Although propane has to be consumed in an incandescent mantle burner to provide a white light, it has several advantages over acetylene:

- it is a by-product in oil refining processes;

- its abundance and low cost;

- propane liquefies at a pressure of 6 atmospheres at 17ºC., and can be transported in quite light and low cost gas bottles;

- propane will maintain a positive pressure down to -40ºC and will not freeze in conditions likely to be encountered at sea;

- placing the bottles in pockets in the buoy or by filling it directly into the body of an buoy, or pressure vessel;

- the comparable containers are the 20 kg propane bottle with gross weight of 48 kg and the 7,000 litre acetylene cylinder, weighing 105 kg;

- furthermore the cost of the propane bottle is only about one-third of that of a acetylene cylinder;

- propane is a particularly safe gas, as only some 6 % of all its possible mixtures with air are explosive against a figure of 80% for acetylene;

- burns cleanly without the risk of sooting that can occur with a poorly adjusted acetylene burner.

Refer to IALA publication: - Practical Notes for the safe handling of gases, October 1993. Refer also to national standards for the safe handling of gas.

## Electric - Non-Renewable Sources

### Primary Cells

Primary cells provide electrical energy by a non-reversible chemical process.  They were used in large numbers up until the 1980s to operate buoys and automatic beacon lights.

#### Zinc-Air Cell

The zinc-air primary cell was a common energy source for operating buoy and beacon applications.  The cell uses a porous carbon block to supply oxygen from the air through an alkaline electrolyte to oxidize a zinc anode.  Individual primary cells have an open circuit voltage of about 1.2 volts and can supply 1000 to 2000 Ah at a maximum rate of about 1 ampere.

#### Others

Add other primary cell types as advised by EEP members.

#### Lithium-Thionyl Chloride Cell

Another type of primary cell in use in buoy applications is the lithium-thionyl chloride cell.  This has a higher energy density and a longer shelf life than the zinc-air cell.

The usage of primary cells has declined sharply since commercial solar power (photovoltaic) modules have become available.  A related issue that hastened the decline of primary cells was the tightening environmental standards in a number of countries that required cells to be recovered from site for disposal in an approved manner.  Disposal compliance costs, and occupational health and safety aspects of the frequent change-out of primary cells have worked in favour of converting to renewable energy sources (eg. solar, wind and wave generators).

#### Sea-Water Cells

The sea-water cell developed for buoy applications in Norway is a primary cell that uses a magnesium anode and a largely inert copper cathode.  The sea water acts both as an electrolyte and the provider of dissolved oxygen for the cathode.

A single cell is installed as part of the buoy tail-tube.  The motion of the buoy has a beneficial effect in agitating the water to provide an oxygen-rich flow through the cell and carry away the reaction products.

Copper was selected for the cathode material because of its inherent antifouling properties.  A magnesium anode was considered environmentally acceptable because it is a naturally occurring element of sea water.  The cell produces a voltage of 0.8 to 1 volt under load.  Components of the cells under evaluated have been sized to provide around 35,000 watt hours of energy.

Since it is impractical to use more than one cell due to the current leakage that would occur, a dc-dc converter is used to raise the voltage to the level required by the load.

### Internal Combustion Engine/Generators

#### Diesel Generators

Diesel engine driven generators are often used as the primary source of electrical power where the location of a lighthouse is too remote to be supplied from a mains electricity grid.  Diesel generators are also used to provide emergency or backup power.

The generator capacity to support the operational and domestic loads of a lighthouse would be in the range of 10 to 30 kW.  Diesel generators of this size could be expected to consume around 0.4 litres/kWh.

The requirement for diesel generators in lighthouses is decreasing as a result of:

- lighthouse automation (destaffing), and because;

- new beacon and lamp technology that enables a light with a nominal range 18 to 20 nautical mile to be operated from a renewable energy source.

#### Petrol-engine Generators

Petrol-engine generators are a useful source of power for maintenance work, but are less common in permanent installations due to:

- fuel and fuel storage safety issues;

- maintenance requirements on the spark-ignition system, and;

- the petrol engine generally being regarded as less durable than a diesel.

### Other

Some of the less common power generating equipment considered for aids to navigation applications are mentioned below.

#### Thermo-electric generator

- This is a solid-state generator in which a heat source, commonly from a propane burner, is directed onto a thermopile (ie. an array of thermocouple type elements).  Since each thermocouple only produces a voltage of around 0.5 volts, a number are connected in series;

- This type of generator has a low thermal efficiency (around 5%) and is rarely used.

#### Stirling engine driven generator

- The Stirling engine is an external combustion engine that can be operated on gas or diesel fuel.  Packaged generator sets are available that could operate a lighthouse.  Typical generators produce 1kW electrical output and 5kW of heat.  The heat output could be a useful by-product for building conditioning in a lighthouse.

#### Fuel cell

- This is a solid-state device that uses a catalytic process to oxidise fuel to generate an electrical current. A common fuel is hydrogen, or hydrogen rich fuels.  It can be thought of as a continuously fed battery.

- The commercial fuel cell is still an emerging technology and at this stage is an expensive power source.  Aids to navigation applications are likely to be limited to situations where solar energy (photovoltaic) is impractical due to limited insolation or icing conditions.

## Electric - Renewable Energy Sources

### Solar Power (Photovoltaic cell)

#### Aids to Navigation Applications

Solar power is an ideal power source for many aids to navigation applications.  It offers:

- a power source with no moving parts;

- no maintenance requirements other than being cleaned;

- negligible deterioration in power output over its life;

- low life-cycle costs;

When used to power a light, the battery recharging process is separated from the operation of the light so that the recharge voltage can be optimised without detriment to lamp life.

The potential difficulties associated with solar power are:

- finding ways to minimise bird fouling;

  - mounting solar modules vertically is probably the best long-term solution for buoys;

- sizing arrays to operate at high latitudes;

- protecting solar modules from;

  - wave damage on buoys;

  - vandalism and theft

  - lightning.

Aids to navigation exposed to icing conditions are perhaps the only applications unsuited to the use of solar modules.

Refer IALA publications: - Specification for solar voltaic systems, June 1988; - Practical note on the application of solar power systems for aids to navigation, September 1993; - Guidelines for renewable energy sources for marine aids to navigation – solar photovoltaic systems, October 1997; IALA is studying the feasibility of developing an IALA Computer program for sizing solar power supplies.

#### Types

There are three common technologies employed in the manufacture of silicon based, solar modules and are listed in Table 7.3 :

Table 7.3 Silicon solar cell technology.

| Type (Technology) | Comments |
| --- | --- |
| Monocrystaline cells | Are made from a thin slice cut from a single large crystal of silicon, usually produced as a circular section rod. Generally have the highest efficiency of the three technologies, If circular wafers of silicon are used the module fill factor is significantly less than with polycrystalline cells. It is now usual for the cells to be trimmed to approximate a square. |
| Polycrystaline cells | Are made from a thin slice cut from a large cast billet of silicon comprising many crystals Are slightly less efficient than the mono crystalline cell but they can be shaped to completely fill the module. |
| Amorphous module | Are made by depositing thin films of silicon directly onto a glass or stainless steel substrate a thin slice cut from a single large crystal of silicon. The cell has a lower efficiency than either of other technologies but can be multi-layered for enhanced performance. Problems have been found with lifetime of these cells. |

In addition to the silicon cell technologies, there are two optional module configurations based on the numbers of series connected cells.  The standard module normally has 36 cells in series to give an open circuit voltage of around 20 volts.  For 12 volt battery charging applications, a voltage (charge) regulator is considered essential.

A self-regulating solar module was promoted as a means of eliminating the voltage regulator that had frequently been found to be the least reliable component in a solar power supply.  The self-regulating module typically has 32 cells in series to give an open circuit voltage of around 18 volts (and a maximum on load voltage of around 15 volts). In a self-regulating solar module, the charging rate is determined by the interaction between the electrical characteristics of the battery and solar module.

- Both concepts can be made to work well.

#### Module or Array Orientation

In the northern hemisphere, solar modules are normally installed facing south and inclined at an angle to the horizontal that is related to the latitude of the site.

The inclination angle for solar modules is often optimised for the particular site as part of the sizing calculations.  Confirm that the table is shown in the solar specification.

One of the main problems experienced with solar powered aids to navigation has been bird fouling.  Numerous ingenious solutions have been trialed, generally with mixed results.  Generally solar modules mounted at an angle or vertically benefit from self washing from rain.

The cost of additional solar modules needed for a vertical installation may be largely off-set by the savings that result from simplifying the mounting arrangements or framework.

### Wind Energy

#### Aids to Navigation Applications

Wind generators (or wind turbines) have been used by a number of IALA Members to power aids to navigation.  The most popular type were horizontal axis machines with a two bladed (propeller type) turbine. The maintenance requirements arising from the moving parts design of wind generators and susceptibility to storm damage has limited the use of wind generators.

Refer IALA publication: - Specification for medium size wind generator systems, June 1988.

#### Installations

Wind generator installations at aids to navigation sites pose a number of problems:

- wind generators tend to require a lot of maintenance if operated in turbulent air flows:

- if the wind generator is installed on a separate mast some distance from the lighthouse, consideration has to be given to cable voltage drop:

- the size of wind generator likely to be considered for operating aids to navigation is at considerable risk of damage if there are populations of birds at the site.

#### Wind Generator Types

A comparison of the typical performance of different types of wind generators is shown in Fig 7.2.

Fig 7.2

### Wave Energy

#### Wave Activated Generator

The wave activated generator (WAG) was developed in Japan and has been successfully used to power lighted buoys.  The interaction between the buoy and wave motions acts as a simple air pump that is used to drive an air turbine and generator.  The WAG is mounted on an extension of a hollow tail tube that passes through the buoy hull.  With wave heights of 0.5 metres, the power output is almost 100 watts.  WAG’s have limited life and current systems suffer from excessive wear.

Site conditions will determine the rate at which the tail tube of the buoy accumulates weed and other forms of fouling, and these aspects need to be taken into consideration when developing the maintenance regime for the WAG. WAG’s can also be very susceptible to storm damage.

## rechargable Batteries

### IALABATT 1 to 4

In recognition of the importance of battery technology in the operation of the modern aids to navigation, IALA has held four workshops on battery related topics for the benefit of members and manufacturers.  The first was held in Bulgaria in 1987 and subsequently in France in 1993 &1997, and combined with IALALITE in Germany, in 2001.  Reports on the proceedings are available as IALA Publications.

### Principal types

There are two main types of storage battery technologies applied to aids to navigation:

- lead acid;

- nickel cadmium.

- Are there other types ???

The lead acid type is generally preferred because of its lower cost and higher energy exchange efficiency (95% vs. 80%) than the nickel cadmium battery.  However, the nickel cadmium battery can operate in lower temperatures and for a greater number of deep discharge cycles.

### Lead Acid

The basic form of this battery uses a lead dioxide positive plate and a lead immersed in an electrolyte of dilute sulphuric acid.  These were originally wet or flooded cells.  However in recent years various forms of “sealed” cell batteries have become available and are quite common in aids to navigation applications.

#### Flooded Lead-Acid Cells

There are three main types of flooded lead acid cell in general use:

- Plante Cells:- that have a pure lead positive plate with a large surface area, in which active material is formed from the lead of the plate itself.

- Tubular Plate Cells:-that use a tubular plate for the positive electrode.  The active material is contained in permeable insulating tubes through which the electrolyte can diffuse.

- Pasted Plate Cells:- in which the positive electrode is a pasted plate consisting of active material formed from oxides or salts of lead pressed into a lead alloy plate of grid form.

#### Valve Regulated Lead-Acid Cell (VRLA)

The VRLA comes in two types:

- AGM:- (absorbed glass-mat) that use a micro glass separator system to absorb the electrolyte.

- Gel Batteries:- that use a gelified electrolyte and polymeric separators to prevent short circuits between the positive and negative plates.

Both types have no “free electrolyte” and the batteries theoretically can be used in any position without acid leakage.  It is recommended that feature be confirmed before system designs are finalised.

The batteries use an oxygen recombination process so that normally there is no gassing. However, a safety valve is provided to release any excess pressure.

Operational characteristics of the battery include:

- a 5-10 year life on float-charge applications;

- tolerance to deep discharge cycles (ie. 80%);

- gradual loss of capacity in cycling applications compared with a flooded cell.

### Nickel Alkaline Battery

These batteries use compounds of nickel and, generally, cadmium with a solution of potassium hydroxide as the electrolyte.

Nickel-cadmium cells use perforated steel plates that hold the active material, mainly a nickel hydroxide in the positive plate and a cadmium compound in the negative plate.  The construction is generally referred to as a "pocket plate” cell.

A range of valve regulated nickel-cadmium batteries that use a recombination process now complement the traditional flooded cell design.  Under normal float charging conditions any gas produced is recombined within the battery and water loss is negligible.  However if the battery is overcharged it will vent but water can be added if necessary.

### Technical Considerations

#### Flooded cell or “Sealed”

The traditional lead acid and nickel cadmium flooded cell technologies have provided good service in many lighthouses and other fixed aids to navigation.  Large capacity flooded cell batteries (or “wet cells”) have a considerable cost advantage for large installations.  However sealed batteries offers some important advantages that include:

- suitability for both floating and fixed aids;

  - facilitates standardisation;

  - reduces on-site maintenance (work, time and cost);

- safer to transport and handle;

  - reduced risk of occupational injuries from electrolyte spills;

  - encourages recovery for appropriate disposal;

  - a less hazardous cargo for helicopters and small boats.

#### Technical Comparison

Table 7.4 shows some of the technical differences between the lead-acid and nickel cadmium batteries.

Table 7.4 A comparison of the performance characteristics of lead-acid and nickel cadmium batteries.

| Type (Technology) | Operating Temperature | Self-Discharge Rate Per Month | Cycle Life to 80% DoD at 40ºC | Ah Charge Efficiency from 20% Discharged to 100% at 25ºC | Relative Cost (1990) |
| --- | --- | --- | --- | --- | --- |
| Tubular-plate flooded | -15 to 55 | 2 | 1800 | 83 | 1 |
| Flat plate flooded | -15 to 55 | 2 | 1200 | 80 | 1 |
| Tubular-plate VRLA | -30 to 50 | 3 | 1000 | >90 | 1.6 |
| Flat plate VRLA | 5 to 50 | 1 | 500 | >90 | 2 |
| Pocket plate VR | -40 to 50 | 2 | 2000 | 71 | 3.9 |
| Pocket plate flooded | -40 to 50 | 4 | 2500 | 71 | 3.9 |

### Battery Disposal

A number of countries now have standards and regulations relating to the safe and environmentally acceptable methods of disposing or recycling of batteries.  The topic was also addressed during IALABATT 3

Refer to IALA publication: - Guidelines on Maintenance and operation of batteries, December 2001.

## Electrical Loads and Lightning Protection

### Electrical Loads

The requirement for IALA to prepare standard methodology for calculating and defining the load profile of electric aids to navigation arose at IALABATT 2 and reconfirmed at IALABATT 3.  IALA has produced a comprehensive Guideline covering the loads for:

- lights;

- racons

- electric sound signals

- fog detectors

- monitoring and telemetry systems

- charge controllers

- signal control equipment

Refer to the IALA publication: - Guidelines on a standard method for calculating and defining the load profile of aids to navigation, December 1999.

### Lightning Protection

To assist those engaged in the design of aids to navigation, IALA has produced Guidelines to describe practical methods for the design, installation, inspection and testing of lightning protection systems.  The information covers lightning protection for aids to navigation structures, equipment and systems.

Refer to IALA publication: - Guidelines for the protection of lighthouses and aids to navigation against damage from lightning, December 2000.

Blue surface colours may be used in inland waterways, estuaries and harbours where the colours may be seen at close range.  See Recommendation E108.

The CIE web site address is: http://www.cie.co.at/cie

That is, a magnifying power of 7 and an objective lens of 50 mm diameter.

The steradian is the equivalent in solid geometry to the definition of radian in plane geometry.  The steradian is defined as the unit of measure of a solid angle with its vertex at the centre of a sphere and enclosing an area of the spherical surface equal to that of a square with sides equal in length to the radius.  There are 4π steradians in a sphere.

This condition is to be met at the outer limits of the useful segment for the minimum meteorological visibility under which the leading lights are to be used.

The term “long flash”, which is used in the descriptions of the long-flashing light and of the light characters reserved for south cardinal marks, means an appearance of light of not less than 2 seconds duration. The term “short flash” is not commonly used and does not appear in the Classification. If an Authority requires discrimination between two flashing lights that only differ in having flashes of different durations, then the longer flash should be described as “long flash” and be of not less than 2 seconds duration, and the shorter flash may be described as “short flash” and should be of not more rhythmic character of such a light is than one third of the duration of the longer flash.

A single fixed light shall not be used on a mark within the scope of the IALA Maritime Buoyage System because it may not be recognized as an aid to navigation light.

A Morse Code white light with the single character "U" is assigned to offshore structures.

The timing of astronomical events can also be applied to calculations (computer programs) for sizing solar power supplies.

Recommendation on the determination of the luminous intensity of a marine aid to navigation light December 1977

The most common of the alternatives is a meteorological visibility of 20 nautical miles.

In these situations the beacon will generally show a colour scheme and topmarks in accordance with the IALA Maritime Buoyage System.

Also known as a Port Entry Light (PEL).

See also IALA Recommendation E 110, The Rhythmic Character of Lights on Aids to Navigation, Paragraph 2.4.

For example, in waterways where the aid may be drifting or destroyed due to ice conditions.

The direction of buoyage is defined generally as inbound (from seaward), and clockwise around the land-mass. See MBS Section 2.1.

The chemistry of the sea-water cell and the prototype light buoys using this cell have been described in papers for the 1990 IALA Conference and IALABATT 2 and 3.

Refer to IALABatt 3  “Fuel cells for aids to navigation”.

Table adapted from IALABATT 2 Overview of PV Battery Charging, Hyperion Energy Systems Ltd, Ireland.