| From: EEP (EEP4) | EEP4/output/03 |
| --- | --- |
| To: ANM | Date: Apr. 20, 2004 |

Liaison Note

The use of Blue Lights for Aids to Navigation

- Introduction

The ANM Committee has been asked to look into developing Guidelines on the provision and identification of AtoN in built-up areas, including the use of blue lights.  In this work they are requesting input from other IALA Committee Members. ANM3 reviewed the liaison note from EEP3 on the use of Blue Lights as an AtoN, including the references to CIE standards.

- Background

ANM3 discussion indicated that many administrations are using Blue Lights, and would like to ensure all relevant information is taken into account in developing the Guidelines. EEP4 make the following input:

Generally, according to CIE Standard:

“Blue is not a suitable colour for long distance visual signals as:

- The eye is relatively insensitive to blue light

- Blue and green are confused by normal observers for signals that have small angular subtense or a low illuminance and particularly when used at the limit of visible range

- Blue is preferentially scattered by the atmosphere.”

Blue lights are suitable for short-range lights but should be used well within their visible range.  Blue lights can be useful in areas where background lighting can cause confusion with other IALA navigation colours, and to provide a conspicuously different signal where necessary.

Research conducted by the GLA Development Department suggests that blue light is best used in conjunction with its reciprocal colour yellow.  This research has also shown that better conspicuity is achieved if blue / yellow lights are alternatively flashed.  Testing in China has shown that blue lights on buoys were very successful for short range lights in conditions of high background lighting.

Blue should be only highly saturated and be chosen with chromaticity coordinates as far from green and white as is possible.

- Requested Action

The ANM Committee is invited to make reference to the comments in their discussions on the use of Blue Lights.