Sections in red need to be reviewed.

Chapter 6	MANAGEMENT ISSUES	3

6.1 Provision, Design and Management of Aids to navigation	3

6.1.1 International Criteria	3

6.1.1.0 International Convention for the Safety of Life at Sea, 1974 (SOLAS)	3

6.1.1.1 SOLAS Chapter V	3

6.1.1.2 Regulation 13 - Establishment and operation of aids to navigation	4

6.1.1.3 Comments	4

6.1.2 LEVEL OF SERVICE (LOS)	4

6.1.2.0 National Obligations	5

6.1.2.1 Level of Service Statement for Quantity	5

6.1.2.2 Level of Service Statements for Quality	8

6.1.2.3 Consultation and Review of LOS	9

6.1.2.4 Mix of Aids to Navigation (Layers of Service)	9

Options	9

6.1.3 Risk Management	11

6.1.3.0 Risk Management	12

6.1.3.1 Risk Management Decision Process	12

6.1.3.2 Levels of Risk	13

6.1.3.3 Coastal Landfall and Waterway Risk Factors	14

6.1.4 Availability Objectives	14

6.1.4.0 History	14

6.1.4.1 Calculation of Availability	15

6.1.4.2 Definition and Comments on Terms	15

6.1.4.3 IALA Categories for Traditional Aids to Navigation	17

6.1.4.4 Availability and Continuity of Radionavigation Services	18

6.1.4.5 Over and Under Achievement	20

6.1.4.6 Continuity	21

6.1.5 Reviews and Planning	22

6.1.5.0 Reviews	22

6.1.5.1 Strategic Plans	23

6.1.5.2 Operational Plans	23

6.1.6 Performance Measurement	24

6.1.7 Quality Management	25

6.1.7.0 quality Systems	25

6.1.7.1 PRODUCT QUALITY AND CERTIFICATION	25

6.1.7.2 SERVICE QUALITY AND CERTIFICATION	25

6.1.7.3 ISO 9000 Series	25

6.1.7.4 ISO 9001 - 2000	26

6.1.7.5 ISO 9004 - 2000	26

6.1.7.6 ISO 14000 Series	27

6.1.7.7 ISO 19011 Environmental Quality	28

6.1.8 Maintenance	28

6.1.8.0 Trends	28

6.1.8.1 Maintenance Intervals	29

6.1.8.2 Improving efficiency	29

6.1.9 Service Delivery	31

6.1.9.0 Current Practice	31

6.1.9.1 Service Delivery Requirements	31

6.1.9.2 Contracting Out	31

Environment	32

6.2	32

6.2.1 Hazardous Materials	32

6.2.1.0 General Issues	32

6.2.1.1 Mercury	33

6.2.1.2 Paints	36

6.2.1.3 Lead	36

6.2.1.4 Anti-Fouling Coatings	36

6.2.2 IALA “PHL” Advisory Panel	37

6.2.3 Lens Size and Terminology	37

6.2.4 Third Party Access To Aids To Navigation Sites	38

6.3 Human Resources Issues	39

6.3.1 Employer Responsibilities	39

6.3.1.0 Source of Skills	39

6.3.1.1 Training for Maintenance Personnel	40

6.3.1.2 Occupational Health & Safety	41

# MANAGEMENT ISSUES

## Provision, Design and Management of Aids to navigation

### International Criteria

#### International Convention for the Safety of Life at Sea, 1974 (SOLAS)

SOLAS is one of the oldest international conventions and originates from a conference held in London in 1914 to address aspects of safety at sea following the sinking of the White Star liner Titanic in 1912.  Since then there have been four other SOLAS Conventions, the latest being the 1974 version that came into force in 1980.

The SOLAS Convention is administered by United Nations through the International Maritime Organisation (IMO).

The 1974 Convention is divided into eleven chapters and within these a series of regulations.  The contents cover:

| Chapter I | General provisions |
| --- | --- |
| Chapter II-1 | Construction – Subdivision and stability, machinery and electrical installations |
| Chapter II-2 | Construction – Fire protection, fire detection and fire extinction |
| Chapter III | Life-saving appliances and arrangements |
| Chapter IV | Radiocommunications |
| Chapter V | Safety of navigation |
| Chapter VI | Carriage of cargoes |
| Chapter VII | Carriage of dangerous goods |
| Chapter VIII | Nuclear ships |
| Chapter IX | Management for the safe operation of ships |
| Chapter X | Safety measures for high-speed craft |
| Chapter XI | Special measures to enhance maritime safety |
| Chapter XII | Additional safety measures for bulk carriers |
| Appendix | Certificates |

#### SOLAS Chapter V

SOLAS Chapter V, and Regulation 13 in particular, defines the obligations on Contracting Governments to provide aids to navigation and related information.  It therefore defines one of the primary roles of IALA National Members.

In December 2000, the 73rd session of the IMO Maritime Safety Committee (MSC) adopted a completely revised SOLAS Chapter V on Safety of Navigation that came into force on 1 July 2002.

#### Regulation 13 - Establishment and operation of aids to navigation

The Regulation states:

1. Each Contracting Government undertakes to provide, as it deems practical and necessary either individually or in co-operation with other Contracting Governments, such aids to navigation as the volume of traffic justifies and the degree of risk requires.

2. In order to obtain the greatest possible uniformity in aids to navigation, Contracting Governments undertake to take into account the international recommendations and guidelines (Reference is made to IALA) when establishing such aids.

3. Contracting Governments undertake to arrange for information relating to aids to navigation to be made available to all concerned. Changes in the transmissions of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.

#### Comments

To satisfy the obligations of Regulation 13, the contracting government has to make assessments on:

- whether or not to provide particular types of aids to navigation;

- the type, number and location of aids to navigation;

- what information services are necessary to adequately inform the mariner.

### LEVEL OF SERVICE (LOS)

In general, there are three levels of service components for aids to navigation services:

- extent which addresses whether a service will be provided by the contracting government or other agencies;

- quantity which addresses the type, size, number and mix of aids to navigation required; and

- quality which addresses the operational reliability of the service.

The SOLAS regulation is an excellent, general level of service statement in terms of the “extent” (where will National Authorities provide the service) of service.  Each contracting government is expected to establish its own service level standards to determine where a service will be provided, based on a measure of benefits, costs, and risks.

The following guidelines address the levels of service provided in terms of “quantity” and “quality” of service wherever a contracting government has decided to provide an AtoN service.

#### National Obligations

To fulfill the obligations imposed by the requirements of the SOLAS Convention, it is recommended that National Authorities prepare Level of Service (LOS) statements as outlined below:

- Review both the quantity and quality of the service and the related LOS standards and Operational Performance Statements (OPS) on a regular basis and update them as necessary to reflect changing patterns of use and reassess risk factors, as well as changing technology and resource levels provided by contracting governments.

- Establish a formal mechanism for promulgating operational parameters and changes which can be accessed by all potential users.  This should include a means of immediate notification, within practical limits, when the failure of an AtoN is identified.

- Where responsibility for the provision of an AtoN service is delegated, for example, to state, territory, or local government organizations, or to port, harbour, or waterway authorities, or to local private groups, responsibility to ensure and/or enforce national obligations shall remain with the National Authority concerned.  (some Authorities may not have sufficient resources to approve and monitor all private and other authorities AtoNs, but will enforce national standards if users report a problem).

#### Level of Service Statement for Quantity

- The Level of Service (LOS) standards and Operational Performance Statements (OPS) for quantity will vary both from country to country and for individual areas depending on usage patterns, degree of risk and the mix of aids to navigation provided.  The LOS/OPS statements represent a commitment from the National Authority to mariners in general who are navigating or operating in the area, and to the government and/or other client groups responsible for funding the provision of the AtoN service.  It is important that the LOS/OPS statements are clearly understood and available to all concerned. The following outlines the stages that must be followed in the preparations of LOS/OPS statements:

- Level of Service/Operational Performance Statements

- It is recommended that LOS/OPS statements be prepared in three stages as follows:

- Step 1: 	Determine the maritime usage patterns for the area concerned and identify risk factors for both vessels and the environment in general, taking into account at least the following:

- Nature and character of the area:

- depth

- siltation

- hazards

- tides and currents

- visibility, weather patterns, and sea-ice conditions

- distinctive shoreline features for both visual and radar navigation.

- background lighting/background environment and relative position of the sun to the traffic routes.

- Traffic Analysis including:

- type of traffic and vessel characteristics by user group

- routes and speed, including route marking requirements

- volume of traffic by user group

- type of cargo, particularly when hazardous

- Risk assessment including:

- basic navigational risks, particularly in high traffic situations

- risk to vessels caused by conflict between user groups

- risk to the environment resulting from a maritime incident

Step 2:	Develop an overall Navigation Plan (NAVPLAN) for the area concerned indicating the mix of AtoN considered necessary to provide the required level of service in the most cost effective manner.  This plan may also include appropriate vessel reporting and control requirements and separation schemes to further reduce the risk of an incident.  As far as possible the NAVPLAN should ensure that failure of a single AtoN does not result in a major penalty to the overall AtoN service, or a substantial increase in risk.

In addition, the NAVPLAN should take into account the likely navigational aids carried by user vessels, including the probability that elements of these onboard navigational aids may themselves suffer failure.  For this reason, the number, type and mix of AtoN types provided must be considered in terms of short-term, navigational aid failure during a critical manoeuvre, or extended period failure.

Step 3:	For each proposed AtoN, or system of AtoNs, prepare an Operational Performance Statement (OPS).  The format for the OPS will vary depending on the type of AtoNs and mix of user groups.

- For a light signal, the OPS should typically comprise information regarding   the probability that the light can be seen at the required range when approached by a vessel at any random time while the AtoN is required for service.  Inputs to this type of statement include the visibility  in the area expressed as a cumulative probability by number of days and the AtoN equipment availability based on meantime between failures and meantime to repair; or

The statement may also be expressed in terms of the maximum operational capability of the light, or totality of lights in an area.  Such an OPS would identify the minimum visibility level at which the light(s) can be seen at the required range when approached by a vessel at night.  The statement would be similar for an unlighted system of aids - eg. AtoN system supports visual navigation until visibility is reduced below 1 nautical mile.

The type of OPS will depend on the method used to measure or establish the LOS standards for the quantity of service.

There are a number of analytical methods available for the establishment of LOS standards and OPS, including:

Relative Risk:

- The level of service standard is based on a relative risk scale using quantified threats or risk factors to determine acceptable risk levels.  Statistical probability ratios or percentages may be the basis for both the LOS and OPS.

Visual Design:

- The level of service standard is based on a standard percentage of time that operational aids will be seen, using predetermined, acceptable risk levels based on professional judgment for each individual, quantified threat or risk factor for each type and size of vessel. The OPS may be expressed in terms of percentage of time that the aids will be visible due to known, local atmospheric conditions, or as the minimum visibility level at which the AtoN system will support visual navigation and below which radar or other electronic systems will be required.

- For a radio AtoN such as a broadcast station providing differential GPS corrections, the LOS should take into account both the expected propagation conditions between the transmitting site and the user and the equipment availability of the AtoN itself.

#### Level of Service Statements for Quality

- Equipment availability

- The equipment availability of an AtoN is governed by the equipment operational reliability measured in terms of meantime- between- failure (MTBF) and the time to complete repairs in the event of a failure, measured in downtime or meantime-to-repair (MTTR).  Where routine maintenance cannot be carried out during non-operational periods, an additional factor for planned downtime must be included.

- Equipment with very high reliability may achieve  required availability standards, even with a long MTTR.  National Authorities should, therefore, consider including a maximum targeted downtime specified in the OPS, where usage and risk factors so dictate.  MTTR depends not only on the ease with which a fault may be diagnosed and repaired on site, but also on access limitations (including weather and sea state conditions), availability of trained staff, and spares holdings, etc.

- Key elements in achieving high equipment availability are:

- choice of equipment which is inherently reliable;

- inclusion of active or passive redundancy features wherever availability targets or maximum downtime targets are not achievable due to remoteness of the site or limited access due to weather or sea state conditions.

- provision of performance measuring facilities, whenever availability or downtime targets are not achievable, will allow potential failures to be identified and corrected before actual failure occurs.  For unwatched automatic AtoN this will normally require provision of a remote monitoring system.

- availability of adequately trained staff and provision of suitable spares.  In this context, National Authorities should address the option of contracting out maintenance and repair tasks with care.  In particular:

- Responsibility for meeting SOLAS Convention requirements cannot be delegated to the contracting organization.

- When contracting out is contemplated, the National Authority concerned should retain sufficient design and performance assessment expertise to be able to audit the performance of the contracting organization in an efficient and effective manner.

- The contracting organization should meet appropriate quality management criteria, including staff training requirements.

Level of service statements for quality of service should be expressed as one, or a combination of the following:

- an overall percentage availability statement reflecting the percentage of time that any AtoN can be expected to be operational.

- percentage availability targets based on importance or type of AtoN.

- maximum downtime targets for AtoN overall, or within a specific area.

#### Consultation and Review of LOS

Development of the LOS targets, the NAVPLAN, and the OPS for each AtoN or AtoN area, should be an iterative process in consultation with all user groups and funding organizations.  It is recommended that the consultation process include:

- meetings with all user representatives conducted during the development of the LOS standards, OPS and NAVPLAN to review proposals and arrive at an equitable balance between service, risk and cost;

- Regular review meetings facilitated by the creation of an Advisory Board, where appropriate, and by feedback from both commercial and pleasure craft operators.

It is recommended that National Authorities put in place procedures to monitor the performance of individual AtoN and that regular reports of achieved equipment availability based on MTBF and MTTR are provided.  As far as is practical, equipment performance monitoring is required in addition to reports of equipment failure received from mariners.

#### Mix of Aids to Navigation (Layers of Service)

#### Options

Tables 6.1 and 6.2 provides a summary of available aids to navigation systems and obtainable accuracies.  It is assumed that radar and visual bearings have an accuracy of 0.5º and radio bearings 2º respectively.

Table 6.1 Indicative accuracies of aids to navigational systems.

| Distance off shore in nautical miles | Obtainable accuracies |  |  |
| --- | --- | --- | --- |
|  | > 500 m | 100 - 500 m | < 100 m |
| Unlimited | Astronomical Fix |  | GPS GLONASS |
| 150 - 800 | Astronomical Fix | LORAN-C | GPS GLONASS |
| 30 - 150 | Astronomical Fix Radio Beacons |  | GPS GLONASS LORAN-C Precision Systems |
| 6 - 30 | Astronomical Fix Radio Beacons Visual Bearings Radio Bearings | Radar Bearings | GPS GLONASS LORAN-C Precision Systems |
| 6 or less |  | Radio Beacons Visual Bearings Radar Bearings | GPS GLONASS LORAN-C Precision Systems |

The various type of aids to navigation have advantages and disadvantages for the user as well as for the provider as indicated in Table 9.3.

Table 6.2 Comparison of the advantages and disadvantages of different types of aids to navigation.

| System |  | Users |  | Providers |
| --- | --- | --- | --- | --- |
|  | Advantages | Disadvantages | Advantages | Disadvantages |
| Visual | Can be used to position Convey immediate information Can be used without a chart if user has a good local knowledge | Range depends on site, height, colour, background Limited by visibility Position of floating aids not always accurate | For hazard warning, traffic regulation, guidance, etc. Placement flexible Maintenance requires little training | Maintenance expensive Planning for maintenance depends on weather conditions Logistic system required Training maintenance personnel |
| Radar | Identification with racon possible in reduced visibility conditions With a racon identification of low coastline Only one aid is required Rapid deployment | Onboard equipment needed Racons may interfere if not placed in an appropriate configuration, aids equipped with radar reflector are difficult to identify | Can replace visual aids Warnings of dangers (New dangers) | Radar reflectors needed Some vessels do not have radar Racon investment expensive Training for maintenance of racons |
| Radionavigation | Wide scale coverage All weather use Automatic navigation Precision possible | On board equipment needed | Reduced maintenance-Automatic monitoring Reduction of visual aids possible | May not be under Lighthouse Authority control Monitoring requirement Training maintenance personnel Large investment |

### Risk Management

This section needs to amended based on the IALA Risk Model. Proposing G.Prosser

Dealing with “risk” is a basic issue of human existence.  The establishment of the early lighthouses represent a tangible way of addressing some of the problems that arose when humans decided to venture out to sea, and then into global trade and the mass transport of people by ships.

The traditional definition of risk is the probability of an unwanted event occurring, multiplied by the impact or consequence of that event.

R = P X C

However no event is completely “Risk Free” and therefore its acceptability by stakeholders will vary in accordance of what they perceive to be in jeopardy. To compensate for this, acceptability is now included in the risk formula.

R = P X C X A

Unwanted events include deprival, loss or injury to persons, property or the environment.

#### Risk Management

Risk management is a term applied to a structured (logical and systematic) process for

- identifying, analysing, assessing, treating, monitoring and communicating risks for any activity, and;

- achieving an acceptable balance between the costs of an incident, and the costs of implementing measures to reduce the risk of the incident happening.

The risk management approach works equally well for identifying the risks at a detailed or broad level.  It can also address the risks from different perspectives.

For example, if the issue is the automation and destaffing of a lighthouse, there are likely to be different sets of risk for:

- service providers (aids to navigation authority, the lightkeepers);

- service users (mariners);

- external groups (politicians, local community, conservation groups).

IALA held a Workshop in Quebec in October 2000 to assist in preparing the publication “IALA Guidelines on Risk Management, December 2000”

A number of countries also have national standards on risk assessment and risk management.

#### Risk Management Decision Process

The Risk Management process described in the IALA Guidelines comprises five steps that follow a standardized management or systems analysis approach:

a. Identify risks/hazards;

b. Assess risks

c. Specify risk control options

d. Make a decision; and

e. Take action.

The diagram in Fig 6.1 provides a guide to the steps involved in the IALA Risk Assessment and Risk Management process.

Fig 6.1 The IALA Risk Assessment and Risk Management process.

#### Levels of Risk

Once the risks have been identified it is generally useful to rank them in order of consequence.  Resources can then be assigned to treating the most serious risks first.  The matrix at Fig 6.2 provides a basis for prioritising risks.

| Impact | High |  |  |  |
| --- | --- | --- | --- | --- |
|  | Medium |  |  |  |
|  | Low |  |  |  |
|  |  | Low | Medium | High |
|  |  | Likelihood |  |  |

|  |  | High Level of Risk |
| --- | --- | --- |
|  |  | Medium Level of Risk with Caution |
|  |  | Low Level of Risk |

Fig 6.2 Risk matrix.

#### Coastal Landfall and Waterway Risk Factors

A risk analysis associated with a coastal landfall, waterway or port approach might consider a range of factors that contribute to the overall risk exposure.  Table 6.3 provides an indication of the factors that could be taken into consideration.

Table 6.3 Indicative risk factors relating to marine navigation.

| Ship traffic consideration | Traffic volume | Navigational conditions | Waterway configuration | Short-term consequence | Long-term consequence |
| --- | --- | --- | --- | --- | --- |
| Quality of vessels | Deep draught | Night/Day operations | Depth | Injuries to people | Health and safety impacts |
| Crew competency | Shallow draught | Sea state | Channel width | Oil spill | Lifestyle disruptions |
| Traffic mix | Commercial fishing vessels | Wind conditions | Visibility obstructions | Hazardous material release | Fisheries impacts |
| Traffic density | Recreational boats | Currents (river, tidal, ocean) | Waterway complexity | Property damage | Endangered species |
| Nature of cargo | High speed craft | Visibility restrictions | Bottom type | Denial of use of waterway | Shoreline damage |
|  |  | Ice conditions | Stability (siltation) |  | Reef damage |
|  |  | Background lighting |  |  | Economic impacts |
|  |  | Debris |  |  |  |

### Availability Objectives

#### History

Members of IALA became interested in the concept of availability around 1975 when significant numbers of lighthouses were being automated and destaffed.  The measurement of ‘Availability’ provided a quantitative measure of performance (or service to the mariner) that was independent of whether an aid to navigation was manned or not.

‘Availability’ is a useful indicator of the level of service provided by individual or defined groups of aids to navigation because it is representative of all the considerations, within the control of the Authority, that have gone into providing and maintaining the facility.  These include:

- quality assurance procedures;

- design and systems engineering;

- procurement;

- installation;

- maintenance procedures;

- failure response;

- logistic arrangements.

In developing the concept of Availability, IALA considered it necessary to measure the long-term performance of an aid to navigation.  To achieve this it was recommended that the calculations should use a time interval greater than 2 years.  The original examples developed for the three availability Categories of lights were based on a 1000 day time interval (most likely to simplify the conceptual calculations).

#### Calculation of Availability

The availability of an aid to navigation may be calculated using one of the following equations, and is usually expressed as a percentage:

Availability =MTBFMTBF+MTTR or Up timeTotal Time or Total time−Down TimeTotal Time

#### Definition and Comments on Terms

Reliability

This is the probability that an aid to navigation, when it is available, performs a specified function without failure under given conditions for a specified time.

Availability

This is the probability that an aid to navigation or system is performing its specified function at any randomly chosen time.

- IALA generally uses the term as a historical measure of the percentage of time that an aid to navigation was performing its specified function.

- The non-availability can be caused by scheduled and/or unscheduled interruptions.

Continuity

This is the probability that an aid to navigation or system will perform its specified function without interruption during a specified time.

- for example, if a DGPS station is functioning correctly when a vessel is about to make its approach into a port, the continuity term states the probability that the DGPS service will not be interrupted in the time it takes the vessel to reach its berth;

- for GNSS systems, IALA has proposed that the time interval for continuity calculations be based on a three-hour time frame.

Redundancy

This is the existence of more than one means, identical or otherwise for accomplishing a task or mission.

Integrity

This is the ability to provide users with warnings within a specified time when the system should not be used for navigation.

Failure

This is the unintentional termination of the ability of a system or part of a system to perform its required function.

Mean Time Between Failures (MTBF)

This is the average time between successive failures of a system or part of a system.  It is a measure of reliability.

- for components, such as lamps, it is usual to determine the MTBF (or life) statistically by testing a representative sample of components to destruction;

- for a system such as a DGPS station the MTBF is determined from the number of failures that have occurred within a given interval.  For example; if four failures occur over a two year interval, the MTBF would be 4380 hours (ie. =24*365*2/4).

Mean Time to Repair (MTTR)

This is a measure of an Authority’s administrative arrangements, resources and technical capability to rectify a failure.

- for a small port, the MTTR times might only be several hours;

- an Authority with a more distributed network of aids to navigation may have MTTR times equivalent to several days because of the distances and transport mobilisation limitations.

Failure Response Time

This is a sub-set of the MTTR and relates to the time it takes to be notified of a failure, to confirm the details and mobilise personnel to depart for the aid to navigation.

#### IALA Categories for Traditional Aids to Navigation

In 2004 ???, IALA published a Recommendations on Availability Objectives of Aids to Navigation Services.  This document states management objectives for operational performance levels of aid to navigation services provided to mariners. It also provides guidance on suitable and realistic levels of operational performance for members to adopt. The 2004 approach for the categorisation of aids to navigation service is different to the earlier version of the recommendation.

| CATEGORY | AVAILABILITY OBJECTIVE | CALCULATION |
| --- | --- | --- |
| 1 | 99.8% | Availability Objectives are calculated over a three-year continuous period, unless otherwise specified |
| 2 | 99.0% |  |
| 3 | 97.0% |  |

Category 1: An Aid to Navigation (AtoN) or system of AtoN that is considered by the Competent Authority to be of primary navigational significance. For example, lighted aids to navigation and racons that are considered essential for marking landfalls, primary routes, channels, waterways or new dangers.

Category 2: An AtoN or system of AtoN that is considered by the Competent Authority to be of navigational significance. For example, it may include any lighted aids to navigation and Racons that mark secondary routes and those used to supplement the marking of primary routes.

Category 3: An AtoN or system of AtoN that is considered by the Competent Authority to be of less navigational significance than Category 1 and 2.

The Recommendation also states that the absolute minimum level of availability of an aid to navigation should be set at 95%.

More details about the availability objectives for conventional or traditional aids to navigation are contained in the publication: IALA Recommendations on availability objectives of aids to navigation, publication ####, published in 2004.

#### Availability and Continuity of Radionavigation Services

The availability objectives for DGNSS (DGPS) services have been handled somewhat differently from traditional aids to navigation. This reflects the broader policy formulation process that includes the IMO Resolution A.815(19) for a World Wide Radionavigation System.

The IALA recommendation on availability objectives for radionavigation aids to navigation are contained in the Recommendation on the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz (R121)

Recommendation R121 retains the original definition of availability, but adds a statement about “non-availability”:

Non-availability is equivalent to “down time” but as proposed includes both scheduled and/or unscheduled interruptions (ie. preventative and corrective maintenance).  The revised equation becomes:

Availability =MTBOMTBO+MTSR

Where:

| MTBO = | Mean time between outages; based on a 2 year averaging period (30 days ocean phase) |
| --- | --- |
| MTSR = | Mean time to service restoration; based on a 2 year averaging period (30 days ocean phase) |

Example (1):

- assume a scheduled maintenance cycle of 6 months:

  - mean time between scheduled maintenance is 0.5 years;

    - ie. 4 scheduled maintenance breaks in 2 years;

- assume a MTBF of 2 years;

  - the average number of failures over 2 years is expected to be approximately 1;

- this gives a total of 5 outages over the two year period;

  - mean time between outages is 2/5 years or approximately 3500 hours;

- if the average out of service time for scheduled maintenance is 6 hours;

  - the total out of service time for scheduled maintenance over the two year period is 24 hours;

  - similarly, if the unscheduled maintenance period is 12 hours, the total time out of service over the two year period is 36 hours;

    - this covers 5 maintenance events and, therefore, the mean time to service restoration is 36/5 hours or approximately 7 hours;

- the overall availability over the two year period is (3500/(3500+7)) or 99.8%.

Example (2):

- assume a scheduled maintenance cycle of 6 months;

  - mean time between scheduled maintenance is 0.5 years;

  - ie. 4 scheduled maintenance breaks in 2 years;

- assume a MTBF of 2000 hours;

  - therefore the average number of failures over 2 years (17520 hours) is expected to be 8.76, rounded up to 9;

- this gives a total of 13 outages over the two year period (4 scheduled + 9 unscheduled);

  - mean time between outages is  17520 hours/13 or 1348 hours;

- if the average out of service time for scheduled maintenance is 6 hours;

  - the total out of service time for scheduled maintenance over the two year period is 24 hours;

  - similarly, if the unscheduled maintenance period is 67 hours, the total time out of service over the two year period is 91 hours.  This covers 13 maintenance events;

    - mean time to service restoration is 91/13 hours or approximately 7 hours;

- the overall availability over the two year period is 1348/(1348+7) or 99.5%.

#### Over and Under Achievement

Issues

As discussed at Section 6.1.2, the actual availability achieved by an individual aid to navigation is a reflection of the quality of the establishment process, the maintenance regime and the skills of personnel involved.

There is a cost penalty associated with prescribing a higher level of availability for a system such as an aid to navigation.  There is also a cost penalty associated with the maintenance of unreliable systems.  The interrelationship is complex, but the objective it to find the minimum cost solution as illustrated in Fig 6.3 .

Fig 6.3 The cost of reliability.

Over-Engineering vs. Unreliability

For a lighthouse in a remote location, the cost of time and transport to rectify equipment failures can be very high.  From this perspective:

- the one-off cost of over-engineering is generally not as expensive in the long term as the ongoing cost of attending to un-reliable equipment and/or poor system designs;

- a conservative design approach has its merits.

If the aid is not achieving its availability objective, the Authority should ascertain the reasons for this and implement actions that remedy the situation.  IALA has recommended that if a facility cannot achieve an availability of 95% (ie. 50 days out per 1000 days) after reasonable endeavours, that consideration should be given to withdrawing the facility (as an aid to navigation)

If a single aid within a group is performing above its availability objective, this could be due to either technical or environmental reasons.  If the performance difference occurs between sites using similar equipment, and this trend has been established for some time, it may be of benefit to investigate the reasons for the difference.

If a group of aids is found to be over performing for a relatively long period of time, there is an opportunity to review the maintenance practices with a view to determining the reasons, and possibly to consider extending the maintenance intervals or reducing the maintenance effort.  This could lead to:

- lower operating costs;

- issues relating to the consequential surplus maintenance capacity.

#### Continuity

IMO uses a more elaborate definition of Continuity than that given in Section 6.1.4.2.  It states that:

Continuity is the probability that, assuming a fault free receiver, a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation within a limited part of the coverage area.

If the service is available at the beginning of the operation, then the probability that it is still available at a time ‘t’ later is:

P = exp (-t/MTBF)

This is the standard expression for reliability and excludes scheduled outages

- it uses MTBF and assumes that planned outages will be notified.

The Continuity, or probability that the service will be available after a continuity time interval (CTI), is then:

C = exp (-CTI/MTBF)

If MTBF is much greater that CTI, the equation approximates to:

C = 1 – (CTI/MTBF)

Where:

| MTBF = | Mean time between failures based on a 2 year averaging period |
| --- | --- |
| CTI = | Continuity Time Interval; in the case of maritime continuity, is equal to 3 hours |

There is no need to include the availability at the beginning of the time period of the operation because if there is no service, then the operation will not commence.

Example (1):

Using the figures in the previous example for a system with a 2 year MTBF, the continuity over a three hour period is 1-(3/17520), or 99.98%

Example (2):

Using the figures in the previous example for a system with a 2000 hour MTBF, the continuity over a three hour period is 1-(3/2000), or 99.85%.

### Reviews and Planning

#### Reviews

In many countries, the network of aids to navigation has been built up over a considerable time, in some cases, centuries.  It should be recognised that the nature of shipping is continually changing and this means that the aids to navigation infrastructure should be reviewed periodically.  The rate of change varies from place to place, but it would be reasonable to adopt a review process using one of the change management tools described LATER ON that provides:

- a Strategic Plan (Navigation Plan) with a suggested 10 year outlook, and;

- an Operational Plan with a suggested rolling 5 year work program.

#### Strategic Plans

A Strategic Plan is the result of an informed and consultative process that sets the long term goals and objectives for an organization.  For an Aids to Navigation Authority it would include decisions on:

- the role of the authority, for example:

  - to promote a high standard of maritime safety;

  - to provide infrastructure and information services to support the safety of navigation in a particular area;

- how the authority will go about discharging its responsibilities, for example:

  - outline of the corporate values of the authority;

  - corporate governance arrangements;

  - funding arrangements;

  - reviews of industry trends;

  - an understanding of the users and navigation requirements.

Because of its importance and its effect on the mariners, any strategic plan should be developed as much as possible in full consultation with the mariners and other stakeholders.

#### Operational Plans

The Operational Plan might cover:

- The implementation of the strategic plan, and may include statements on current policy issues such as:

  - maintenance;

  - current and new technology;

  - the design life of new infrastructure

  - remote monitoring and control;

  - historic lighthouses;

  - environmental culture and safety;

  - the program for aids to navigation reviews;

  - contract services;

  - transport services;

  - sources of revenue;

  - external relationships;

  - information, communication  and consultation management.

- a list of changes to individual aids to navigation, including any new facilities.  The list would reflect:

  - decisions resulting from user and stakeholder consultations;

  - reviews, including those that use:

- - risk analysis, risk management procedures (see section 6.1.3), or;

- - a level of service process, (see section 6.1.2), or;

- - the authority’s quality management procedures (see section 6.1.7)

  - the authority’s technical and maintenance policies etc.

- project schedules that reflect known priorities, such as:

  - government policies;

  - user requirements;

  - available resources;

  - budget (revenue) forecasts and constraints.

### Performance Measurement

Performance measurement is a management tool that can be used to measure, analyse and monitor the performance of a network of aids to navigation and/or specific systems and equipment.  The information obtained can be used to:

- show accountability to government and stake holders;

- demonstrate the efficiency and effectiveness of the service being provided;

- compare the performance of:

  - similar systems or equipment in different locations;

  - contract and internally provided services ;

- amend:

  - system designs;

  - procurement decisions;

  - equipment choices;

  - maintenance procedures and practices;

- increase or reduce maintenance effort;

- extend maintenance intervals.

SHOULD add more here about Perf. Measurement and its advantages.

### Quality Management

#### quality Systems

Quality Management Systems have been developed and introduced by numerous businesses, but increasingly are being based on the ISO9000 series of standards. These standards provide a broadly accepted framework for implementing a quality management system.

A generic quality management system is process focused and defines procedures for how things are to be done and what resources are necessary.  It addresses:

- who does what?

- what skills and qualifications are necessary?

- what processes have to be followed to get consistent outcomes?

- what resources are necessary to do the work efficiently?

Refer also to: - The IALA Quality Assurance Guideline for the procurement, maintenance and repair of aids to navigation equipment and systems. - IALA Guidelines for the establishing of Quality Management in aids to navigation administrations, April 1997.

#### PRODUCT QUALITY AND CERTIFICATION

To be completed by Juan F Rebollo based on the guideline to be developed by IMC

#### SERVICE QUALITY AND CERTIFICATION

To be completed by Juan F Rebollo based on the guideline to be developed in the “workshop on service quality management. First draft for ANM5

#### ISO 9000 Series

The 1994 quality standard series of ISO 9001, 9002 and 9003 have been jointly revised and amalgamated into ISO 9001-2000.

The new series of standards designated as ISO 9000 comprises:

- ISO 9000 Quality management systems - Fundamentals and vocabulary

- ISO 9001 Quality management systems - Requirements

- ISO 9004 Quality management systems - Guidance for Performance Improvement

#### ISO 9001 - 2000

ISO 9001 specifies requirements for a quality management system that can be used for internal application by organizations, or for certification, or for contractual purposes.  It focuses on the effectiveness of the quality management system in meeting customer requirements.  See Fig 6.4.

#### ISO 9004 - 2000

ISO 9004 gives guidance on a wider range of objectives of a quality management system than does ISO 9001, particularly for the continual improvement of an organization's overall performance and efficiency, as well as its effectiveness.  ISO 9004 is recommended as a guide for organizations whose top management wishes to move beyond the requirements of ISO 9001, in pursuit of continual improvement of performance.  However, it is not intended for certification or for contractual purposes.

Fig 6.4 Diagram from ISO 9001 showing the emphasis on

satisfying customer requirements.

#### ISO 14000 Series

##### ISO 14000

This is a collection of voluntary consensus standards that have been developed to assist organizations to achieve environmental and economic gains through the implementation of effective environmental management systems.

There are three standards that deal with Environmental Management Systems (EMS).  These are ISO 14001,14002 and 14004.  ISO14001 is the only standard intended for third party accreditation.  The other standards are for guidance.

##### ISO 14001

ISO 14001 specifies the requirements for an environmental management system, to enable an organization to:

- formulate a policy and objectives taking into account legislative requirements and information about significant environmental impacts;

- it applies to those environmental aspects that the organization can control and over which it can be expected to have an influence;

- it does not itself state specific environmental performance criteria;

- demonstrate to itself, and to other interested parties, conformance with its stated environmental policy;

- seek certification/registration of its environmental management system by an external organization;

- manage and measure a program of continual improvement.

#### ISO 19011 Environmental Quality

- To be developed by Pedro Rodriguez from Venezuela & JFRebollo

    - Maintenance

#### Trends

While few maintenance statistics have been collected in recent years, IALA Conference papers and Bulletin articles indicate a general interest in ways to extend maintenance intervals.  The automation and destaffing of major lighthouses has shifted the maintenance functions from a daily activity to less frequent events.

The optimal maintenance interval for aids to navigation is determined from a consideration of national priorities and the Authority’s administrative, technical and environmental constraints.  Where cost efficiency and effectiveness is the driving issue, Authorities are:

- using automation and renewable power supplies to contain or reduce costs;

- addressing the potential for new technology to:

  - reduce acquisition and operating costs;

  - extended maintenance intervals;

- reviewing transport service options.

IALA has published - Guidelines on Lighthouse Maintenance, and; - Quality Assurance Guideline for the Procurement, Maintenance and Repair of Aids to Navigation Equipment and Systems

#### Maintenance Intervals

The maintenance intervals for aids to navigation varies from daily in the case of a manned lighthouse to perhaps five years for a light buoy.  It is difficult to establish a clear view of typical maintenance intervals other than what is stated in conference and workshop papers.  Some examples include:

- major facilities are being inspected on a monthly basis;

- automated lights are being inspected less frequently (typically, one of the following: quarterly, semi-annually, annually).

Advances in self contained beacons, lamps, self-powered LED lanterns, and solar power supplies make it relatively easy for a well designed system on a fixed structure to achieve annual or biannual servicing intervals.  Systems that can be maintained in multiples of a year can be set up to take advantage of the times of the year that minimise the weather risk on work schedules and disturbance to flora and fauna.

However, a balance has to be found since longer maintenance intervals affect the authority’s knowledge of storm damage, general deterioration to aids to navigation, and control over vegetation growth that could increases the risk of obscuration and fire damage etc.  There may also be a detrimental affect on the detailed level of knowledge held by maintenance personnel.

#### Improving efficiency

Authorities have been able to achieve significant cost savings by:

- automating of lighthouses:

  - reduces the work load for lightkeepers;

  - facilitates destaffing;

- destaffing aids to navigation reduces:

  - staff costs (payroll)

  - power consumption;

  - the frequency of stores replenishment;

  - commitments on infrastructure such as houses or accommodation facilities, water and fuel storage and in some cases jetties and cargo handling equipment;

  - the requirements for station vehicles, plant and equipment;

- using more reliable equipment, better system designs, with "fail safe" or “fail by stages” features coupled with;

  - longer intervals between maintenance visits;

  - a review of maintenance management procedures;

- converting aids to navigation that operates on oil, gas or primary battery to solar power or self-powered LED lanters may provide:

  - greater flexibility in scheduling maintenance visits because of the renewable energy source;

  - opportunities for extending maintenance intervals;

- replacing floating aids with fixed structures in waterways of moderate depth;

  - particularly if it also allows a dedicated buoy tender to be replaced by some other means of transport such as smaller vessel or launch.

- introducing low maintenance materials such as high density polyethylene, GRP, stainless steel, etc to reduce maintenance time on site.  It may also:

  - decrease the number of ship-day requirements;

  - reduce the need for construction (or structural maintenance) skills;

- standardising equipment to simplify of spares management, and:

  - benefit the purchasing power of the organisation;

  - reduce the range of skills required by maintenance staff;

  - give more flexibility on the choice of basic qualification when recruiting maintenance staff;

  - provide more opportunity to understand the inherent deficiencies in particular pieces of equipment and for remedial actions to be implemented;

- remote monitoring (and control) of distant or isolated aids to navigation can save on the cost of responding to what is later found to be a false outage report;

- analysis of aids to navigation systems using risk analysis / risk management techniques may produces cost savings from a rearrangement and or reduction of the aids to navigation within a nominated area.

    - Service Delivery

#### Current Practice

Authorities with the responsibility for the provision of aids to navigation are generally government owned.  They are usually the sole national regulator of marine aids to navigation infrastructure and services, but are not necessarily the sole provider of these services.  In some countries there is a division of responsibility between the authority representing the national government and other organisations that include:

- state and territorial authorities;

- local government organisations;

- port, harbour or waterway authorities, or;

- local private groups.

#### Service Delivery Requirements

The SOLAS Convention applies to a range of vessels over 500 gross tonnage (and over 300 gross tonnage from July 2002) that are engaged in international voyages:

- where more than one local authority provide aids to navigation services, the Contracting government has ultimate responsibility for obligations under the SOLAS Convention.

The requirements for vessels operating on coastal and interstate voyages, and those that are less than 500 gross tonnage are determined by the national or state governments.

Aids to navigation may be provided to meet the specific needs of these vessels and it is these that are often operated by state, territories and local government organisations or private groups.

#### Contracting Out

In some parts of the world there have been moves by governments to sell off government business activities to the private sector.  The motivation for this varies, but includes:

- adding flexibility to how work is carried out;

- breaking down entrenched work practices that are perceived to be inefficient;

- accessing a wider range of skills and resources on demand;

- recognition that as aids to navigation become more reliable and maintenance intervals are increased, it becomes more difficult to:

  - justify having permanently staffed maintenance depots;

  - maintain the currency of work skills;

- using on-call contractors in regional locations to improve fault rectification times through reducing the travelling time to the aid.

The key elements to success when contracting out are:

- to retain sufficient skills within the Authority to understand the functional requirements of the aids to navigational network.  This includes:

  - good contract management skills to handle the day-to-day operational issues;

  - personnel to engage in user consultation and forward planning;

  - the knowledge to act as an “informed purchaser” of services;

- to retain control of intellectual property such as:

  - original drawings;

  - documentation covering the design and configuration of individual aids to navigation;

  - a register of assets and spares;

- defining a set of key performance indicators to measure the performance of the contractor.

## Environment

This section 6.2 needs to be revised by EEP

### Hazardous Materials

#### General Issues

As an employer and in the wider context of an aids to navigation authority being a provider of safety services, there are obligations on each Authority to inform staff, temporary employees and contractors of known hazards when working on and around lighthouses and other aids to navigation.  This section provides information and guidance on some of the hazardous materials.

#### Mercury

A number of historic lighthouses are still operating rotating glass lenses and mercury float pedestals.  This was a clever method for providing a heavy lens with an almost frictionless bearing so that it could be turned by a clockwork mechanism.  However, given the toxic and corrosive properties of mercury, the following information may assist Authorities to implement appropriate safety procedures.

The mercury-bath pedestal for a first-order rotating lens contains about 13 litres of mercury.  Quantities of mercury can also be found in the electrical slip-ring units in rotating lamp array lighting equipment, some tilt-action switches, high current contact breakers, manometers and thermometers.

Physical Properties

General properties of Mercury.

| Appearance | Silvery metallic liquid |
| --- | --- |
| Chemical Symbol | Hg |
| Specific gravity | 13.546 |
| Boiling Point | 357ºC (630ºK) |
| Freezing Point | - 38.7ºC (234.3ºK) |

Spill Risk

The mercury in a lighthouse optic system does not present a significant hazard, unless personnel come into contact with "uncontained" mercury as a result of accidental spills.  Such events are usually the result of a mishap during maintenance work, or as a result of a natural disaster such as an earth tremor that displaces mercury from its containment bath.

If spilt, the mercury can enter cracks in floors, and is readily absorbed into porous surfaces such as concrete, masonry and timber.  When broken into small globules or droplets, the surface area and vaporisation rate rises rapidly.  Minute droplets will adhere readily to dust and can form respirable particles.

Mercury is a corrosive substance if it comes into contact with metals such as zinc and aluminium.

Occupational Hazard

The occupational hazard associated with mercury relates to:

- the threshold limit value (TLV) which is the concentration of the substance that most workers can be exposed to without adverse effects.

- the permissible exposure limit (PEL) expressed as a time-weighted average.  This is the concentration of the substance that most workers can be exposed to without adverse effects averaged over a normal 8 hour day or a 40 hour week.

- Vapour inhalation: Some vaporization from a free mercury surface will occur at normal room temperature and this is the most likely first contact that lighthouse personnel will have with mercury.  Unless the mercury vapour levels have been measured, personnel are unlikely to be aware of the hazard.  If the work-space around lighthouse equipment containing mercury is not well ventilated, the concentration levels can rise above recommended limits and there is potential for mercury poisoning:

  - the typical TLV and PEL concentrations for elemental mercury vapour is 0.05 mg per m3 ;

  - it is suggested that personnel should not knowingly be subjected to mercury vapour concentrations exceeding 40% of the TLV (ie. 0.02 mg per m3) without respiratory protection;

  - mercury vapour is heavier than air and in still air will tend to concentrate in low parts of the work-space.

- Ingestion:- is less common than vapour inhalation but can lead to acute mercurial poisoning;

- Absorption through the skin:- mercury is not easily absorbed through the skin and generally health authorities do not quote a TLV for skin contact.

Precautions

It is advisable that:

- personnel assigned to service equipment containing mercury should have a medical check prior to the commencement of work to establish a base-line for mercury levels in their bodies.  Further checks should take place periodically to monitor the mercury levels;

- personnel should receive training on mercury safety and spill cleanup, decontamination and waste disposal procedures;

- mercury vapour levels at the work site should be determined prior to the commencement of work to establish a base-line value;

- mercury vapour concentrations should be measured periodically during the course of the work, and action taken if values rise above safe levels;

- lighthouse towers and the equipment work space should be well ventilated when personnel are present;

- the work space should be kept clean and dust levels minimised.

Spill Cleanup

A typical approach for a mercury spill cleanup would include:

- keeping temperatures as low as practicable to limit the rate of vaporization.

- supplying personnel with disposable protective clothing and respirator. For example:

  - an appropriate disposable respirator;

  - disposable overshoes;

  - disposable coveralls;

  - eye protection;

  - disposable gloves;

- checking mercury vapour concentrations with a correctly calibrated mercury vapour meter;

- recovering all visible mercury from surfaces, using the special vacuum cleaner or an approved mercury vacuum pump;

- using sulphur powder to neutralise small spills and a solution of sulphur powder and calcium hydroxide for washing down surfaces.

Consignment

Whenever consigning mercury, the following details should be provided on the outside of the package using the approved label for the type of transport:

UN No.2809

Technical name: Mercury

Dangerous Goods Class: 8 (Corrosive)

HAZCHEIVI: 2Z

Note:

- Both IMO and the International Air Transport Association (IATA) have regulations covering the transportation of mercury.

#### Paints

Aids to Navigation authorities use a significant quantity and variety of paints and related surfacing materials.  There is potential for hazardous situations to arise and for environmental pollution.  For example:

- storage of inflammable paints and solvents;

- during surface preparation and removal of paint prior to repainting;

- contact with vapours and solvents during application;

- clean-up and waste disposal.

#### Lead

Lead based paints have been widely used in the past, but are now restricted or prohibited in some countries.  Authorities maintaining older lighthouses are likely to be faced, at some stage, with having to remove lead based paints and dispose of the waste.

Members are encouraged to assess the risks and to adopt appropriate measures to safeguard maintenance personnel and the environment.

#### Anti-Fouling Coatings

Antifouling paints contain biocides and are applied to vessels and floating aids to navigation to reduce the accumulation of marine organisms.

For service vessels the antifouling paint assists to minimise fuel consumption.

On buoys and lightvessels the build-up of marine growth is not particularly detrimental.  In view of the concentration of these types of aids to navigation in port approaches and internal waterways, less toxic paint systems may be preferred to minimise environmental pollution.

A particular group of antifouling paints using Tributyltin (TBT) has come under scrutiny in recent years due to its adverse affects on the genetics of marine shellfish and findings on TBT concentrations in waterway sediments.  The IMO Marine Environmental Protection Committee is studying a proposal for these antifouling paints to be phased out over a ten-year interval.

### IALA “PHL” Advisory Panel

The IALA Advisory Panel on the Preservation of Lighthouses, Aids to Navigation, and Related Equipment of Historic Interest (PHL) was established by the IALA Council in 1996 in response to membership interest in the cultural value of lighthouses.

The objectives of the Panel are to:

- encourage deeper commitment by members to preserve historic values;

- encourage member countries to see the preservation of their own lighthouses in an international context;

- share information on the subject between both members and non-members, with particular attention being given to the alternative use of lighthouses.

The work of the Panel has produced:

- the format of an IALA database for recording details of historic lighthouses;

- a book, titled “Lighthouses of the World” was published in 1998 with English, French, German and Spanish versions.  It features over 180 historic lighthouses from around the world;

- a Workshop to gather information on the range of alternative uses of lighthouses.

Future work for the Panel will be directed at preparing two IALA Guidelines that will address:

- the technical aspects of operating and maintaining historic lighthouses, and;

- the Policy, Planning and Practical aspects of preserving historic lighthouses.

### Lens Size and Terminology

The following table provides information on terminology for historical glass lens systems and the typical amount of mercury held in mercury bath pedestals (for rotating lens systems).

Terminology for historical glass lens systems and associated quantities of mercury used in rotating lens systems.

| Description | Focal distance | Typical quantity of mercury for mercury-bath pedestals |  |
| --- | --- | --- | --- |
|  | mm | kilograms | litres |
| Hyper-radial | 1330 |  |  |
| Meso-radial | 1125 |  |  |
| First Order | 920 | 175 | 12.9 |
| Second Order | 700 | 126 | 9.3 |
| Third Order | 500 | 105 | 7.7 |
| Small Third Order | 375 | 96 | 7.0 |
| Fourth Order | 250 |  |  |
| Fifth Order | 187.5 |  |  |
| Sixth Order | 150 |  |  |

### Third Party Access To Aids To Navigation Sites

In 1998, IALA conducted a survey to investigate the extent to which Authorities were permitting aids to navigation sites to be used for collecting “non-aids to navigation” data.  This study was associated with investigations of the Advisory Panel on the Preservation of Historic Lighthouses into alternative uses of lighthouses and other aids to navigation.

The responses came from a wide range of IALA members and shared several common themes:

- the predominant applications were for the collection of meteorological data (i.e. weather, wind speed and direction), tidal/ current data and for telecommunication installations;

- data collected for or by other governmental agencies generally did not attract a fees, but fees were often charged for data obtained for commercial purposes;

- data acquisition equipment had to have its own separate power supply unless that aids to navigation site had mains power available.

IALA acknowledges that Authorities face an increased demand to share aids to navigation sites with  “third parties”.  While it is important to ensure that the integrity and security of aids to navigation are maintained, the presence of a third party may be beneficial:

- in reducing the risk of vandalism;

- as a source of revenue or sharing of operational costs (eg power, road maintenance etc);

- as a means of monitoring the operation of the aid.

If an Authority receives a request for a third party installation, it should first establish whether such involvement is permitted in the Authority’s legislation.  If there are no impediments the Authority may consider negotiating an agreement with potential third party to clearly establish the responsibilities and liabilities of each party.  The agreement may also address:

- conditions to apply to the third party installation and operation to ensure that the equipment does not compromise the integrity and security of the aids to navigation and other property owned by the Authority;

- access to electrical power.  At sites with mains power, it may advisable for the Authority to require separate metering of the third-party supply so that electricity costs can be recovered;

- if no mains power is available, it is reasonable to require that the third-party provide its own power supply;

- where practical, the installation of the third-party equipment should take into consideration and preserve the heritage value of the aid to navigation.

Authorities should reserve the right to cancel any third party agreement if continued use jeopardizes the performance or functionality of the aid to navigation.

The above list of terms and definitions do not adequately cover all of the situations that an Authority might want to use when issuing a Navigation Warning.  An expanded set of definitions of terms for use in Navigation Warnings is provided for consideration in

## Human Resources Issues

### Employer Responsibilities

Aids to navigation authorities should ensure that all employees have the knowledge, skills and training to perform their duties effectively, and with safety.  The term, ‘employees’ includes newly hired, part time and temporary employees.

The ISO 9001 Quality Management standard places considerable emphasis on competence, awareness and training.  (See Section 6.1.7.3)

#### Source of Skills

Skill development processes for aids to navigation work.

| Skill | Process |
| --- | --- |
| Education | - school - tertiary institution |
| Experience | - work experience - related work experience |
| Training | - induction training - on-the-job training - apprenticeships - specific training programs - refresher courses |

#### Training for Maintenance Personnel

A number of IALA surveys have indicated that, in some areas, maintenance personnel lack the training to perform their tasks effectively.  The situation can be addressed by:

- the authority developing a written maintenance philosophy;

- carrying out a skills audit to identify the gaps between available skills and skills required for the various maintenance tasks;

- arranging for training programs to fill the gaps, noting that:

  - modular training courses are available or can be adapted to the particular needs of each trainee or group of trainees;

  - computer based training and distance learning methods are a useful ways of achieving continuity of training when personnel are also engaged in field activities.  Some adjustment to work schedules would be necessary to allow for training time;

- using training courses are accredited with a recognised institution.  This has several benefits:

  - a trainee may be more highly motivated if he/she can see accredited courses leading on to a formal qualification (ie. career path prospects);

  - accredited courses are “portable” and are of benefit to those changing jobs;

  - recognised courses could be referenced in position descriptions to broadening the range of applicants for job vacancies.

#### Occupational Health & Safety

- To be developed by Pedro Rodriguez from Venezuela & JFRebollo

END OF DOCUMENT

-

-

IALA Questionnaire Information

The IALA Engineering Committee has been monitoring the types of computer programs being used or developed by members.  A report was prepared on this topic in 1996 entitled “Computer Programs in Lighthouse Services”.  The Committee has proposed that programs that can be shared with other members could be listed on the IALA website.

IALA Computer Programs and Databases

The IALA Engineering Committee has embarked on projects to develop computer programs for:

Leading (Range) Line computations;

-

-

Training for Maintenance Personnel

A number of IALA surveys have indicated that, in some areas, maintenance personnel lack the training to perform their tasks effectively.  The situation can be addressed by:

the authority developing a written maintenance philosophy;

carrying out a skills audit to identify the gaps between available skills and skills required for the various maintenance tasks;

arranging for training programs to fill the gaps, noting that:

modular training courses are available or can be adapted to the particular needs of each trainee or group of trainees;

computer based training and distance learning methods are a useful ways of achieving continuity of training when personnel are also engaged in field activities.  Some adjustment to work schedules would be necessary to allow for training time;

using training courses are accredited with a recognised institution.  This has several benefits:

a trainee may be more highly motivated if he/she can see accredited courses leading on to a formal qualification (ie. career path prospects);

accredited courses are “portable” and are of benefit to those changing jobs;

recognised courses could be referenced in position descriptions to broadening the range of applicants for job vacancies.

|  | - |
| --- | --- |

#### Sp

The above list of terms and definitions do not adequately cover all of the situations that an Authority might want to use when issuing a Navigation Warning.  An expanded set of definitions of terms for use in Navigation Warnings is provided for consideration in Table 10.5.

Positions and Bearings

Positions

The Joint IMO/IHO/WMO Manual on Maritime Safety Information (IMO COMSAR/Circ 15) states that positions should always be given in

|  | - |
| --- | --- |

Spill Risk

The mercury in a lighthouse optic system does not present a significant hazard, unless personnel come into contact with "uncontained" mercury as a result of accidental spills.  Such events are usually the result of a mishap during maintenance work, or as a result of a natural disaster such as an earth tremor that displaces mercury from its containment bath.

If spilt, the mercury can enter cracks in floors, and is readily absorbed into porous surfaces such as concrete, masonry and timber.  When broken into small globules or droplets, the surface area and vaporisation rate rises rapidly.  Minute droplets will adhere readily to dust and can form respirable particles.

Mercury is a corrosive substance if it comes into contact with metals such as zinc and aluminium.

Occupational Hazard

The occupational hazard associated with mercury relates to:

the

Example include Canada and Australia / New Zealand.

Or any nominated system or component.

IMO Resolution A.860(20) Appendix 1.

Irrespective of whether or not the increased availability is required by the mariner.

This is the same definition as “mission reliability”.

Some aids to navigation are in reality monuments to historical accidents and are of little value to modern shipping.

For example with national, state, territory, and local governmental bodies and international organisations.

Only where the opportunity arises and where both are engaged in substantially similar work.

The quantity of mercury used in higher order optics is shown in Section 6.2.3.

The US Environmental Protection Agency has a website reference to mercury:  www.epa.gov/mercury

The 3M respirator type 9008 has been used for this purpose.

Twenty six responses were received.

Also known as Narrow Band Direct Printing, or NBDP.

Or any nominated system or component.

IMO Resolution A.860(20) Appendix 1.

Large aid to navigation buoys.

Irrespective of whether or not the increased availability is required by the mariner.

This is the same definition as “mission reliability”.

Ships’ routeing is covered by Regulation 10 in the new Chapter V that comes into effect on 1 July 2002.

IMO has developed the “International Convention on Standards of Training, Certification and Watchkeeping for Seafarers” to prescribe minimum standards of training and certification for seafarers. The latest amendments to the Convention (STCW 95) will be fully implemented by 31st January 2002.

Example include Canada and Australia / New Zealand.

Some aids to navigation are in reality monuments to historical accidents and are of little value to modern shipping.

For example with national, state, territory, and local governmental bodies and international organisations.

Only where the opportunity arises and where both are engaged in substantially similar work.

Or any nominated system or component.

IMO Resolution A.860(20) Appendix 1.

Large aid to navigation buoys.

Irrespective of whether or not the increased availability is required by the mariner.

This is the same definition as “mission reliability”.

Ships’ routeing is covered by Regulation 10 in the new Chapter V that comes into effect on 1 July 2002.

IMO has developed the “International Convention on Standards of Training, Certification and Watchkeeping for Seafarers” to prescribe minimum standards of training and certification for seafarers. The latest amendments to the Convention (STCW 95) will be fully implemented by 31st January 2002.

The quantity of mercury used in higher order optics is shown in Section 10.1.6.

The US Environmental Protection Agency has a website reference to mercury:  www.epa.gov/ttn/uatw/hlthef/mercury

The 3M respirator type 9008 has been used for this purpose.

Twenty six responses were received.

Also known as Narrow Band Direct Printing, or NBDP.

The quantity of mercury used in higher order optics is shown in Section 10.1.6.

The US Environmental Protection Agency has a website reference to mercury:  www.epa.gov/ttn/uatw/hlthef/mercury

The 3M respirator type 9008 has been used for this purpose.

Only where the opportunity arises and where both are engaged in substantially similar work.

Or any nominated system or component.

IMO Resolution A.860(20) Appendix 1.

Large aid to navigation buoys.

Irrespective of whether or not the increased availability is required by the mariner.

This is the same definition as “mission reliability”.