A methodology for Cost Benefit Analysis of buoys

AMSA has recently performed a cost benefit analysis of options for replacement buoys in Australia taking into consideration a number of presently available plastic based alternatives to their current steel based buoys. The analysis resulted in a decision to continue with steel buoys for the next five years.

Whilst the decision to stick with steel buoys is very interesting in it self, the methodology used to arrive at this decision is even more interesting for EEP committee.

Basically the approach used for evaluating the capital cost savings was as follows:

- Select which scenarios to analyze

- Define any basic assumptions made for the analysis

- Select a timeframe for the analysis

- Calculate the initial replacement costs

- Calculate the maintenance costs each year

- Calculate the Net Present Value (NPV) of all costs within the selected timeframe

- Compare the NPV of each scenario

The following scenarios where selected for the AMSA analysis:

- Continue to use the current design of buoys of steel construction

- Change to plastic construction buoys which have reduced maintenance requirements.

The following basic assumptions where made for the analysis:

- Costs common to all scenarios can be omitted

- Maintenance cycle of steel hull buoys requires purchase of 1.5 times the number in service

- Maintenance cycle of plastic hull buoys requires purchase of 1.25 times the number in service

A time frame of 20 years was selected.

The following factors where taken into consideration for calculating the initial costs:

- Unit Cost

- Maintenance Spares (proportion in store)

- Installation components/fittings

- Cost of ship time for installation

While calculating the maintenance costs each year, the following factors where considered:

- Minor maintenance costs at interval of X years

- Major maintenance costs at interval of Y years

- Buoy replacement costs at interval of Z years (buoy lifetime)

In each case the costs where split into the cost of ship time and other costs (materials and services).

Comparing the NPV of each scenario revealed what potential capital cost savings could be obtained for each scenario relative to the present scenario.

A spreadsheet example of this basic NPV methodology is enclosed at the end of this paper.

The above mentioned factors can more or less be characterized direct cost drivers. In addition to these there may be a number of indirect cost drivers such as workshop facilities that may be affected the alternative scenarios.

Thanks to Allan Crossing and Gary Prosser from AMSA for providing this information.

Ómar Frits Eriksson

| Item | Current Steel Buoy | Buoy A | Buoy B | Buoy C | Buoy D |
| --- | --- | --- | --- | --- | --- |
| Initial Costs |  |  |  |  |  |
| Unit Cost | 9600 | 9700 | 14000 | 25000 | 55000 |
| Maintenance Spares (proportion in store) | 0,5 | 0,25 | 0,25 | 0 | 0 |
| Installation components/fittings | 6300 | 6300 | 6300 | 1800 | 1800 |
| Ship time for installation (days) | 0,5 | 0,5 | 0,5 | 2 | 2 |
| Installation costs | 2550 | 2550 | 2550 | 10200 | 10200 |
| Daily Ship Cost | 5100 |  |  |  |  |
| Maintenance Costs |  |  |  |  |  |
| Minor maintenance costs at interval of:- (years) | 2 | 2 | 2 | 2 | 2 |
| Ship Time (days) | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 |
| Ship Cost | 2550 | 2550 | 2550 | 2550 | 2550 |
| Other Costs (materials & services) | 2700 | 2700 | 2700 | 1500 | 1500 |
| Major maintenance costs at interval of:- (years) | 4 | 4 | 4 | 6 | 6 |
| Ship Time (days) | 0,5 | 0,5 | 0,5 | 1 | 1 |
| Ship Cost | 2550 | 2550 | 2550 | 5100 | 5100 |
| Other Costs (materials & services) | 3000 | 2400 | 2400 | 3000 | 3000 |
| Buoy Changeout costs at interval of:- (years) |  |  |  | 10 | 10 |
| Ship Time (days) |  |  |  | 2 | 2 |
| Ship Cost |  |  |  | 10200 | 10200 |
| Other Costs (materials & services) |  |  |  | 6000 | 9000 |
| Year | Current | Buoy A | Buoy B | Buoy C | Buoy D |
| 0 | -23250 | -20975 | -26350 | -37000 | -67000 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | -5250 | -5250 | -5250 | -4050 | -4050 |
| 3 | 0 | 0 | 0 | 0 | 0 |
| 4 | -5550 | -4950 | -4950 | -4050 | -4050 |
| 5 | 0 | 0 | 0 | 0 | 0 |
| 6 | -5250 | -5250 | -5250 | -8100 | -8100 |
| 7 | 0 | 0 | 0 | 0 | 0 |
| 8 | -5550 | -4950 | -4950 | -4050 | -4050 |
| 9 | 0 | 0 | 0 | 0 | 0 |
| 10 | -5250 | -5250 | -5250 | -16200 | -19200 |
| 11 | 0 | 0 | 0 | 0 | 0 |
| 12 | -5550 | -4950 | -4950 | -8100 | -8100 |
| 13 | 0 | 0 | 0 | 0 | 0 |
| 14 | -5250 | -5250 | -5250 | -4050 | -4050 |
| 15 | 0 | 0 | 0 | 0 | 0 |
| 16 | -5550 | -4950 | -4950 | -4050 | -4050 |
| 17 | 0 | 0 | 0 | 0 | 0 |
| 18 | -5250 | -5250 | -5250 | -8100 | -8100 |
| 19 | 0 | 0 | 0 | 0 | 0 |
| 20 | -5550 | -4950 | -4950 | -16200 | -19200 |
| Total | -77250 | -71975 | -77350 | -113950 | -149950 |
| NPV at |  |  |  |  |  |
| 5% | -56.033 | -52.023 | -57.398 | -80.619 | -113.591 |
| 10% | -45.084 | -41.709 | -47.084 | -64.214 | -95.817 |

(The figures shown are not the real figures obtained by AMSA)