IALA Guideline No. XXXX

On

Comparison of Competing Buoy Technologies

Edition #

[Date approved by Council]

[Date of previous edition]

AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  www.iala-aism.org

Table of Contents

1	Introduction	5

2	Examples of different buoy technologies	5

3	Primary points for consideration	5

4	Financial analysis and the Net Present Value method	7

5	Notes on costs for use in NPV or other financial analysis	7

6	Examples	8

6.1	Example 1 – AMSA, Australia	9

6.2	Example 2 – Japan	12

6.3	Example 3 – Trinity House Lighthouse Service, United Kingdom	16

16

17

Appendix 1 - Advantages and Disadvantages of the use of large plastic buoys (1.8 to 3.0m diameter)	19

IALA Guidelines on Comparison of Competing Buoy Technologies

# Introduction

Aids to navigation services are faced with a choice of buoy technologies. These technologies employ various materials and manufacturing methods. A change of buoy technology might allow a service to reduce its operating costs. However the decision to change from the existing buoy technology to a different technology must be made with care. This paper sets out some points which should assist the decision making, and recommends the use of Net Present Value [NPV] analysis for financial evaluation.

# Examples of different buoy technologies

Various materials are used for the manufacture of floating AtoN buoys. These include

- Steel

- Aluminium

- Plastics

These materials may be used alone. They can also be used in combination to manufacture buoys which combine the benefits of different materials. For example:

- Steel buoy hull with aluminium tower

- Plastic buoy float with steel centre tube and aluminium tower

- All plastic hull and tower, with internal metal structural members

- Aluminium buoy with plastic foam filling within the hull

- GRP buoy hull with steel or aluminium tower

These are simply a few of the possible choices. The materials choices are influenced by the size of buoy required, and by the environmental conditions under which it must operate. Changing to a different technology may offer the AtoN operating authority a reduction in overall costs, via reduction in servicing costs or reduction in first costs.

This subject should also be extended to compare conventional buoys with fixed beacons (usually single pile) or spar buoys which are moored at the seabed with a universal joint or short chain. [These are know by various names, including “spar buoys”, “resilient beacons”, “buoyant beacons”, and “elastic beacons”.] This type of buoy can offer potential cost savings by reducing the frequency of mooring inspection and replacement (if local tidal regime and water depth allow the use of these alternatives). Refer to the Japan Coast Guard example  following.

# Primary points for consideration

If an Authority is to consider using a different buoy technology then the following questions should be addressed:

- AtoN signal – it is important to compare equal with equal

  - Is the new buoy big enough as a day-mark?

  - Is it big enough and stable enough to carry a lantern with the required range, and other required equipment [e.g. racon, AIS, monitoring, etc.], and the associated power system?

  - Is the motion of the buoy on station too rapid or of too great an angular displacement for the AtoN function to be clear to the mariner [e.g. vertical divergence]

  - Is the focal height sufficient for local sea conditions?

  - Can a suitable radar reflector be mounted?

  - In summary, is the new buoy equal to the old? If not there are two different questions to be answered.

    - Can the service to the user be reduced?

    - What is the best means of providing this new signal?

- Serviceability

  - Is it possible to connect a suitable mooring to the buoy?

  - Are the buoy’s motion and its stability acceptable for maintenance personnel to safely and efficiently work on the buoy?

  - On larger buoys, is there adequate safe access to the lantern?

  - Is the new buoy suitable for deployment and servicing by the authority’s existing or projected service vessels, or will new vessels or assets be required?

- Local conditions

  - If there are ice conditions, will the buoy be suitable?

  - If self coloured materials (usually plastic) are used, how long will the coloured surface last in the local UV environment?

  - Is the solar system design compatible with the latitude and local weather conditions?

- Service intervals

  - At what interval must the day-mark components be replaced in order to retain correct signal colours?

  - What are the inspection and maintenance intervals of parts such as mooring system, energy system, and lantern? You must be able to identify the critical items that will wear out and define the maintenance periods for the new buoy.

  - What are the shore-side maintenance requirements and frequencies [e.g. shot-blasting, painting, hull and tower repairs, etc.]?

- Purchase costs

  - What are the costs of the purchase of the existing and the new buoys?

  - What level of spares holding will be required for the new buoys?

- Maintenance costs

  - What time, resources, and personnel are required to maintain the existing and the new buoys – and how much does each cost?

  - Will there be any saving in vessel costs from using a different buoy?

  - Will there be a saving in vessel time at each servicing visit?

  - Will the number of servicing visits be reduced?

  - Can a different mooring type be used?

  - Will the new buoy mean a reduction in mooring component costs?

  - What are the costs of disposal?

  - What are the costs of repair? Usually, steel can be repaired “anywhere”. Structural steel specialists are widely available; aluminium repair may be difficult away from industrial centres, high specification paint systems for steel and aluminium require sophisticated surface preparation and application facilities, etc. Repairs to some plastics may require the contractor to have special tools and skills.

- Infrastructure changes

  - Will changing to the new buoy allow for elimination or reduction of the maintenance infrastructure [ships, shore facilities, personnel, etc.], or will it require the purchase of new facilities? For example, full cost advantage may only be possible if the entire buoy inventory can be changed from the old to the new, possibly allowing shore facilities to be closed down and smaller servicing vessels to be used.

# Financial analysis and the Net Present Value method

Financial analysis for the comparison of different buoy strategies should be used. The recommended method is Net Present Value [NPV] analysis. NPV is an approach used in capital budgeting where the present value of cash inflow is subtracted from the present value of cash outflows. It is accepted internationally as a suitable and popular method for making financial analysis to assist decision making, and often used for commercial decision making, where there are costs and income streams, and where the decision must be made whether to go ahead with a project.  Where estimated costs are used in the NPV calculation, a sensitivity analysis should be made to validate the NPV result. Explanation of  NPV theory is not given here. The reader is referred to the Internet or to textbooks, which can be found on the topic in most languages. An additional step in the financial analysis could be to compute annual costs, when comparing buoys which have very different service lives.

For most buoy decision analysis there will be the comparison of costs, but income could arise from sales of redundant items, or from marketing ‘spare’ ship time.

# Notes on costs for use in NPV or other financial analysis

The following are some points to consider when making a financial analysis. The list is not all-inclusive, and the Authority may have specific local issues which the analysis must also consider.

- Discount rate and study period

  - Set the discount rate in accordance with your organisation’s accounting procedures.

  - The study period should normally be the life of the longer-lived buoy

- Costs

  - Previously-spent sums [sunk costs] must not be included – this is a common accounting convention

  - Existing buoys, with remaining life, should be treated as sunk costs; however consider in what year they must be replaced, repaired, etc.

  - Take account of the costs of any staff training related to the use and maintenance of the new buoy

  - Occupational safety and health

    - Are there any special costs, e.g. special clothing, etc.?

    - Do staff need special safety and health training? What will this cost?

  - Disposal costs and environmental costs

    - For disposing of certain scrap, there may be disposal charges

    - Environmental regulations may require complete re cycling of new buoys. This may influence choice of new product

    - Take account of any costs for special facilities relating to disposal of materials [e.g. solvents, scrap plastic]

  - If staff reduction is being considered, the related costs must be included

- Income

  - What accounting rules related to income are used in the service, and how do these affect the financial analysis?

  - Disposal income from scrap, or disposal of un-required assets

    - In some cases, the sale of scrap, e.g. steel, will be income in the NPV study

    - Surplus facilities may also generate income

  - Existing service vessels, facilities, staff, and crews

    - Only count genuine income opportunities

    - Income from selling ship time must be treated carefully. How secure is this income stream? Conservative figures should be used in the financial analysis. Will this income benefit the authority, or will it accrue elsewhere?

# Examples

Examples of application of the above methodology are included as annexes.

- Australian Maritime Safety Authority (AMSA) compared conventional steel buoys with plastic buoys

  - AMSA decided to retain its conventional steel buoys

- Japan Coast Guard (JCG) compared conventional steel buoys with resilient beacons

  - At early 2005, JCG had converted 105 of its approximately 1,450 steel buoys to resilient beacons, and plans to convert approximately a further 300

- Trinity House Lighthouse Service, United Kingdom (THLS) compared conventional steel buoys with lightweight buoys

  - THLS decided to convert all 44 of its 1.5 to 2 metre diameter buoys (THLS class 3, 4, and 5 buoys) to lightweight buoys

## Example 1 – AMSA, Australia

AMSA has recently performed a cost benefit analysis of options for replacement buoys in Australia taking into consideration a number of presently available plastic alternatives to their current steel buoys. The analysis resulted in a decision to continue with steel buoys for the next five years.

Whilst the decision to stick with steel buoys is very interesting in itself, the methodology used to arrive at this decision is even more interesting for other AtoN authorities.

The approach used for evaluating the capital cost savings was as follows:

- Select which scenarios to analyze

- Define any basic assumptions made for the analysis

- Select a timeframe for the analysis

- Calculate the initial replacement costs

- Calculate the maintenance costs each year

- Calculate the Net Present Value (NPV) of all costs within the selected timeframe

- Compare the NPV of each scenario

The following scenarios where selected for the AMSA analysis:

- Continue to use the current design of buoys of steel construction

- Change to plastic construction buoys which have reduced maintenance requirements.

The following basic assumptions where made for the analysis:

- Costs common to all scenarios can be omitted

- Maintenance cycle of steel hull buoys requires purchase of 1.5 times the number in service

- Maintenance cycle of plastic hull buoys requires purchase of 1.25 times the number in service

A time frame of 20 years was selected.

The following factors where taken into consideration for calculating the initial costs:

- Unit Cost

- Maintenance Spares (proportion in store)

- Installation components/fittings

- Cost of ship time for installation

While calculating the maintenance costs each year, the following factors where considered:

- Minor maintenance costs at interval of X years

- Major maintenance costs at interval of Y years

- Buoy replacement costs at interval of Z years (buoy lifetime)

In each case the costs where split into the cost of ship time and other costs (materials and services).

Comparing the NPV of each scenario revealed what potential capital cost savings could be obtained for each scenario relative to the present scenario.

A spreadsheet example of this basic NPV methodology is found at the end of this example.

The above mentioned factors can more or less be characterized as direct cost drivers. In addition to these there may be a number of indirect cost drivers such as workshop facilities that may be affected the alternative scenarios.

The NPV example submitted had been used for a broad strategy paper on floating aids, hence the number of comparison options.  For further work it would be appropriate to reduce the number of columns.

Re the box for ‘Initial Costs’:

- The ‘unit cost’ is the purchase price (new or used) of the buoy – for AMSA, this would typically be the bare hull and tower.

- For AMSA, the ‘installation components and fittings’ are the beacon, solar power supply top mark and fitting the mooring.

The composition of unit cost and installation components and fittings may vary for other authorities as they may purchase buoys ready for installation or installed as a ‘turn-key’ solution.

Re the box for ‘Maintenance Costs’:

- The ‘other costs’ for minor maintenance covers the replacement of the mooring chain and shackles (on site).

- The major maintenance ‘other costs’ cover the refurbishment of a chain buoy following change out.  The columns for Buoy C and D are for spar buoys and in this case the maintenance was envisaged as being replacement of anodes and touch-up painting.  (Change out cost estimates for spar buoys were identified separately).

The NPV result at the bottom of the page is actually the sum of the initial Cost + NPV (annual maintenance costs).

The end of life scrap values of buoys have been ignored in the AMSA NPV example as they are generally negligible.

AMSA example continued

| Item | Current Steel Buoy | Buoy A | Buoy B | Buoy C | Buoy D |
| --- | --- | --- | --- | --- | --- |
| Initial Costs |  |  |  |  |  |
| Unit Cost | 9600 | 9700 | 14000 | 25000 | 55000 |
| Maintenance Spares (proportion in store) | 0,5 | 0,25 | 0,25 | 0 | 0 |
| Installation components/fittings | 6300 | 6300 | 6300 | 1800 | 1800 |
| Ship time for installation (days) | 0,5 | 0,5 | 0,5 | 2 | 2 |
| Installation costs | 2550 | 2550 | 2550 | 10200 | 10200 |
| Daily Ship Cost | 5100 |  |  |  |  |
| Maintenance Costs |  |  |  |  |  |
| Minor maintenance costs at interval of:- (years) | 2 | 2 | 2 | 2 | 2 |
| Ship Time (days) | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 |
| Ship Cost | 2550 | 2550 | 2550 | 2550 | 2550 |
| Other Costs (materials & services) | 2700 | 2700 | 2700 | 1500 | 1500 |
| Major maintenance costs at interval of:- (years) | 4 | 4 | 4 | 6 | 6 |
| Ship Time (days) | 0,5 | 0,5 | 0,5 | 1 | 1 |
| Ship Cost | 2550 | 2550 | 2550 | 5100 | 5100 |
| Other Costs (materials & services) | 3000 | 2400 | 2400 | 3000 | 3000 |
| Buoy Changeout costs at interval of:- (years) |  |  |  | 10 | 10 |
| Ship Time (days) |  |  |  | 2 | 2 |
| Ship Cost |  |  |  | 10200 | 10200 |
| Other Costs (materials & services) |  |  |  | 6000 | 9000 |
| Year | Current | Buoy A | Buoy B | Buoy C | Buoy D |
| 0 | -23250 | -20975 | -26350 | -37000 | -67000 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | -5250 | -5250 | -5250 | -4050 | -4050 |
| 3 | 0 | 0 | 0 | 0 | 0 |
| 4 | -5550 | -4950 | -4950 | -4050 | -4050 |
| 5 | 0 | 0 | 0 | 0 | 0 |
| 6 | -5250 | -5250 | -5250 | -8100 | -8100 |
| 7 | 0 | 0 | 0 | 0 | 0 |
| 8 | -5550 | -4950 | -4950 | -4050 | -4050 |
| 9 | 0 | 0 | 0 | 0 | 0 |
| 10 | -5250 | -5250 | -5250 | -16200 | -19200 |
| 11 | 0 | 0 | 0 | 0 | 0 |
| 12 | -5550 | -4950 | -4950 | -8100 | -8100 |
| 13 | 0 | 0 | 0 | 0 | 0 |
| 14 | -5250 | -5250 | -5250 | -4050 | -4050 |
| 15 | 0 | 0 | 0 | 0 | 0 |
| 16 | -5550 | -4950 | -4950 | -4050 | -4050 |
| 17 | 0 | 0 | 0 | 0 | 0 |
| 18 | -5250 | -5250 | -5250 | -8100 | -8100 |
| 19 | 0 | 0 | 0 | 0 | 0 |
| 20 | -5550 | -4950 | -4950 | -16200 | -19200 |
| Total | -77250 | -71975 | -77350 | -113950 | -149950 |
| NPV at |  |  |  |  |  |
| 5% | -56.033 | -52.023 | -57.398 | -80.619 | -113.591 |
| 10% | -45.084 | -41.709 | -47.084 | -64.214 | -95.817 |

(The figures shown are not the real figures obtained by AMSA)

## Example 2 – Japan

## Example 3 – Trinity House Lighthouse Service, United Kingdom

Appendix 1 - Advantages and Disadvantages of the use of large plastic buoys (1.8 to 3.0m diameter)

Advantages:

- The plastic does not corrode.

- Less maintenance on station; only removal of marine growth, no painting on station.

- Less maintenance on shore in the workshops for the plastic component (no grid blasting, no paint system of 3 layers or more) therefore fewer personnel and smaller workshops, lower personnel and equipment costs.

- Lower weight (1/2 to 1/3 of the weight of steel buoys in the diameter range 1.8 – 3.0 m) therefore the service can use smaller buoy tender.

- CETMEF and Rijkswaterstaat have more than 10 years experience of operating rotationally moulded polyethylene (PE) plastic buoys in the 1.8 – 3.0 m diameter size range.

- At least 6 commercial companies offer large rotationally moulded plastic buoys.

- Linear PE is recyclable (if not foam filled with polyurethane).

- It is possible to hot-flame-spray PE if the surface colour needs to be changed, but this is not necessary in many cases.

- Large plastic buoys are typically of modular construction. It is therefore possible to change individual body segments if they are damaged.

- Easier to transport and store the sections of the body when compared with conventional single component bodies.

- It is possible to install a radar reflector within a plastic buoy’s superstructure.

- The superstructure can be rotationally moulded.

Disadvantages:

- A large plastic buoy will usually contain metal components such as the tail tube, mooring-eyes, lifting-eyes, connecting bolts, lantern-mounting, etc. These metal items will still need maintenance.

- Less weight will result in more rapid motion on station when compared with the heavier steel buoy.

- Repairs are not simple or may often not be possible: it is more difficult to weld PE than steel. It is very difficult to monitor the weld quality.

- There is little experience in ice conditions or in very cold waters although Sweden has some good experience with PE spar-buoys in ice conditions.

- It is more difficult to change the colour of a plastic buoy, as conventional painting is not reliable. This may increase the number of spares needed, compared with steel buoys, i.e. red, green, black and yellow spares may have to be carried rather than re-painting steel components.

- Shorter lifetime than steel buoys –PE lifetimes are estimated to be better than 10 years compared with proven steel lifetimes often considerably in excess of 30 years.

- Less resistance to collision damage than steel.

- Plastic buoy components will be specific to each manufacturer compared to steel components that can be manufactured by any competent steel fabricator.

*****