From:  EEP (EEP6)						EEP6/output/04

## To: ANM (ANM6)							Date: April 5, 2005

# Liaison Note

Colour Signal Standard for Blue Lights

# Introduction

The EEP Committee noted paper EEP6/6i/3 (formerly ANM5/7/1) requesting EEP to specify a chromaticity region for Blue Lights.

- Background

ANM discussion indicated that many administrations would like to use Blue Lights, and would like to ensure all relevant information is taken into account in developing the Guidelines.

- Considerations

The EEP Committee felt that there are a number of issues that should be considered when using blue lights as Aton:

- Blue is not a suitable colour for long distance visual signals

- The eye is relatively insensitive to blue light

- Blue and green are confused by normal observers for signals that have small angular subtense or a low illuminance and particularly when used at the limit of visible range

- Blue is preferentially scattered by the atmosphere. Blue light is scattered by the air molecules roughly 16 times more than red light.

- Blue light has a tendency to penetrate into the sea, with the result that the light is absorbed rather than reflected from the sea surface.

- The rhythmic characters of marine AtoN blue lights could be restricted to steady lights or long ‘on’ periods to further reduce the risk of colour confusion and to avoid confusion with lights exhibited by emergency services.

- In addition to addressing problems arising from background lighting, street lights and dock-side industries, blue lights could be used instead of lateral mark lights to mark the ends of piers.

Research suggests that blue light is best used in conjunction with its reciprocal colour yellow. Some testing has shown that better conspicuity is achieved if blue / yellow lights are alternatively flashed when viewed within luminous range.

- Requested Action

The colour blue should be highly saturated and chosen with chromaticity coordinates as far from green and white as is possible. An ad-hoc Group has been formed to review and update recommendations on lights, including Recommendation on the Colours of Light Signals on Aids to Navigation, Dec 1977. This review will include recommendations regarding the chromaticity co-ordinates for blue lights. It is expected that the Ad-hoc Group will complete this work by EEP7.

The ANM Committee is invited to take the comments on blue lights into account as they develop guidelines for the use of blue lights in aids to navigation.