| From: EEP6 | EEP6/output/05 |
| --- | --- |
| To: ANM6 | 6 April 2005 |

Liaison Note

Graphical Representation of Rhythmic Lights

- Introduction

The EEP Committee has developed a graphical representation of rhythmic lights for inclusion in the IALA Navguide.

- Comments

In preparing this representation, the EEP Committee has removed the references to ‘Interrupted’ lights.  This class of light is not included in the graphic representation.

- Requested Action

The ANM Committee is invited to include the graphic represetnation of lights in the NAVGUIDE, noting the fact that references to ‘Interrupted’ lights has been removed.

| CLASSIFICATION OF THE RHYTHMIC CHARACTERS OF LIGHTS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Class | Abbreviation | General description | IALA Specification |  |  | Particular use in the IALA Maritime Buoyage System |
| 1 | FIXED LIGHT | F | A light showing continuously and steadily. | A single fixed light should be used with care because it may not be recognized as an aid to navigation light. |  |  | A single fixed light shall not be used. |
| 2 | OCCULTING LIGHT |  | A light in which the total duration of light in a period is longer than the total duration of darkness and the intervals of darkness (eclipses) are usually of equal duration. | A light in which the total duration of light in a period is clearly longer than the total duration of darkness and all the eclipses are of equal duration. |  |  |  |
| 2.1 | Single-occulting light | Oc | An occulting light in which an eclipse is regularly repeated | The duration of an appearance of light should not be less than three times the duration of an eclipse. The period should not be less than 2 s |  |  | A single-occulting White light indicates a safe-water mark. |
|  |  |  |  |  | Example: l = 3 s; d = 1 s; p = 4 s | 1 ≥ 3 d p ≥ 2 s |  |
| 2.2 | Group-occulting light | Oc(#) eg. Oc(2) | An occulting light in which a group of eclipses, specified in number, is regularly repeated. | The appearances of light between the eclipses in a group are of equal duration, and this duration is clearly shorter than the duration of the appearance of light between successive groups. The number of eclipses in a group should not be greater than four in general, and should be five only as an exception. The duration of an appearance of light within a group should not be less than the duration of an eclipse. The duration of an appearance of light between groups should not be less than three times the duration of an appearance of light within a group. In a group of two eclipses, the duration of an eclipse together with the duration of the appearance of light within a, group should not be less than 1 s. In a group of three or more eclipses, the duration of an eclipse together with the duration of an appearance of light within the group should not be less than 2 s. |  |  | A group-occulting Yellow light indicates a special mark. |
|  |  |  |  | Oc(2) | Example: l’ = 6 s; l = 2 s; d = 1 s; c = 3 s; p = 10 s | l' ≥ 3 l 1 ≥ d c ≥ 1 s |  |
| 2.3 | Composite group-occulting light | Oc(# + #) eg.Oc(2 + 1) | A light similar to a group-occulting light except that successive groups in a period have different numbers of eclipses. | This class of light character is not recommended because it is difficult to recognize. |  |  |  |
|  |  |  |  | Oc(2+1) | Example: l’’ = 9 s; l’ = 3 s; l = 1 s; d = 1 s; c = 2 s; p = 16 s | 1’’ ≥ l’ l’ ≥ 3 l l ≥ d c ≥ 1 s |  |
| 3 | ISOPHASE LIGHT | Iso | A light in which all the durations of light and darkness are clearly equal. | The period should never be less than 2 s, but preferably it should not be less than 4 s in order to reduce the risk of confusion with occulting or flashing lights of similar periods. |  |  | An isophase White light indicates a safe-water mark. |
|  |  |  |  |  | Example: l = d = 2 s; p = 4 s | l = d p ≥ 2 s |  |
| 4 | FLASHING LIGHT |  | A light in which the total duration of light in a period is shorter than the total duration of darkness and the appearances of light (flashes) are usually of equal duration. | A light in which the total duration of light in a period is clearly shorter than the total duration of darkness and all the flashes are of equal duration. |  |  |  |
| 4.1 | Single-flashing light | Fl | A flashing light in which a flash is regularly repeated (at a rate of less than 50 flashes per minute). | The duration of the interval of darkness (eclipse) between two successive flashes should not be less than three times the duration of a flash. The period should not be less than 2 s (or not less than 2.5 s in those countries where a quick rate of 50 flashes per minute is used). |  |  | A single-flashing Yellow light indicates a special mark. |
|  |  |  |  |  | Example: d = 3 s; l = 1 s; p = 4 s | d ≥ 3 l p ≥ 2 s |  |
| 4.2 | Long-flashing light | LFI | A single-flashing light in which an appearance of light of not less than 2 s duration (long flash) is regularly repeated. |  | Example: d = 8 s; l = 2 s; p = 10 s | d ≥ 3 l l ≥ 2 s | A long-flashing White light with a period of 10 s indicates a safe-water mark. |

| CLASSIFICATION OF THE RHYTHMIC CHARACTERS OF LIGHTS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Class | Abbreviation | General description | IALA Specification |  |  | Particular use in the IALA Maritime Buoyage System |
| 4.3 | Group-flashing light | Fl(#) eg. Fl(2) | A flashing light in which a group of flashes, specified in number, is regularly repeated. | The eclipses between the flashes in a group are of equal duration, and this duration is clearly shorter than the duration of the eclipse between successive groups. The number of flashes in a group should not be greater than five in general, and should be six only as an exception. The duration of an eclipse within a group should not be less than the duration of a flash. The duration of an eclipse between groups should not be less than three times the duration of an eclipse within a group. In a group of two flashes, the duration of a flash together with the duration of the eclipse within the group should not be less than 1 s. In a group of three or more flashes, the duration of a flash together with the duration of an eclipse within a group should not be less than 2 s (or not less than 2.5 s in those countries where a quick rate of 50 flashes per minute is used). |  |  | A group-flashing White light with a group of two flashes, in a period of 5 s or 10 s, indicates an isolated-danger mark. A group-flashing Yellow light with a group of four, five or (exceptionally) six flashes indicates a special mark |
|  |  |  |  | Fl(2) | Example: d’ = 6 s; d = 2 s; l = 1 s; c = 3 s; p = 10 s | d’ ≥ 3 d d ≥ l c ≥ 1 s |  |
| 4.4 | Composite group-flashing light | Fl(# + #) eg Fl(2 + 1) | A light similar to a group-flashing light except that successive groups in a period have different numbers of flashes. | Light characters should be restricted to (2 + 1) flashes in general, and should be (3 + 1) flashes only as an exception. |  |  | A composite group-flashing Red or Green light with a group of (2 + 1) flashes indicates a modified lateral (preferred-channel) mark. A composite group-flashing Yellow light indicates a special mark. |
|  |  |  |  | Fl(2+1) | Example: d’’ = 9 s; d’ = 3 s; d = 1 s; l = 1 s; c = 2 s; p = 16 s | d’’ ≥ d’ d’ ≥ 3 d d ≥ l c ≥ 1 s |  |

| CLASSIFICATION OF THE RHYTHMIC CHARACTERS OF LIGHTS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Class | Abbreviation | General description | IALA Specification |  |  | Particular use in the IALA Maritime Buoyage System |
| 5 | QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 50 flashes per minute but less than 80 flashes per minute. | A light in which identical flashes are repeated at the rate of 60 (or 50) flashes per minute. The higher rate of flashing is preferred. |  |  |  |
| 5.1 | Continuous quick light | Q | A quick light in which a flash is regularly repeated. |  | Example: l = d = 0.5 s; p = 1 s | d ≥ l l s ≤ p ≤ 1.2 s | A continuous quick White light indicates a north cardinal mark. |
| 5.2 | Group quick light | Q(#) eg Q(3) eg Q(9) eg Q(6) +LFl | A quick light in which a specified group of flashes is regularly repeated. | The number of flashes in a group should be three or nine. An exceptional light character is reserved for use in the IALA Maritime Buoyage System to indicate a south cardinal mark. |  |  |  |
|  |  |  |  | Q(3) | Example: d’ = 7.5 s; l = d = 0.5 s; c = 1 s ; p = 10 s | d ≥ l d’>d 1 s ≤ c ≤ 1.2 s | A group quick White light with a group of three flashes, in a period of 10 s, indicates an east cardinal mark. |
|  |  |  |  | Q(9) | Example: d’ = 6.5 s; l = d = 0.5 s; c = 1 s; p = 15 s | d ≥ l d’>d 1 s ≤ c ≤ 1.2 s | A group quick White light with a group of nine flashes, in a period of 15 s, indicates a west cardinal mark. |
|  |  |  |  | Q(6)+LF1 | Example: d’ = 7 s; l’ = 2 s; l = d = 0.5 s; c = 1 s.; p = 15 s | d' ≥ 3 l’ l’ ≥ 2 s d ≥ l 1 s ≤ c ≤ 1.2 s | A group quick White light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 15 s, indicates a south cardinal mark. |

| CLASSIFICATION OF THE RHYTHMIC CHARACTERS OF LIGHTS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Class | Abbreviation | General description | IALA Specification |  |  | Particular use in the IALA Maritime Buoyage System |
| 6 | VERY QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 80 flashes per minute but less than 160 flashes per minute. | A light in which identical flashes are repeated at the rate of 120 (or 100) flashes per minute. The higher rate of flashing is preferred. |  |  |  |
| 6.1 | Continuous very quick light | VQ | A very quick light in which a flash is regularly repeated. |  | Example: l = d = 0.25 s; p = 0.5 s | d ≥ l 0.5 s ≤ p ≤ 1.6 s | A continuous very quick White light indicates a north cardinal mark. |
| c | Group very quick light | VQ(#) eg VQ(3) eg VQ(9) eg VQ(6)+LFl | A very quick light in which a specified group of flashes is regularly repeated. | The number of flashes in a group should be three or nine. An exceptional light character is reserved for use in the IALA Maritime Buoyage System to indicate a south cardinal mark. |  |  |  |
|  |  |  |  | VQ(3) | Example: d’ = 3.75 s; l = d = 0.25 s; c = 0.5 s; p = 5 s | d' ≥ 1,5 s d ≥ l 0.5 s ≤ c ≤ 0.6 s | A group very quick White light with a group of three flashes, in a period of 5 s, indicates an east cardinal mark. |
|  |  |  |  | VQ(9) | Example: d’ = 5.75 s; l = d = 0.25 s; c = 0.5 s; p = 10 s | d' ≥ 1.5 s d ≥ l 0.5 s ≤ c ≤ 0.6 s | A group very quick White light with a group of nine flashes, in a period of 10 s, indicates a west cardinal mark. |
|  |  |  |  | VQ(6)+LF1 | Example: d’ = 5 s; l’ = 2 s; l = d = 0.25 s; c = 0.5 s; p = 10 s | d' ≥ 1.5 l’ l’ ≥ 2 s d ≥ l 0.5 s ≤ c ≤ 0.6 s | A group very quick White light with a group of six flashes followed by a long flash of not less than 2 s duration, in a period of 10 s, indicates a south cardinal mark. |

| CLASSIFICATION OF THE RHYTHMIC CHARACTERS OF LIGHTS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Class | Abbreviation | General description | IALA Specification |  |  |  | Particular use in the IALA Maritime Buoyage System |
| 7 | ULTRA QUICK LIGHT |  | A light in which flashes are repeated at a rate of not less than 160 flashes per minute. | A light in which flashes are repeated at a rate of not less than 240 flashes per minute and not more than 300 flashes per minute. |  |  |  |  |
| 7.1 | Continuous ultra quick light | UQ | An ultra quick light in which a flash is regularly repeated. |  |  |  |  |  |
| 8 | MORSE CODE LIGHT | Mo(#) eg. Mo(A) | A light in which appearances of light of two clearly different durations are grouped to represent a character or characters in the Morse Code. | Light characters should be restricted to a single letter in the Morse Code in general, and should be two letters only as an exception. The duration of a "dot" should be about 0.5 s, and the duration of a "dash" should not be less than three times the duration of a "dot". |  |  |  | A Morse Code White light with the single character "A" indicates a safe-water mark. A Morse Code Yellow light, but not with either of the single characters "A" or "U"*, indicates a special mark. |
|  |  |  |  | Mo(A) | Example: l’ = 1.5 s; l = 0.5 s; d = 0.5 s; d’ = 4.5 s; p = 7 s |  | l' ≥ 3 l d ≥ l l = 0.5 s |  |
| 9 | FIXED AND FLASHING LIGHT | FFl | A light in which a fixed light is combined with a flashing light of' higher luminous intensity. | This class of light character should be used with care because the fixed component of the light may not be visible at all times over the same distance as the rhythmic component. |  |  |  |  |
|  |  |  |  |  |  | Example: d = 3 s; l = 1 s; p = 4 s | d ≥ 3 l l ≤ 1 s |  |
| 10 | ALTERNATING LIGHT | Al## eg A1WR | A light showing different colours alternately. | This class of light character should be used with care, and efforts should be made to ensure that the different colours appear equally visible to an observer. |  |  |  |  |
|  |  |  |  | AlWR | Example: l = d = 2 s; p = 4 s |  | l  d |  |

The term “long flash”, which is used in the descriptions of the long-flashing light and of the light characters reserved for south cardinal marks, means an appearance of light of not less than 2 seconds duration. The term “short flash” is not commonly used and does not appear in the Classification. If an Authority requires discrimination between two flashing lights that only differ in having flashes of different durations, then the longer flash should be described as “long flash” and be of not less than 2 seconds duration, and the shorter flash may be described as “short flash” and should be of not more rhythmic character of such a light is than one third of the duration of the longer flash.