| From: EEP | ## EEP6-output / 06 |
| --- | --- |
| #### To: ANM | ## 5th April 2005 |

Note

from the IALA EEP Committee to

The IALA ANM Committee

Marking of Emergency Wreck Marks

# Introduction

China Maritime Safety Administration (CHINA MSA for short), has proposed a new type of Emergency Wreck Marks using light of Yellow alternating with Blue, daymark colour of Yellow alternating with Blue, and with a blue and a yellow topmark ball (a total of 2 topmark balls).  (ANM6 input paper number reference)

# Engineering Considerations

The EEP Committee offer the following comments regarding this proposal.

Horizontal blue and yellow bars could be confused as black and yellow bars on a cardinal buoy in poor light conditions at long range. While the shape of the topmark may help to define the type of buoy mark, the use of vertical colour bands may also help to avoid confusion.

In poor visibility daylight conditions, yellow penetrates better than blue. This can result in the yellow topmark ball being visible while the blue is no longer visible. However if the surrounding background is dark, yellow can be seen better than blue due to contrast. Similarly, if the background is bright, blue can be seen better than yellow. Consequently colour in the topmark can be deceptive under certain conditions.  Topmarks are often seen in silhouette and colours may not be visible.  The silhouette of two topmark balls could be confused with existing topmark for isolated danger buoy.

Blue light is preferentially scattered by the air molecules roughly 16 times more than red light. This may result in the blue light being no longer visible at longer range while the yellow is still visible.

When using blue and yellow alternating flashing light with no eclipse (1.0+1.0) for Emergency Wreck Marks, the light rhythm is not very pronounced at ranges more than 1.2 nautical miles due to the lack of dark eclipse intervals. It is therefore suggested that the light character of 0.5(yellow) +0.5(dark) +0.5(blue) +0.5(dark) be adopted.

If the precise location of a light is lost, blue light is detected before yellow due to the sensitivity of peripheral scotopic vision of the human eye to blue light (night time vision).

# Requested Action

EEP request ANM to take account of these issues when considering the use of blue and yellow lights and daymarks for wreck marking.