| From: EEP (EEP6) | EEP6/output/07 |
| --- | --- |
| To: Ad Hoc Group on Photometry and Colour | April 7, 2005 |

Liaison Note

Draft document – use of lights as marine aids to navigation signals

Introduction

After IALABATT/IALALITE5, an ad hoc group on Photometry and Colour was formed.  The EEP Committee is providing input to the ad hoc group as requested.

Background [as required]

The EEP Committee reviewed EEP6 input paper EEP6/6k/info2.  The Committee feels that this is an appropriate outline and provided amendments.  The revised document is attached as Annex A.

Action Requested

The ad hoc group on Photometry and Colour is invited to use the attached annex as an outline as they continue their work.

AISM Association of Internationale de Signalisation Maritime                IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

Annex A

IALA Recommendation E-###

On

Use of Lights as Marine Aids to Navigation Signals

DRAFT

Edition 1

April 2005

20ter, rue Schnapper     -     78100     Saint Germain en Laye        -       France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05 Telex 695499 ialaism f

E-mail          :         iala-aism@wanadoo.fr      Internet      :      hjttp://iala-aism.org

EEP6/WG4/WP4

Use of Lights as Marine Aids to Navigation Signals

1	Objective	4

2	Scope	4

3	Definitions	4

4	Characteristics of Lights as Marine Aids to Navigation Signals	4

5	Rhythmic Character	5

5.1	Rhythmic characters of lights on aids to navigation	5

6	Colour	5

6.1	Spectral characteristics (Colours) of light signals on aids to navigation	5

7	Range	5

7.1	Determination of the luminous intensity of a marine aid to navigation light	5

7.2	Calculation of the effective intensity of a rhythmic light	5

7.3	Notation of luminous intensity and range of lights	5

7.4	Definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day	5

8	Metrology	5

8.1	Photometry of marine aids to navigation signal lights	5

8.2	Colourimetry (Measurement of colour)	5

Use of Lights as Marine Aids to Navigation Signals

(Recommendation E-###)

# Objective

The objective of this standard is to provide an approved methodology to promote uniformity in determining and reporting the performance of marine aids-to-navigation signal lights.

# Scope

Lights are used throughout the world as Aids to Navigation signal lights to assist mariners to navigate safely in all weathers. This recommendation sets out the characteristics and method of design of lights when used as marine Aids to Navigation.

The recommendation incorporates and supercedes the following previous recommendations:

IALA Recommendation for the colours of light signals on aids to navigation, Dec. 1977

IALA Recommendation on the determination of the luminous intensity of a marine aid to navigation light, Dec. 1977

IALA Recommendation on the calculation of the effective intensity of a rhythmic light, Nov. 1980

IALA Recommendation E-122 on the photometry of marine aids to navigation signal lights, October 2004

IALA Recommendation for the notation of luminous intensity and range of lights, Nov 1966

IALA Recommendation for a definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day. April 1974

IALA Recommendation for the colours of light signals on aids to navigation. December 1977

IALA Recommendation E110 for the rhythmic characters of lights on aids to navigation. May 1998

# Definitions

# Characteristics of Lights as Marine Aids to Navigation Signals

When using lights as Aids to Navigation, the lights must be visible within the hazard area and must be uniquely identifiable. The following are the essential characteristics that define a light when used to assist marine navigation.

- Range. Determines the area over which the light can be used by mariners.

- Rhythmic Character. Allows a light to be uniquely identified.

- Colour. Defines some characteristics of the marked hazard and helps to uniquely identify a light.

Each of these characteristics may be considered under three headings – desired performance, prediction of performance, measurement of performance.

# Rhythmic Character

## Rhythmic characters of lights on aids to navigation

IALA Recommendation E-### Part 5.

# Colour

## Spectral characteristics (Colours) of light signals on aids to navigation

IALA Recommendation E-### Part 7.

# Range

## Determination of the luminous intensity of a marine aid to navigation light

IALA Recommendation E-### Part 1.

## Calculation of the effective intensity of a rhythmic light

## Notation of luminous intensity and range of lights

IALA Recommendation E-### Part 3.

## Definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day

# Metrology

## Photometry of marine aids to navigation signal lights

IALA Recommendation E-### Part 6.

## Colourimetry (Measurement of colour)