Product Template 1 – Lanterns for buoys and light-beacons, including enclosed rotating beacons

# Purpose

This product template identifies the parameters that should be defined and the standards against which each parameter should be tested to enable a realistic comparison to be made between competing products.

# Scope

The template is applicable for lanterns used on buoys and light beacons including rotating beacons, powered via external power supplies or via integrated solar power supplies.

## Product Template 1 – Lanterns for buoys and light-beacons, including enclosed rotating beacons

| Ref. No | Parameter category | Parameter | Test Method | Comments |
| --- | --- | --- | --- | --- |
| 1.1 | Optical | Effective luminous intensity | IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’ |  |
| 1.2 |  | Flash duration | IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’ | Time between points of 50% peak intensity. Define stability of rotation for a rotating beacon. |
| 1.3 |  | Horizontal beam uniformity | IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’ | State maximum variation from the mean intensity over of any angle in the horizontal plane. Better than 60% peak intensity. |
| 1.4 |  | Vertical divergence | IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’ | Arc through the centre of the lantern in the vertical plane, between 50% points of peak intensity. State minimum divergence. |
| 1.5 |  | Signal colour |  | Colour boundaries in the preferred region as defined in IALA Recommendation ??? (1977) ‘Recommendation for the colours of light signals’ using the CIE 1931 chromaticity coordinates chart. |
| 1.6 |  | Nominal range |  | IALA Recommendation for the notation of luminous intensity and range of lights. (1966) State applicable atmospheric transmission factor |
| 1.7 |  | Lens Material |  |  |
| 1.8 |  | Filter material |  | Colour boundaries in the preferred region as defined in IALA Recommendation ??? (1977) ‘Recommendation for the colours of light signals’ using the CIE 1931 chromaticity coordinates chart |
| 2.1 | Electrical – externally sourced | Power supply normal and extreme voltage | IEC 600945 section 7 | State voltage tolerance |
| 2.2 |  | Power consumption |  |  |
| 2.3 |  | Reverse polarity circuit protection | IEC 600945 section 7.2 |  |
| 2.4 |  | Power supply and control and monitoring terminations/connectors |  | Define connector and/or terminal type |
| 3.1 | Electrical – integrated solar | Solar panel type |  | State type – mono-crystalline; poly crystalline; |
| 3.2 |  | Peak Power |  |  |
| 3.3 |  | Battery type |  |  |
| 3.4 |  | Nominal voltage |  |  |
| 3.5 |  | Battery Ah capacity |  | Define discharge rate. |
| 3.6 |  | Autonomy |  | Minimum no sun operational hours for defined character and range. |
| 3.7 |  | Limiting Latitude range |  | Range within which operational parameters are applicable. |
| 4.1 | Control | Daylight control |  | Lux level switching range |
| 4.2 |  | Monitoring |  |  |
| 4.3 |  | Programming |  |  |
| 4.4 |  | Light source regulation |  |  |
| 5.1 | Physical | Maximum Height |  |  |
| 5.2 |  | Maximum Diameter |  |  |
| 5.3 |  | Maximum Weight |  |  |
| 5.4 |  | Focal Height |  |  |
| 5.5 |  | Materials |  |  |
| 5.6 |  | Finish |  |  |
| 5.7 |  | Ingress Protection |  | Define applicable IP number to EN60529 |
| 6.1 | Environmental | Temperature – operational and storage |  |  |
| 6.2 |  | Humidity – operational and storage | IEC 600945 section 8.3; IEC 60068-2-3; JIS C 5024; MIL-STD-202G-103B |  |
| 6.3 |  | Immersion | IEC 600945 section 8.9; JIS C 0920; MIL-STD-202G-104A |  |
| 6.4 |  | Salt air and sea water spray | IEC 600945 section 8.12; MIL-STD-202G-101E |  |
| 6.5 |  | Ultraviolet exposure |  |  |
| 6.6 |  | Shock and vibration | IEC 600945 section 8.7; JIS C 0911; MIL-STD-202G-201A/202D |  |
| 6.7 |  | Electromagnetic interference | IEC 600945 sections 9 & 10 |  |