PRODUCT CERTIFICATION TEMPLATE 1

Lanterns for buoys and light-beacons, including enclosed rotating beacons

# Purpose

Product templates identify parameters that should be tested and the standards against which each parameter should be tested to enable a comprehensive evaluation of products.

# Scope

The template is applicable for lanterns used on buoys and light beacons including rotating beacons, powered via external power supplies or via integrated solar power supplies.

Applicable IALA Recommendations and Guidelines

- IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’

- IALA Recommendation E-200-1 (2005), Part 1 – Colour

- IALA Guide To Ambient Light Levels At Which ATON Lights Should Switch On And Off.(No. 1038)

# Test Standards

The Templates quote international standards. If an IALA Product Certification Authority (IALA-PCA) wishes to use an alternative standard, it is the responsibility of the IALA-PCA to confirm that the standard used conforms to the IALA quoted standard. Where no test standard is mentioned the manufacturer should state the test standard used, preferably with reference to international or national standards.

## Product Certification Template 1 – Lanterns for buoys and light-beacons, including enclosed rotating beacons

Product Type / Manufacturer Reference Number:

| Ref. No | Parameter category | Parameter | Measured Value | Test Method | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | ### Optical |  |  |  |  |
| 1.1 |  | Effective luminous intensity |  | IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’ | For LEDs the effective luminous intensity varies depending on duty cycle |
| 1.2 |  | Flash duration and flash shape (LED lights included) |  | IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’ | Time between points of 50% peak intensity. Define stability of rotation for a rotating beacon. For LED define frequency modulation of light, intensity profile of the flash |
| 1.3 |  | Horizontal beam uniformity (omnidirectional light) or horizontal divergence (range lights and rotating beacons) |  | IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’ | For omnidirectional lights , state maximum variation from the mean intensity over any angle in the horizontal plane. For range lights and rotating beacons for the horizontal beam, state the horizontal divergences at 50% and 10% of peak intensity. |
| 1.4 |  | Vertical divergence |  | IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’ | From the vertical beam, state the vertical divergences at 50% and 10% of peak intensity as referenced in IALA Recommendation E-122 (2001) |
| 1.5 |  | Signal colour |  | IALA Recommendation E-122 | Colour boundaries and colour shift (for LED, variation with duty cycle) in the regions as defined in IALA Recommendation E-200-1, Part 1 - Colour |
| 1.6 |  | Nominal range |  |  | IALA Recommendation for the notation of luminous intensity and range of lights. (1966) IALA Recommendation for a definition of the Nominal Daytime Range of Maritime Signal Lights Intended for the Guidance of shipping by day (1974) State applicable atmospheric transmission factor |
| 2 | ### Electrical – externally sourced | Power supply normal and extreme voltage |  | IEC 60945 section 7 | State voltage tolerance |
| 2.1 |  | Power consumption |  |  |  |
| 2.2 |  | Reverse polarity circuit protection |  | IEC 60945 section 7.2 |  |
| 2.3 |  | Power supply and control and monitoring terminations/connectors |  |  | Define connector and/or terminal type |
| 3 | ### Control |  |  |  |  |
| 3.1 |  | Daylight control |  |  | Lux level and switching range, reference to IALA Guide To Ambient Light Levels At Which ATON Lights Should Switch On And Off.(No. 1038) |
| 3.2 |  | Monitoring |  |  | State parameters being monitored |
| 3.3 |  | Programming |  |  | State parameters that can be programmed |
| 3.4 |  | Light source regulation |  |  | State the accuracy of voltage regulation to light source in % |
| 4 | ### Physical |  |  |  |  |
| 4.1 |  | Maximum Height |  |  |  |
| 4.2 |  | Maximum Diameter |  |  |  |
| 4.3 |  | Maximum Weight |  |  |  |
| 4.4 |  | Focal Height |  |  |  |
| 4.5 |  | Materials |  |  | State the material for the lantern and lens and UV protection |
| 4.6 |  | Ingress Protection |  |  | Define applicable IP number to EN60529 |
| 4.7 |  | Description of lantern mounting hole pattern |  |  |  |
| 5 | ### Environmental |  |  |  |  |
| 5.1 |  | Temperature – operational and storage |  | Manufacturer to state the test standard | State the operational and storage temperature range |
| 5.2 |  | Humidity – operational and storage |  | IEC 60945 section 8.3; IEC 60068-2-3 | JIS C 5024; MIL-STD-202G-103B |
| 5.3 |  | Salt air and sea water spray |  |  | MIL-STD-202G-101E |
| 5.4 |  | Shock and vibration |  |  | JIS C 0911; MIL-STD-202G-201A/202D |
| 5.5 |  | Electromagnetic interference |  | IEC 60945 sections 9 & 10 |  |
| 6 | ### Service |  |  |  |  |
| 6.1 |  | Maintenance free period |  |  |  |
| 6.2 |  | Rework at stated service intervals |  |  |  |
| 6.3 |  | Assured Service life of all components |  |  |  |
| 7 | ### Disposal |  |  |  |  |
| 7.1 |  | Obligatory and recommended disposal method |  |  | IALA guideline 1036 – Environmental Considerations in Aids to Navigation Engineering |

 Where no test standard is mentioned the manufacturer should state the test standard used, preferably with reference to international or national standards.