Replace Section 3.3.3 of the Navguide with the following:

### 3.3.3 Signal Colours

IALA has made recommendations on colours for lighted aids to navigation and for surface colours for visual signals on aids to navigation.

- Lights use a five-colour system comprised of white, red, green, yellow and blue that conforms to the International Commission on Illumination (CIE) Standard S 004/E 2001 “Colours of Signal Lights”. For the regions quoted in the standard, it is recommended that only three colours are used in any signal system.

- Recommended surface colours for visual signals on aids to navigation are as follows:

  - Ordinary colours should be limited to white, black, red, green, yellow or blue.

  - Orange and fluorescent red, yellow, green or orange may be used for special purposes requiring high conspicuity.

Refer to IALA publications: - Recommendation for the colours for light signals on aids to navigation, December 1977; - Recommendation for the use of retro-reflecting material on aids to navigation marks within the IALA Maritime Buoyage System (E106), May 1998; - Recommendation for the surface colours used as visual signals on aids to navigation (E108), May 2004.

The CIE standard on the measurement of colours (colorimetry) is based on three reference colours (ie. a tri-stimulus system) that in varying combination can generate the spectrum of colours.  A particular colour function is described by the symbols; X, Y and Z that represent the proportions of the reference colours.

Using ratios of the tri-stimulus values, such that: X + Y + Z = 1, colours can be defined in terms of chromaticity using just the x [= X / (X+Y+Z)] and y [= Y/ (X+Y+Z)] values.  The advantage of this arrangement is that colours can be mapped on a two-dimensional chromaticity diagram.

CIE colour standards for marine signalling can be depicted as areas on the chromaticity diagram.  These areas are defined by boundaries expressed as functions of x and y (equations).

If the chromaticity co-ordinates of a coloured light, filter material or a paint product are known, its acceptability for marine signalling applications can easily be determined.

Fig 3.1 Illustrates the colour zones on the 1931 CIE chromaticity diagram.

(Note that the colour rendering is indicative only).

[contact Ian Tutt / Malcom to see about a cleaner version of the diagram]

The CIE standard for signalling colours has recently been revised, with some adjustments to the boundaries of signal colours.  Further information on surface colours can be found in IALA Recommendation E108 on the surface colours used as visual signals on aids to navigation.  Information for light signal colours is shown in IALA Recommendation for the colours for light signals on aids to navigation, December 1977. For further details on this issue please refer to CIE S 004/E-2001 [confirm reference to CIE] Colours of Light Signals

Blue surface colours may be used in inland waterways, estuaries and harbours where the colours may be seen at close range.  See Recommendation E108.

The CIE web site address is: http://www.cie.co.at/cie