Replace section 3.3.11.7 of the Navguide (and all associated subparagraphs) with the following:

#### Light Measurement of Marine AtoN Signal Lights

##### Photometric Measurement of Light

IALA Recommendation E122 on the Photometry of Marine Aids to Navigation Signal Lights provides definitive information on the measurement of marine signal lights.

The behaviour of light, in physics, is normally seen in the context of either a form of electromagnetic radiation or particle motion.  The latter includes the concept of “rays” of light that are used in analyzing the interactions of light and lenses.

The units of interest for electromagnetic applications of light are generally metres (wavelength) and watts (power).

The study of photometry and the use of lights for signal application has necessitated a parallel set of units to be developed to account for the physiological aspects of how the human eye evaluates a light source, as shown in Table 3.2.

The spectral sensitivity of the human eye (or the response of the eye to different coloured light) has been evaluated in tests of large numbers of people.  The results have been presented as a standard spectral sensitivity distribution or V( curve for photopic observers and V’(  for scotopic observers.

Fig 3.3 Spectral sensitivity distributions or V(  and V’(  curves

for the human observer also showing the difference between day and night vision.

##### Units of Measurement

Table 3.2 Photometric units of measurement.

| Term | Description | Unit | Abbreviation |
| --- | --- | --- | --- |
| Luminous flux | This is the total light emitted from the source (ie. lamp) The peak sensitivity of the human eye occurs at about 555 nanometres, a wavelength that corresponds to green. At this wavelength, the photometric equivalent of one watt is defined as 680 lumens. | lumens | lm |
| Luminous intensity | This is the part of the luminous flux in a particular direction. Also expressed as the luminous flux per solid angle (or steradian) | candela | cd |
| Luminance (Brillance) | This is the portion of the luminous flux emitted in a specific direction by the surface area of a luminous body. This variable is an important term for rating the brightness impression of light sources and illuminated objects. | candelas per square meter and also as candelas per square centimetre | cd/m2 cd/cm2 |
| Illuminance | This is the density of the luminous flux incident on a surface. It is the quotient of the luminous flux by the area of the surface when the surface is uniformly illuminated | lux (lumens/square metre) | lx |
| Luminous efficacy | This is the ratio of luminous output to radiometric output of a light source. It can also be applied to the efficiency with which electrical power is converted to visible radiation. | lumens per watt of electrical power consumed |  |
| Colour temperature | This related to the temperature of a black body. As a body heats up, it goes through a series of different colours from red through yellow and white, to blue white. The colour appearance of a tungsten filament lamp is similar to a black body at the same temperature. | Kelvin | ºK |
| Colour rendering index CRI | Characterises the colour rendering quality of the light from a lamp. It is the same for all incandescent lamps by definition and equal to the maximum value of 100. |  |  |

##### Threshold of Illuminance

In physical terms, the threshold of illuminance is the lowest level of illuminance from a point source of light, against a given background level of luminance, that causes a visual response at the eye.

For visual signalling applications, the threshold of illuminance (E) is taken to be 0.2 micro lux at the eye of the observer.

In the case of leading lights of limited range and with a high level of shore illumination, the above figures may be found too low. It is recommended that to observe the relative position of the lights easily and to derive the maximum possible accuracy from leading and sector lights, it is generally necessary to have a minimum illuminance of 1 micro lux [] at the eye of the observer. This condition is to be met at the outer limits of the useful segment for the minimum meteorological visibility under which the leading lights are to be used. Refer to IALA publication: - Recommendation for Leading Lights (E-112), May 1998.

IALA Recommendation on the Definition of the Nominal Daytime Range of Maritime Signal Lights, Intended for the Guidance of Shipping by Day (1974) provides the method of designing AtoN lights for use in daylight.

For lights on floating aids, care must  be taken to provide adequate vertical divergence so that the minimum illuminance at the observer is maintained as the floating aid rolls and pitches.

##### Luminous intensity

The luminous intensity of a navigation light is directly proportional to the luminance of the light source.  The size of the light source is inversely proportional to its luminance and directly proportional to the divergence of the optic system. Candela (cd) is the measurement unit used to quantify the luminous intensity of a lighted aid to navigation. The measurement of luminous intensity is described in IALA Recommendation E122 on the Photometry of Marine Aids to Navigation Signal Lights

##### Inverse Square Law

Light emitted from a source radiates out in all directions. For a point source, the wave fronts of light can be imagined to generate a series of spherical surfaces.  As shown in Fig 3.4, the further the light travels from the source, the greater is the surface area of the sphere and consequently, the lower the illuminance.  Since illuminance is measured in lumens/metre2 and the surface area of a sphere increases in proportion to the square of the radius, the illuminance decreases in proportion to the square of the distance from the source.  The decline in illuminance with distance is described as an inverse-square law.

.

Fig 3.4 Illustration of the Inverse-Square-Law concept.

##### Allard’s Law

The illuminance of a light source reaching an observer’s eye determines whether the light is seen.  The relationship between the illuminance produced at the observer’s eye, the luminous intensity of the light source, the distance to the observer and the atmospheric transmissivity is given by Allard’s Law:

E = I×Tdd2

where:

| E = | illuminance at the observer’s eye (lm/m2) |
| --- | --- |
| I = | effective intensity of the light source (cd) |
| T = | atmospheric transmissivity |
| d = | distance between the light source and the observer. [Because T is measured per nautical mile, d in the numerator must also be in nautical miles. In the denominator, d is in metres] |

Allard’s law applies only when the luminance of the background is small compared to the average illuminance of the light.

Where average background luminance is large, as typically happens during the daytime, the equation becomes:

E = [I − (L − L')A] × Tdd2

where:

| L = | luminance (cd/m2) of the background light, measured in the direction of the line of sight from a position near the AtoN light (e.g. a patch of sky next to the lighthouse) |
| --- | --- |
| L' = | average luminance (cd/m2) of the unlighted AtoN measured in the direction of the line of sight from a position near the lighthouse (e.g. a luminance measurement of a lighthouse optic lens with the lamp switched off) |
| A = | area (m2) of the AtoN light beam projected onto a plane normal to the line of sight (e.g. a measurement of the lit area of a lighthouse optic) |
| (L - L' )A= | Since I = LA, this is the light intensity required to make the average luminance of the projector equal to that of the background (cd) |

Note 1: 	Luminance is equal to Intensity divided by Area (L = I / A). Luminance measurements can be carried out with a luminance meter; typically these are devices that measure a quantity of light through a fixed aperture of known area.

Note 2: 	When the AtoN light exhibited is of a significantly different colour to the background light, Allard’s Law may not apply.

##### Colorimetric measurement of lights (Colour measurement)

The measurement of the colour of lights is described in CIE Publication No 15.2 (1986) Colorimetry. There are two main types of instrument for measuring the colour of a light: one is a colorimeter; the other is a spectroradiometer.

Colorimeters usually comprise three photoreceptors, each with a coloured filter. Each filter is matched to the response of one of the three eye receptors, red green and blue and such devices are called ‘tristimulus’ colorimeters. The colorimeter gives three outputs, one for each filtered receptor, and these correspond to the X, Y and Z functions of the human observer.

Spectroradiometers consist of a monochromator and photoreceptor. The monochromator splits the light into individual wavelengths (much like a prism makes a rainbow) and is usually rotated in steps past an exit slit. The photoreceptor, behind the exit slit, measures different sections of the spectrum as the monochromator is rotated. The output is a series of readings enabling a graph of power against wavelength to be displayed. Results may then be weighted with the X, Y and Z functions of the human observer to produce colour information.

Stepping monochromators of the type described above are fairly slow in operation and are not suitable for measuring flashing lights. Tristimulus colorimeters, on the other hand, enable much faster measurements of colour. A new type of spectradiometer, known as ‘array-based’ spectroradiometers, are now available. Instead of a single photoreceptor and a rotating monochromator, a fixed monochromator has its output directed at an array of charge-coupled devices (CCDs). Such devices are capable of much faster measurement speeds than stepping monochromators.

Further recent developments in colour measurement have resulted from the technology of digital cameras. ‘Imaging photometers’, as they are known, are little more than calibrated digital cameras, some with tristimulus filtering. They are capable of fast measurement of a whole scene, making them useful for work outside the laboratory. However, the accuracy of some cheaper devices leaves much to be desired.

In summary, tristimulus colorimeters are fast but cheaper models suffer errors when measuring narrowband light sources such as LEDs. Stepping monochromators are expensive and slow but very accurate. Array-based spectroradiometers are fast relatively inexpensive but can suffer with stray light errors. Imaging photometers are expensive and not very accurate but can record a whole scene and not just one light.

Resultant data from colour measurements is usually displayed on a chromaticity chart, developed by the CIE in 1931. The three X, Y, Z values are reduced to two x, y values as shown in the chart below.

x

y

The steradian is the equivalent in solid geometry to the definition of radian in plane geometry.  The steradian is defined as the unit of measure of a solid angle with its vertex at the centre of a sphere and enclosing an area of the spherical surface equal to that of a square with sides equal in length to the radius.  There are 4π steradians in a sphere. For further explanation see IALA Recommendation E122.

This condition is to be met at the outer limits of the useful segment for the minimum meteorological visibility under which the leading lights are to be used.