- Add a new section to the Navguide as follows (renumber and reformat as appropriate):

- 6.4  Lighthouse Preservation

    - IALA Engineering Committee –Working Group 5, “Preservation of Historic Lighthouses”, formally known as the“PHL” Advisory Panel

The IALA Advisory Panel on the Preservation of Lighthouses, Aids to Navigation, and Related Equipment of Historic Interest (PHL) was established by the IALA Council in 1996 in response to membership interest in the cultural value of lighthouses.  In 2002, this Panel became part of the IALA Committee on Engineering, Environment, and Preservation (EEP).

Objectives:

- to encourage deeper commitment by members to preserve historic values;

- to encourage member countries to see the preservation of their own lighthouses in an international context;

- to share information on the subject between both members and non-members, with particular attention being given to the alternative use of lighthouses.

Examples of work accomplished to date:

- the format of an IALA database for recording details of historic lighthouses;

- a book, titled “Lighthouses of the World” was published in 1998 with English, French, German and Spanish versions.  It features over 180 historic lighthouses from around the world;

- a Seminar on the “Practical Aspects of Lighthouse Preservation” in August of 2005

- a Workshop to gather information on the range of alternative uses of lighthouses and a report on the findings of the Workshop .

Future work planned:

- a Manual on “Lighthouse Preservation”, to be released in the Spring of 2006

    - Lens Size and Terminology

The following table provides information on terminology for historical glass lens systems and the typical amount of mercury held in mercury bath pedestals (for rotating lens systems).

Table 6.4.1  Terminology for historical glass lens systems and associated quantities of mercury used in rotating lens systems.

| Description | Focal distance | Typical quantity of mercury for mercury-bath pedestals |  |
| --- | --- | --- | --- |
|  | mm | kilograms | litres |
| Hyper-radial | 1330 |  |  |
| Meso-radial | 1125 |  |  |
| First Order | 920 | 175 | 12.9 |
| Second Order | 700 | 126 | 9.3 |
| Third Order | 500 | 105 | 7.7 |
| Small Third Order | 375 | 96 | 7.0 |
| Fourth Order | 250 |  |  |
| Fifth Order | 187.5 |  |  |
| Sixth Order | 150 |  |  |

    - Third Party Access To Aids To Navigation Sites

In 1998, IALA conducted a survey to investigate the extent to which Authorities were permitting aids to navigation sites to be used for collecting “non-aids to navigation” data.  This study was associated with investigations of the Advisory Panel on the Preservation of Historic Lighthouses into alternative uses of lighthouses and other aids to navigation.

The responses came from a wide range of IALA members and shared several common themes:

- the predominant applications were for the collection of meteorological data (i.e. weather, wind speed and direction), tidal/ current data and for telecommunication installations;

- data collected for or by other governmental agencies generally did not attract a fees, but fees were often charged for data obtained for commercial purposes;

- data acquisition equipment had to have its own separate power supply unless that aids to navigation site had mains power available.

IALA acknowledges that Authorities face an increased demand to share aids to navigation sites with  “third parties”.  While it is important to ensure that the integrity and security of aids to navigation are maintained, the presence of a third party may be beneficial:

- in reducing the risk of vandalism;

- as a source of revenue or sharing of operational costs (eg power, road maintenance etc);

- as a means of monitoring the operation of the aid.

If an Authority receives a request for a third party installation, it should first establish whether such involvement is permitted in the Authority’s legislation.  If there are no impediments the Authority may consider negotiating an agreement with potential third party to clearly establish the responsibilities and liabilities of each party.  The agreement may also address:

- conditions to apply to the third party installation and operation to ensure that the equipment does not compromise the integrity and security of the aids to navigation and other property owned by the Authority;

- access to electrical power.  At sites with mains power, it may be advisable for the Authority to require separate metering of the third-party supply so that electricity costs can be recovered;

- if no mains power is available, it is reasonable to require that the third-party provide its own power supply;

- where practical, the installation of the third-party equipment should take into consideration and preserve the heritage value of the aid to navigation.

Authorities should reserve the right to cancel any third party agreement if continued use jeopardizes the performance or functionality of the aid to navigation.