Product Template 2 – Lighthouse Lanterns Systems Components

# Purpose

Product templates identify parameters that should be tested and the standards against which each parameter should be tested to enable a comprehensive evaluation of products.

# Scope

The template is applicable for lanterns used on buoys and light beacons including rotating beacons, powered via external power supplies or via integrated solar power supplies.

Applicable IALA Recommendations and Guidelines

- IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’

# Test Standards

The Templates quote international standards. If an IALA Product Certification Authority (IALA-PCA) wishes to use an alternative standard, it is the responsibility of the IALA-PCA to confirm that the standard used conforms to the IALA quoted standard. Where no test standard is mentioned the manufacturer should state the test standard used, preferably with reference to international or national standards.

#### Product Certification Template 2 - Lighthouse Lanterns Systems Components

Product Type / Manufacturer Reference Number:

| Ref. No | Parameter category | Parameter | Measured Value | Test Method | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Light source |  |  |  |  |
| 1.1 | ## LED |  |  |  |  |
| 1.1.1 | Operational | Type |  |  | Detailed description (semi-conductor, low power / high power) |
| 1.1.1.1 |  | Colour characteristics |  | CIE 127 – 1997 | Depending on binning, current, duty cycle, ambient temperature. This is only important for LED source in the traditional optics. |
| 1.1.1.2 |  | Intensity Profile |  | IALA Recommendation E-122 | Depending of binning, current, duty cycle, ambient temperature. This is only important for LED source in the traditional optics |
| 1.1.1.3 |  | Life time - Degradation Colour vs Time - MTBF |  |  | Current management |
| 1.1.1.4 |  | Thermal Management - Operational temperature Range - Temperature regulation system |  |  | Detailed description of temperature regulation |
| 1.1.1.5 |  | Operational temperature range |  |  | Only for LED source in traditional optics. |
| 1.1.2 | Electrical |  |  |  |  |
| 1.1.2.1 |  | Electronic driving scheme |  |  | Series or parallel, pulse (frequency, luminous signal shape) or DC. |
| 1.1.2.2 |  | Junction Temperature range |  |  | Depending of binning, current, duty cycle, ambient temperature |
| 1.1.2.3 |  | Power and Voltage |  |  | Only for LED source in traditional optics |
| 1.1.3 | Environmental |  |  |  |  |
| 1.1.3.1 |  | Vibration |  | IEC 60945 section 8.7 | Only for LED source in traditional optics |
| 1.1.3.2 |  | Shock |  |  |  |
| 1.1.3.3 |  | Electromagnetic compatibility |  | IEC 60050 |  |
| 1.1.3.4 |  | Storage temperature |  |  |  |
| 1.1.4 | Physical |  |  |  |  |
| 1.1.4.1 |  | Size characteristics |  |  | position of LEDs; only for LED source in traditional optics |
| 1.1.4.2 |  | Weight |  |  |  |
| 1.1.4.3 |  | Solder Method |  |  |  |
| 1.2 | Filament lamps |  |  |  |  |
| 1.2.1 | Operational |  |  |  |  |
| 1.2.1.1 |  | Size and position of Filament |  |  | scheme and accuracy |
| 1.2.1.2 |  | Lifetime - Degradation Flux vs Time - MTBF |  |  |  |
| 1.2.1.3 |  | Type of Filament |  |  | Single, double, spiral, … |
| 1.2.1.4 |  | Focal Height |  |  |  |
| 1.2.1.5 |  | Distribution of Intensity |  | CIE 70-1987 | Vertical & Horizontal |
| 1.2.1.6 |  | Flux, accuracy of flux and illuminance |  | CIE 84-1989 |  |
| 1.2.2 | Electrical |  |  |  |  |
| 1.2.2.1 |  | Nominal Voltage |  |  |  |
| 1.2.2.2 |  | Power or Current |  |  |  |
| 1.2.3 | Physical |  |  |  |  |
| 1.2.3.1 |  | Socket |  |  | Dimensions and service temperature |
| 1.2.3.2 |  | Size of envelope |  |  |  |
| 1.2.3.3 |  | Weight |  |  |  |
| 1.2.3.4 |  | Material |  |  |  |
| 1.2.3.5 |  | Gas |  |  |  |
| 1.2.4 | Environmental |  |  |  |  |
| 1.2.4.1 |  | Vibration |  | IEC 60945 section 8.7 |  |
| 1.2.4.2 |  | Shock |  |  | Manufacturer to provide test standard |
| 1.2.4.3 |  | Storage temperature and humidity and time limit |  |  |  |
| 1.2.5 | Personal Safety |  |  |  |  |
| 1.2.5.1 |  | Handling requirements |  |  | Explosion and UV exposure |
| 1.3 | Discharge lamp |  |  |  |  |
| 1.3.1 | Operational |  |  |  |  |
| 1.3.1.1 |  | Lifetime - degradation flux vs time - degradation colour vs time - MTBF |  |  |  |
| 1.3.1.2 |  | Type of Gas |  |  |  |
| 1.3.1.3 |  | Material of discharge tube |  |  | Quartz / Ceramic |
| 1.3.1.4 |  | Chromaticity coordinates |  |  |  |
| 1.3.1.5 |  | Size of Luminous Source |  |  |  |
| 1.3.1.6 |  | Spectral Distribution |  | CIE 63 - 1984 | Normalised |
| 1.3.1.7 |  | Focal Height |  |  |  |
| 1.3.1.8 |  | Distribution of Intensity |  | CIE 70-1987 |  |
| 1.3.1.9 |  | Accuracy of discharge tube Position |  |  |  |
| 1.3.1.10 |  | Flux and illuminance |  | CIE 84-1989 / CIE 69-1987 |  |
| 1.3.1.11 |  | Re-ignition Time |  |  |  |
| 1.3.1.12 |  | Lamp position |  |  |  |
| 1.3.1.13 |  | Discharge tube Position |  |  |  |
| 1.3.2 | Electrical |  |  |  |  |
| 1.3.2.1 |  | Nominal Current |  |  |  |
| 1.3.2.2 |  | Power |  |  |  |
| 1.3.3 | Physical |  |  |  |  |
| 1.3.3.1 |  | Socket |  |  |  |
| 1.3.3.2 |  | Size of discharge tube |  |  |  |
| 1.3.3.3 |  | Size of envelope |  |  |  |
| 1.3.3.4 |  | Weight |  |  |  |
| 1.3.3.5 |  | Material |  |  |  |
| 1.3.3.6 |  | Gas |  |  |  |
| 1.3.4 | Environmental |  |  |  |  |
| 1.3.4.1 |  | Shock |  |  | Manufacturer to provide test standard |
| 1.3.4.2 |  | Vibration |  | IEC 60945 section 8.7 |  |
| 1.3.4.3 |  | Storage temperature range and time limit |  |  |  |
| 1.3.5 | Personal Safety |  |  |  |  |
| 1.3.5.1 |  | Handling requirements |  |  | Explosion and UV exposure |
|  | Auto transformer ballast circuit |  |  |  |  |
| 1.3.6 | Operational |  |  |  |  |
| 1.3.6.1 |  | Type |  |  | Electronic or electromagnetic |
| 1.3.6.2 |  | Constitution |  |  | Description of different elements (generator, converter) |
| 1.3.6.3 |  | Ingress protection (IP) |  |  |  |
| 1.3.6.4 |  | Maximum distance between the different elements |  |  |  |
| 1.3.6.5 |  | Operational temperature range |  |  |  |
| 1.3.7 | Electrical |  |  |  |  |
| 1.3.7.1 |  | Input voltage |  |  | Frequency, voltage range authorized |
| 1.3.7.2 |  | Output voltage |  |  | Frequency, signal shape, voltage range |
| 1.3.7.3 |  | Voltage protection |  |  |  |
| 1.3.7.4 |  | ignition voltage |  |  | Description of ignition cycle |
| 1.3.7.5 |  | Power factor |  |  |  |
|  |  | efficiency |  |  |  |
| 1.3.8 | Physical |  |  |  |  |
| 1.3.8.1 |  | Size of the different elements |  |  |  |
| 1.3.8.2 |  | Weight of the different elements |  |  |  |
| 1.3.9 | Environmental |  |  |  |  |
| 1.3.9.1 |  | Storage temperature |  |  |  |
| 1.3.9.2 |  | Shock |  |  | Manufacturer to provide test standard |
| 1.3.9.3 |  | Vibration |  | IEC 60945 section 8.7 |  |
| 1.4 | Lamp Changer |  |  |  |  |
| 1.4.1 | Operational |  |  |  |  |
|  |  | Maximum Lamp Capacity |  |  |  |
| 1.4.1.1 |  | Type of lamp |  |  | socket |
| 1.4.1.2 |  | Number of rotation in life cycle |  |  | Definition of life |
| 1.4.1.3 |  | Operational Mechanism |  |  | Auto reset or manual |
| 1.4.1.4 |  | MTBF |  |  |  |
| 1.4.1.5 |  | Equipment Life |  |  |  |
| 1.4.1.6 |  | Ingress protection (IP) |  |  |  |
| 1.4.1.7 |  | Lamp Positioning Accuracy |  |  |  |
| 1.4.1.8 |  | Operational temperature range |  |  |  |
| 1.4.2 | Electrical |  |  |  |  |
| 1.4.2.1 |  | Maximum Current |  |  |  |
| 1.4.2.2 |  | Maximum voltage |  |  |  |
| 1.4.3 | Environmental |  |  |  |  |
| 1.4.3.1 |  | Vibration |  | IEC 60945 section 8.7 |  |
| 1.4.3.2 |  | Shock |  |  | Manufacturer to provide test standard |
| 1.4.3.3 |  | Storage Temperature and humidity range |  |  |  |
| 1.4.3.4 |  | Terminal Barrier |  |  |  |
| 1.4.3.5 |  | Electromagnetic compatibility |  | IEC 60050 |  |
| 1.4.4 | Physical |  |  |  |  |
| 1.4.4.1 |  | Maximum Height |  |  |  |
| 1.4.4.2 |  | Maximum Diameter |  |  |  |
| 1.4.4.3 |  | Maximum Weight |  |  |  |
| 1.4.4.5 |  | Focal Height |  |  |  |
| 1.4.4.5 |  | Materials |  |  |  |
| 2 | ## Daylight Switch |  |  |  |  |
| 2.1 | Operational |  |  |  |  |
| 2.1.1 |  | Type |  |  | e.g. LDR |
| 2.1.2 |  | Spectral Response |  | CIE 64-1984 |  |
| 2.1.3 |  | Accuracy |  | CIE 64-1984 |  |
| 2.1.4 |  | Temporization |  |  |  |
| 2.1.5 |  | Lifetime |  |  |  |
| 2.1.6 |  | Ambient Level Regulation |  |  | Mechanical or electronically |
| 2.2 | Electrical |  |  |  |  |
| 2.2.1 |  | Hysteresis |  |  |  |
| 2.2.2 |  | Input Power |  |  |  |
| 2.3 | Physical |  |  |  |  |
| 2.3.1 |  | Dimension of Detector |  |  |  |
| 2.3.2 |  | Weight |  |  |  |
| 2.4 | Environmental |  |  |  |  |
| 2.4.1 |  | Shock |  |  | Manufacturer to provide test standard |
| 2.4.2 |  | Vibration |  | IEC 60945 section 8.7 |  |
| 2.4.3 |  | Ambient Resistance |  |  |  |
| 2.4.4 |  | Storage Temperature and humidity range |  |  |  |
| 3 | Lighthouse Glass/ Screening |  |  |  |  |
| 3.1 | Operational |  |  |  |  |
| 3.1.1 |  | Type |  |  | Flat or Curved, Surface Treatment, colour |
| 3.1.2 |  | Material |  |  |  |
| 3.1.3 |  | Colour - Spectral curve of absorption - Chromaticity and transmission factor of glass associated with lamp |  | CIE 130-1998 |  |
| 3.1.4 |  | Colour degradation of UV |  | IEC 60060-2-5 |  |
| 3.1.5 |  | Dimension of and orientation of junction |  |  |  |
| 3.1.6 |  | Operational Temperature |  |  |  |
| 3.2 | Physical |  |  |  |  |
| 3.2.1 |  | Dimensions, Thickness |  |  |  |
| 3.3 | Environmental |  |  |  |  |
| 3.3.1 |  | Storage Temperature and humidity range |  |  |  |
| 4 | ## Optic-Drive |  |  |  |  |
| 4.1 | Operational |  |  |  |  |
| 4.1.1 |  | Speed rotation |  |  | Range, interval |
| 4.1.2 |  | Speed rotation set up |  |  | (pwm, …) |
| 4.1.3 |  | Operational temperature |  |  |  |
| 4.2 | Electrical |  |  |  |  |
| 4.2.1 |  | Optic Drive Controls |  |  |  |
| 4.2.2 |  | Current and Power |  |  |  |
| 4.2.2 |  | Type of motors |  |  | (brushless, continuous current, gearless) |
| 4.3 | Physical |  |  |  |  |
| 4.3.1 |  | Dimensions |  |  |  |
| 4.3.2 |  | Weight |  |  |  |
| 4.3.3 |  | fixation |  |  |  |
| 4.3.4 |  | Maximum weight for lenses |  |  |  |
| 4.3.4 |  | Maximum torque |  |  |  |
| 4.4 | Environmental |  |  |  |  |
| 4.4.1 |  | Electromagnetic compatibility |  | IEC 60050 |  |
| 4.4.2 |  | Ingress protection (IP) |  |  |  |
| 4.4.3 |  | Storage temperature and humidity range |  |  |  |
| 5 | ## Flashers |  |  |  |  |
| 5.1 | Operational |  |  |  |  |
| 5.1.1 |  | Details of programmed rhythm |  |  |  |
| 5.1.2 |  | Program method |  |  | IR, wire … |
| 5.1.3 |  | Synchronisation mode |  |  |  |
| 5.1.4 |  | MTBF |  |  |  |
| 5.1.5 |  | Operational temperature |  |  |  |
| 5.2 | Electrical |  |  |  |  |
| 5.2.1 |  | Input voltage interval |  |  |  |
| 5.2.2 |  | Output signals |  |  | Remote information |
| 5.2.3 |  | Output voltage |  |  | range |
| 5.2.4 |  | Maximum output current |  |  |  |
| 5.2.5 |  | Day light detection |  |  |  |
| 5.2.6 |  | Reversed polarity protection |  |  |  |
| 5.2.7 |  | Detection of lamp failure |  |  | Define detection mode (intensity measure, photocell,…) |
| 5.2.6 |  | Voltage regulation accuracy |  |  |  |
| 5.3 | Environmental |  |  |  |  |
| 5.3.1 |  | Storage Temperature and humidity range |  |  |  |
| 5.3.2 |  | Electromagnetic compatibility |  | IEC 60050 |  |
| 5.3.3 |  | Ingress protection |  |  |  |
| 6. | Convergence system |  |  |  |  |
| 6.1 | Operational |  |  |  |  |
| 6.1.1 |  | Focal length |  |  |  |
| 6.1.2 |  | Type |  |  | Lens, mirror |
| 6.1.3 |  | Material |  |  | Colour if coloured |
| 6.1.4 |  | Lens surface - dioptric surface - catadioptric surface |  |  |  |
| 6.1.5 |  | Colour degradation of UV |  |  |  |
| 6.1.6 |  | Operational Temperature |  |  |  |
| 6.2 | Physical |  |  |  |  |
| 6.2.1 |  | Dimensions, Thickness |  |  |  |
| 6.3 | Environmental |  |  |  |  |
| 6.3.1 |  | Storage Temperature and humidity range |  |  |  |
| 7 | Service |  |  |  |  |
| 7.1 |  | Maintenance free period |  |  |  |
| 7.2 |  | Assured service life |  |  |  |
| 7.3 |  | Description of inspection methods |  |  |  |
| 8 | Disposal |  |  |  |  |
| 8.1 |  | Obligatory and recommended disposal method |  |  | IALA guideline 1036 – Environmental Considerations in Aids to Navigation Engineering |