Guideline on Safety Manual
 at Aids to Navigation Maintenance Work

1    Purpose

This guideline defines standard contents and others of a safety manual at aids to navigation (AtoN) maintenance work which is established by each Coast Guard office in order to conduct the work safely and smoothly.

2    Scope

The scope of the safety manual is applied to all maintenance works of equipments and facilities that are conducted by the Coast Guard staff or contractors in order to maintain performance of AtoN.

3    Contents

The guideline should consist of the following points as the standard taking account of the special environment of the site in addition to the characteristic of the work.

3.1  Assignment of Officer in Charge and Clarification of Duty

- The Commanding Officer (CO) of the Office should form a maintenance team taking account of the details of the work, the specialty, capability and health of the staff and assign the Officer in Charge (OIC) of the team.

- The OIC should instruct the staff of the team in the procedure and cancellation of the work and countermeasures against emergency situation.

3.2  Standard safety procedure (Refer ANNEX 1)

The CO should establish a standard safety procedure at each AtoN in order to enable the maintenance team to work safely.

3.3  Discontinuance criteria (Refer ANNEX 2)

- The CO should establish discontinuance criteria that are quantified to maximum extent taking account of the contents of the work and the special environment of the site.

- The OIC should decide the discontinuance of the work by judging comprehensively information such as weather, sea state, the safety check sheet and the discontinuance criteria or report to a CO of buoy tender when working with the tender.  If necessary, the OIC should report to the CO of the office and receive the direction from the CO.

3.4  Safety check sheet (Refer ANNEX 3)

The CO should establish a safety check sheet in order to confirm whether a preliminary briefing is held among the maintenance team and all concerned for checking the work procedure, the safety measure, each responsibility, etc. and to check the health condition of the team members and preparation, function and wearing of safety gears such as safety hat, life jacket, safety shoes, safety belt which are necessary for the safety of the work.

3.5  Emergency drill

The CO should establish first aid, evacuation, communication and other emergency measures at the office and conduct an emergency drill at least one time in a year for preparing an accident to the staff that carries out the maintenance work of the equipment and facilities or an emergency situation that will possibly cause the accident.

3.6  Safety education

The CO should conduct a safety education to the staff regarding regulations, working methods, accident prevention measures, etc. that are necessary for the work when the education is required due to the change of the staff, the change of the job, etc.

3.7  Safety check meeting

The CO should hold a safety check meeting regularly in order to maintain or improve the safety awareness of the staff that carries out the maintenance work of the equipment and facilities and confirm the present safety management system and the safety measures and if necessary, should conduct the review and the continuous improvement of the system and measures.