Using Bayesian Networks

in Aids to Navigation Engineering

Author:	Ómar Frits Eriksson

Position:	Head of Technical Division

Organisation:	Royal Danish Administration of Navigation and Hydrography

Postal address:	Overgaden Oven Vandet 62B, Postboks 1919

DK-1023 Copenhagen K, Denmark

Phone:	+45 32 68 95 98

Fax:	+45 32 68 96 34

E-Mail	ofe@frv.dk

Topic most

appropriate

for this paper:	Development and introduction of digital techniques in Aids to Navigation

Abstract

The powerful computers of today have opened up a whole range of new possibilities in relation to mathematical modelling of engineering problems in the field of Aids to Navigation.

Some IALA members have undertaken to develop their own engineering software, and in some cases it has been possible and practical to share the result with other IALA members.

This paper first gives a brief description of Bayesian networks and then gives an example of how these may be utilized in Aids to Navigation engineering and related decision making.

It is argued that two of the more fundamental design criteria for traditional Aids to Navigation engineering are system/equipment reliability and ultimately the probability that the mariner actually detects the Aid.

It is shown how a problem of selecting a suitable vertical divergence for a given waterway with certain characteristics can be simulated and solved using a Bayesian network.

In conclusion it is recommended that IALA endeavours to support and encourage the further use of Bayesian networks through the development, verification, and publication of relevant probabilistic models for relevant aspects of Aids to Navigation.

Using Bayesian Networks

in Aids to Navigation Engineering

Ómar Frits Eriksson

Head of Technical Division

Royal Danish Administration of Navigation and Hydrography

Introduction

Today’s powerful computers have opened up a whole range of new possibilities in relation to the mathematical modelling of complex engineering problems in the field of Aids to Navigation. Sophisticated commercial software packages that support almost every aspect of Aids to Navigation engineering are available at a price, and simple computer programs such as spreadsheets are readily available and widely used by IALA members to deal with a variety of problems in Aids to Navigation engineering.

Some IALA members have undertaken to develop their own software, and in many cases it has been possible and practical to share the result with other IALA members. Sharing such results and indeed sharing more or less sophisticated mathematical models and methods, useful for problem solving and decision making in the field of Aids to Navigation, is one of the fundamental mechanisms of IALA. This is to a large extent done through the activities of IALA technical committees, and the results are communicated to the broader membership of IALA through recommendations and guidelines.

This paper sets out to give an example of using the so-called Bayesian networks (also known as “belief networks” and “influence networks”) in the field of Aids to Navigation.

Bayesian Networks

Bayes' theorem relates the conditional and marginal probability distributions of random variables. The probability of an event A conditional on another event B is generally different from the probability of B conditional on A. However, there is a definite relationship between the two, and Bayes’ theorem describes that relationship. A full description of Bayes’ theorem, its strengths and weaknesses is outside of the scope of this paper, but a good introduction may be found on Wikipeda [1].

A Bayesian network is a model that encodes probabilistic relationships among variables of interest utilizing Bayes’ theorem. In contemporary Bayesian software packages, this is done graphically, which gives a good illustration of the interrelationships between the variables.

Bayesian networks are today widely used, for example in Risk Assessment, where there is a tradition for a probabilistic approach. Bayes’ theorem was first introduced in a IALA publication in 1989, in the IALA Guideline on Availablity and Reliablity of Aids to Navigation, and the idea of Bayesian networks was first introduced in IALA in the revised version of this document published in 2004 [2]. The mathematical model within IALA Waterway Risk Assessment Program (IWRAP) uses Bayes’ theorem, so Bayes is by no means new to IALA, however the author is not familiar with any prior application of Bayesian networks within IALA.

Problem to be modelled

Certainly, two of the more fundamental design or decision criteria in visual Aids to Navigation engineering are equipment and/or system reliability and designing the AtoN for a certain level of probability that the mariner actually detects and/or recognizes the Aid.

Since 1989, IALA has provided excellent guidance on how to design for suitable system and/or equipment reliability, taking into account availability objectives set out by management as well as the capabilities of the casualty recovery functions in terms of response and repair time(s) [2]. Thus, with respect to reliability and availability, IALA provides system level guidance to its members.

When it comes to probability of detection and/or recognition, the situation is different. While IALA provides good and detailed information in some specific areas, such as the propagation of light in the atmosphere and threshold of vision, hitherto limited system level guidance has been provided.

Modelling probability of detection (for example for a maritime signal light at night time) encompasses a number of variables that may vary considerably with time within a given numerical range. These include the illumination level at the observer’s eye and the observer threshold of vision. These variables are functions of a number of other variables such as the luminous intensity of the light, meteorological visibility, buoy inclination angle, the vertical divergence of the lantern and even parameters such as the fitness and psychophysical state of the observer. Modelling probability of recognition is even more complex, including also the relatively advanced cognitive processes of the observer.

Within the IALA EEP Committee, the need to define two or three vertical IALA divergence classes (narrow, medium, and wide) for lanterns has been discussed for some time. This would simplify the design process for industrial members. For AtoN authorities this would simplify the process of comparison between lanterns from different manufacturers, for example when selecting a lantern to be fitted on a certain type of buoy to be deployed in a given waterway. In spite of whatever divergence limits ultimately will be assigned to such classes of vertical divergence, AtoN service providers will still have to ask the question: “What vertical divergence (or divergence class) should be chosen for a particular buoy type in a particular waterway with a given probability distribution of current, meteorological visibility etc.”

This leads to the following problem definition:

What vertical divergence should be selected for a light on a given buoy in a given waterway to give a certain probability of detection at a given distance at night time?

In this paper an attempt is made to describe a simple Bayesian Network capable of modelling the interrelationships between a number of variables deemed to be significant for the abovementioned problem definition exemplifying the use of Bayesian networks in AtoN engineering. Note that for simplicity, the problem definition is limited to probability of detection and does not include the probability of recognition.

A Bayesian network modelling the problem

The figure below illustrates a relatively simple Bayesian network modelling the interrelationship between the variables found significant for this first order model of the problem definition.

The ellipses represent nodes describing the statistical properties (probability functions) of a particular variable. Rectangles represent so-called decision nodes or tools describing a number of states that can be selected for those variables while the arrows between the nodes define the causal relationship between the nodes they connect.

Fig. 1

A simple Bayesian Network modelling the problem defined

The variables modelled by the ellipses typically encompass a number of well defined states for the variable in question as well as the (best guess) probability that the variable assumes each particular state. The software used to create this network is Hugin Developer [3].

Creating the graphical view of the network itself as illustrated in fig. 1 is a relatively simple task. The hard bit is to come up with a good description of the statistical properties of each variable. In some cases statistical data are available, in other cases some assumptions have to be made.

One of the strengths of Bayesian networks is that until you have gathered and verified real world statistical data for a given variable, you can make do with temporary probability distributions based on preliminary assumptions. Once better data becomes available, the nodes in question can be updated. Another strength is that more variables can be added at any time by adding new nodes, as long as it is possible to model the probabilities of the individual states of these variables and to define their causal interrelationships with other nodes.

In the following text, the modelling of the interrelationships between the variables in the model as well as the statistical properties of these are described in some detail.

Waterway properties

Any given waterway has different characteristics with respect to prevailing meteorological and hydrographical conditions. For the purpose of this example, the Sound between Denmark and Sweden has been chosen. Good observation data is available in the northern part of the Sound at a location called Lappegrund, where a Light Ship was positioned for many years. The observation data used in this example is from 1931-1960.

| Fig. 2 Lappegrund in the entrance of the Sound between Denmark and Sweden | M. Visibility Naut. miles; Probability. 0-0.1; 0.007. 0.1-0.3; 0.011. .3-0.5; 0.015. .5-1; 0.014. 1-2; 0.012. 2-5; 0.114. 5-10; 0.170. 10-30; 0.657. >30; 0.000. Table 1 Meteorological visibility statistics at Lappegrund 1931-1960 (nautical miles) |
| --- | --- |

The data for meteorological visibility can be utilized directly for the model node describing meteorological visibility. These are therefore entered into the node in the form of a numerical table. It is interesting to note that at the location in question, the probability of visibility being more that 10 nautical miles, corresponding to the nominal meteorological visibility used to calculate the nominal range of a light, is only about 66%. This means that about 34% of the time, the visual range of a light at this position will indeed be less than its nominal range.

As can be seen in fig. 2, the Sound is quite narrow at Lappegrund. The narrowest part of the Sound is just south of Lappegrund, where the width is about 2 nautical miles. Therefore the water current in the area can be quite strong. Data from 61248 observations were compiled and the probability distribution these modelled with a two parameter Weibull distribution where β = 1.8 and η =1 .6 and x is water velocity in metres/sec.

Fig. 3

Water velocity probability distribution modelled with a Weibull distribution 
where β = 1.8 and η =1 .6 and x is water velocity in metres/sec.

This distribution is entered into the node for water velocity in the form of a numerical table.

Probability of detection

As already mentioned, modelling the probability of detection is a complex task. For the purpose of this example, the probability of detection is modelled by comparing the excitation level at the eye op the observer (illumination at observer) to the threshold level of the eye (observer threshold of vision). If the excitation level exceeds the threshold value, detection occurs, otherwise detection does not occur.

Fig. 4

Probability of detection is based on a comparison of the 
excitation level at the eye of the observer with the threshold state of the observer eye.

Since both input parameters are expressed in probabilistic terms, the two output states, detection or no detection, can also be expressed in probabilistic terms.

Observer threshold of vision

Observer threshold of vision depends on the level of background luminance in the observer’s field of view. Background luminance depends on time of day as well as meteorological conditions.

Having no data at hand for the probability of different levels of night time background luminance at the give geographic location, some plausible values have been assumed as a first order approximation:

| Approximate Luminance of the sky near the horizon in cd/m2 (Middleton,1952) |  |  |
| --- | --- | --- |
| Conditions | Luminance Level (cd/m2) | Assumed Probability (night) |
| Clear sky, a quarter hour after sunset | 1 | 0.05 |
| Clear sky, a half hour after sunset | 0.1 | 0.05 |
| Night sky - Full moon | 0.01 | 0.1 |
| Night sky - Clear without moon | 0.001 | 0.3 |
| Night sky - Cloudy without moon | 0.0001 | 0.5 |

Table 2

Probabilities assigned to background luminance values

These probabilities are entered into the background luminance node.

Having determined the probability distribution of background luminance, the relationship between background luminance and threshold of vision needs to be described. One model for this relationship was given by IALA in 1974 [4]:

ET= 0.242⋅10−6⋅( 1−0.4⋅L )2		(eq. 1)

where L is background luminance and ET is the threshold value. This model is suitable for daytime background luminance levels [4], and may be rather conservative for night time luminance levels. However it serves well as a first approximation for the purpose of the example at hand and the equation is therefore used in the node for observer threshold of vision.

Illumination at the observer

The resultant illumination level E at the eye of the observer depends on the observation distance D, meteorological visibility V, and the luminous intensity I0 of the light source. This relationship is known as Allard’s law [5]:

E= I0⋅0.05DVD2			(eq. 2)

This equation is used in the node for illumination at the observer. Luminous intensity of the light and meteorological visibility are both expressed probabilistically while the observation distance is obtained an absolute value from a decision node (see fig. 1). The implication of the latter is that when running the Bayesian network, an absolute value for the observation distance has to be selected. Output results, in the form of probability of detection, are then valid for the chosen observation distance.

Luminous intensity of the light source

There are four input variables to the Luminous Intensity node:

- Vertical divergence class

- Peak luminous intensity

- Buoy inclination

- Equipment reliability

The first two input variables are obtained from decision nodes, so one has to select one or another state for these at run time. The two remaining variables, buoy inclination and equipment reliability, are expressed probabilistically.

Two states are assigned to the equipment reliability node: “working” and “not working”. The probability of “not working” is arbitrarily assumed to be 0.003 corresponding to an availability of 99% [6] and an mean time between failure (MTBF) of about 26000 hours.

Modelling the relationship between equipment reliability and luminous intensity is fairly straight forward: given the state “not working”, luminous intensity is set to zero, otherwise luminous intensity is as determined by the remaining input variables to the node.

Three classes of vertical divergence have been defined:

| Divergence Class | Divergence (50%) | Standard Deviation |
| --- | --- | --- |
| Narrow | 5˚ | 2.12 |
| Medium | 10˚ | 4.24 |
| Wide | 20˚ | 8.49 |

Table 3

Vertical divergence classes defined and standard deviation of

Gaussian probability distribution models for these.

Each class may be modelled by a Gaussian distribution using the standard deviations σ from table 3 above:

Fig. 5

Vertical divergence classes modelled as Gaussian distributions

The advantage of this approach is that the area under each of the three curves is the same, i.e. the luminous flux (and power consumption) is similar, making them comparable, energy wise. However, as a consequence their peak intensity differs.

The peak luminous intensity decision node is used to scale the luminous intensity in order to achieve a suitable luminous range at run time.

Buoy inclination angles translate directly to the vertical angles illustrated in figure 5, so this relationship is modeled as 1:1 however we need to express buoy inclination in probabilistic terms.

Modelling the statistical properties of the inclination angle of a moored floating object in varying sea states is not trivial. There are a number of forces that go into the equation: drag due to water current, wave forces, wind forces, and mooring forces. These are all dynamic forces which also vary as a function of geographic location, time of year etc. Depending on the physical properties of the object, it may also oscillate at frequencies determined by its hydrodynamic properties.

It is however entirely possible to develop a detailed model, using the basic Morrison equation [7] comprising of a drag term and an inertial term. A good discussion of the various forces that act upon a moored buoy may be found in the IALA Recommendation on the design of normal moorings (E-107) [8].

For the purpose of this example, a very simple model is used. It is inspired by the basic Morrison equation, including only “semi static” inclination states induced by the fairly “stable” force of the average water current (simplified drag term of Morrison equation), discarding the more rapid dynamic variation in inclination as well as any inertial effects.

In this model, the force of the water current upon the buoy is assumed to vary with the square of the water velocity v. The inclination angle θ of the buoy is assumed to vary linearly with the force, and inclination “sensitivity” to water velocity is adjustable with the factor k. Different values for k would then correspond to different types of physical buoys:

φ=k⋅v2			(eq. 3)

This is indeed a very simple model, but sufficient to demonstrate the principle. Note that the model only addresses the inclination in one direction, which is assumed to be directly towards or away from the observer. Note also that when a better model for the buoy inclination is available, it can very easily be substituted into the network.

A description of the probability distribution of water velocity at Lappegrund has been given earlier in the text.

The luminous intensity of the light source has now been modelled as described above. Note that the effective intensity of the light also is dependent upon the rhythmic character of the light [9] however this effect is not included in the present model.

Results of analysis performed with the Bayesian network

Having modelled the interrelationship between all these variables and expressed most of these in probabilistic terms, the Bayesian network now is ready for use.

Glancing back at the problem definition, it seems useful to plot the probability of detection, for each of the vertical divergence classes versus observation distances.

A suitable peak intensity scaling value was selected, resulting in a corresponding peak intensity of 100 candela (narrow class), 50 candela (medium class) and 25 candela (wide class), still bearing in mind that all three classes have similar total luminous flux (and thus power consumption).

The corresponding nominal range of these peak intensities (at zero inclination angle) are approximately 5.5 (narrow), 4.4 (medium), and 3.5 (wide) nautical miles.

Fig. 6

Probability of detection versus observation distance for a 100 candela (5.3 nm) narrow class, a

50 candela (4.4 nm) medium class light and a 25 candela (3.5 nm) wide divergence class light. 
Note that the total luminous flux and thus power consumption of all three lights is similar.

Viewing fig. 6, the first observation to be made is that there seems to be an interesting break even point for all three divergence classes at a distance of about 3 nautical miles, where the probability of detection is about 50%.

Another observation is that not surprisingly, the wide divergence class yields much better probability of detection than the narrow class at distances shorter than 3 nautical miles.

A third observation is that at their nominal ranges, all three classes exhibit low probabilities of detection in the range 20-35%. This clearly demonstrates that prevailing local conditions should always be taken into account during the system design phase. It is insufficient to base the design on nominal range only.

A fourth observation is that if one wants to design for a high probability of detection, say 80%, there is a significant difference between the three divergence classes. While the wide divergence class would result in a range of approximately 2 nautical miles, medium class would result in about 1.5 nautical miles while the narrow class would result in less than 0.5 nautical miles. All this is of course only valid for Lappegrund and locations with similar local conditions.

Conclusions

It has been demonstrated, that Bayesian networks can be useful for simulation during AtoN design and system planning.

Models based on Bayesian networks are relatively simple to develop, they are flexible, the may be based on a number of assumptions in the first instance, and they are updatable as new and validated data becomes available.

In theory, there is no limitation to the possible complexity of Bayesian Networks, as long as the variables included can be expressed in probabilistic terms and that the causal nature of their interrelationships is expressible.

In practice very complex relationships can be modelled, limited only by the software and hardware used, and the availability of mathematical models available.

Simulations performed with the Bayesian network described in this paper, revealed, that for the prevailing conditions at Lappegrund in the Sound between Denmark and Sweden, it seems best to use lights with a fairly wide vertical divergence, in order to maximize the probability of detection at ranges up to 3 nautical miles at night time.

The simulation also revealed that basing AtoN systems on nominal range of lights is insufficient when a relatively high degree of probability of detection is desired.

The main shortcoming of the Bayesian network developed is the lack of good (probabilistic) mathematical models in certain areas.

In conclusion it is recommended that IALA endeavours to support and encourage the further use of Bayesian networks through the development, verification, and publication of useful probabilistic models for all relevant aspects of Aids to Navigation.

Ómar Frits Eriksson

References

[1]	Wikipeda “Bayesian Probability” 
http://en.wikipedia.org/wiki/Bayesian_probability

[2]	“IALA Guideline 1035 To Availability and Reliability of Aids to Navigation, Edition 2, December 2004.”

[3]	http://www.hugin.com

[4]	“IALA Recommendation for a definition of the nominal daytime range of maritime signal lights intended for the guidance of shipping by day”, April 1974.

[5]	Allard E. “Memoire sur l’intensité et la portée des phares”, Imprimerie Nationale, Paris, 1876.

[6]	“IALA Recommendation O-130 on Categorization and Availability Objectives for Short Range Aids to Navigation”, Edition 1, December 2004.

[7]	“Morrison’s equation”, MIT Handouts by Prof. A.H. Techet

http://web.mit.edu/13.42/www/handouts/reading-morrison.pdf

[8]	“IALA Recommendation E-107 on the design of normal moorings”, May 1998.

[9]	“IALA Recommendation for the calculation of the effective intensity of a rhythmic light”, November 1980.

Further reading on Bayesian networks

[10]	Bayesian Networks as a Decision Support Tool in Marine Applications, PHD Thesis by Andreas Friis-Hansen, Department of Naval Architecture and Offshore Engineering, Technical University of Denmark, December 2000

[11]	“A Brief Introduction to Graphical Models and Bayesian Networks”, Kevin Murphy, 1998. http://www.cs.ubc.ca/~murphyk/Bayes/bnintro.html

[12]	Google’s list of Bayes net software:

“http://directory.google.com/Top/Computers/Artificial_Intelligence/Belief_Networks/Software/