SPANISH METHODOLOGY FOR RANGE CALCULATION ON LED LANTERNS

--------------------------------------------------------------------------------------------------------

JUAN F. REBOLLO  (Puertos del Estado-Spain)

A visual aid to navigation (AtoN) always means two different types of marks: Day mark and night mark and both are important from a theoretical point of view although the latter (lighted beacon) is more chritical for the navigator due the absence of avalaible sourrounding visual references.

It is necessary for an observer identifying a light to be inside of the lighting cone sourced by the beacon and also that the flashing code is easily perceived. The basic parameters for that are the following:

- Vertical divergence (and furthermore the pitching angle in the  case of lighted buoys, how chritical is the leading lines and, eventually, the accuracy  of  sector lights)

- Lamp ON delay time and flashing length

- Range

In all cases the navigator will claim that does not see the light. Previously to any evaluation about such a claim and before further actions are taken all the possible causes must be analized. These might be due to the equipment itself, its installation and maintenance, the observer and the weather conditions.

- Vertical Divergence

The vertical divergence (δv ) is just the angle of the vertical lighting cone section as emitted by the lighting source. Horizontal divergence  (δh )  is the angle of the horizontal lighting cone of the said source.

The vertical divergence conditions the  minimum distance use of a AtoN and the horizontal divergence is related to the light holding or retention at the observer´s eye. In other words, a wider horizontal divergence improves the observer´s light perception.

Divergence calculation is:

v = b / f                                     h = a / f

Where

“a” is the breadth of the lighting source (horizontal dimension)

“b” is the height of the lighting source (vertical dimension)

“f”  is the focal distance of the lens (in cases of allround lenses this is the half of their diameter at the optical plane)

The best solution is to get this information from experimental data related to pairs combination of standard lamp and lens or from complete LED´s beacon. Anyhow, the manufacturer must  provide this data for some standard sets.

It must be kept on mind that this divergence value refers to the complete lighting cone angle but only the area facing the sea, say the half of this cone, is the relevant point. It makes that this value is normally of just a few degrees and due to that it is very important to make sure that the beacon optical plane is perfectly horizontal.

An error on that could make the light being not correctly identified by the observer at the expected distance as the observer would be outside the lighting cone.

The formula is

D = H / tg (V/2)

Where D is the minimum  distance for using the beacon and H is the height of the optical plane over the sea level. Both values are in meters.

Beside this case there is another special one: When the beacon is fitted on a floating element or  buoy. These will pitch, making the beacon optical plane to pitch as well, what will cause important identification difficulties for the mariners.

It is essential that the pitching angle is smaller than the vertical divergence, otherwise the light could be not identify as a AtoN.

In cases of leading lights, beside the vertical divergence and the essential need that the optical plane is totally horizontal, it is also necessary that the parameter known as “sensivity” should be higher than 1.5. A lower value could make the observer unable to identify the AtoN.

Finally, regarding sector lights, there is another element to be considered: The accuracy angle. It must be defined geometrically at least and verify from the sea since these are normally operating under seriously chritical conditions for navigation. The LED´s beacons could be not usable in convencional installations with a quite high accuracy angles if the sector were too narrow.

It is indispensable to know:

- Optical system vertical divergence

- Optical plane accuracy angle in respect the horizontal plane

- Buoys pitching angle

- Leading line sentivity

- Sector lights accuracy angle

  - Lamp ON delay time and flashing length

Both the range and the flashing identification are conditioned by the flashing length and, when incandescent lamps are used, by the lamp ON delay time as well.

The lamp ON delay time on each flash is a matter related to the current intensity passing through the incandescent lamp so that all the darkness periods must be higher than those values of the graph´s 90% straight line as shown on page 124 of “Nornas Técnicas sobre Obras e Instalaciones de Ayudas a la Navegación – 1986”.

Otherwise the observer will not identify the individual flashes and, in consequence, will not identify the light as a AtoN.

On the other hand the flashing length greatly affects the intensity efficiency value hence, for an identical luminous source, a longer flashing length produces a better intensity efficiency and, in consequence, a longer range too.

The said lamp ON/OFF delay time must be taken into account in order to consider the actual flashing length. The adjusted value of such a length (50% of the formerly mentioned graph´s straight line must be used) must be the intensity efficiency calculation base. Shorter periods will produce shorter ranges.

The ON/OFF delay time does not affect the LED´s technology and did not exist too when gas powered flashers were used.

The filament current intensity is important for the this effect but it is also important for the luminous range since it is directly related to the luminous intensity. Such a filament current intensity is a function which belongs to the system  power voltage (assuming that circuit electrical resistance does not change) so that the power voltage regulation could also affect the luminous range.

  - Luminous range

The luminous range is the distance at which a light can be seen by an observer. This distance depends basically of the relative heights between the luminous source and the observer´s eye,  the luminous intensity of such a source and the metheorological conditions.

The former points such the vertical divergence, horizontal divergence, ON/OFF delay time and flashing length are luminous range limitating factors.

The direct line-of-sight  from the observer to the luminous source is limited by the Earth curvature. The concept of “geographical range” appears so and this is related to the respective observer and luminous source heights, in both cases at the medium sea level. The maximum distance at which a light can be seen is its geographical range.

In all cases the geographical range must be longer than the luminous range.

Now, a few words about the luminous intensity. This is a part of the luminous flux in a given direction.  Its unit is “candela (cd)” and this is equivalent to lumen/sterorradian.  In our case we will consider the intensity values at the optical plane directions and, specially, at the direction between the luminous source and the observer. In principle we will take the optical system as isotropic so that the intensity is identical in all optical plane directions.

We have to take three intensity types:

- Stationary intensity

- Efficient intensity

- Calculation intensity

The path from one of these types to each other is done through factors which value is always less than the unit; in other words, for a given stationary intensity the efficient intensity is lesser and at the same time the calculation intensity is lesser than the latter.

The luminous range is calculated with the Allard’s formula:

E0 * D2  =  Ic * cD

Where E0  is the light perception threshold in Luxes (illuminance or illumination level),  D  is the luminous range in meters,, Ic is the calculation intensity in candelas and “c” is the atmospheric transparency coefficient which connection to the atmospheric transmission factor “T” is:  T = c1852.

This function is presented in calculating frames designed for dark backgrounds conditions so that the perception threshold takes the value E0 = 0.2*10-6  Lux, These calculating frames are well known and can be found in “Normas Técnicas sobre Obras e Instalaciones de Ayudas a la Navegación– 1986”. It relates the  “calculation intensity” (Ic, in candelas) and the ” luminous range”  (D, in nautical miles) depending of the atmospheric transmission factor (T).

For an atmospheric visibility of 10 miles the atmospheric transmisión factor takes the value T=0.74 and the subsequent range is called “nominal range”, under night conditions and for a dark background (E0=0.2*10-6 Lux)

On the light of the said, both atmospheric visibility and background conditions are very important elements to be considered at the time of a lighted AtoN range calculation.

The calculation intensity is a consequence of the efficient intensity and the following factors:

- Intensity lose through a glass: F1 = 0.85 (Only in case that an external glass surface to the own light source plus optical lens exists).

- Intensity lose for duty conditions: F2 = 0.75. This value is for an average of cleaning and degradation conditions but it could be adjusted to other values if the information for it were avalaible. In case of LED lanterns such a value should be evaluated.

- Intensity lose for coloured filter: F3R = 0.20 (Red), F3V = 0.30 (Green), F3A = 0.6 (Yellow). These values are approximate since the right value should be provided case by case by the lens manufacturer on the light of the luminous source colour temperature. The LED´s beacons have not a filter so that the factor F3 will take the value 1.

- Intensity lose for background lighting: F4 = 1 for dark background and F4 = 0.20 for a high background lighting. The value F4 will take intermediate values depending of the background lighting in respect the line-of-sight between the luminous source and the observer.

Then, the calculation intensity will be:   Ic = F1 * F2 * F3 * F4 * Ie, where  Ie is the effective intensity.

The effective intensity is related to the system own stationary intensity as well as to the flashing length so that its value will be higher with a longer flash and/or with a higher stationary intensity. In both cases the power consumption optimization will act as a limit, sometimes, for these parameters.

The efficient intensity calculation will be obtained with the Blondel-Rey-Douglas method (III):

Ie=Iott+awhere “t” is the flashing length (it is assumed that all are identical otherwise the worst case must be taken, say, the shortest flashing length) and “a” takes the value 0.2 (which is a Blondel-Rey constant value related to the images retention threshold at the retine). Io  is the flashing intensity peak, equivalent to the stationary intensity.

The used method is important since relevant differences could happen if   either method I (Schmidt-Clausen) or method II (Allard) or method III (Blondel-Rey Douglas) were applied.  For LED’s the factor F of method I is F=1, and then the method I give the same results that the method III.

Regarding LED´s beacons it would be necessary to search for a new effective intensity calculation procedure, bearing in mind the flashing manner of this kind of beacons. Notwithstanding the said methods will be applied meanwhile.

Now, the stationary intensity is given by the system itself, seen as a set of luminous source plus optical lens. Despite a  calculation way is going to be shown, this value should be obtained from laboratory tests. In cases of LED´s beacons or encapsulated systems on which the luminous source and the optical lens are a solid set, this value should be given by every manufacturer.

The stationary intensity appproximate value as obtained from a calculation method is:    Io = K * L * b * h    , where

K= 0.89 cos α

L= Illuminance or luminous source brightness (cd/cm2)

B = Height or vertical dimension of the source (cm)

h = Height (vertical projection) of dioptric (cm)

α = Angle between the optical axe and the farest dioptric element

The lamps manufacturers normally give the luminous flux in lumens and it is necessary to become in cd/cm2, depending of the flux distribution type (spherical, toroidic, etc). In order to simplify the procedure only spherical distribution will be taken into account. In consequence, for a lamp with dimensions “b”, “a” where “a” is the width or horizontal projection of the luminous source (filament) and  “b” is the vertical dimension of the said source, the total flux is   (lumens):

L=φ4πab

The calculation procedure to get to stationary intensity of an optical set for a certain nominal range is the following:

- Go into the calculating frame “luminous range – calculation intensity” applying the Allard’s formula. Then the value Ic to the value T= 0.74 is obtained.

- Calculate the effective intensity value, by mean of the formula

Ie = Ic / (F1* F2 * F3 * F4 )

- Io=Iet+atCalculate the stationary intensity value  by mean of the formula,

The value Io (cd)  is the value to be provided by the system (luminous source plus optical lens) and must be either granted by the manufacturer or calculated on the light of the lens and lamp to be used.

An exemple :

Nominal range = 5 nautical miles

Green colour; high level of background lighting; flashing length 0.5 seconds (ON/OFF delay time is not taken into consideration here)

Ic = 80 cd (on the Allard’s formula calculating frame)

F1 = 1, no additional external glass (conventional beacon)

F2 = 0.75 (médium cleaning and degradation degree))

F3V = 0.30 (green lens; LED’s: F3 = 1)

F4 = 0.2 (high background lighting)

Ie = 80 / 0.75*0.30*0.2  = 1.778 cd  (533 cd, estimated in case of LED’s)

Io = 1778 (0.5+0.2)/0.5 = 2.489 cd. (746 cd, estimated in case of LED’s)

In other words, we will need a beacon (lamp plus optical lens) with an stationary intensity of at least 2489 cd (746 cd estimated in case of LED´s) to provide a nominal range of 5 miles with a green lens beacon, at a place with a high background lighting and with 0.5 seconds flashes.

Finally, although all the calculation conditions are fullfiled, the luminous range to be perceived by a navigator will depend of the atmospherical visibility between the beacon and himself since all the installation is under the 10 miles in terms of atmospherical visibility conditions.

In consequence, if the atmospherical visibility were normally lower than this value, a mariner will not perceive the lighting signal at the expected range. Then the navigator should know the atmospherical visibility in this zone and at this time and make the necessary corrections to learn the distance at which the beacon is visible, knowing its nominal range.

The atmospherical visibility can be estimated by trying to see surrounding well known points which distance is also known. So, if atmospherical visibility were 7 nautical miles the installation of our exemple, with a perfect nominal range of 5 nautical miles, would be visible at only 4 nautical miles.

So that a luminous signal perception value data limit  must be toghether the atmospherical visibility value in the zone as well as with the course on which the observer can see the beacon since the atmospherical visibility probably changes depending of the course to the beacon.

February, 2004