PRODUCT CERTIFICATION TEMPLATE 1

Lanterns for buoys and light-beacons, including lanterns with enclosed rotating optics

# Purpose

Product templates identify parameters that should be tested and measured and the standards against which each parameter should be tested and measured to enable a comprehensive evaluation of products.

# Scope

The template is applicable for lanterns used on buoys and light beacons and  includes lanterns with enclosed rotating optics, powered via external power supplies or via integrated solar power supplies.

Applicable documents, Recommendations and Guidelines

Iala documents

- IALA Recommendation E-122 (2001) ‘Photometry of Marine Aids to Navigation Signal Lights’

- IALA Recommendation E-200-1 (2005), Part 1 – Colour

- IALA Guide To Ambient Light Levels At Which ATON Lights Should Switch On And Off.(No. 1038)

- IALA Recommendation for the notation of luminous intensity and range of lights. (1966)

International Electrotechnical Commission (IEC) documents

- IEC 61215 Crystalline silicon terrestrial photovoltaic (PV) modules - Designqualification and type approval

- IEC 60945 Maritime navigation and radiocommunication equipment and systems - General requirements - Methods of testing and required test results

- IEC 60068 Environmental testing

Japanese International Standards (JIS)

- JIS C 0028 - Basic environmental testing procedures Part 2: Tests Test Z/AD: Composite temperature/humidity cyclic test

- JIS C 0911 Vibration test method of small-sized electrical machinery & apparatus

United States Military Standards (MIL-STD)

- MIL-STD-202G - Test Method Standard, Electronic and Electrical Component Parts.

# Test Standards

The Templates quote national and/or international standards. If an IALA Product Certification Authority (IALA-PCA) wishes to use an alternative standard, it is the responsibility of the IALA-PCA to confirm that the standard used conforms to the IALA quoted standard. Where no test standard is mentioned the manufacturer shall state the test standard used, preferably with reference to international or national standards.

## Product Certification Template 1 – Lanterns for buoys and light-beacons, including lanterns with enclosed rotating optics

Product Type / Manufacturer Name and Reference Number:

| Para No. | Parameter category | Parameter | Measured Value | Test Method | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | ### Optical |  |  |  |  |
| 1.1 |  | Effective luminous intensity | Candela | Photometric measurement IALA Recommendation E-122 (2001) | For LEDs the effective luminous intensity varies depending on duty cycle |
| 1.2 |  | Flash duration and flash shape (LED lights included) |  | Photometric MeasurementIALA Recommendation E-122 (2001) | Time between points of 50% peak intensity. Define stability of rotation for a rotating beacon. For LED define frequency modulation of light, intensity profile of the flash |
| 1.3 |  | Horizontal beam uniformity (omnidirectional light) or horizontal divergence (range lights and rotating optics) |  | IALA Recommendation E-122 (2001) | For omnidirectional lights , state maximum variation from the mean intensity over any angle in the horizontal plane. For range lights and rotating optics for the horizontal beam, state the horizontal divergences at 50% of peak intensity. The manufacturer shall provide a light distribution curve. |
| 1.4 |  | Vertical divergence | Degrees | Photometric Measurement IALA Recommendation E-122 (2001) | From the vertical beam, state the vertical divergences at 50% of peak intensity as referenced in IALA Recommendation E-122 (2001) |
| 1.5 |  | Signal colour |  | IALA Recommendation E-122 | Colour boundaries and colour shift (for LED, variation with duty cycle) in the regions as defined in IALA Recommendation E-200-1, Part 1 – Colour The manufacturer shall provide the measured coordinates on a chromaticity diagram. |
| 1.6 |  | Nominal range | ## Nautical Miles |  | The nominal range is derived from the effective intensity (see 1.1.) in accordance with the applicable IALA recommendations listed below: IALA Recommendation for the notation of luminous intensity and range of lights. (1966) IALA Recommendation for a definition of the Nominal Daytime Range of Maritime Signal Lights Intended for the Guidance of shipping by day (1974) State applicable atmospheric transmission factor |
| 2 | ### Electrical – externally sourced | Power supply normal and extreme voltage | Volts | IEC 60945 section 7 | State design voltage range |
| 2.1 |  | Power consumption | Watts |  | For flashed light sources quiescent and ‘lamp on’ power should be quoted. Power consumption for each available flash character and in the case of LED lanterns each available power level should be made available upon request. |
| 2.2 |  | Reverse polarity circuit protection |  | IEC 60945 section 7.2 | Indicate protection method. |
| 2.3 |  | Power supply and control and monitoring terminations/connectors |  |  | Indicate connector and/or terminal type |
| 3 | ### Control |  |  |  |  |
| 3.1 |  | Daylight control | Lux |  | Lux level and switching range, reference to IALA GuideN°. 1038 Indicate the hysteretic level and time delay. |
| 3.2 |  | Monitoring |  |  | State parameters that can be monitored |
| 3.3 |  | Programming |  |  | State parameters that can be programmed and method of programming |
| 3.4 |  | Light source voltage regulation |  | As defined by manufacturer | State the accuracy of voltage regulation to light source in % and indicate the method of testing |
| 4 | ### Physical |  |  |  |  |
| 4.1 |  | Maximum Height | mm |  | State the dimension |
| 4.2 |  | Maximum Diameter | mm |  | State the dimension |
| 4.3 |  | Maximum Weight | kg |  | State the weight |
| 4.4 |  | Focal Height | mm |  | Measure from base to focal plane |
| 4.5 |  | Materials of construction |  |  | State the material for the lantern, lens, and glazing. |
| 4.6 |  | Ingress Protection ( water resistance ) |  |  | Define applicable IP number to EN60529 |
| 4.7 |  | Description of lantern mounting hole pattern |  |  | Diameter and spacing of holes for fixing bolts |
| 5 | ### Environmental |  |  |  |  |
| 5.1 |  | Temperature – operational and storage |  | Manufacturer to state the test standard | State the operational and storage temperature range. Manufacturer to submit the test results and the method of testing |
| 5.2 |  | Humidity – operational and storage |  | IEC 60945 section 8.3; IEC 60068-2-3 JIS C 0028; MIL-STD-202G-103B | State the operational and storage humidity range. Manufacturer to submit the test results and the method of testing |
| 5.3 |  | Salt air and sea water spray |  | MIL-STD-202G-101E | Manufacturer to submit the test results and the method of testing |
| 5.4 |  | Shock and vibration |  | JIS C 0911; MIL-STD-202G-201A/202D | Manufacturer to submit the test results and the method of testing |
| 5.5 |  | Electromagnetic interference |  | IEC 60945 sections 9 & 10 | Manufacturer to submit the test results and the method of testing |
| 6 | ### Service |  |  |  |  |
| 6.1 |  | Designed Service life. | Years |  | State if any components are designed to be replaced or serviced during the service life of the lantern Indicate UV stability of lens and glazing (if applicable) |
| 6.2 |  | Maintenance intervals | Hours |  | State the maintenance interval for each replaceable component |
| 6.3 |  |  |  |  |  |
| 7 | ### Disposal and recycling |  |  |  |  |
| 7.1 |  | Recycling |  |  | Indicate whether the product is recyclable. State whether the manufacturer will undertake the recycling of the product Reference: IALA guideline 1036 |
| 7.2 |  | Disposal |  |  | State any material content that would harm the environment, if the product is disposed of as normal waste Reference: IALA guideline 1036 |
| 8 | Safety |  |  |  |  |
| 8.1 |  | Worker safety |  |  | Manufacturer to state any special requirement to ensure workers’ safety in the use of the product |

 Where no test standard is mentioned, the manufacturer shall state the test standard used, preferably with reference to international or national standards.