The General Lighthouse Authorities, Research and Development Department

# Abstract for IALA Shanghai 2006

Lighthouses have long been established around the coast of the British Isles. Changes in technology have affected their use and mode of operation meaning that they have become more efficient, more reliable (current availability is 99.8%) and have a greater variety of use. The GLAs (General Lighthouse Authorities) have had to respond to these changes in a positive manner. The use of modern technology has encouraged mariners to venture into waters not previously navigated and this has increased requirements for visual AtoNs (Aids to Navigation). The growth in residential areas has meant an increase in street lighting and when viewed from the sea it is sometimes difficult to distinguish the AtoN lights. In other words the light emitted by the lighthouse has become less conspicuous than it was originally, through no fault of its own. Some examples have been identified and attempts have been made to increase the conspicuity of the light, but as yet no specific recommendations have been made for dealing with this problem. This project sets out to identify the factors involved and develop recommendations for future use when considering the conspicuity of traditional marine aids to navigation signal lights.

Depuis longtemps des phares sont présents le long des côtes des îles britanniques. L’évolution de la technologie a affecté leur utilisation et leur mode de fonctionnement, les rendant plus efficaces et plus fiables (leur disponibilité actuelle est de 99,8%) et leur offrant une gamme plus large d’utilisations. Les Administrations générales des phares du Royaume Uni (GLAs) ont eu à répondre à ces changements de façon positive. L’utilisation de la technologie moderne a encouragé les marins à s’aventurer dans des eaux où l’on ne naviguait pas auparavant, ce qui a accru la demande en aides à la navigation visuelles. L’élargissement des zones résidentielles a, aussi, entraîné une augmentation des éclairages publics et, vu de la mer, il est parfois difficile de distinguer les feux des aides à la navigation. En d’autres termes la lumière émise par le phare est moins discernable qu’à l’origine, bien qu’on ne puisse l’en accuser directement. Quelques exemples ont été identifiés et l’on a essayé d’améliorer la visibilité des feux, mais jusqu’à maintenant aucune recommandation n’a été formulée pour remédier à ce problème. Le projet consiste donc à déterminer les facteurs influents et à développer des recommandations à suivre lors de l’étude de la visibilité des feux des aides à la navigation traditionnelles.

Malcolm Nicholson

Lights Specialist

(Colour and Vision)

## Introduction

If we describe conspicuity as how well something stands out from its surroundings, then  aids to navigation (AtoN) with constant characteristics are insufficient, as in some situations they will be conspicuous, but in others they will not. If a standard AtoN could adapt to its situation within predetermined boundaries it would be possible to provide effective AtoN in all situations. This, however requires a greater understanding of what the surroundings are and how the AtoN can be most effective.

As yet there is no measure for conspicuity, but by attempting to measure,  objectively or subjectively, all the factors relating to conspicuity a model can be built that can be applied to all situations.

## Conspicuity factors

The first thing to consider is what is the purpose of the AtoN. In most situations an AtoN provides the mariner with location information via time coding of information. In other words it gives the mariner an idea of his position with respect to the AtoN and the flash character is specific to the AtoN. It is recognised that in the modern world of digital navigation, visual signals provide a confirmation of position, rather than a primary fix. In many ways this could help in the provision of conspicuous visual signals as a unique flash character is perhaps no longer necessary, allowing sequenced or synchronised AtoN, for example.

The second thing to consider is under what conditions the observer is viewing the AtoN. One factor is the state of adaptation of the observer’s eye. Recent research suggests that cool white, blue and some green light signals appear to be brighter as the eye becomes dark adapted, whereas  warm white, red, yellow and some other green light signals  appear less bright as the eye becomes less sensitive to these colours. Another factor is  the meteorological conditions under which the AtoN is being viewed. Meteorological Visibility – as defined by IALA is ‘the greatest distance at which a black object of suitable dimensions can be seen and recognised by day against the horizon sky.’ The World Meteorological Organisation recommends that this distance is when the object is reduced to 5% of its original size (contrast threshold) for a safe limit of contrast. A subjective assessment of visibility can be made in these conditions, but it is very difficult to make an assessment during darkness.

In darkness the safe limit of contrast for the recognition of light signals is 0.2 microlux received by the eye of the observer. The figures for the nominal range of light signals are published in the Admiralty List of Lights. It gives the luminous range of the light signal when the meteorological visibility is 10 sea miles.

Allard’s Law is used to calculate visibility. Although the distance at which the light can no longer be seen is subjective, the true visibility lies within a small distance span.

The equation for Allard’s Law is written:-

E = ( I x TD) / D2

Where Illumination (E) = sea-mile candela (lumen/M2)

Intensity (I) = candela (cd)

Distance (D) = nautical miles (M) and

Transmissivity (T) = per nautical mile

Allard’s Law dates from 1890 and is based on the work of Koschmeider, circa 1865. Koschmeider’s extinction co-efficients are different for different colours. This was omitted from Allard’s work, however it can be taken into account in modern  calculation software.

The amount of light received by the observer from a beacon is affected by ambient and background lighting conditions as well as transmissivity.

During normal daylight conditions, in order for a light signal to be detected, the safe limit of contrast is 0.1milli-lux? (for 10000cd/m2 ambient light).

To achieve a nominal range of 4M during normal daylight conditions the light signal must have an intensity of 182,000cd. This intensity has a nominal range of 21M during darkness. Only 36cd is required for a nominal range of 4M during darkness.

Ambient light is omni-directional (all-round). Background light differs in that it is directional (from the direction that the observer is looking). However background lighting also has an omni-directional component, which makes it a complex issue. Current practice is based on luminous contrast, using different levels of illumination at the observer’s eye for different levels of background lighting and again, not considering colour.

Historically, AtoN light signals have been flashed to give them a unique identification. This has led to various models for calculation of effective intensity from Allard through Blondel-Rey to Schmidt-Clausen. The latter model has been shown not to work for complex flashes i.e. pulse- width modulated flashes. Much of this work has been based on threshold levels, however, there are a number of  physiological effects that can be utilised (with the advent of modern technology) to enhance conspicuity at supra-threshold levels which may be more pertinent in today’s environment.

Perhaps the most prominent factor is change. The eye is very adept at detecting change in level. Flashing lights are more conspicuous than fixed or steady lights in a two dimensional plane, whereas a chain of sequenced, flashing lights provides three dimensional spatial awareness.

## The Model

It is the intention of this project to combine all of the above-mentioned factors in a two tiered software model for use in the selection of AtoN when conspicuity is an issue. The top-level tier will default to all the current recommendations in practice, i.e. nominal range, photopic intensity etc. However for those wishing to further enhance the conspicuity of AtoN, a lower tier will be available for selection of special effects. As and when these effects are fully quantified and approved they can migrate to the top tier.