LEDs Light with Flicker

Japan Coast Guard

Japan Aids to Navigation Association

Overview

Flashing light with flicker is known by its unique stimulus to the visual sensation, which has better conspicuity. Focusing on such a characteristic, this study was conducted by a committee consisting of academic and maritime experts with the personnel of the Japan Coast Guard regarding possibility to utilize the flashing light with flicker for AtoN as a countermeasure against background lights.

The committee found excellent conspicuity and recognizability of LEDs Light with flicker by a field survey in a coastal area with many background lights.

Method of Experiment

Two types of lights were lit simultaneously to compare how those lights were seen. On one hand, an LEDs lantern currently deployed (standard LEDs Light) flashed with the duration of 0.4 second. On the other hand, an LEDs lantern with a flickering function newly employed in this study was lit with the Pulse-Width-Modulated (PWM) lighting method. The LEDs Light with flicker was lit four times with higher luminous intensity for shorter lighting durations to equalize the light quantity (the product of luminous intensity and its duration) of both lights.

The port of Yokohama was selected for the site to conduct a field survey because it has many background lights. 20 persons including nine persons with experience as a navigation officer participated in the survey conducted on November 30, 2006.

Outcome

Conspicuity and Recognizability

The results of the survey conducted at distances of 2.0, 1.5, 1.0 and 0.5 nautical miles as well as of 300 meters showed excellence of the LEDs Light with flicker both in conspicuity and recognizability compared to the standard LEDs Light. The difference was especially significant in the distance within 1.0 nautical mile.

“Irritation” by flicker

Regarding the “irritation” caused by the flicker, majority of the participants answered “No inconvenience for navigation” at distances of 2.0, 1.5 and 1.0 nautical miles. Even at the closest distance of 300 meters, the participants who answered “Irritated/Uncomfortable” were less than the half of all participants.

Brightness

The brightness of the LEDs Light with flicker was on a par with that of the standard LEDs Light at distances of 2.0, 1.5, 1.0 and 0.5 nautical miles as well as of 300 meters.

Future Tasks

The committee identified the following tasks to be resolved and will consider them toward practical applications of the LEDs Light with flicker.

- Confirmation of brightness

In the field test, the light quantity of the LEDs Light with flicker was set the same as the standard LEDs Light; and its brightness appeared to be equivalent to the standard LEDs Light. However, it is required to confirm the results by laboratory tests in various conditions.

- Method for effective luminous intensity measurement

Since the method for effective luminous intensity measurement has not established for the LEDs Light, the study of luminous intensity must be continued.

- Practical lighting conditions

It must be examined that the most economical (efficient) and effective lighting conditions by varying parameters such as the frequency of the PWM lighting method and duty ratio.

- Study on applicable scope

Based on the practical lighting conditions, scale of the light and the applicable scope for AtoN (available distance) must be determined.

- Other applications of the Lights with the PWM Lighting Method

An application to the fixed and flashing light will be studied.