RAPPORTEUR REPORT ON ADVANCES IN BATTERY TECHNOLOGY

Submitted by Wayne Danzik, U.S. Coast Guard, as input to EEP9

The following are the activities now being carried out by the U.S. Coast Guard:

- Ice buoys will be converted to Lithium-Thionyl-Chloride primary (non-rechargeable) technology, replacing alkaline batteries.  Battery capacity is reduced by 2/3 as a result of lower power requirements from LED lanterns, and the physical package is 9" diameter by 3" tall versus 9" diameter by 30" tall.  Cost is similar to the alkaline batteries.

- One vendor (BWT Lighting) of self contained 4 mile LED lanterns is using rechargeable Lithium-Ion technology replacing lead-acid.  Weight reduction is about 75%.

- The required UN testing of lithium packs has made them much safer to use, and built in protection features avert the catastrophic failures that have occurred in the past.

Relevant web sites are listed below:

http://www.bwtek.com/

http://www.tadiranbatteries.com/home.php