EEP 9    Developments in Aids to Navigation

There are considerable changes underway in the operating principles of ships radar.

Although this is a development in a navigation aid rather than an Aid to Navigation I believe it will have relevance to our future work as it will change the interaction between the mariner and many of the AtoN that we provide.

There have been extensive developments in military radar but commercial ships radar works on basically the same principals as when it was introduced 60 years ago.

The magnetron has remained at the heart of the system and developments in signal processing have gradually improved radar performance.

Kelvin Hughes have introduced “Sharp Eye” which borrows many features from military radars and no doubt other manufacturers will follow with similar products. Sharp Eye uses solid state power amplifiers in the place of the magnetron and utilises Doppler effect to determine the velocity of targets. The radar can send longer pulses than is possible with a magnetron and fine target resolution can be obtained by encoding these long duration pulses. Very small targets can be illuminated with sufficient energy for detection.

These new radars will provide greatly enhanced detection of small targets and should significantly improve the radar recognition of buoys and beacons, enabling mariners to place greater reliance on the identification of  small AtoNs fitted with simple radar reflectors.