- Effective Intensity of LED Based Light

- Tadayoshi Imai

- Japan Coast Guard

- Abstract

Effective intensity of pulse-width-modulated LED based light is studied. First, visibility test was conducted with various duty ratio and frequency at different illuminance levels. Second, effective intensity was calculated based on Schmidt-Clausen’s formula. It was found that there were differences between calculated effective intensity and experimentally determined effective intensity. The formula seems to underestimate effective intensity of pulse-width-modulated LED based light.

- I  Introduction

The Japan Coast Guard organized a research committee with learned persons and experts and the committee is conducting a study on evaluations of effective intensity of Pulse-Width-Modulated lighting method with LED light sources and its utilizations.

LED is recently becoming a mainstream as a light source of AtoN rather than incandescent lights because a main characteristic of LED is excellent at electric response, in other words, LED generates sharp square-wave flashing lights with complete control.

In these years, a light apparatus which takes full advantage of the characteristic of LED lighting method called Pulse Width Modulation (PWM, Fig.1) is commercially available and it can emit multi intensity light. It controls emissions by varying duty ratios (Du) in high-frequency regions over critical fusion frequency.

- Fig. 1: Pulse Width Modulation (PWM). F13s is usually used for a lateral mark in Japan.

- II  Examined items

- (1) Effective intensity of LED

・Effective intensity of LED using PWM lighting method

・Effective intensity of square wave flashing light using LED (verification of Schmidt-Clausen’s formula).

- (2) Further use of PWM light

・Utilizations of a PWM light for AtoN as a measure for power savings, visibility improvements and background luminance.

- Utilization of a PWM light in a flicker frequency region.

- III  Visibility test

As for effective intensity of PWM light, the committee analyzed and verified the data obtained from the visibility test conducted indoors.

- Outline

| Date and hour | Aug. 1-8, 2005(Except 6 and 7) |
| --- | --- |
| Place | The third light measurement laboratory of Japan Coast Guard Research Center |
| Observer | 10 (Male:7 Female:3 Average age 24) Eyesight 0.8-1.5 (Naked or Corrected) |
| Distance | 10m |
| Time for dark adaptation | Above 20mins |

・All observers are tested simultaneously in two ways, in ascending order (from dark to bright) and in descending order (from bright to dark), under a same observation condition.

- Method

Light intensity steps of the PWM light were nine; 60%, 70%, 80%, 90%, 100%, 120%, 140%, 160% and 200% of the intensity of a reference light. Light intensity was controlled by current. The relation between electric current in LED and light intensity is accurately measured prior to the observation.

The reference light (fixed light) and the PWM light (F13s, Flashing duration: 0.4sec) were shown to each observer simultaneously. The light intensity of the PWM light was varied and the observers press a switch wired to the indicator at the moment that the two lights were seen with the same brightness. (Results were recorded by an operator)

The experimental conditions were determined according to light color, frequency and Du as shown in Table 1. The number of measurements was 360. (Only white and red were used for the glare level)

- Table 1: Experimental conditions

| Illuminance level | Color | Frequency | Duty ratio |
| --- | --- | --- | --- |
| Threshold （2×10-7lx） Useful （5×10-６lx） Glare （1×10-３lx） | White Red Green Yellow | 100Hz 50Hz 40Hz 30Hz 20Hz 10Hz | 90% 70% 50% 40% 30% 10% |
| 3 | 4 | 6 | 6 |

(3) Experiment apparatus

A simplified circuit diagram is shown in Fig. 2 and the experiment scene is shown in Fig.3.

- Indicator

A lamp lights up when observers press the switch.

- LED

- White: Toshiba TLSH157P

Red: Nichia NSPW500BS

Green：Nichia NSPG500S

Yellow：Toshiba TLYH17TP

- Fig. 2 Simplified Circuit Diagram

Controller     　　　　　　　　　　　　　　　Indicator

Front view of the experiment apparatus　 　   　　    　Experiment scene

- Fig.3 Experimental equipment

- IV  Results

Peak intensity (Io) of the PWM light was measured when observers think that it is the same brightness as that of the reference light. Then, geometric mean values of the data observed by the ten observers were calculated. The measurement was done in ascending order (from dark to bright) and in descending order (from bright to dark) under same observation conditions (color, illuminance level, frequency and duty ratio)

Based on this data, light quantity ratios P of the reference light and the PWM light when they were seen with the same brightness were calculated by the equation (3) below and characteristics of various observation conditions in different colors, illuminance levels, frequencies and Du were also calculated (Fig. 3).

- Fig. 3

P=Io⋅Du⋅tIe⋅t=Io⋅DuIe　・・・・・・(3)

where

Io： Peak intensity of the PWM light

Du：Duty ratio

Ie：Intensity of the reference light

t：Flashing duration (0.4sec）

Here, the reference light is a fixed light which is sensed with the same brightness as that of the PWM light therefore the intensity Ie shows effective intensity of the PWM light.

- (1) Threshold and useful level

Fig. 4 and 5 are graphs which show characteristics at a threshold level by Du for the horizontal axis and light quantity ratio (P) for the vertical axis

Almost the same trend were seen in all colors therefore all the light color data were mixed, averaged geometrically and characteristics of each frequency were obtained as shown in Fig. 4.

- Fig. 4: Light quantity ratio, duty ratio and frequency at a threshold illuminance level

- Fig. 5 shows characteristics obtained by mixing and averaging all the data geometrically.

- Fig. 5: Averaged light quantity ratio and duty ratio at a threshold illuminance level

As seen in the equation (3), if P is smaller than 100%, the PWM light is perceived with the same brightness as those of reference lights by smaller light quantity than those of reference lights and if P is more than 100%, the PWM light is sensed with the same brightness of those of the reference light therefore larger light quantities are required for the PWM light comparing with the reference light.

As understood by Fig. 4 and 5, the result from a threshold illuminance level revealed that PWM light and the reference light require almost the same light quantity at any Du.

Also, as for a useful illuminance level, the same trend was seen as shown in Fig. 6 and 7.

- Fig. 6: Light quantity ratio, duty ratio and frequency at a useful illuminance level

- Fig. 7: Averaged light quantity ratio and duty ratio at a useful illuminance level

- (2) Glare level

White and red were used for the measurement at glare level (corneal illuminance: 1.0×10-3lx). The data of these two colors were mixed and averaged geographically. Fig.8 shows characteristics of each frequency.

Also, Fig. 9 shows intensity at 20Hz and 10Hz which are below the critical fusion frequencies that almost all the observers perceived flickers.

Fig. 8: Light quantity ratio, duty ratio and frequency at a glare illuminance level

- Fig. 9: Light quantity ratio, duty ratio, frequency(20Hz and 10 Hz) at a glare illuminance level

As mentioned in the previous section, at threshold and useful levels, the PWM light required the light quantities which are the almost same as those of reference lights for any Du, however, at the glare level, light quantity ratios were below 100% for any frequencies and Du as shown in Fig. 8.

Moreover, focusing on 20Hz and Du=30% shown in Fig. 9 (lighting pattern is shown in Fig. 10), the PWM light was seen with the light quantity of approximately 60% of those of the reference light.

- Fig. 10: Lighting pattern (Fl3s, the lighting time of 0.4sec consists of eight flashes of 15ms)

This is considered to be Brucke-Bartley Effect. As shown in Fig. 9, light quantity ratios reduce as Du becomes smaller therefore in case the light quantity is the same, increasing peak illuminances is more preferable for the better vision than increasing Du.

- V  Discussions

- Effective intensity of LED

For evaluating the effective intensity of the PWM lighting method, it seems suitable to use the following method.

From the equation (3);

P=Io⋅DuIe≒１

∴Ie=Io×Du

From the equation (1)

Ie≒Iot1t1+t2　・・・・(4)

Equation (4) is called Talbot’s law. It is suggested that the equation (4) is appropriate to evaluate effective intensity of PWM lighting method using LED.

- Effective intensity of square wave flashing light using LED (Verification of Schmidt-Clausen’s formula)

As one of the methods to calculate effective intensity of PWM flashing lights, it is appropriate to follow Talbot’s law as mentioned above.

Moreover, the effective intensity of a non-PWM square-wave flashing light is expressed as equation (5) which is derived from both by Schmidt-Clausen and Blondel-Ray.

Ie=Io×tC+t・・・・(5)

Ie：Effective intensity

Io：Peak intensity

t：Flashing period

C：Time constant of optical sensation（Nighttime: 0.2）

According to the equation (3), the effective intensity of a flashing period of 0.4sec (Du=100%) is 0.66 times as large as that of a fixed light.

The inverse number of 0.66 is 1.51, however, as seen from the PWM experiment shown in Fig. 5, even flashing light of Du=90% (flashing period: 0.4sec) were perceived with almost the same brightness as that of the reference light, with the intensity of 1.13 times as large as that of the reference light.

Since the effective intensity of square-wave flashing light calculated from the equation (5) may be underestimated compared with actual visions, it should be verified by further experiments.

- VI  Additional experiment

As mentioned above, it was found that evaluated effective intensity of LED flash lights were different from those of theoretical values calculated from Schmidt-Clausen and Blondel-Rey. Therefore, this additional experiment was conducted to clarify the relation between flashing time and effective intensity.

- Outline

| Date and hour | Nov.24-29,2005(Except 26th and 27th ) |
| --- | --- |
| Place | The third light measurement laboratory of Japan Coast Guard Research Center |
| Observer | AtoN Engineering Division Staff: 3 (Male:3 Averaged age: 24) Eyesight 1.0-1.2 (Corrected) |
| Distance | 10m |
| Time for dark adaptation | Above 20mins |

- Observers are tested one by one in two ways, in ascending order(from dark to bright) and in descending order(from bright to dark), under a same observation condition.

- Experimental method

A reference light (fixed light) and a PWM light (F13s, flashing duration: 0.1-2.0sec and fixed light) are shown to each observer using the same equipment as used for the previous test.

In this experiment, to enhance the accuracy, luminosity of the PWM light was changed continuously.

Experimental conditions for the PWM light are shown in Table 2

(Number of measurements: 60)

- Table 2

| Color | Illuminance level | Frequency | Duty ratio | Flashing period(sec) |
| --- | --- | --- | --- | --- |
| White | Threshold （2×10-7 lx） Useful （5×10-6 lx） | 100Hz | 100% 70% 50% | Fixed light，2.0，1.5，1.0，0.7，0.5，0.4，0.3，0.2，0.1 |
| 1 | 2 | 1 | 3 | 10 |

- Experimental result

An evaluation method used for the test was the same as that of the test in section 4. Fig. 11 and 12 show the results. These results suggest that effective intensity calculated by Schmidt-Clausen’s formula is underestimated with a square-wave flashing light of LED. The results also suggest that a flashing light of the period of 0.4 sec may be evaluated as fixed lights.

Fig. 11 Light quantity ratio, flashing duration and duty ratio at a threshold illuminance level

- Fig. 12 Comparison of light quantity ratio at a threshold illuminance level

- VII  Conclusions

- (1) Effective intensity of PWM lighting

At the corneal illuminance at threshold and useful levels, reference lights and PWM lights were perceived equally by almost the same light quantity and outstanding variations were not observed even when the frequency and Du were varied.

Therefore, the effective intensity of PWM lighting may be evaluated by Talbot’s law.

- (2) Effective intensity of a LED square-wave flashing light

Schmidt-Clausen’s formula may underestimate effective intensity of a LED square-wave flashing light. Furthermore, flashing lights with flashing duration greater than or equal to 0.4 sec are regarded as fixed lights.

- (3) Flicker frequency region

In a flicker region, large amount of gains are obtained for effective intensity and light quantity (electric power consumption). Since extremely excellent conspicuity exists in this region, it may be possible to use the flicker frequency region for short ranges as a countermeasure for background luminance.

However, flashing lights in the flicker region may be uncomfortable or burdensome for some people thus they must be treated with care.

- VIII  Future use of LED’s PWM lighting method

Considering the results mentioned above, it will be possible to obtain advantages shown below by using LED’s PWM lighting method.

・It can be expected to save power consumption if evaluation methods for effective intensity are established.

・Fixed and flashing light are generated by extremely low power consumption comparing to electric bulbs therefore it will be easier to employ fixed and flashing lights even for AtoN for which it is difficult to secure electric power.

・Light control is easier therefore the number of types of light equipment may be reduced

・It is possible to utilize the significant conspicuity of the flicker frequency region as a countermeasure for background luminance.