FLOOD LIGHTING OF STRUCTURES

Background

The Swedish Maritime Adm. (SMA) maintain and run 1100 lighthouses and lighted beacons. 350 of these light houses and beacons have their structure and rock-base flood-lighted. SMA have in addition to this abt. 800 flood-lighted marks.

Flood-lighting is considered by the mariner to be an important AtoN function, in many cases considerably more important than the flashing light. The projected indirect reflection gives the user a good support to judge the distance to the object.

Flood-lighted objects are normally used within a distance up to 1 M.

Conventional constructions

Light houses and beacons

The light source in these constructions is Seald Beam lamp PAR 36 SB4589 28V 50W. To prolong the lifetime of the lamps we are running them on 22V. SMA normally use 4 lamps on a beacon (2 m in diameter) and abt. 8–12 lamps on our bigger caisson light houses, i.e.total illumination power 200– 600W.

Marks

Marks consist in the most cases of a plastic cone, which is illuminated from the top downwards. When yellow light is needed Low Pressure Sodium lamp is used alt. a Mercury Vapour lamp when white light is needed.

Research and development

Due to energy consumption flood lighting has to be runned by the mains supply.

Some installations, which by remote control are switched on when needed can be run by PV.

Installations connected to sea cables mean high investments and running cost and therefore we are looking for alternative installations which can run on PV (see progress report Photovoltaic supplied LED-floodlighting and extended light sources)

070411 Christian Lagerwall