Input paper:   ENAV27-n.n.n

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					□  Input

X  ENAV	□  VTS							X  Information

Agenda item 			n.n

Technical Domain / Task Number 2	Digital Communications

Author(s) / Submitter(s)			Jan Šafář, Alan Grant, Martin Bransby

# VDES Testing in the Times of COVID-19

# Summary

This document reports on the development of a (COVID-19 secure) VHF Data Exchange System (VDES) testbed off the coast of East Anglia. After a description of the testbed area and the intended time frame, the document describes the testbed system architecture and equipment setup and considers co-existence with co-located VHF systems. Information on system calibration and radio licensing is included and details of the test methodology are described. Preliminary results from a sea trial conducted in January 2021 are then presented and discussed. Finally, a summary of the key findings and conclusions from the work to date and recommendations for further work is provided.

## Purpose of the document

The purpose of this document is to inform members of the IALA e-Navigation Committee of on-going VDES trials being carried out by the General Lighthouse Authorities of the United Kingdom and Ireland (GLA), and to elicit feedback on the preliminary findings, conclusions and recommendations presented herein.

## Related documents

IALA Guideline 1139, “Technical Specification of VDES”.

# Introduction

This work follows on from the VDES channel sounding campaign [1] and other VDES-related activities carried out by the GLA Research & Development Directorate (GRAD) in previous years. The aim of the current work was to establish a semi-permanent e-navigation testbed off the east coast of England to allow GRAD researchers to test prototype VDES equipment at sea, collect scientific data to support coverage and performance modelling and, in the future, demonstrate the provision of GLA e-navigation services using VDES. Given the ongoing pandemic, an additional constraint on the design of the testbed was that GRAD personnel had to be able to execute trials remotely, minimising or eliminating the contact with ship and office-based staff.

# Method

## Testbed Area

Permission has been obtained from the UK spectrum regulator to carry out over-the-air VDES testing within a radius of 45 NM of Harwich, Essex, United Kingdom, as indicated in Figure 1.

Harwich has a rich maritime heritage dating back hundreds of years. Today, there are two major ports in the area. The Harwich International Port lies on the south bank of the River Stour one mile upstream from the town of Harwich. The main users of the port are the regular ferry services to continental Europe, cruise ships, tankers and general cargo vessels. The Port of Felixstowe, to the north-east of Harwich, is Britain’s biggest and busiest container port, dealing with over 40% of UK’s containerised trade. The port serves approximately 3,000 ships each year, including the largest container vessels afloat today. Roll on/Roll off cargo ships sail from the Navyard Wharf on the northern tip of the Harwich Peninsula to various ports in continental Europe and Trinity House (TH) lighthouse tenders berth at the Trinity Pier at the western end of the Harwich Quay. During the summer months a foot ferry operates between Harwich, Shotley and Felixstowe and a wide range of leisure craft can be seen in the area.

For the purpose of this work, an experimental VDES shore station was set up in the GLA Radio Lab in Harwich. A second VDES station was installed on-board Trinity House Vessel (THV) Alert. During the sea trial reported on later on in this paper, THV Alert sailed from Harwich through the Felixstowe approach channel and further north-east, along the coast of Suffolk.

Testbed area.

## Time and Duration

The sea trial reported on here took place on 27th January 2021. The test equipment remains in place and further trials will be conducted in 2021. In the longer term, it is intended to integrate the testbed into a wider e-navigation service demonstrator currently under development by GRAD, based on the Maritime Connectivity Platform [2].

## Testbed System Architecture

A testbed system was developed consisting of a shore-based, shipborne and “work-from-home” station. The shore and shipborne stations can each act as a transmitter and a receiver, and each comprises the following key components (see also Figure 2):

VDES Transceiver – CML Microcircuits VDES Evaluation Platform with the VDES1000 transceiver module;

VHF Antenna – Amphenol-Jaybeam End-fed Dipole with a stated gain of 0 dBd (Ship Station); Skymasts Collinear Antenna with a stated gain of 3 dBd (Shore Station);

GNSS Antenna used for VDES Transceiver synchronisation and positioning;

Laptop / Headless PC to control the VDES equipment and record test data;

ADSL/4G Router to enable remote control over the internet.

The CML VDES Evaluation Platform consists of the VDES1000 transceiver module, power supply and associated cabling fitted in a 1U 19” rack-mountable enclosure. The VDES1000 is a software-defined radio and as such allows new features to be added via firmware updates, as the VDES specification continues to evolve. The version of firmware used in this trial included the full Class A AIS stack and basic VDES Application Specific Message (VDES ASM) and VHF Data Exchange (VDE) functionality. The platform allows testing to be performed at all frequency channels and using all modulation and coding schemes currently being considered for use in terrestrial VDES.

The VDES1000 can be configured and controlled via an Engineering Control Application supplied with the units, which was installed on the Laptop and Headless PC at the Shore and Ship Station. The applications can be accessed either locally, or via the internet, from the Work-from-home (WFH) Station, as indicated in Figure 2. Sea trials can thus be executed by a single GRAD Engineer working either from the lab or from home.

Simplified system architecture.

## Equipment Setup and Co-existence with Other Systems

### Shore Station

The shore-based equipment was set up in the GLA Radio Lab in the TH Buoy Yard Building in Harwich. The VHF and GNSS antennas were mounted on the “rotunda” on the roof of the building, as shown in Figure 3 and Figure 4. The VHF Antenna was mounted at a height of 23 m above mean sea level.

There are several other VHF antennas installed on the rotunda, which are used for testing purposes. None of these were understood to be in use at the time of the trial. At the centre of the rotunda is a mobile communications mast, as can be seen in Figure 3 (right), which is understood to be in active use.

Photos of the VDES and IT equipment used at the Shore Station are included in Figure 5.

VHF Antenna installation on the TH Buoy Yard Building rotunda; 
left: looking west; right: looking south, from below the rotunda.

GNSS Antenna installation on the TH Buoy Yard Building rotunda.

Shore Station equipment setup, with the Work-from-home Station laptop shown in top left.

### Ship Station

The shipborne equipment was installed on-board Trinity House’s rapid intervention vessel Alert. THV Alert is 39 m long and has a maximum speed of 17 knots.

The VHF Antenna was mounted on the vessel’s mast, at the top of the port-side ladder at a height of approximately 14 m above the water line. The GNSS Antenna was fitted to a handrail to the left of the wheelhouse ladder. The mounting locations are shown in Figure 6.

The presence of numerous co-located radio systems on-board maritime vessels creates a particularly challenging radio environment. THV Alert carries six VHF antennas that are used for AIS, radiotelephony and DSC, all installed in the area above the wheelhouse or on the mast. A spectrum survey was carried out to make sure that the power coupled from these systems into the VHF Antenna does not exceed the maximum in-band input power of the VDES1000 unit (for details see Section 3.5.3).

The VDES Transceiver was mounted in a 19” rack cabinet on the bridge deck and the Headless PC with an integrated 4G Router was fitted to the vessel’s planning table, as shown in Figure 16.

Antenna installation on THV Alert.

Ship Station equipment setup; left: VDES Transceiver; 
right: Headless PC with an integrated 4G Router.

## System Calibration

A system calibration was performed in order to obtain an accurate measurement of the transmitter power and to allow the Received Signal Strength Indicator (RSSI) readings produced by the VDES Transceivers to be converted to a received power value (in dBm). A spectrum survey was also performed at each location to prevent damage to the VDES equipment and the insertion loss of the VHF antenna feeders was measured so that the effective radiated power (ERP) could be determined.

The results of the system calibration were recorded in a custom data file format and used during post-processing. Further details are provided below.

### Bench Calibration

The transmitter power calibration was performed using an Antenna Analyser equipped with spectrum analysis, power meter and vector network analysis options. Appropriately power-rated attenuators were used to reduce the signal power to a level safe for the equipment used. The received power corrections were produced using a pair of (calibrated) VDES1000 units connected by a transmission line with a known insertion loss.

The transmitter power corrections were measured for both the low (1 W) and high (12.5 W) power settings of the VDES1000 units, all terrestrial VDES waveforms and two operating frequencies appropriate for each given VDES waveform (both in the lower and upper part of the maritime mobile VHF band). The actual transmitter power levels were found to be lower than the requested values by a factor dependent on the waveform used. The power levels for the AIS waveform (using the constant-envelope GMSK modulation) were within a fraction of a dB of the requested value. The power for the ASM waveforms and VDE-TER Link IDs 11, 12, 14, 15 and 17, 18 (using the Pi/4-QPSK and 8-PSK modulations) were found to be approximately 2 dB below the requested level, and the VDE-TER Link IDs 13, 16 and 19 (using the 16-QAM modulation) were approximately 4 dB below the nominal value. The reduction in the average transmitter power levels observed with the Pi/4-QPSK, 8-PSK and 16-QAM modulations is likely required as a consequence of the increased Peak-to-Average Power Ratio (PAPR) of these waveforms as compared to the AIS waveform.

RSSI-to-received-power conversion factors were generated for all waveforms and two frequencies appropriate for each waveform. The RSSI readings for the VDE Link IDs were found to be within approximately 1 dB of the actual received power but significantly larger deviations were measured for some of the ASM Link IDs and the AIS waveform (the reason for these is being investigated).

### Shore Station Calibration

The VHF antenna feeder loss measurements were performed after installation using the ‘1-port Cable Attenuation’ function of the Antenna Analyser. At the Shore Station, a feeder loss of 5.6 dB was measured in the maritime mobile VHF band.

The strongest (non-VDES) VHF signals observed at the Shore Station were received at a power of -28 dBm and a frequency of approximately 159.8 MHz. These were likely the signals of the PAKNET system transmitted from a nearby TH building. The TH PAKNET station sends burst data transmissions at a rate that varies throughout the day. The second strongest signals detected were those of the AIS, received at up to -35 dBm. These were likely coming from AIS AtoN stations being tested on TH premises as well as vessels operating in the area. The signals observed were of a sufficiently low level not to cause damage to the VDES equipment used but may have the potential to cause receiver desensitisation.

### Ship Station Calibration

Due to a relatively short feeder length and a low-loss cable type used, the antenna feeder loss at the Ship Station was measured to be only 1.0 dB.

The strongest VHF signals observed at the Ship Station were emitted from an AIS AtoN unit installed on the vessel and were received at a power of 10 dBm. The AtoN unit is rarely in use and the duty cycle of the transmissions is relatively low; therefore this is not expected to cause any significant interference to the VDES trials. The second strongest source of VHF energy were the vessel’s VHF voice radios, received at a power of up to -4 dBm. Although relatively high, the signal levels observed were still below the maximum input power limit of the VDES equipment used, leaving a safety margin of approximately 10 dB.

## Licensing

An Innovation and Research Licence was obtained for the purpose of these trials from the UK spectrum regulator (Ofcom), allowing GRAD personnel to operate VDES equipment within 45 NM of Harwich, at all frequency channels currently being considered for use in terrestrial VDES (with the exception of the AIS channels), at a maximum ERP of 14 dBW (25 W). Confirmation has also been obtained from the Maritime and Coastguard Agency that, in the UK, the VDES channels are currently not assigned to any maritime users.

## Test Methodology

### Data Collection

The VDES Engineering Control software supplied with the VDES1000 units includes a ‘Ping Test’ application which can be used to assess VDE performance in a controlled environment, such as a laboratory or field trial. The Ping application uses a PC connected to a Requesting Device (in our case the VDES Transceiver at the Shore Station) to send request messages to a Responding Device (the Ship Station’s transceiver). The Responding Device operates autonomously and returns a response message carrying various diagnostic data, as detailed further below.

The user can specify a number of test parameters, such as the operating frequency, ping rate and the VDE Link ID to be used. The progress of the test can be monitored in real-time using the Ping Test application and data can be logged to a text file for further processing and analysis. The logging data includes:

- Position of the Requesting and Responding Device;

- Request and Response RSSI;

- Request and Response Signal-to-(Interference-and-)Noise Ratio (SNR);

- Number of Requests Sent / Received; and

- Number of Responses Received.

In addition to the data file produced by the Ping Test application, a custom meta-data file format was used to record the system configuration for each test carried out and the environmental conditions (weather and sea state) prevailing during the test.

### Data Processing and Visualisation

Recorded data was processed to extract the following additional characteristics of the VDES data link:

Station Separation (NM) – the distance between the Requesting (Shore) Station and Responding (Ship) Station derived from the GNSS positions of the terminals.

UTC – the UTC time of each Request/Response message derived from the Unix timestamps produced by the VDES units.

Transmitter Power (dBm) – accurate transmitter power values obtained using the bench calibration data.

Request / Response Power (dBm) – the power of the signal at the output of a loss-less receiving antenna, calculated using the Request / Response RSSI, calibration data and stated gain of the VHF antennas used. Comparison was also made with predicted values obtained using the radio wave propagation model described in Recommendation ITU-R P.2001 [3].

Noise and Interference Power at the Ship Station (dBm) – the power of noise and interference observed at the Ship Station, referenced to the output of the receiving antenna. Calculated by subtracting the Request SNR from the Request Power.

Noise and Interference Power at the Shore Station (dBm) – the power of noise and interference observed at the Shore Station, referenced to the output of the receiving antenna. Calculated by subtracting the Response SNR from the Response Power.

Shore-to-Ship Packet Error Rate (%) – the Packet Error Rate (PER) observed at the Ship Station. The Shore-to-Ship PER was calculated by dividing the Number of Requests Received at the Ship Station over the past 60 seconds by the Number of Requests Sent over the same time interval.

Ship-to-Shore Packet Error Rate (%) – the PER for the Ship-to-Shore link. The Ship-to-Shore PER was calculated by dividing the Number of Responses Received at the Shore Station over the past 60 seconds by the Number of Requests Received at the Ship Station (and therefore the number of responses sent) over the same time interval. It should be noted that this is likely a biased estimate of the actual PER in the Ship-to-Shore direction as a Response is sent from the ship only when a Request is successfully received (i.e. when the channel conditions are favourable); therefore, the actual PER in this direction may be expected to be somewhat higher than the estimates shown here.

Roundtrip Packet Error Rate (%) – the combined PER for the (bi-directional) Shore-Ship link. The Roundtrip PER was calculated by dividing the Number of Responses Received at the Shore Station over the past 60 seconds by the Number of Requests Sent.

The following visualisations were produced:

Link Characteristics vs. Time – observation of the VDES link characteristics described above and in the preceding section over time.

Link Characteristics vs. Position – Google Earth plots of selected link characteristics as a function of the Ship Station’s location.

# Results and Discussion

This section presents the results of the first dynamic test carried out using the system described in this paper. The test was carried out on 27th January 2021, while THV Alert was sailing from Harwich, along the coast of Suffolk towards Lowestoft.

## Test Settings

Test transmissions were sent at a rate of approximately one per second using the Ping Test application.

VDE Link ID 11 was used (corresponding to a 25 kHz channel bandwidth and the Pi/4-QPSK modulation).

An operating frequency of 157.2 MHz was chosen, consistent with the VDES channel plan agreed at the World Radiocommunication Conference in 2019.

The transceivers were set to the low-power setting (30 dBm nominal), corresponding to a corrected transmitter power value of 27.9 dBm (0.617 W) for the Requesting Device (Shore Station) and 27.8 dBm (0.603 W) for the Responding Device (Ship Station). Taking into account the feeder losses and antenna gains, the ERP for the Shore and Ship Station was 25.3 dBm (0.339 W) and 26.8 dBm (0.479 W), respectively.

## Environmental Conditions

The weather and sea state conditions prevailing during the test are summarised in Table 1.

Environmental Conditions

| Temperature (⁰C): | 8 |
| --- | --- |
| Humidity (%): | 93 |
| Pressure (hPa): | 1011 |
| Clouds (%): | 54, later 43 |
| Rain (mm): | 0 |
| Wind speed (kt): | 7, later 9 |
| Wind direction: | W, later SW |
| Significant Wave Height (m): | 0.2, later 0.4 |
| Wave period (s): | 3, later 4 |
| Wave direction: | S, later SW |

## Maximum Communication Range

The data link characteristics are plotted here as a function of time rather than distance between the stations as the propagation channel is not necessarily line-of-sight and using distance as the independent variable may be misleading. Figure 8 relates the UTC time of day to the distance between the stations.

As can be seen from the figure, the maximum communication range was approximately 16.5 NM (30.5 km). It should be noted that this range was achieved with relatively low antenna heights and ERP of only around 0.4 W. Future trials will use high-power transmissions and may be expected to achieve a significantly longer communication range.

Shore Station – Ship Station separation vs. time.

## Received Power

Figure 9 and Figure 10 plot the signal power received at the Ship and Shore Station over time (note that the values measured at the Ship Station have a reduced resolution as the data is sent back to the Shore Station via VDES and must fit within the Response message). A Google Earth plot of the Request Power is also provided in Figure 11.

As may be expected, the power-vs-time characteristics measured at the two stations have a very similar shape, with the values measured at the Ship Station being slightly lower due to the higher feeder loss at the Shore Station (recall that the received power is referenced here to the output of the receiving antenna, excluding the insertion loss of the feeder at the receiving end). The measurements show a number of distinct nulls and peaks in the received power, hinting at the presence of multipath propagation. Some of these fades appear to be more than 10 dB in depth.

The figures also show predicted power values generated using two different radio wave propagation models – the terrain-specific model described in Recommendation ITU-R.P 2001 [3], and a simple two-ray sea surface reflection model assuming an all-sea-water propagation path with no obstructions.

The ITU-R model follows the general trend in the data quite well but overestimates the measurements by approximately 5 dB on average. This may be due to the VHF antennas having a lower gain than stated by the manufacturers, clutter surrounding the antennas mainly at the Ship Station and any inaccuracies in the terrain elevation database used. Further, the ITU model predicts the received power exceeded for a given percentage of time in an average year (the median level is shown here). It may be necessary for a larger dataset to be collected before the trends predicted by the ITU model fully emerge.

Interestingly, the two-ray model provides a better fit to the data than the ITU model at short to medium range but, not surprisingly, it overestimates the received power at long range (see approximately from 16:05 UTC onwards) where the vessel sailed in the shadow of a land mass (note that the ITU model correctly predicts the drop in signal level in this region).

The main reason for using the two-ray model was to see whether the fading (null-peak) pattern observed in the measured data could be replicated analytically. However, the phase difference for a VHF signal between the line-of-sight ray and the ray reflected off the sea surface was found to be too small to produce any nulls within the range of distances considered here. This suggests that the multipath fading observed in this test was, at least partly, due to reflectors further afield, such as objects on land or the landmass itself or, perhaps, irregularities in the troposphere. Indeed, the fading pattern could be replicated with some success at short range by increasing the distance of the reflecting surface in the two-ray model (however, the modified model did not provide a good fit at long range and therefore is not included here). Further investigation is required to better understand the mechanisms that give rise to multipath fading in maritime environments.

Request Power over time (as measured at the Ship Station).

Response Power over time (as measured at the Shore Station).

Request Power as measured at the Ship Station.

## SNR and Noise (plus Interference) Power

Figure 12 and Figure 13 show the SNR and Noise and Interference Power, respectively, as measured at the Ship Station. The increased noise floor and interference spikes observed at the start of the voyage may have been due to the Vessel’s own VHF voice transmissions or VHF transmissions from other stations in the harbour area. The lower-level interference spikes that persisted for the whole duration of the test were likely due to the Vessel’s own AIS transmissions. The cause of the increase in noise floor observed between approximately 15:22 and 15:42 UTC is unknown but was likely due to an on-board electronic or electromechanical system being in use at that time.

Figure 14 and Figure 15 show the SNR and Noise and Interference Power, respectively, as measured at the Shore Station. The interference spikes visible in both figures were likely due to the PAKNET and AIS AtoN stations operating at TH premises.

The median Noise and Interference Power measured at the Ship Station was -105 dBm. Assuming that the VDES Transceiver was using a measurement bandwidth of 25 kHz (consistent with the channel bandwidth for Link ID 11), and making the simplifying assumption that the noise and interference power was uniformly distributed across the receiver’s passband, this corresponds to a median noise (and interference) power spectral density of -149 dBm/Hz, that is 25 dB above the thermal noise floor (kT).

The median Noise and Interference Power measured at the Shore Station was -114 dBm, which is 9 dB lower than the noise power at the Ship Station. Under the same assumptions as above, this corresponds to a median noise power spectral density of -158 dBm/Hz, or 16 dB above kT.

Note that the noise levels quoted here include the effects of the receiver’s noise figure and the feeder loss, although these are expected to be minor compared to the effects of the external radio noise in the VHF band. The noise levels observed at both the Ship and Shore Station are consistent with findings of earlier noise-floor surveys carried out using a high-sensitivity spectrum analyser setup. It is also noted that THV Alert is a relatively small vessel and as such is likely to experience higher levels or radio noise compared to larger vessels as the sources of noise and the victim systems are packed in a smaller volume.

Request SNR over time (as measured at the Ship Station).

Noise and Interference Power over time as measured at the Ship Station.

Response SNR over time (as measured at the Shore Station).

Noise and Interference Power over time as measured at the Shore Station.

The SNR plots also show the Forward Error Correction (FEC) Decoder Threshold for the VDE Link ID 11, determined from the theoretical energy-per-bit-to-noise-density-ratio curve for the Pi/4-QPSK modulation in Additive White Gaussian Noise (AWGN) and a coding gain value obtained from simulation results provided by Bronk et al. [4], [5]. A target Packet Error Rate (PER) figure of 1% was chosen and an SNR measurement bandwidth of 25 kHz was assumed. It is noted that IALA Guideline 1139 provides carrier-power-to-noise-density ratio (C/N0) thresholds for the FEC protected Link IDs; however, these were generally found to be several dB lower than equivalent thresholds determined by the lead author using the coding gain values sourced from Bronk et al. [4]. These discrepancies may merit further investigation by IALA ENAV WG3.

The decoder threshold is useful information when assessing the PER performance and geographical coverage of the VDES data link.

## Packet Error Rate

Figure 16, Figure 17 and Figure 18 plot the Shore-to-ship, Ship-to-shore and Roundtrip PER over time, respectively. By comparing Figure 16 with Figure 12, it can be seen that there is a strong correlation between the times when the SNR drops below the Detector Threshold and instances of increased PER. Isolated errors occurred during the first part of the voyage, most likely as a result of interference and signal fading. From around 16:05 UTC onwards, when the Vessel entered into the radio shadow of the coast of Suffolk, the Detection Threshold was being consistently breached and the PER rose steadily from that point on until the end of the test. Reliable communication was thus possible up to a range of approximately 10 NM.

The Ship-to-shore PER plotted in Figure 17 is lower than the PER measured in the opposite direction; this is likely due to a combination of a lower noise floor at the Shore Station compared to the Ship Station (see Section 4.5) and the suspected bias of the estimation method used (as explained in Section 3.7.2).

The Roundtrip PER plotted in Figure 18 is then dominated by the Shore-to-ship PER, as could be expected.

Shore-to-ship PER over time.

Ship-to-shore PER over time.

Roundtrip PER over time.

## Coverage Prediction

The data produced by the testbed will provide valuable input into the development of VDES coverage and performance tools.

Figure 19 compares the predicted coverage area of the Harwich Shore Station (operating at the low-power setting) with the PER measured during the sea trial. The contour of the coverage area is defined as a set of points where the predicted SNR (or rather C/N0) drops below the Decoder Threshold. In theory, the PER at the contour should be 1%. Recall, however, that the threshold was calculated assuming a non-fading propagation channel with AWGN and that the propagation model appears to overestimate the signal power measured during this particular trial. Consequently, the PER measured at the predicted contour of the coverage area is higher than expected, as can be seen from Figure 19. As further trials are conducted and additional data become available, the model will continue to be developed to ensure it provides accurate predictions of real-world coverage and performance.

As mentioned previously, the current trial was performed using reduced transmitter power; however, future trials will be conducted at high power. For illustration, Figure 20 shows the predicted coverage area using the identical equipment and vessel as in the current trial but with the VDES Transceiver set to the high-power setting (with Link ID 11, this corresponds to an average transmitter power of approximately 7.8 W). The coverage area for this configuration is predicted to extend up to 22 NM from Harwich.

Predicted coverage area (shown in red) and measured PER (shore to ship).

Predicted coverage area for the high-power transmitter setting.

# Conclusions and Recommendations

A COVID-19 secure VDES testbed has been developed allowing over-the-air testing of VDES equipment to be performed in the area around Harwich, United Kingdom. VDES communications can be carried out in both the ship-to-shore and shore-to-ship direction, on any of the frequency channels and using any modulation and coding schemes currently considered for use in terrestrial VDES ASM and VDE. System deployment is consistent with real-world operating conditions.

The testbed allows a range of propagation channel and data link parameters to be characterised, including the received signal power, signal-to-noise-ratio and packet error rates. Results can be plotted over time or displayed in Google Earth and provide valuable input into the development of VDES coverage and prediction tools, as well as the development of the VDES itself.

The first sea trial was carried out in January 2021, using a low-power transceiver configuration (ERP of approximately 0.4 W).

The maximum communication range was 16.5 NM, with reliable, bi-directional communication possible up to a range of approximately 10 NM. Modelling suggests that, in the high-power configuration, the testbed will provide VDES coverage up to approximately 22 NM from Harwich.

Received power was slightly lower than theoretical predictions, possibly due to the VHF antenna gain being lower than stated by the manufacturers and clutter surrounding the antenna on the vessel. Further data is required before a definitive conclusion can be made.

The received power data shows evidence of multipath propagation, with numerous distinct “nulls” observed, some more than 10 dB in depth. Analysis using a two-ray propagation model suggests that the multipath signal components responsible for the fading observed originated either from land or irregularities in the troposphere rather than from reflections off the sea surface.

SNR data shows evidence of short-duration burst interference being present at both the Shore and Ship station (likely originating from the AIS and PAKNET). This does not necessarily result in packet loss but may have the potential to desensitise the VDES receiver.

The noise floor on the Vessel was found to be around 9 dB higher than at the Shore Station, consistent with the findings of previous noise-floor surveys carried out by GRAD. This makes shore-to-ship communication somewhat more challenging than ship-to-shore.

The C/N0 thresholds for the FEC protected terrestrial VDES Link IDs provided in IALA Guideline 1139 were found to be lower than expected based on simulation results published by Bronk et al. [4], [5].

It is recommended that further sea trials be conducted in order to gain a better understanding of the maritime propagation environment, in particular with respect to multipath propagation and its impact on the VDES performance. Such information would be valuable not only for the modelling of the VDES communication performance but also in determining the achievable accuracy of VDES ranging (R-Mode).

It is further recommended that noise-floor and interference surveys be carried out on maritime vessels of different sizes and classes to gain a better understanding of the VHF radio environment on-board vessels.

Finally, IALA ENAV WG3 may wish to revisit the assumptions used in determining the C/N0 decoder thresholds in light of new data and, if necessary, amend G1139 accordingly.

# Acknowledgements

GRAD would like to acknowledge the Second Officer and Chief Engineer of THV Alert and staff of the Trinity House Marine Operations department and Operations and Planning Centre for their understanding and support in conducting this trial. GRAD would also like to acknowledge the technical support given by CML Microcircuits Ltd and Stone Three Venture Technology.

# References

[1]	ITU, ‘VHF Data Exchange System Channel Sounding Campaign’, ITU, Geneva, Switzerland, Report ITU-R M.2317-0, Nov. 2014. [Online]. Available: https://www.itu.int/pub/R-REP-M.2317-2014.

[2]	‘Maritime Connectivity Platform (MCP)’, Feb. 2018. http://maritimeconnectivity.net/.

[3]	ITU, ‘A General Purpose Wide-range Terrestrial Propagation Model in the Frequency Range 30 MHz to 50 GHz’, Recommendation ITU-R P.2001-3, Aug. 2019. [Online]. Available: https://www.itu.int/dms_pubrec/itu-r/rec/p/R-REC-P.2001-3-201908-I!!PDF-E.pdf.

[4]	K. Bronk, M. Mazurowski, D. Rutkowski, and B. Wereszko, ‘Badania Symulacyjne Warstwy Fizycznej Naziemnego Segmentu Systemu VDES’, Przegląd Telekomunikacyjny i Wiadomości Telekomunikacyjne, vol. 2016, no. 6, pp. 467–470, 2016.

[5]	K. Bronk, P. Koncicki, A. Lipka, D. Rutkowski, and B. Wereszko, ‘Simulation and Measurement Studies of The Vdes System’s Terrestrial Component’, Polish Maritime Research, vol. 26, no. 1, pp. 95–106, Mar. 2019, doi: 10.2478/pomr-2019-0011.

# Action requested of the Committee

The Committee is requested to:

- Note the contents of this paper.

Provide feedback to the lead author (jan.safar@gla-rad.org).

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain

In VDES, the term ‘Link ID’ refers to a particular combination of signal bandwidth, modulation and forward error correction code rate.