Input paper:   ENAV27- 5.1.6.0

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					R  Input

R  ENAV	□  VTS							£  Information

Agenda item 			n.n

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)  China Maritime Safety Administration,Dalian Maritime University

Research of AIS VDL Integrity Monitoring

# Abstract

With the popularization of AIS applications, the types of AIS messages,services, and equipment are increasing. The number of users also increased quite fast. However, the problem of AIS VDL traffic load becomes increasingly serious,and the vulnerability of AIS itself has gradually emerged. Therefore, in the context of the AIS channel protection issue, real-time monitoring of AIS VDL is considered in order to react timely to AIS VDL signal abnormalities, so as to realize the reliable application of AIS channel environment in high-density areas of coastal ships, thereby ensuring the safety of ship navigation and protecting the maritime radio environment.

## Purpose of the document

The purpose of this document is to discuss the AIS VDL monitoring service and its necessity, and to draw the attention of the ENAV committee to AIS VDL channel protection measures.

## Reference documents

- IMO MSC 74(69) Annex 3,《Recommendation on Performance Standards for an Universal Shipborne Automatic Identification System (AIS) 》

- ITU-R M.1371-5,《Technical characteristics of automatic identification system using time division multiple access in the VHF maritime mobile frequency band》

- IMO Resolution MSC.140 (76),《Recommendation for the protection of the AIS VHF data link》

- IALA Guideline 1050, 《The Management and Monitoring of AIS Information》IALA,《Maritime Radio Communications Plan (MRCP)》

# Background

As one of the main methods of ensuring maritime traffic safety, AIS has increasingly highlighted its importance. In the context of e-Navigation strategy and the implementation of maritime autonomous surface ships (MASS), the reliability, stability and authenticity of AIS data, as well as the smoothness of AIS channels, especially in coastal waters, are important basic supports for future maritime navigation safety.

The international AIS technical standards defines technical specifications and related application operation guidelines for AIS air interfaces, physical layer, link layer, network layer, transport layer. In the MSC.140 (76) resolution, the IMO also proposed that "Administrations should take steps necessary to ensure the integrity of the radio channels used for AIS in their waters". Although AIS channel protection has become one of focusing discussion in the industry, there is still no guideline or recommendation on AIS integrity monitoring services at the international level as the basis for the implementation of AIS channel integrity monitoring by the competent authority.

With the gradual maturity and further application of VDES, as well as the future research and implementation of maritime full-band digital VHF communication systems, the integrity monitoring of maritime VDL channels is facing this important issue.

Under these backgrounds, in order to better guide members to protect AIS VDL channels with policies or technical means, it is necessary to do some research on the AIS VDL channel integrity monitoring at service level to provide implementation basis for maritime VDL channel protection.

# Discussion about AIS VDL integrity monitoring

As a continuous broadcast communication system, AIS has open and transparent air interfaces; in the coastal area, the channel management function of the AIS base station provides convenience for the rational use of regional AIS channels, but also increases the vulnerability of the system. Therefore, the necessity of AIS VDL channel integrity monitoring is discussed from the following aspects.

## Vulnerability of AIS VDL management messages

At present, AIS VDL management related messages include Msg16, Msg20, Msg22 and Msg23, especially Msg22 and Msg23, which can configure the regional operating channels, operating mode, transmission power and other parameters by AIS stations. Since these AIS messages are all subject to standard formats, under the operating mechanism of the AIS technical system, any AIS station has the possibility to broadcast management messages on the AIS channel without authorization, deceiving the standard AIS stations into switching the operating channel, leads to the risk of regional AIS applications.

Actual case:

In 2017, the VTS Center of Dalian found that a large number of ships AIS data disappeared. Through investigation and analysis, it was found that there was an unauthorized AIS base station broadcasting the AIS msg22, and the channel parameters were set as follows:

- Northeast: （126°50′，39°1.6′）

- Southwest: （8°34.4′，38°8.4′）

- Channel A: 2016

- Channel B: 2948

- Power: low

Effect:

As the existence of the illegal AIS base station, all ships passing through the control area of the illegal base station would switch channels. Because the AIS base station in Dalian Port waters would not change its operating channels according to Msg22 broadcast by the illegal base station and still operate in the default channels, the VTS center cannot monitor ships tracks, and some ship stations cannot be identified between each other.

播发基站

Conclusion:

1) The AIS management message can lead to the unavailability of the remote AIS shore-based system, which in turn affects the identification of ships entering and leaving the port by VTS, and affects the navigation safety.

2) Any AIS station can be modified to an illegal AIS channel management station, causing difficulty in supervision.

## DOS attack on AIS channel

According to the standard AIS messages, some modified AIS stations can simulate a large amount of static and dynamic AIS messages, occupying most of the AIS channel slots, making normal AIS equipment unable to allocate slots, and then lead to denial-of-service attack (DOS attack) on the AIS channel.

Actual case:

In 2016, it was found that someone used a modified AIS transmitting device to simulate or replay a large number of AIS messages, occupying more than 60% of the regional AIS slots in west Bohai area.

Effect:

1) Although shore-based management authorities can still find a large number of ships moving in the area, these ships may not actually exist;

2) Strong concealment, without sensitive AIS channel monitoring facilities, it is difficult to find it out in a short time;

## Illegal AIS Stations

Some illegal ships (smuggling, etc.) can evade supervision by applying legal ship AIS information. It brings difficulties to maritime law enforcement.

Actual case:

During boarding inspection, it was found that different ships used same MMSI several times.

Conclusion:

1) If cross-regional monitoring is not carried out, it is difficult to find fake-plate ships.

# AIS VDL monitoring service function requirements

Based on the above discussion of AIS VDL channel integrity monitoring, the relevant authorities should monitor and manage the integrity of AIS VDL channels in real time.Therefore, the channel integrity monitoring service need to be provided, which should meet but not limited to the following requirements:

(1) Ability to monitor illegal AIS Msg22 and Msg23 in the whole maritime area;

(2) Real-time monitoring capability of AIS signals in key coastal traffic areas;

(3) Real-time monitoring and analysis capabilities of AIS information for fake-plate ships;

(4) AIS station authenticity monitoring capability, etc.

# ACTION REQUESTED

(1) Taking the content of this input file as a reference, considering the necessity of formulating relevant guidelines or proposals for AIS VDL channel integrity monitoring services, further investigations on AIS VDL channel integrity monitoring requirements will be carried out to support AIS VDL channel protection.

(2) Representatives who are interested, are invited to notice the information file in Appendix 1 of this proposal, and looking forward to conduct relevant discussions.

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain