Appendix 1

Introduction of China AIS VDL Integrity Monitoring Platform

# Abstract

Consider increasingly serious vulnerability in the AIS VDL and response to the IMO resolution in MSC.140 (76):“Administrations should take steps necessary to ensure the integrity of the radio channels used for AIS in their waters.” It is required to realize real-time monitoring, analysis, and alerting of VDL management messages, malicious false stations, and autonomous maritime radio equipment (AMRD), etc. China Maritime Safety Administration carried out the research and development of the VDL integrity monitoring project based on the AIS big data platform to improve the management and control capabilities of AIS VDL, enhance the operation efficiency of AIS, and cope with systemic risks faced by AIS.

## Purpose of the document

The purpose of this input document is to provide relevant information about China's AIS VDL integrity monitoring system and its test results, and to draw the ENAV Committee's attention to the current situation of AIS VDL and its protective measures.

## Reference documents

- IMO MSC 74(69) Annex 3,《Recommendation on Performance Standards for an Universal Shipborne Automatic Identification System (AIS) 》

- ITU-R M.1371-5,《Technical characteristics of automatic identification system using time division multiple access in the VHF maritime mobile frequency band》

- IMO Resolution MSC.140 (76),《Recommendation for the protection of the AIS VHF data link》

- IALA Guideline 1050,《The Management and Monitoring of AIS Information》

- IALA,《Maritime Radio Communications Plan (MRCP)》

# BACKGROUND

In order to ease serious vulnerability in the AIS VDL, it is necessary to realize real-time monitoring, analysis and alerting of VDL management messages, malicious false stations, autonomous maritime radio equipment (AMRD), etc. China Maritime Safety Administration carried out the research and development of the VDL integrity monitoring project based on the AIS big data platform to improve the management and control capabilities of AIS VDL, enhance the operation efficiency of AIS, and cope with systemic risks faced by AIS.

In recent years, with the popularization and application of AIS, AIS stations have been used in various offshore facilities, such as offshore construction, drilling platforms, offshore bridge collision avoidance, wind farms, aquaculture and fishing nets. Following this, AIS channel interference and deception information gradually emerged, affecting the safety of maritime navigation.

In response to the requirements of the IMO MSC.140 (76) resolution, to protect the integrity of the AIS channel in China's waters and ensure the safety of navigation, from 2017 to 2020, China Maritime Safety Administration led the research and development of China AIS VDL integrity monitoring platform, achieved a series of effects.

# Technical architecture of China's AIS VDL integrity monitoring system

The technical architecture of China's AIS VDL integrity monitoring platform is based on a big data platform and designed with a service-oriented concept, covering technical services, data services and application services. The data exchange between services adopts the data exchange framework specified by S-100 standard. The specific technical architecture is shown in Figure 1.

- Figure 1 Technical structure of VDL integrity monitoring platform

## AIS base station technical services

AIS base station technical services are based on the current AIS shore station system, designed accordance with the technical framework of the IALA A-124, and provide basic raw AIS data for the AIS VDL integrity monitoring service platform.

## AIS raw data pre-processing and analysis service

The AIS raw data pre-processing and analysis service performs verification and pre-processing on the data sent from the AIS base station technical services, and analysis the AIS messages based on the AIS big data platform, and the results are submitted to the AIS VDL integrity monitoring service in real time.

## General big data platform

The general big data platform is used for AIS raw data processing and analysis as well as the VDL integrity data processing. The platform can support big data analysis in post-processing mode or real-time processing model, according to data service requirements.

## AIS VDL integrity monitoring service

AIS VDL integrity monitoring service includes AIS VDL management messages (Msg16 Msg20 Msg22 and Msg23) monitoring and processing service, AIS radio real-time monitoring and analysis service,fake AIS ship analysis services, and duplicated AIS ship analysis services, etc.

## User Application Service

The user application service provides authenticated users with access to the AIS VDL integrity monitoring platform.

# Development results

The AIS VDL integrity monitoring platform is jointly designed and developed by China Maritime Safety Administration and relevant partners, and deployed in the private cloud.

## Management message monitoring and processing

The base station management message detection service mainly monitor Msg20, Msg22, and Msg23sent by the base station. VDO messages are analyzed to identify whether the base station is built by maritime authority; Basic information such as the base station’s IP address and port number are imported to the database in advance; Msg20, Msg22, and Msg23 sent by the base station are analyzed, displayed and stored; Msg22 which has a major safety impact on the navigation is displayed and approved by the management personnel, and the Msg22 that has not passed the approval is neutralized by recovery message manually (according to the IP address of the base station).

## Fake station identification

The fake station identification analyzes the ship's position and the base station's location to verify the ship's actual location; the service mainly based on two types of approaches, the first is to calculate the position information of the ship based on the power data sent and received in the message, and determine the degree of deviation between its calculated position and the position reported from the ship, so as to determine the match between the reported position data and the actual position data; the second is to confirm the authenticity of the location information according to the match between the ship's location information and the base station position source.

（a）

（b）

- Figure 2 Station information real-time display

## AMRD device identification

This project mainly uses equipment monitoring and identification services to identify the AMRD fishing nets. According to AIS static and dynamic messages and other AIS data, real-time AMRD device identification monitoring is performed on the received AIS data, and the results are output to the user application service.

- Figure 3 Fishing net recognition and monitoring result