#### Specifications for E-Navigation Fundamental Elements

Hydrographic Data (ENCs)

The following requirement for hydrographic information will be necessary at the time that e-Navigation is implemented:

- A full coverage of official ENCs for navigational waters made readily available to all e-Navigation users.

The following is a list of issues pertaining to the provision of hydrographic data:

- A full coverage of ENCs for navigational waters will require considerable effort from the world’s hydrographic community, and additional attention and effort will be needed to facilitate this).

- The current nautical chart (paper, raster and ENC) distribution system may need to be reviewed to ensure easy and ready global access to ENCs.

- An IMO mandatory ECDIS carriage requirement as soon as practicable is expected to significantly contribute to improving the rate of ENC production.

- The current excessive cost of ENCs must be addressed and so far as possible a cost neutral migration from paper charts should be achieved.

It is recommended that a liaison note be provided to the IHO by the ANM committee asking the IHO to clarify the situation regarding the adoption of the WGS 84 datum as the standard geodetic reference frame for use in e-Navigation.

Position Fixing

The following is a list of key elements required for e-Navigation position fixing:

- appropriate accuracy (already included in IMO Res 915).

- continuity

- reliability

- adequate redundancy

- integrity

- compatibility between systems

- appropriate datums (vertical and horizontal)

All current GNSS systems have a common weakness in that they can all be accidentally or intentionally interfered with.  Hence alternative and independent position fixing capabilities need to be considered.

Consideration should be given to independent non-GNSS Electronic Position Fixing System and sensors as an optional part of e-Navigation.

e-Navigation systems should enable the electronic capture of radar ranges, radar bearings and visual bearings etc. and the use of this information for position fixing.

Mariner’s training must ensure that mariners maintain reversionary skills and are always capable of using alternative position fixing methods.

The time signal acquired from GNSS is used in other systems (e.g. communications and AIS) and this needs to be considered as e-Navigation is developed from the point of view of considering a backup time signal.  The use of alternative time signals, perhaps from Loran or by enhancing the IALA DGPS network could mitigate this risk.

Communications

There are two general areas under this heading for consideration; technical and content.  The following is a list of key communication elements required for e-Navigation:

- autonomous acquisition and mode switching (ie. minimal mariner involvement needed)

- common messaging formats

- sufficiently robust (eg. signal strength, frequency aspects)

- adequate security (eg. encryption etc)

- sufficient bandwidth (data capacity)

- growth potential

- automated report generation

- global coverage (could be achieved with use of different modes)

- the use of a single language, or alternatively with other languages permitted as options.

WG1 suggests that the following issues are among those that will require resolution to achieve the above:

- It seems likely that a satellite broadband link will be required to achieve the above requirements, and consideration must be given to how this will be achieved.

- The question of cost and who pays for the provision of a satellite broadband link must be resolved early in development of e-Navigation.