# Draft Vi.2

# IALA AIS GUIDELINES

# (VOL. II)

BASIC AIS SERVICES (BAS)

# Basic AIS Services

## Introduction

The purpose of this document is to provide first level understanding of an AIS Service and the Basic AIS Services (BAS). It will present an organization diagram illustrating loose relationships, then consolidate this same information into a relational representation in the form of a cube. Then we will relate this with a method for communicating the Functional Requirements using the Unified Modeling Language (UML).

## Lets start with the organizational representation

We have a Competent Authority (CCG). It has policies, regulations, resources and provides service (public service in our case). To provide this service it needs other resources that work within its management framework (policy, regulations etc.). Our example has Office of Boating Safety (OBS) Service, MCTS Service, AtoN Service – it could also have included all services NWPA, Fleet, SAR etc. Each of these services use other services to deliver their service i.e. MCTS needs Communications Service, Radar Service to get or distribute their information. Each of these services are comprised of basic services. Radar provides the basic service of Radar position, Radar range, point of coincidence. AIS is also a service – it provides Basic AIS Services in the same manner as other services i.e. AIS position, AIS range, AIS speed over ground etc. This illustrates the relationship that exists between all of the services from Basic to General.

Each service on every level can be described – CCG Service, MCTS Service, etc. There are many methods to describe them – verbal, pictorial, text, combinations of multi-media. There is also the possibility to describe or represent the service using a modeling language called UML. This is an established and mature language that can provide the building blocks of description in a language (the L in UML) that is universal (the U in UML is actual Unified but is derived from a series of previous modeling tools hence Unified).

Each service can be represented by a UML “Component”. Each service or Component will have a relationship to the other Components – the AIS Service “belongs to” the MCTS Service and a BAS “belongs to” the AIS Service. Each Component then has only to list the interface criteria for each relationship, one from the point of view of an input the other as an output. These criteria need to match.

It therefore follows that if we build a system of components with their interface we describe a total Service eventually.

CCG Service

Office of Boating Safety

(OBS) Service

MCTS Service

AtoN Service

Ais Service

DR Service

Radar Service

Radar Services

BAS Services

B

AtoN

S

BAS

BAS

BRS

Figure 1

## Relational representation

Figure 2 has taken the organization view of Figure 1 and transformed it into a more “logical” presentation with respect to the relations. The lines in Figure 1 that usually mean “flows to” are replaced by blocks that denote “belongs to”. The third dimension indicates that the system of relationships has to be duplicated for each instance of the service. It should also be noted that there are often multiple interfaces to a Component (manager of it, other services, conditional influence).

CCG Service

Office of Boating Safety

(OBS) Service

MCTS Service

AtoN Service

Ais

Service

Radar

Service

Basic Radar Services

BRS

Ais Service

DR

Service

Basic DR Services

Basic OBS

Services

Basic AIS Services

Basic AtoN Services

Ais Service

BAS

BOBSS

BAS

BRS

BDRS

BAtonS

Site # 1

Figure 2

## So why consider UML?

By taking the time to work in a modeling language we eliminate much ambiguity. The rules are fairly rigid for UML and they are logical. The same can not be said for any oral/written language – even this text that I am writing is being understood differently by each reader. The UML is less ambiguous. Figure 3 shows the symbol for a UML Component with an interface. There are other relational elements available to represent relationships to other Components and if a Component “belongs to” another it can be nested. The purpose of this paper is not to give a lesson on UML, it only attempts to illustrate that it is a language that can serve our purpose. The main reason to use UML lies in the fact that if we describe a service using UML and another Competent Authority provides the same service, it can use the same model (the M in UML) to describe its service and not have to develop a new description. On an international level this could be used to standardize our services.

Figure 3   UML Component with Interface

## Subdivision of the Basic AIS Services

The BAS are subdivided into "external" and "internal". The "external" BAS are those BAS which deliver net information from and to ships and shipboard applications, the direction of the data flow depending on the nature of the BAS.

The "internal" BAS are those which are needed in particular to perform the "system management" functionality as defined by Recommendation ITU-R M.1371-1, Annex 2, Table 13. They do not deliver net information about ships and shipboard applications, but some "internal" BAS may be needed to provide "external" BAS at all under certain circumstances. The "internal" BAS are contained and managed within the AIS Service and may be triggered and / or configured by higher levels of shore-based infrastructure.

## Motivation for the description of the Basic AIS Services

The description of the Basic AIS Services can serve the following purposes.

### The Basic AIS Services as the interface between operational and technical considerations

"External" BAS produce, transfer, and accept information to and from higher levels that are not themselves part of the AIS Service.  The functionality of higher level services and AIS applications are built upon the capabilities of these services. This is illustrated by

The "external" BAS are the services the AIS Service delivers to the recipient's side ashore at the interface between the AIS Service and "higher", application-oriented levels. Therefore, their description also takes into account the processing at the fixed AIS stations, the Physical AIS Shore Station (PSS), the Logical AIS Shore Stations (LSS), and the AIS Service Management (ASM).

Therefore, the BAS are also the basis for an assessment of the usefulness of the AIS Service for a particular intended application in terms of accuracy, frequency, reliability etc.

### The Basic AIS Services as a means to convey consistent relevant information from the recipient's point of view

When making use of the potential of the AIS by setting up an AIS shore infrastructure, which is integrated into e. g. a VTS environment, the VTS (re-)designer, who formerly had not been involved in shipborne equipment issues at all, will now need to receive and use information on the AIS shipborne installation.

This is due to the fact, that the introduction of the AIS as an automated, co-operative system triggers the integration of shipborne and VTS technologies, which formerly existed more or less independently of each other. The same applies – in principle, but more on an operational level with less technical sophistication – to the VTS operator(s).

The situation is further complicated by the fact, that relevant information is distributed in a variety of relevant documents, the number of which may still be growing and which are maintained by various international or regional organisations. It becomes obvious that it may be difficult to identify all relevant documents for a particular issue at hand, keep track of them or even control them to reflect the intent and current status of the AIS.

The BAS descriptions aim at making available relevant information for a given issue at hand for a system (re-)designer using the AIS Service.

This description of the BAS does not make redundant the referenced documents, i.e. the appropriate international standards, nor do they introduce new system features. However, each description of a BAS integrates, in a comprehensive and highly accurate manner, all information items from various sources into functional terms that are essential to understand what this BAS is delivering.

### The Basic AIS Services as the functional, technical interface between the AIS Service and higher technical levels

Besides being the means to assess the operational usefulness and features of the AIS Service to higher levels and applications, the BAS – in their precise technical description – constitute the functional, technical interface between the AIS Service and higher technical levels. This is indicated in Figure 25.1 in regard to the only physical accesssible interface between the higher levels, applications, and the AIS data transfer network. For the same reason, the BAS constitute the major use cases of the Logical AIS Shore Station layer.

### The Basic AIS Services as a means to encapsulate complexity

The BAS descriptions are also the basis for an assessment of the characteristics of a particular AIS service for a particular intended application in terms of accuracy, frequency, reliability, fallback arrangements etc. An individual BAS, considered as one consistent and prudently partitioned set of functionality of the whole AIS Service may be investigated on its own, thus allowing a concentrated assessment of this particular functionality.

This holds true although there are interactions between certain BAS. Since these interactions will be clearly identified, they may be considered one by one, instead of being overwhelmed by the sheer complexity of the AIS Service as a whole.

### The Basic AIS Services as a means to understand and handle complex configuration states of the AIS Service as a whole

The BAS may also serve as a convenient means to understand and handle complex configuration states of the AIS Service as a whole because of the above features of the BAS, in particular because they encapsulate complexity.

## Concluding remarks

As such, Volume II will be of benefit to manufacturers, system integrators, and users alike.  It provides a basic understanding of what services AIS can deliver.  What services it cannot deliver, and what are the proper conditions under which the service can be used.  It will describe the basic AIS services that support the design of AIS based applications that directly serve the needs of the mariner, vessel traffic service, and littoral state.

## The functional interface between the shore-based applications and the AIS Service

The requirements for the interface between shore-based applications and the AIS Service can be defined in functional terms as follows:

A) This functional interface should provide the AIS Service to shore-based applications as a set of Basic AIS Services (BAS).

B) The functional interface should facilitate the integration of the AIS Service shore-based applications, such as radar-based VTS environments.

C) The functional interface encapsulates the technical details of both the AIS technology and of the layout and local configuration of shore-based AIS. (This is a state-of-the-art engineering principle. It protects application software from unnecessary changes due to configuration or technology improvements within the AIS Service.)

The interface between the AIS Service and its clients e.g. the VTS environment would also comprise a list of available Basic AIS Services (selection or all) and would consist of a definition of all Logical AIS Shore Stations (LSS) set up in this particular VTS environment.

#### External BAS

A_STAT		Static ship data from Class A shipborne mobile AIS stations

A_DYN		Dynamic ship data from Class A shipborne mobile AIS stations

A_VOY		Voyage related ship data from Class A shipborne mobile AIS stations

SAFE_AD		Safety related addressed message

SAFE_BR		Safety related broadcast message

INT_TDMA		interrogation via AIS VDL

INT_DSC 		interrogation via DSC Ch 70

B_DAT		Ship data from Class B shipborne mobile AIS stations

SAR_DAT		Data from SAR airborne AIS stations

ATON_DAT		Data from AtoN AIS stations

TRANS_IAI		Transparent transmission within the International Application Identifier branch

TRANS_RAI		Transparent transmission within the Regional Application Identifier branch

#### Internal BAS

BASE_DAT		Data on base station (base station's own data)

ASGN_RATE	Assignment of reporting rate to mobile station(s)

ASGN_SLOT		Assignment of transmission slots to mobile station(s)

DGNS_COR		DGNSS corrections to mobiles

CH_TDMA		Channel management by TDMA

CH_DSC		Channel management by DSC

PWR_LEV		Power level setting to mobiles

FATDMA		(announce) configuration of FATDMA reservations of base station(s)

## Structure of service descriptions

To facilitate the uniform description of each of the services identified in List 1 - Basic AIS Services (section 4.2) the following format is used.

Service Description

Service Overview

- Service Name:

- Purpose and General Description:

"Who, what, where, when, and why? - newspaper like description"

Early decision, "How do we want to pitch this service?"

Define and describe the service

What does the service deliver?

Why is this service important?

Brief description of service products/benefits

When/where is this service used?

Circumstances under which this service should not be used?

- Use:

Recommendations concerning exploitation of this service.  This service could be used to support the following applications: (missions, operations, surveillance,...)

- Risks:

What are risks - business, safety, technical,...?

How significant is their impact?

Are there measures that can limit the negative effects?

[when using this service mistakenly or with wrong data.

Evaluation of these risks.

Possible measures (on higher levels) that can be used to limit possible negative effects of risks]

- Operation and Support:

Describe start ("trigger event"), support [dependencies, external, internal, ...], and end of this service

Prior Initialization needed for this BAS

Time

Location

Platform condition change

Interdependence of this BAS [service] with another [internal service] BAS

Service Details

- Operating Characteristics:

Timing and Behavior with regard to time

Queuing delays

Conditional behavior

Time of Generation

Time elapsed since Transmission (e.g. aboard) to Reception at point of use (e.g. VTS Center) = Latency issues

Update Interval

- Information:

- Detail of service products and support, Data formats, and Data Content - explicit and / or implicit

- example for explicit information: Latitude in degrees; data format: integer value between ...

- example for implicit information: Use of Message #3 instead of Message #1 indicates that the source AIS now operates in assigned mode = implicit acknowledgement of Assigned Mode Command from Base Station

- Shipboard generation of data item: Automatic generation using sources internal to the shipborne AIS; automatic generation using sources external to the shipborne AIS; manual input at installation; manual input during voyage, etc.

- Resolution and / or accuracy of Data Content (e.g. +/- ... m; binary; etc.)

- Default Value of Data Item

- Messages of M.1371-1 used to realize this BAS

- Integrity precautions taken when generated automatically

- Estimated reliability of information with regard to generation of that data item: high, low, etc. (from the recipient's point of view)

- Implicit or explicit integrity information transmitted

- Consequential fault conditions:

technical

- Shipboard/shore prerequisites for use:

- Technical

[examples: DTE (date terminal equipment), AIS unit revision, AIS class A/B/base, external application specific capabilities,...]

- Regulatory

[examples: ...?]

- Side effects:

- positive

- negative

- Fall back arrangements to other AIS services

List in a priority scheme

- Possibilities / necessity to indicate to higher level state of BAS performance

- Logging possibilities

## Basic AIS Service (BAS) Descriptions - External Basic AIS Services

### A_STAT: IMO SOLAS Static Information

#### Service Overview

This basic service broadcasts the following static information about ships.

-MMSI Number

-AIS type number

-Call Sign

-Name of ship

-Type of ship

-Length and beam of ship

-Height above keel

-Reference location of position sensor

-Type of position sensor

-Functionality of entering and displaying AIS data

The AIS mobile station automatically broadcasts the above static ship information except "height over keel" every 6 minutes. To enter and display this information the AIS mobile station provides, at a minimum, a minimal display function.

Use

-The static ship information is used to identify and classify ships.

-The ship's dimensions are important in order to determine and evaluate the maneuvers of a ship.

-The details regarding the ship's position sensor are important in order to assess the accuracy of the position information.

-The "height-over-keel" information together with other data can be used to determine the height over water level.

Risks

-Because static information on board has to be entered manually it is possible that this data may be faulty.

-The limited precision of the ship's dimensions and the location of the position sensor have to be taken into consideration when evaluating this data.

Operation and Support

-Following normal start-up and synchronization this basic service starts automatically.

-After that, all data will be broadcast every 6 minutes.  If any individual data element is changed the entire data string in immediately transmitted.

-This basic service cannot be turned off either by a base station or by the mobile AIS station itself.

a) The MMSI number, the AIS type number, the IMO number, length and beam of ship have been entered during the original installation of the AIS and cannot be changed by the ship's personnel.

b) Call sign, name, type of ship are pre-assigned during the installation phase and should not be changed by the ship's personnel.

Height-over-keel is a basic initial data element and should not be changed by the ship's personnel. A VTS centre can obtain the height-over-keel information only via polling.

c) The types of the installed position sensors and their reference locations have been predetermined at the time of installation of the AIS unit.  During operation the AIS-unit determines automatically which sensor is being used and what data it is to transmit.

Interaction with other Basic AIS Services

- The type of position sensor and its reference location position as well as the extended use of the length and beam of the ship are important parameters for the ships' data which can be obtained by using A_DYN.

### A_DYN: IMO SOLAS Dynamic Information

#### Service Overview

Purpose and General Description:

The BAS transmits dynamic information from mobile AIS Stations onboard SOLAS V vessels. The current dynamic information is derived onboard the vessel.

Data that is derived from the actual traffic situation and is not static or voyage related is defined to be dynamic information.

- Latitude und longitude of geographical position

- Intigrity of position

- Accuracy of the position

- Time tag of position determination

- SOG and COG

- Heading

- ROT (if available)

- Navigational status

The dynamic information is transmitted in defined data frames periodically in fixed intervals. All sensor data is generated automatically and transferred to the AIS-station except the navigational status. The position, SOG and COG derive from an Electronic Positioning Fixing Device (EPFD).

The BAS is always enabled and operates continuously and autonomous. The information is always broadcasted.

The transmission period (time between reports) depends on the navigational status, the SOG and the maneuvering of the vessel. The transmission rate can be 2 times per second for a vessel with a SOG of 23 knots while maneuvering reduces to 3 times per second while anchoring. Only base stations are capable to change the transmission rate by using ASGN_SLOT or ASGN_RATE.

Use:

The dynamic information combined with the data derived from A_STAT (Static information) and A_VOY (Voyage Information) can be used to visualize the traffic situation.

The information transmitted by A_DYN can by used by other mobile stations and base stations. In a VTS centre it can support the radar tracking and ships data processing.

For the first time it will be possible to control traffic of AIS equipped vessel in region not covered by radar.

Risks:

None

Operation and Support:

Two minutes after switch on a class A mobile station is operational and it works in the autonomous mode. Depending on the navigational status, the maneuvering and the speed of the vessel A_DYN will operate continuously in the autonomous mode.

It is not foreseen to turn off the mobile station during anchoring.

Interaction with other BAS:

The A_DYN depends on:

- A_STAT concerning the type of EPFD and concerning the reference of position to calculate the position, the COG and SOG.

- BAS no 29 concerning secondary synchronization on mobile station

### A_VOY: IMO SOLAS Voyage Information

[optional, Exchange Waypoints and Routeplans (both directions) (IAI FI #17+18)]

#### Service Overview

Purpose and General Description

This AIS Basic Service describes the transmission of voyage related ship data. Only SOLAS V ships (class A) have to transmit voyage related ship data.  The basic service broadcasts  voyage related information.  This information includes the following ship data.

-Draft

-Port of Destination

-ETA

-Type of Cargo

-Waypoints/ Routeplans (optional)

-Number of persons on board

The voyage related data, except for the route plans and persons on board, are broadcast every 6 minutes together with the static ship data.  When required these data can additionally be requested from another AIS unit (re Interrogation see INT_TDMA.  The way points/ route plan information as well as the "number of persons on board" are not broadcast regularly. This broadcast takes place only when the ship's personnel initiates it.  Again, if required, other AIS units can poll for this data.

This AIS Basic Service is enabled at all times.

All data is entered manually and is often subject to the discretion of the ship's personnel.

-The draft must always be entered.

-The entry of port of destination and ETA is at the discretion of the captain. Free form text will be used for the port of destination.  Agreements concerning ways of writing, abbreviations, or codes do not exist.

-The entry of ETA is based on date and time at the port of destination.

-Providing information on the type of cargo is required if the responsible agency so demands it.  The designation of a code for dangerous cargo, or a designation in clear text and quantity is not envisaged.

-Waypoints and route plans may be entered directly into the AIS device.  This process requires additional device functionality.

-The number of persons on board cannot be entered directly into the AIS device.  This, too, requires additional device functionality.

Use:

[Note: item is already described adequately under purpose.]

Risks:

-Manual entries depend on the 'human factor" and are, therefore, only partially reliable.  Since an AIS device only forwards data but does execute any verification of the transmitted data, the sensor (here: person = ship's personnel) has to ensure that only valid data is communicated.

-The time of data entry or the respective decision of the captain is not known. The use of the manually entered data includes, therefore, a certain risk that should not be underestimated that some of the data is old not corrected and up-to-date or even missing. Verification of data or missing up-dates is not possible.

-When waypoints are up-dated on board no automatic transmission by the AIS device takes place.

Operation and Support

The transmission of voyage related data, except waypoint information and number of persons on board, takes place regularly every 6 minutes.  The data are being transmitted independently of the routine schedule if specifically polled for.

Interaction with other Basic AIS Services

INT-TDMA (interrogation of the VDL); TRANS_IAI

### SAFE_AD: IMO Short Safety-related Addressed Messages

(with acknowledgement)

#### Service Overview

Purpose and General Description:

[This message can be used to address to a specific recipient information about safety in readable format. This message is automatically acknowledged by the AIS station.  However, this does not mean that the operator has read the message (acknowledgement does not indicate that the operator has been informed). It should be noted that the AIS unit can only process messages in be pilot position, buoy outage information and a temporary speed limitation.  This service should be used prudently]

- Transmission of the onboard operational (navigational) status in the case of safety onboard is affected or endangered.

- The transmission of navigation information which could mutually affect the navigation of vessels

- Transmission of addressed safety related traffic information from the VTS center to a selected vessel

The IMO defines the safety related message in their requirements as tool for traffic control and collision avoidance. The information is generated automatically or manually at any time. The contents of the message is arbitrary ASCII-text. In case of transmission failure the message is repeated 3 times. The number of repeats may be set by an application. An acknowledgement of the receipt is given only to the AIS system layer.

Class A and Class B mobile stations differ in the performance of the message (Class B maximum length is 2 slots, Class A is 5 slots). The message can be displayed in case of Class B on a minimum display if the DTE flag is not set; in case of Class A the minimum display and keyboard is mandatory. This BAS must accessible under all conditions.

## Use

This BAS can be used for transmission of safety related messages from ship to ship and from ship to shore e.g. fire onboard or failure in a technical system needed for navigation. Applications are recommended to automatically generated and transmitted. The VTS center should invent indicators on the application level for e.g. crossing borders of safety areas (shallow waters, restricted areas). This function can be combined with [DE: initiate] automatic transmission of traffic information addressed to the specific vessel.

Risks:

A minimum display and keyboard is required in SOLAS V vessels. The acknowledgement is given only on the VDL level but no acknowledgement for the contents of the message on higher levels. A safety related information may be missed on board. The transmission starts immediately after initiation of the BAS with a maximum delay of [XXX] sec. Long and often repeated messages stress the capacity of the VHF channel. They also may result long lasting transmission. During VHF disturbance these message may be entirely disturbed. Planning of applications with automatic traffic information should be done with respect to the VDL capacity. Operations on highly frequented VDL-channels may require time slot management to control the system

## Operation and Support

Due to the technical feasibilities of the minimum display and keyboard onboard and the  number of occupied slots the safety related messages should be as short as possible. The message is variable in length and contents. The maximum number of time slots is limited to 5.

Interactions with other BAS:

ASGN_SLOT: Control slot allocation of one or more AIS stations

### SAE_BR: IMO Safety-related Broadcast Messages

#### Service Overview

Purpose and General Description:

This message can be used to broadcast information about safety in readable format to all receivers in the area. This message is limited to maximum "chunks of" about 160 alphanumeric characters. Some examples of this information are real time meteorological and hydrological information, traffic situation, position of hazards. This service should be used prudently (explain prudently).

### INT_TDMA: Interrogation via AIS VDL

#### Service Overview

[Any AIS can send an interrogation to the units in the area requesting a position report and static and voyage related data. This would normally be provided automatically but the user may want the information immediately. The base station may be interrogated for DGNSS correction information, if available, channel management information, position information etc. AtoNs may also be interrogated for their aids to navigation report (name, position, type, status …)]

- Purpose and general description:

- The user can interrogate an AIS station for position information and static and voyage related information, providing the interrogator is permitted and the interrogated stations is in the reception range.

- Vice versa every mobile station can be interrogated according the permission of the interrogating station.

- Additionally mobile stations can interrogate base stations for UTC-time, DGNSS-corrections and Channel and frequency management parameters. With the latter the mobile station can be controlled.

- As a maximum 2 stations can be interrogated with each two set of information like position data and static/voyage data.

Use:

- A base station may use the BAS to interrogate stations in between the periodical transmission to get position and static/voyage information only with slight delay. The stations may be SAR Aircraft, other crafts, base stations or AtoN equipped with AIS systems.

- A base station may also get extended information, especially extended static and voyage information from other stations.

- Mobile stations can interrogate a base station for

- UTC time

- DGNSS correction

- Frequency and channel parameters to control their VDL channel

Risks:

- Many interrogation and answers may overload the VDL channel. The AIS system turns itself off at overload.

- Especially for the (extended) ships data [DE: means Static/Voyage Information] there is a possibility that they are not right or current.

Operation and Support:

- The interrogator has to select the information he wants to interrogate. Either the user defines the delay time or the answer or AIS system defines the delay automatically. After transmission of the interrogation the communicating stations control the process of data exchange automatically.

- If being interrogated the AIS system answers automatically without user action (provided the information is available).

- Interaction with other BAS

- The BAS includes the functionalities of the interrogation and the answer to interrogation.

- The following BAS may be started to get dynamic, static and voyage data:

- A_STAT (Static Information)

- A_DYN (Dynamic Information)

- A_VOY (Voyage Information)

- B_DAT (Class B Dynamic Information)

- SAR_DAT (SAR Aircraft Dynamic Information)

- ATON_DAT (A to N Dynamic Information)

### INT_DSC: Interrogation via DSC(Tampa)

#### Service Overview

Purpose and General Description:

A base station can interrogate an AIS Unit requesting a DSC response

### ASGN_SLOT: Control slot allocation of one or more AIS stations (not only mobile) via assignment (Tampa)

#### Service Overview

Purpose and General description:

AIS stations basically operate in 3 different modes: Autonomous Mode, Assigned Mode, Polling Mode (Interrogated). The default is the autonomous mode. The referring reporting intervals are listed in ITU-R M1371 §4.2. The autonomous mode uses the self organized slot allocation.

Only base stations are capable of slot allocation to have access to the VDL capacity for the use of certain services (assigned mode). This way the reception of far away stations is guaranteed.

Use: If a base station only sporadically receives messages for one mobile station, the base station can assign a mobile station specific slot numbers for use to prevent multiple use of one slot number

### ASGN_RATE: Control reporting rates of one or more AIS stations (not only mobile) via assignment (Tampa)

#### Service Overview

Purpose and General Description:

A base station can control the rate of reporting for AIS stations. This would be done to follow more closely the movement of a vessel (increase the number of the reports) or to prevent interference (decrease the number of reports).

### B_DAT: Current vessel status of ships carrying a Class B shipborne mobile AIS device

#### Service Overview

[This service broadcasts specific static and dynamic information about the ship and the position source used by the AIS unit. This information includes the ship's: name, call sign, length, beam, type, position, COG, SOG, HDG.  The AIS unit also broadcasts the reference point on the ship to which the reported position information is referenced. The dynamic information reporting interval can range from as short as every five seconds for a vessel with a speed over 23 knots, or as long as every three minutes for a vessel not moving faster than 2 knots. The static information is contained in a single report that is broadcast every six minutes, or in response to an interrogation from another AIS unit.  The information is used to increase situation awareness during navigation planning and for collision avoidance.  This information can also be used to improve the waterway management function of a VTS.  The service is normally active.]

Purpose and general description:

This service is used for transmission of current vessel status from non SOLAS V ships (CLASS B).

The BAS transmits in broadcast mode dynamic and static information, and the information with which technique and accuracy the position is determined.

The information includes following data about the ship: MMSI- Number, Ships Name, Call Sign, Length and beam, Type of ship, cargo type, geographical position, COG, SOG, HDG and the position reference point.

If only dynamic information is transmitted, AIS message #18 is used

In the table below the possible reporting intervals are listed.

[Insert M1371 table 1b without the last 3 lines- class B only]

If dynamic and static information is transmitted, AIS message #19 is used. In that case the reporting interval is 6 minutes.

When necessary mobile stations can be interrogated for both messages. (see BAS No.8 Interrogation)

The information is used for the purpose of navigation planning and collision avoidance to be aware of other ships in the area and their behaviour.

The fast and precise presentation of the information about the ships behaviour increases the safety on waterways.

The information can be used to improve the display of the traffic situation in a VTS.

As default the BAS is always enabled.

- The information transmitted by messages #18 or #19 is supplied by:

- the AIS station itself

- set during installation like e.g. MMSI number, Ships Name, Type of Ship

- derived from the system like e.g. time tag, availability of the integrity control or the  availability of the data terminal [DE means:  min. display and keyboard], transmission mode, synchronisation status, slot usage etc.

- sensors, e.g. position, position accuracy (external EPFD)

- manual input, e.g. change of static information (e.g. ships name) or sensor failure

- The automatic delivery of data can produce onboard of all ship and in VTS centres an automatic display of the current traffic situation. The quality of display depends on the quality of the delivered data. It should be required that sensors only should deliver valid data because an AIS station only transfers data without examine its plausibility.

- In VTS centres the manual workload associated with the corrections of radar data and ships data can be reduced.

- On the interface the explicit data delivered from the ship and implicit data derived from the system is available.

Use:

The BAS is used to make available the static and dynamic data of non SOLAS V vessels equipped with AIS.

No voyage data is required from Class B AIS stations.

No information is available about ship not carrying an AIS Station respectively the information can only be derived by verbal interrogation via VHF radio and after the data have to be processed manually. It is unknown up to now, how the data will be processed on higher application levels. Nothing at all there is the risk of faults because of manual input. Therefore the traffic situation display of various users may show different information.

In this case the verification of the information by verbal VHF radio interrogation, radar or by observation in very important.

Operation and Support:

After a maximum delay of 2 minutes the BAS is operational and operates until the AIS station is switched off.

Interaction with other BAS:

BAS9  no.7 (User Interface), BAS No. 8 (Interrogation via VDL), [BAS No.9 (Interrogation via DSC???)], [BAS No.16 (DTE-Flag)], [BAS No.22 (Interrogation specific message) ]  [require further work]

### SAR_DAT: Current SAR aircraft status

#### Service Overview

Purpose and General Description:

AIS units can be installed aboard aircraft and voluntarily used to provide information about the aircraft's status in a fashion similar to vessels.  This information is provided in a way that allows it to be easily identified and separated from vessel and other surface information. This facility is intended to support the coordination and communications between surface vessels and aircraft involved in a common search and rescue operation.  This service provides information that includes; aircraft name, call sign, location with altitude, speed, and course.  This capability should only be used for activities that require the immediate coordination of air and surface platforms.  It is not intended to be a general aviation tracking service.  [Note that "aircraft name" and "call sign" require the use of message 5.  This requires an IALA clarification for the use of message 5.]

Risks: The range of an airborne AIS unit is considerably greater than vessel units.  Limiting the power and use of airborne AIS units serves to reduce any impact this extended range might have on the normal use of AIS by surface platforms.

### ATON_DAT: Current AtoN AIS station data

#### Service Overview

Purpose and General Description:

An AIS transponder fitted to an Aid to Navigation will provide the same positive identification as a radar beacon. This service broadcasts specific aids to navigation information about type, name, dimension, position and off-position information. The AIS can broadcast the current status of the aid. It concerns both fixed and floating aids.

### BASE_DAT: Current base station status

#### Service Overview

Purpose and General Description:

The position, UTC time and date, and current status of the base station is transmitted automatically every 10 seconds. This rate can be increased to every 3.33 seconds when necessary for synchronisation.

### TRANS_IAI: Broadcast text telegram (IAI FI #0)

#### Service Overview

Purpose and General Description: This message can be used to broadcast information in readable format to all receivers in the area. This message is limited to maximum "chunks of" about 163 alphanumeric characters. This is used to transmit free text. The use of this feature can be regulated by the competent authority to avoid overloading of the system.

### TRANS_RAI: Not applicable at this time pending and IMO decision.

### DGNS_COR: Provide DGNSS-Corrections via AIS VDL

#### Service Overview

Purpose and General Description: A base station can transmit differential GNSS corrections if it is connected to a DGNSS reference station. This service can be broadcast automatically and is a back-up service for the DGNSS beacons.

### CH_TDMA: Switch AIS VDL frequencies via AIS VDL

#### Service Overview

[If there is a need to switch over one or both of the frequencies in use to another frequency(s), a service message is available to do this. When both channels must be switched, there will be a transition zone to avoid both channels will be switched at the same time.]

Purpose and General Description

This BAS provides GNSS correction factors. These correction factors are transmitted via base stations.  [BT: The German text is unclear as to which correction factor will actually be transmitted by the AIS base station.  An ideal sentence would be:

"A VTS centre selects the correction factors that will be transmitted by the AIS.  The VTS center in turn selects the correction factors to be transmitted according to the following rule…………..etc.]

AIS mobile stations can request this BAS.

This BAS is offered in order to provide vessels DGNSS correction factors under following conditions:

-when the ship's own receiver for DGNSS correction factors (beacon-receiver) has failed or

-when the ship does not have a receiver for DGNSS correction factor in the first place.

Use: This BAS will be employed in order to deliver to all users of a GNSS that is within reach of a specific base station improved position data and integrity data.

Risk: If there is no interface connection between the AIS device and the GNSS receiver then no correction factors can be transmitted.  In that case the position accuracy is only >10 m.

[BT: Don't agree with the last sentence.  The ship's own independent GPS system could well have differential corrections.  Consequently, it would feed differentially corrected position data to the AIS device.]

Operation and Support: When it is desired to transmit DGNSS correction factors via a base station, they can be sent automatically and continuously in the broadcast mode or in response to the polling mode.  The optimum rate of transmission in the broadcast mode has still to be determined.

Interaction with other Basic AIS Services

INT_TDMA

FATDMA

### CH_DSC: Switch AIS VDL frequencies via DSC Channel 70 (Tampa)

#### Service Overview

- Purpose and General Description:

In an area where the international designated AIS channels may not be available and the AIS is not under control of another AIS pair of frequencies, DSC channel 70 will be used to force the AIS unit to switch over to designated frequencies.

### PWR_LEV: Set transmitter power level of mobile AIS stations

#### Service Overview

Purpose and General Description: This BAS ensures that all AIS stations (mobile stations and base stations) within the same coverage area are synchronized.

Timing Process:

There are several synchronization states. These states depend on the availability of UTC.  UTC time can be obtained directly from the ship's own AIS device via the GNSS link or, via the VHF data channel,  from another AIS station (base station, mobile station) that can receive UTC direct.

If these two possibilities are not available, a mobile station can synchronize itself to another base station whose signals it can receive.  A prerequisite for this mode that the other bases station is transmitting the semaphore status i.e. it is receiving the largest number of other AIS stations.  When an AIS mobile station receives signals from several other base stations that indicate semaphore status then it selects the one with the lowest with the lowest MMSI number.

If all previous methods have been exhausted then synchronization will be based on that AIS mobile station that indicates that it receives the largest number of other AIS Stations (AIS mobile station which signals semaphore status).

For the purpose of synchronization a base station sends message 4 at least very 10 sec. For the time during which one or more AIS mobile stations synchronize themselves to that base station, message 4 is transmitted every 3 1/3 seconds.

(For further details on this process see the corresponding flow diagrams)

Messages used: Message 4, 11, 10

Integrity:

A base station transmits regularly, among other data, its synchronization state and UTC time via message 4.

In response to an interrogation via message 10 an AIS mobile station uses message 11 to communicate its synchronization state and UTC time.

In case the base station has no UTC time it can set a  semaphore status or substitute another time reference for UTC.

[BT: The last sentence doesn't sound right, needs to be verified].

Advantages of Semaphore: Cost effective i.e. no expansion of the base station is required.

Disadvantages of Semaphore: No absolute  time reference.  Slot numbers are unknown which can lead to interference problems when transmitting certain messages.

Advantages of Time Standard: Absolute time relationship remains intact.

Disadvantage of Time Standard: Additional investments are required e.g. for Loran-C receivers or Cesium clocks.

Onboard Ship Requirements: None, are contained in the AIS equipment.

Onboard Ship generation: Automatic

Requirements for VTS centres: No special requirements exist. The parameter "Synch State" must be derived from the Communication State.

Risk

Probabilities:

Fault Indications:

Affected Areas: (Technical operation of base station)

Affected Area: (Technical operation of AIS network)

Affected Area: (Technical operation of radar data processing)

Affected Area: (Technical operation of ships' data (static and voyage related only) processing.

Man Machine Interface (MMI) for the mariner is the VTS centre: (The synch state of own base station and, as required, that of the received AIS mobile station can be evaluated and indicated for possibly required consequences .

Data Format:

Precision/ Accuracy:

[BT: Here follow a number of detailed flow diagrams which have to be processed]

#### Service Overview

FATDMA: FATDMA Slot Management

1.39.2.1 Service Overview

Purpose and General Description

The purpose of this BAS is to allow making reservations for time slots for the base stations (for repetitive tasks).

It follows that time slots reserved by base stations will note be used by AIS mobile stations.

A base station requires time slots for the transmission of:

- Base station report to support synchronization processes

- Assignment of time slots to AIS mobile stations

- Assignment of repetition rate for specific AIS mobile stations

- Channel management messages

- DGNSS correction factors

- Safety related messages (non-repetitive)

- Application specific messages (non-repetitive)

Timing behavior

Reaction Time

- Time of generation: configuration

ITU-R M.1371 Message used: Message 20: Data Link Management Message

- 4 blocs of different size, separation distance, timeout

ITU-R M.1371 References: A2, para 3.3.4.3 FTDMA

Reference Document: M.1371, A2, para. 3.3.8.2.16 Message 20

Integrity, Verification, navigational reliability

Reliability is high due to system management function in case of automatic generation

Default value

………….etc.

If DSC has been implemented as part of the AIS service

A message that is initiated by the vessel is said to be broadcast