| From: IALA-IEC WG14-13 | WG14-13-output-02 |
| --- | --- |
| To: IALA e-Navigation Committee (e-NAV1) | August 25, 2006 |

Liaison Note

AIS AtoN / Use of VDL Messages by an AIS AtoN Station

# Introduction

In developing the draft testing standard 62320-1, the IALA-IEC WG14 has discussed the use of AIS messages by an AIS AtoN station.

# Use of VDL Messages by AIS AtoN Station

IEC Working Group 14 has developed the following table (working document) and proposes the following use of AIS messages by an AIS AtoN station.

| Message No. | Name of Message | Use Internally | Output on PI | Transmit | Access mode | Remark |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Undefined | No | Opt | No Not Allowed |  | Reserved for future use. If CRC checksum is correct, output on PI. |  |
| 1 | Position Report (Scheduled) | Yes * | Opt | No Not Allowed |  | * Only applicable if RATDMA access scheme is used. |  |
| 2 | Position Report (Assigned) | Yes * | Opt | No |  | * Only applicable if RATDMA access scheme is used. |  |
| 3 | Position Report (When interrogated) | Yes * | Opt | No |  | * Only applicable if RATDMA access scheme is used. |  |
| 4 | Base Station Report | Yes * | Opt | No |  | * Only applicable if RATDMA access scheme is used. |  |
| 5 | Static and Voyage Related Data | No | Opt | No |  |  |  |
| 6 | Addressed Binary Message | Yes (1) | Opt | Opt | FATDMA CSTDMA RATDMA | (1) Only if received, and if addressed to own station and AIS AtoN Station can follow instructions in the message. |  |
| 7 | Binary Acknowledge | Opt (6) | Opt | Opt | FATDMA CSTDMA RATDMA | (6) Only if AIS AtoN Station can transmit Message 6. |  |
| 8 | Binary Broadcast Message | Opt Yes (7) | Opt | Opt | FATDMA CSTDMA RATDMA | (7) Only if AIS AtoN Station can follow instructions in the message. |  |
| 9 | Standard SAR Aircraft Position Report | Yes * | Opt | No |  | * Only applicable if RATDMA access scheme is used. |  |
| 10 | UTC and Date Inquiry | No | Opt No | No |  |  |  |
| 11 | UTC/Date Response | No | Opt No | No |  |  |  |
| 12 | Addressed Safety Related Message | No Opt (10) | Opt (3) | Opt | FATDMA CSTDMA RATDMA | (3) Only if received, and if addressed to own station. (10) For the purpose of repeating safety related messages. |  |
| 13 | Safety Related Acknowledge | Opt (9) | Opt | Opt | FATDMA CSTDMA RATDMA | (9) Only if AIS AtoN Station can transmit Message 12. |  |
| 14 | Safety Related Broadcast Message | No | Opt | Opt | FATDMA CSTDMA RATDMA |  |  |
| 15 | Interrogation | No Yes | Opt No | No |  | An AtoN AIS unit shall respond to interrogation for message 21 but shall not interrogate other units. |  |
| 16 | Assigned Mode Command | No Yes | Opt No | No |  |  |  |
| 17 | DGNSS | Yes (5) | Opt | No |  | (5) Only if internal GNSS receiver is capable of processing DGNSS corrections or PI contains a DGNSS output port. |  |
| 18 | Standard Class B Equipment Position Report | Yes * | Opt | No |  | * Only applicable if RATDMA access scheme is used. |  |
| 19 | Extended Class B Equipment Position Report | No | Opt | No |  |  |  |
| 20 | Data Link Management Message | Yes * | Opt | No |  | * Only applicable if RATDMA or CSTDMA access schemes is used. |  |
| 21 | Aids-to-Navigation Report | No | Opt | Yes | FATDMA RATDMA |  |  |
| 22 | Channel Management Message | Yes No | Opt | No |  |  |  |
| 23 | Group Assignment Command | Yes No | Opt | No |  |  |  |
| 24A/24B | Static Data Report | No | Opt | No Yes (11) | FATDMA CSTDMA RATDMA | (11) Only applicable to Type 3 AIS AtoN Station |  |
| 25 | Single Slot Binary Message | Opt (8) | Opt | Opt | FATDMA CSTDMA RATDMA | (8) Only if AIS AtoN Station can process configuration over VDL. |  |
| 26 | Multiple Slot Binary Message with Comm. State | Yes * | Opt | Opt No | SOTDMA | * Only applicable if RATDMA access scheme is used. |  |
| 27-63 | Undefined | No | Opt | No |  | Reserved for future use. If CRC checksum is correct output on PI. |  |
| Use Internally, e.g. act upon the received information, and use the received information internally. This column is only applicable if AIS AtoN Station has TDMA receiver. [REFERS TO COLUMN “USE INTERNALLY”] |  |  |  |  |  |  |  |
| Only applicable if PI is implemented. [REFERS TO COLUMN “OUTPUT ON PI”] |  |  |  |  |  |  |  |

# Requested Action

The e-NAV Committee is invited to review the proposal and take it into account during the development of further IALA documentation on AIS.