The interfacing of older GNSS units with AIS

# Issue

There is a lack of awareness amongst mariners of the danger in interfacing older GNSS units with AIS and other systems on board ships.

# Background

There are currently two IMO performance standards for GNSS receiver equipment:

- if installed on board on or after 1 July 2003, it needs to conform with performance standard MSC 112(73) (the "new" standard);

- if installed before 1 July 2003, it needs to conform to standards specified in Res. A.819 (19)  (the "old" standard).

Differences between the two standards are considerable.  Under the new standard, there is a requirement for integrity monitoring (RAIM)*, satisfactory operation in typical interference conditions, higher accuracies for position, Course Over Ground (COG) and Speed Over Ground (SOG) output, higher update display rate (every second, as opposed to every two seconds).  The most significant of these are the RAIM requirement and interference rejection standards.

A number of vessels are equipped with GNSS equipment installed before it became a mandatory requirement (SOLAS 2002 amendments).  Therefore, these are mostly units that conform to the old standard.

*  Receiver Autonomous Integrity Monitoring (RAIM) is a technique whereby the performance of the GNSS system is determined within the receiver itself.  Four pseudo ranges are needed to compute a position at any particular instance.  If five are available then five different position solutions can be computed by using all combinations of four satellites (Al-Nakib, 1992). By examining these five solutions, ones which are using an erroneous pseudo ranges will be similar and different from the remaining solution.  In this way poor ranges can be isolated, but the technique obviously relies on a full geometry to be able to detect multiple errors.

Originally, AIS was to be fully implemented by 2008.  This would have given the shipping industry enough time to replace the older standard GNSS receivers.  The accelerated introduction of AIS may have resulted in IMO losing sight of the importance of this crucial detail.

# Discussion

With the increasing reliance on GNSS by current generation systems such as ECDIS, AIS and INS, it is vital that the maritime community is made fully aware of the pitfalls of interfacing such systems with older GNSS units.  The unresolved issue of integrity monitoring represents a 'latent pathogen' in the system.  In the case of AIS, ships with newer GNSS' will isolate a faulty satellite and transmit a correct position.  However, ships fitted with older receivers will transmit an incorrect position.  Unless the OOW is vigilant he/she will be unaware that incorrect own ship’s positions are being transmitted.

Ideally, only GNSS RAIM enabled equipment, with DGNSS functionality to further enhance integrity monitoring, should be permitted on SOLAS ships.

The attached Annex gives information on a real incident in 2004.

# Progress so far

Initially, IALA raised awareness of this issue (documents AIS 14/13/2 and AIS 14/13/2 refer) and then finalized a paper outlining the problems (AIS14/output/01).

Subsequently, UK raised this issue at NAV 50 (July 2004) along with other AIS concerns including poor data entry and installation problems (NAV 50/4/2 refers).

The Technical WG of IMO NAV 50 stated the following:

The TWG noted that recent feedback was showing that AIS problems were declining as experience with AIS was achieved.  The TWG did not therefore see an immediate need to review IMO Resolution A.917 (22)* but requested that this item be added to the work programme of the Sub-Committee so that experience could be evaluated after the full implementation of AIS on 31 December 2004.  Moreover the TWG urged all Members to stress the importance to ship owners of correctly following the installation guidelines in SN/Circ.227 and correctly entering the ship data when installing the AIS.

*This Resolution refers to IMO Guidelines for the onboard operational use of AIS

Note IMO Res A.917 (22) makes no reference to the GNSS issue.  Further, the SN/Circ.227 is silent on this as well.  So, this important issue has evidently “fallen through the cracks”.

# Proposed way forward

AIS 17 to decide

*********

ANNEX

#### A REAL INCIDENT

#### GPS provided misleading information

A significant GPS anomaly occurred on I January 2004.  The anomaly affected precise timing and navigation over a large portion of Europe, Africa, Asia, Australia, and the far northern reaches of North America.  The anomaly was due to a failed Atomic Frequency Standard (AFS) on one of the satellites. The GPS system relies heavily on the accuracy and stability of its AFS.  A failed AFS not only affects precise timing, but also can also significantly degrade navigational accuracy.  Apparently, the above anomaly adversely affected a UK cable ship.

This has highlighted the critical importance of Integrity Monitoring for the shipping industry.  A lack of hard failure indications in satellite telemetry, coupled with satellite visibility limitations in the Master Control Station's L – Band monitor station network, made this anomaly difficult to characterize and resulted in the transmission of misleading information.  The position error from this satellite was around 300 km by the time it was set to 'unhealthy'.

#### Commentary

Above episode highlights the problem of using GNSS without integrity monitoring.  It is sometimes forgotten that the GPS system does not have in-built Integrity Monitoring.  Integrity Monitoring is essential for critical applications like ECDIS, AIS and marine pilotage.  In this instance, the GPS system continued to provide erroneous positions for 2 hours and 45 minutes before the problem was rectified.  If the receiving set had been enabled for Integrity Monitoring, the faulty satellite in the above situation would have been flagged and isolated from the final position solution, with no loss of accuracy.