| From: ANM7 | ANM7-WG2-output-liaison_RNAV.doc |
| --- | --- |
| To: RNAV Committee | October 2005 |

Liaison Note

- Discussion paper on continuity requirements from RNAV23

- Introduction

The ANM Committee was asked to review the requirements for Continuity and provide comments for further discussion.

- Background

RNAV considers that the existing IMO targets for reliability/availability on maritime GNSS applications are unrealistic due to the difficulty to achieve. Also, the comparison between aviation and maritime targets seems to be discrepant.

| Context | Risk level | CTI | Permitted MTBF |
| --- | --- | --- | --- |
| Aviation (Non Precision Approach) | 0.8.10-6 | 15 seconds | 217 days |
| Marine (High Risk Areas) | 3.10-4 | 3 hours | 416 days |

In particular, the ANM Committee was invited to comment the following proposal and respond to the next questions:

“It is therefore proposed that a single level of continuity (99.97%), over a Continuity Time Interval (CTI) of 15 minutes should be applied for all relevant phases of a voyage i.e. harbour entry and approach, port manoeuvres and inland waterways.”

The ANM Committee was also invited to consider and respond to the following questions:

- What are the phases of a manoeuvre?

- Which are the critical phases?

- How long would each phase last?

- What would your requirements be for navigation systems in each phase?

- General Comments

Reference: Navguide, 5th Edition – Draft Edition

Navigational phases:

Typically, navigation has been divided into three phases: ocean navigation, coastal navigation and restricted waters navigation. More recently, some documents have introduced other phases of restricted waters, namely harbour approaches, port and inland waterways navigation.

The performance of a GNSS is crucial in all restricted waters navigation.

IMO Resolution A.915(22), adopted in January 2002, established the following accuracy standards for maritime navigation.

| Application | Absolute horizontal accuracy (95%) (m) |
| --- | --- |
| General Navigation: Ocean Coastal Restricted waters Port Inland waterways | 10-100 10 10 1 10 |
| Hydrography | 1-2 |
| Oceanography | 10 |
| Aids to navigation management | 1 |
| Port operations: Local VTS Container/cargo management Law enforcement Cargo handling | 1 1 1 0.1 |

- The three phases of manoeuvres are turning, recovery and track keeping.

- The critical phases of manoeuvres in restricted waters navigation are turning and recovering, although track keeping could be critical in some situations.

- Although it is not possible to define the time each phase is lasting, because it depends on a lot of internal and external factors such as vessels size, draught, rate of turn, speed, water depth, weather etc., experience says that maximum times of manoeuvring in restricted waters are:

  - Turning: 10 minutes

  - Recovering: 3 minutes

  - Track Keeping: 3 hours

- The requirements for the navigation system depend on each phase of  a manoeuvre in restricted waters could be:

  - Turning and Recovering:

    - Availability: 99.97%

    - CTI: 15 minutes

    - Accuracy: 1-10 metres

  - Track Keeping:

    - Availability: 99.97%

    - CTI: 5 minutes

    - Accuracy: 10 metres

- Requested Action

Given the short amount of time and the limited resources to perform adequate research, the ANM members are not in a position to provide a final answer on the questions from RNAV23. As it stands now, ANM members support in principle the proposal from the RNAV committee for a single level of continuity of 99.97%, over a CTI of 15 minutes to be applied for all relevant phases of a voyage i.e. harbour entry and approach, port manoeuvres and inland waterways. It is suggested that the RNAV committee prepare a more precise recommendation based on the above information and submit it to the ANM committee for another review.

The ANM members consider this liaison note as preliminary view of the ANM Committee.  The Committee recognizes the difficulty in quantifying the requirements, and supports further discussion with users (e.g. Nautical Institute, IMPA).  The ANM Committee looks forward to working with RNAV as they develop this further.

IMO Resolution A.915(22) includes in its Appendix 2 the requirement for an accuracy of 10 m on ocean navigation, while IMO Resolution A.953(23) mentions that “where a radionavigation system is used to assist in the navigation of ships in ocean waters, the system should provide [an accuracy of] 100 m with a probability of 95%”.