Draft SN/Circ.XXX

July 2006

# INTERFACING OLDER GNSS UNITS WITH AIS

There is lack of awareness amongst mariners of the danger in interfacing older GNSS units with AIS and other systems on board ships.  Member Governments are therefore requested to bring this information to the attention of the relevant authorities and all seafarers for guidance and action, as appropriate.

# Background

There are currently two IMO performance standards for GNSS receiver equipment:

- if installed on board on or after 1 July 2003, it needs to conform with performance standard MSC 112(73) (the "new" standard);

- if installed before 1 July 2003, it needs to conform to standards specified in Res. A.819 (19)  (the "old" standard).

The differences between the two standards are considerable.  Older GNSS receivers do not have Receiver Autonomous Integrity Monitoring (RAIM), and are therefore unable to offer an indication of the integrity of the positional information being broadcast (DGNSS can mitigate against this).  They also lack course over ground and speed over the ground output.  Older units have a lower update rate (two seconds, as opposed to one second updates for newer units) and are less able to cope with signal interference

A number of vessels are equipped with GNSS equipment installed before it became a mandatory requirement (SOLAS 2002 amendments).  Therefore, these are mostly units that conform to the ‘old’ standard.

Originally, AIS was to be fully implemented by 2008.  This would have given the shipping industry enough time to replace the older GNSS receivers.  The accelerated introduction of AIS has meant that this important issue has not been fully addressed.

# The issue

With the increasing reliance on GNSS by current generation systems such as ECDIS, AIS and INS, it is vital that the maritime community is made fully aware of the pitfalls of interfacing such systems with older GNSS units.  The unresolved issue of integrity monitoring represents a 'latent pathogen' in the system.  In the case of AIS, ships with newer GNSS' will isolate a faulty satellite and transmit a correct position, as long as sufficient satellites are visible.  However, older receivers can continue to use a faulty satellite and ships fitted with them may transmit an incorrect position.  Unless the OOW is vigilant he/she will be unaware that incorrect own ship’s positions are being transmitted.

Ideally, only GNSS RAIM enabled equipment, with DGNSS functionality to further enhance integrity monitoring, should be permitted on SOLAS ships.

Note that IMO Resolution A.917 (22) (Guidelines for the onboard operational use of AIS) makes no reference to the GNSS issue.  Further, the SN/Circ.227 (installation guidelines) is silent on this as well.

The question then arises - what is to be done about these two documents?  Comments and suggestions, please.

*******