| From: ANM (ANM7) | ANM7 / output / 0X |
| --- | --- |
| To: AIS / IEC WG14 | Oct. 14, 2005 |

Liaison Note

Items for consideration in revision of IALA Recommendation A-126 (AIS as an Aid to Navigation)

- Introduction

The AIS Committeee and IEC WG14 provided a liaison note to the ANM Committee requesting user information on two items.

- Information Requested

  - Single channel AIS AtoN’s

The ANM Committee was asked to consider whether there is a functional requirement and justification for a single channel receiver.  Would it be an option to create a single channel device?  (discussion included the concept of a ‘disposable’ AtoN e.g., AtoN placed on an iceberg)

The ANM Committee noted that there may be advantages and disadvantages of a single channel receiver, which could include cost saving / power saving.  It was determined that an inexpensive option would be desirable, however the specific user applications were not identified.

The option to place an AIS-device on an iceberg or another floating obstacle could be required, however this does not necessarily need to be an AIS AtoN.  (An AtoN typically has fixed position; an iceberg or other floating obstacle is a moving object.)

  - Use of short safety related messages

Additional short safety messages do not necessarily need to be sent by an AIS AtoN device.  It would be sufficient if Message 21 included the information on “off position” and “Lantern off (failed condition)” or “Lantern on (faulty condition i.e. wrong character).”  For this, according to A-126 5.1.2., a regional or local application identifier (8 bits reserved according to A-126, Table 34) or the message 21 could be used.  As it is understood in ANM committee, the discussion relates to Message 6, 8, and 12.

Message 6 – from the user point of view and from the management point of view – this message was not part of the original purpose of AIS and should only be used for AtoN, if it is an economic alternative to other devices for monitoring and/or remote control.

Message 8 – was not part of the original purpose of AIS and should be broadcasted by AIS Base Stations instead of “overloading” the functionality of AIS AtoN devices.

Message 12 – should not be sent automatically by AIS AtoN but by AIS base station because of the necessity of checking the conditions by the service provider prior to sending the message (A-126; 5).

In further discussion it was noted that there could be different types of A to N-AIS stations (devices):

- A very simple one on a buoy (taking into account that a floating A to N can, due to failure of mooring, float uncontrolled quite a distance from its advertised position and could, feasibly, send its message in a fixed time slot in the area of several different AIS base stations).

- An AIS AtoN station with greater functionality (nearly that of a base station) for use on a fixed AtoN, which provides operational communication to AtoN services and VTS.  These could be used to broadcast or send messages in an area which is not covered by an AIS base station.  If such an AIS AtoN is used on floating AtoN there will be special requirements to the infrastructure e.g. energy supply.

- Action Requested

The AIS Committee / IEC WG14 are invited to take these comments into account as they proceed with their work on both A-126 and the IEC AIS AtoN Station Testing Standard 62320-? (number not yet assigned).