## IALA E-NAV Committee input to the IMO NAV Correspondence Group TOR on e-NAV

Definition

The e-Nav Committee uses IALA’s working definition of e-nav as a starting point:

“e-Navigation is the collection, integration and display of maritime information onboard and ashore by electronic means to enhance berth-to-berth navigation and related services, safety and security at sea and protection of the marine environment.”

This should be harmonised with any definition agreed upon by the IMO Correspondence Group.  It may be more appropriate to use “presentation” than “display” since the interpretation of display includes the notions of presentation, portrayal, audible alarms, etc.  The definition could also mention “efficiency” of the waterways.

Key issues and priorities

Considering the wide range of options and benefits that could become part of e-Navigation, the primary value of e-Navigation is to join the ship’s bridge team and VTS team to create a unified navigation team that would achieve safer navigation through shared information. For full implementation of such a system it would need to be mandatory for SOLAS vessels and scaleable to all users.

It was suggested that before the primary benefits and value-added services could be realised, an architecture comprising three fundamental elements should first be in place. These are:

- Electronic Navigation Chart (ENC) coverage of all navigational areas,

- a robust electronic position-fixing system (with redundancy), and

- an agreed infrastructure of communications to link ship and shore.

Specifications for these fundamental elements are contained in the Annex.

Human Element

Some observations were made on the human element issues that need to be addressed when developing an e-Navigation strategy:

- Man/machine interface (i.e., balance between standardisation and allowing for innovation and development).

- Modes of information display/portrayal

- Appropriate communication of situation awareness

- Equipment should be designed to engage both the bridge team and VTS operator, maintaining high levels of attention and motivation without causing distraction.

Annex

Hydrographic Data (ENCs)

A full coverage of ENCs for navigational waters will require considerable effort from the world’s hydrographic community.

It has further been noted that the existence of proprietary updating software in many ECDIS systems has become a key cost issue when implementing ENC data. It is thought that if, through IMO, an open architecture system could be agreed, this would allow a more competitive environment in the purchase, and maintenance of ECDIS systems thus reducing the overall costs of ENC’s and increasing the global rate of acceptance.

The current nautical chart (paper, raster and ENC) distribution system may need to be reviewed to ensure easy and ready global access to ENCs.

An IMO mandatory ECDIS carriage requirement should contribute to improving the rate of ENC production.

The current excessive cost of ENCs must be addressed and so far as possible a cost-neutral migration from paper charts should be achieved.

There is a need for clarification regarding the adoption of the WGS 84 datum as the standard geodetic reference frame for use in e-Navigation.

Position Fixing

The following is a list of key elements required for e-Navigation position fixing:

- appropriate accuracy, availability,  continuity, and integrity (already included in IMO Res A.915(22)).

- adequate redundancy

- compatibility between systems

- appropriate datums (vertical and horizontal)

Current GNSS have a common weakness in that they are all subject to accidental or intentional interference.  Hence, alternative and independent position fixing capabilities need to be considered.

Consideration should be given to independent non-GNSS Electronic Position Fixing System and sensors as a potential component of e-Navigation.

e-Navigation systems should enable the electronic capture of radar ranges, radar and visual bearings, etc. for position fixing.

Mariners should maintain skills through regular training so they are always capable of using alternative position fixing methods.

A backup time signal should be considered since GNSS is used for timing in other systems (e.g., communications and AIS).

Communications

The following is a list of key communication aspects required for e-Navigation, relating to both technical and content:

- autonomous acquisition and mode switching (i.e., minimal mariner involvement needed)

- common messaging formats

- sufficiently robust (e.g., signal strength, resistance to interference)

- adequate security (e.g., encryption)

- sufficient bandwidth (data capacity)

- growth potential

- automated report generation

- global coverage (could be achieved with more than one technology)

- the use of a single language, perhaps with other languages permitted as options.

The following communications issues are among those that will require resolution to achieve the above:

- It seems likely that a satellite broadband link will be required to achieve the above requirements, and consideration must be given to how this will be achieved.

- The question of cost and who pays for the provision of a satellite broadband link must be resolved early in development of e-Navigation.