| From: IALA-IEC WG14-13 | WG14-13-output-01 |
| --- | --- |
| To: IALA e-Navigation Committee (e-NAV1) / ITU-R WP8b | August 25, 2006 |

Liaison Note

Assignment, Channel Management and Interrogation for AIS AtoN Stations

# Introduction

The current recommendations for AIS AtoN Stations (ITU-R M.1371; IALA A-126) require that they respond to Assignment, Channel Management and Interrogation messages transmitted by local base stations or mobile stations. However, IEC TC80 WG14 has the view that these messages are of limited use in the AIS AtoN application.

The concern of the working group is that, to conserve power on the physical AtoN, most Stations will be of the type that does not have a receiver or sleeps (receiver off) between transmissions. The implication is that AIS AtoN Stations will not be able to respond to any messages transmitted in these configurations.

# Proposal

The WG has concluded that an AIS AtoN Station should never respond to an interrogation since an interrogating station cannot detect if an AIS AtoN Station is capable of responding. This could lead to confusion if a response is received only from AIS AtoN Station in an apparently random fashion.

The proposal of WG14 is to remove the requirement for AIS AtoN Stations to respond to Message 15 (Interrogation), Message 16 and 23 (Assignment), and Message 22 (Channel Management).

# Requested Action

ITU-R WP8b is invited to review the proposal and take it into account during the development of further ITU documentation on AIS.

The e-NAV Committee is invited to review the proposal and take it into account during the development of further IALA documentation on AIS.  The e-NAV Committee is also invited to provide comments back to WG14 as input to WG14-14 (Oct 16-20, 2006).

Comments should be forwarded to the Chairman of WG14, A. Stewart at alans@nlb.org.uk and the Convenor for WG14, J. Carson-Jackson at jillian.carson@gmail.com