IALA e-Navigation Committee					e-NAV 1

1st Session								18 Sept 2006

Background

The Test and Performance Standard for Class A AIS is in the maintenance cycle at IEC and the CDV includes a Communication Test in clauses 6.11.2 and 14.10.3 (see Annex).

This Communication Test verifies the operation of the Class A AIS Station on the VDL for both transmission and reception giving the effective operational range. It is fully backward compatible with all existing IEC 61993-2 Class A Stations.

Requested Action

IALA is asked to consider the application of this Communication Test to other AIS devices (AtoN Stations, Base Stations, General Purpose Base Stations, Repeater Stations and AIS SART) in the IALA Recommendations, Guidance and Technical Clarifications for AIS.

ANNEX

### Communication test

The AIS shall have a means of testing communication over the VDL by transmitting an addressed Message 10 and verifying the response of the addressed station, Message 11.

It shall be possible to initiate the communication test manually by MKD input and via Presentation Interface input using an ABM sentence (indicating message10 in the “message ID” field and null field for encapsulated data).

The result of the communication test shall be displayed. The operation manual shall provide guidance on use of this feature such that the selected addressed target is at a suitable range e.g. between 15 and 25 NM.

### 14.10.3 Communication test

(6.11.2)

Method of measurement

Set up standard test environment and operate EUT in autonomous mode. Select a suitable target and initiate the Communication Test function (transmit Message 10) by

- MKD and

- ABM message.

Required results

- Confirm that EUT transmits Message 10 addressed to the selected target and that the communication test result is correct for both a successful and unsuccessful response.

- Confirm that EUT transmits Message 10 addressed to the selected target