| Source: Document 8B/410 | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8B/TEMP/223(Rev.1)-E |
|  |  | 12 September 2006 |
|  |  | English only |

Working Party 8B

LIAISON STATEMENT TO IMO (COMSAR) and IALA

AIS Search and Rescue Transmitter (AIS-SART)

Subject:				AIS-SART

Status:				For action

Date for Action:		May 2007

Contact:			Joe Hersey
E-mail:				Joe.Hersey@uscg.mil

1	Working Party 8B would like to thank the IMO for the liaison statement requesting comments on its preliminary draft performance standards for AIS-SART. An AIS-SART would use AIS technology for locating survival craft and would have the same basic purpose as a radar-SART.

2	WP 8B agrees with the preliminary draft performance standard and suggests the following changes:

An AIS-SART will require a receiver so that it will operate properly in the AIS to ensure the integrity of the VHF data link. This should not impact the 96 hour battery requirement. We suggest the following change:

–	The AIS Search and Rescue Transponder  (AIS-SART), in addition to meeting the requirements of the relevant ITU-R Recommendation and the general requirements set out in Resolution A.694(17)1, should comply with the following performance standards.

2.2	That section 2.1.6 be modified so that every AIS position report contains the current position. This should not impact the 96 hour battery requirement.

2.3	That an AIS-SART should have a unique identifier. Every device operating on AIS needs a unique identifier to ensure the integrity of the VHF data link.

2.4	That an AIS-SART should be backward compatible to existing Class A and Class B mobile AIS systems on ships.

3	WP 8B is of the opinion that the current preliminary draft performance specifications with the modifications suggested in section 2 are adequate for the AIS-SART to have the same basic purpose as a radar-SART and is looking forward to IMO completing these specifications.

4	WP 8B is of the opinion that there are other future SAR capabilities that could be developed in an AIS-SART by taking advantage of AIS technology and the future AIS shore infrastructure. The possibility of these future capabilities should not delay the completion of the current AIS-SART performance standard. These future capabilities should always take into consideration the need for 96 hour battery life. WP 8B kindly requests IMO to provide comments on the following items that may be technically feasible and could be considered for future study.

4.1	In addition to having the same basic purpose as a radar-SART the AIS-SART could include the same capability as an EPIRB and act as a distress alerting device. When the initial AIS shore infrastructure is in place, a device such as this could operate in sea area A1. The device could provide SAR authorities with the same distress alert information as other GMDSS devices.

4.2	WP 8B is studying long range detection of AIS using terrestrial and satellite techniques as a possibility to satisfy the IMO LRIT (Long Range Identification and Tracking) requirement. This may allow a device such as an AIS-SART to be detected in sea areas A2, A3 and A4.

___________