ANNEX 2

IALA Recommendation R-xxx

On

the Future of DGNSS

##### DRAFT

##### March 2006

IALA Recommendation on the Future of DGNSS

### THE COUNCIL

NOTING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment,

NOTING ALSO IMO Resolutions A.915(22) on Maritime Policy for the Future Global Navigation Satellite System (GNSS) and A.953(23) on the World Wide Radio-navigation System,

NOTING FURTHER ITU-R Recommendation M.823-2 on the Technical Characteristics of differential transmissions for Global Navigation Satellite Systems (GNSS) from maritime DGNSS sites in the frequency band 285-325 kHz (283.5-315 kHz in Region 1),

RECOGNISING the need to ensure that Differential GNSS (DGNSS) services in the frequency band 283.5-325 kHz continue to meet the minimum requirements set out in relevant ITU-R recommendations and IMO resolutions,

RECOGNISING ALSO that the equipment providing the DGNSS service in several members states requires replacement and that the present service will not provide for all the applications described in IMO Resolution A.915(22).

RECOGNISING FURTHER that some maritime applications are not fully met by the existing GNSS services.

HAVING CONSIDERED the proposals made by the e-Navigation Committee:

ADOPTS the strategy proposed for the Future of the IALA DGNSS, set out in the annex to this Recommendation; and

RECOMMENDS National Members and other appropriate Authorities providing or intending to provide DGNSS services in the band 283.5-325 kHz, to implement the strategy set out in the annex to this Recommendation.

ANNEX

### Strategy for the Future of IALA DGNSS

1. General

The IALA DGNSS beacon system was developed in the 1990s and systems were installed in many countries over the period 1995-2000. The system has been adopted as the international, maritime standard for providing differential corrections to Global Navigation Satellite Systems (GNSS). The Radio-navigation (presently e-Navigation) Committee assessed the current and potential use of the system, and concluded that there will be a requirement to recapitalise older systems. There is also potential to develop the system for the benefit of existing users and to enhance GNSS capabilities to take account of technical innovations, in accordance with IMO Resolution A.915 (22).

2. Alternative Technologies

There are potential alternatives to the IALA beacon system for the distribution of safety related differential services, in particular Satellite Based Augmentation Systems (SBAS) and the Automatic Identification System (AIS). SBAS, such as WAAS and EGNOS are now becoming operational and are being promoted for maritime use. The use of AIS as a means of broadcasting differential corrections has also been proposed and demonstrated. There are other ways of providing enhanced services, including Real Time Kinematic (RTK), Eurofix and pseudolites. All these options have been considered and a subjective cost–benefit analysis carried out, a summary of which is presented below.

| System | Accuracy | Coverage | Integrity/Continuity |
| --- | --- | --- | --- |
| IALA DGNSS | 1-3 m | local/regional | Yes/High |
| SBAS | 1-3 m | regional/global | Yes/High |
| AIS# | 1-3 m | local | ?/Moderate |
| Pseudolites | sub-meter | local | Yes/Moderate |
| Eurofix | 1-3 m* | regional | Yes/High |
| RTK | sub-meter | local | ?/Low |

# questions concerning VHF Data Link (VDL) loading have still to be addressed

* assuming Loran system is available

?  – level of integrity depends on implementation technique

Secretarit to change symbols to numbers in this table

| ## System | Provider cost | User cost | Marine Standard |
| --- | --- | --- | --- |
| IALA DGNSS | moderate | low | Yes |
| SBAS | very high | low | No |
| AIS | low | low | Yes |
| Pseudolites | high | moderate | No |
| Eurofix | low* | moderate | No |
| RTK | moderate | high | No |

3. Factors Affecting Future of DGNSS

Taking all the above factors into account, modernisation of the IALA beacon DGNSS is the preferred option for maritime applications, particularly where existing infrastructure can be re-used. However, the time for a decision on its future is approaching. Driving factors include the age of equipment - the original computing and communications equipment has already become obsolete and been replaced in many cases. Some manufacturers of reference station and integrity monitor equipment have withdrawn from this market, although maintenance contracts will still be supported for a few years.

Normal re-capitalisation periods for such equipment would be 10 years. Budgetary requirements in most administrations dictate up to four years lead-time for planning re-equipment.

Developments in GNSS (GPS L2C, L5, and Galileo) will require the introduction of new message types and new equipment.  A concept of operations needs to be developed for the use of multiple frequencies for DGNSS services, to determine whether frequencies should be used separately or the data should be combined.

With the emergence of new technologies, the way in which the original DGNSS system was implemented needs to be re-evaluated. For example, instead of specialised, dedicated hardware units for reference stations and integrity monitors, it may be more cost-effective to consider off-the-shelf receivers with the reference station and integrity monitor functions implemented in software.

Receiver Autonomous Integrity Monitoring, RAIM, is a method that provides GNSS integrity information on the user receiver without external augmentation. RAIM is not continuously available with the present GNSS satellite constellation and is not likely to be fully available until the full implementation of Galileo. The current IMO GNSS receiver performance standards call for fault detection (RAIM). Future standards for receivers using combinations of GNSS will almost certainly contain the same requirements.

4. Applications

IMO Requirements for future GNSS cannot all be met by the existing system, e.g. 1 m accuracy. New approaches could meet those requirements and provide greatly enhanced services, but must preserve backward compatibility. Some new applications that could be served are hydrographic survey, port operations, berthing and Aids-to-Navigation management. The introduction of AIS and new telematic applications on inland waterways increase the requirements for higher integrity and accuracy than is provided for in the present IMO A.915(22).

5. Options for Future Development

If no action is taken the service from older systems will become unreliable, difficult to maintain and will no longer meet international requirements, therefore this is not an acceptable approach. Putting off a decision will make it more difficult and expensive to re-engineer or replace the system when it becomes essential to do so.

Systems could be modernised or replaced using the same architecture, but there are few potential suppliers and this is unlikely to meet future needs. This could lead to a proliferation of additional systems to meet specialised applications.

Development to meet future requirements and changes in the core GNSS would be the strategy most likely to result in a single integrated system meeting a wide range of applications, whilst maintaining backward compatibility and compliance with existing standards.

Discontinuing the service and replacing it with something else (SBAS, AIS) could render existing user equipment obsolete and would require the development of new standards. It is unlikely that this would be acceptable to administrations, providers or users.

6. Recapitalisation Requirements

The baseline requirements and principles for the re-capitalisation of the IALA DGNSS service are as follows:

- Maintenance of legacy signals (backward compatibility)

- Flexibility to support future service requirements

- Internationally applicable solution

- Life-time of at least 10 years

7. Re-engineering Options

Action must be taken to prevent the service becoming unreliable and unable to meet international requirements.

Even if a decision is made to close down the IALA DGNSS system at some point in time, an adequate period of notice is required to allow users to update to alternative technologies. Since this period could be several years, older systems have to be recapitalised whether or not a decision is made to close it down eventually.

Recapitalisation offers the opportunity to provide enhanced performance to serve high integrity and high precision applications (IMO A.915 (22)). Furthermore, the system could be designed to support emerging and upgraded GNSS.

Replacement of existing hardware with similar dedicated Reference Stations and Integrity Monitors (RSIM) would appear to be a low-risk option – the technology is known and procedures are in place. However, the choice of suppliers is limited and once this route is chosen, it will be difficult to develop the system to meet future needs. This could lead to an undesirable proliferation of non-standard systems to meet specialised applications.

There are at least three other options that should be considered: a software RSIM, a Virtual Reference Station and integration with SBAS. These options for re-engineering the IALA DGNSS system are considered here and further investigations proposed to enable the best choice to be made.

7.1 Software RSIM

An alternative already investigated by some administrations is to implement the Reference Station, Integrity Monitor, Communications and Control in

software, using commercial, off-the-shelf GNSS receivers for position and time input. This has the advantage of flexibilityas the hardware would be easy to replace and reconfigure. However, the long-term maintenance of the software would be a concern. This might be partially alleviated by using software modules developed in a widely-used application language. This would also facilitate continuing development of the system to meet changing needs. Comparisons show that this is a cheaper option than like-for-like replacement of existing equipment.

7.2 Virtual Reference Station

Another option is to develop a Virtual Reference Station (VRS) network. In this case the corrections and integrity messages are computed at a central site using data fed from remote receivers. The receivers could be off-the-shelf, although with high specification. The software at the central site could be based on well-known principles and standard algorithms. This would offer performance advantages, since accuracy could be uniform over the coverage area, but the availability of the service would then be dependent on the communication links, which would also introduce extra running costs. Use of the internet and provision of backup systems would need to be considered. The use of data from an existing network such as those implemented by national survey organisations would be a possibility.

7.3 SBAS Integration

Integration with an existing SBAS, such as WAAS or EGNOS could offer a low-cost solution to service providers. This could be achieved by the use of SBAS receivers on each site, or by direct data links to the SBAS control centre.

The service would then be dependent on the SBAS provider so that a Service Level Agreement would be needed and possibly backup systems would have to be considered. Performance would be limited to that provided by the SBAS-accuracy of a few metres at present. There are also limitations in coverage, especially at high latitudes, and there may be time to alarm (TTA) implications from this approach.

There are two ways to use SBAS data. The first is to reprocess the data at the beacon site and transmit legacy beacon messages. In this case the user’s receivers would not have to be changed.

The second way to use SBAS data is to re-broadcast it.In this case the IALA DGNSS stations would become terrestrial alternatives to the geo-stationary satellites. The data capacity of the channel would need to be increased. The hardware of the user’s receiver would have to be changed to accommodate this new transmission, but the receiver’s software would perform the normal WAAS or EGNOS operations to generate an RTCM output.

8. Enhanced Performance

The recapitalisation solution should be able to enhance the level of service. Enhanced performance would include the ability to transmit additional messages, to cater for multiple GNSS and give better than 1 m accuracy, meeting harbour navigation, docking and other specialised requirements.

The capability of providing messages for Galileo, GLONASS M and GPS L2C and L5 would be required, even though these signals will not be fully available for some years.

A Real Time Kinematic solution using carrier-phase corrections would have the most potential for enhanced accuracy, but there are constraints imposed on data rates by the bandwidth of the present datalink. New modulation techniques could be used to broadcast additional messages, which are too long to fit into the existing low rate data stream. A hybrid data-link, capable of accommodating both legacy and new data channels could be employed, for example, with a 50-200 baud low rate channel for legacy services and a new high rate channel capable of providing RTK observables, detailed Troposphere & Ionosphere models, precise orbit data and emergency messaging.

The legacy DGNSS signal typically broadcasts RTCM SC-104 type 9-3 messages regularly to provide timely corrections and to provide integrity to meet the IMO requirement of 10 second time to alarm.  Typically DGNSS sites broadcast Type 9-3 messages for 12 satellites every 8.4 seconds at 100 bps. The repetition rate of these corrections was partially based on the dynamics of selective availability (SA).  With SA removal in 2000, the latency of the signal may be increased to meet the same accuracy standards.  Legacy standards provides for the increase of typical transmission rate to 200 bps. In addition, compression algorithms and alternative modulation schemes may be employed to harvest additional capacity in the data channel. With additional data channel capacity, additional capabilities may be available to the DGNSS system.

Reducing the frequency of type 9 message transmission will affect the system ability to meet the legacy integrity time to alarm (TTA) requirements.   This concern may be mitigated allowing the type 9 messages to have priority to interrupt other broadcasts as necessary to meet TTA requirements.   Additionally, messages may be checked prior to transmission to ensure quality.  This pre-integrity concept may reduce the potential interrupts to near zero. If needed, additional messages or integrity flags in other message headers can be devised to fulfill integrity requirements.

Over the next several years, additional GNSS signals will be available to the public.   These signals include Galileo, GPS L5, GPS L1C, GPS L2C and GLONASS M.  Future DGNSS may augment these as well as GPS L1 to provide a higher level of accuracy and integrity to the user.

There may be merit in broadcasting corrections for the individual GNSS error components instead of only providing a single pseudorange correction encompassing all error components.   Error components such as the tropospheric and ionospheric errors are common to all GNSS systems and may be broadcast independently of components specific to the individual GNSS systems.   Independent error components such as precise orbits and precise time may be broadcast for each GNSS satellite.

With the availability of additional capacity in the DGNSS data channel, type 16 or similar messages may be utilised to broadcast non-positional information to the user community.

Range Rate Corrections (RRC) should be removed as in some conditions the error may become larger by using RRC. With new message types, this could reduce data link loading by 20% thereby reducing data latency and improving position accuracy.

New message types, such as message type 27 for improving station selection, should be implemented.

The integrity of DGNSS can be further improved by comparison with other augmentation services.

9. DGNSS Network Techniques

Various techniques are under consideration for the networking of DGNSS sites. In the Regional Area Augmentation System (RAAS) mode the radio-beacon would be left unchanged, but a new type of receiver would be required by users. This would be compatible with the existing standard, but able to take advantage of RAAS-mode operation by virtue of its multi-channel reception capability and RAAS software.

In transmitted RAAS each DGNSS station would acquire the messages transmitted by its neighbours, by means of a receiver, and process all messages as a combined solution. It would transmit the combined solution using an extended version of the present broadcast standard. This should be well within the data capacity of the present system. The hardware of the user’s differential receiver would be unchanged, but its software would generate an enhanced RTCM output to the associated GNSS receiver, using the combined set of messages.

In addition to the traditional MF data link, alternative data channels may be considered to transmit correction data as well as additional high rate information such as GNSS observables.  Alternative data transmission methods include internet gateways and broadcasting over other RF bands that can carry other DGNSS data.

10. Further Work

Further investigations are required before proposals for the enhanced future system can be put forward for international consideration, so that specifications can be drawn up. In particular, the cost and feasibility of the re-engineering options must be investigated. The advantages and disadvantages of the DGNSS networks need to be explored, the new modulation techniques require evaluation and new message types have to be agreed and standardised.

11. International Standards

Any solution proposed must meet international requirements, since the present service is the maritime standard. The technical characteristics of the system and any changes to spectrum allocations must be agreed by ITU; message formats are generally developed and approved first in RTCM and then the ITU Recommendation is revised accordingly. The operational requirement and receiver performance standards must be approved by IMO.

System performance standards and operating procedures are dealt with by IALA and IEC produces receiver specifications. A lead-time of several years must be allowed for the regulatory process if any substantial change is planned to the existing system.

DGNSS is not yet recognised as a mandatory carriage requirement under the IMO SOLAS Convention. Also, most countries have not submitted their DGNSS as a component of the World Wide Radio-navigation System. These issues should be addressed in parallel with any re-engineering of the system.

As long as international standards are met, the exact method of implementation remains a decision for the individual administration, although there may be advantages in sharing technical developments.

12. Recommended Strategy

It is recommended that

- IALA members should submit their DGNSS as a component of the World Wide Radio-navigation System.

- IALA members providing DGNSS services should plan the re-engineering of older services to enhance performance and meet future maritime requirements.

- Alternative development routes for IALA DGNSS e.g. software RSIM, VRS and SBAS integration, should be investigated in detail and costed.

- Whichever option is chosen it should meet the minimum requirements set out in Section 6 and conform to the standards mentioned in Section 11.

It is further recommended that concurrent studies be conducted in the following subjects:

- Specify the intended performance levels (accuracy, availability, integrity,  continuity, coverage).

- Determine the data link capacity required for the enhanced DGNSS service.

- Determine the merit of pre-integrity, identify the rules for integrity interrupt and develop any additional integrity messages or RTCM integrity message headers that may be required.

- Develop the basic message types and transmission schemes for independent error components to provide for new and existing GNSS signals.

- Investigate alternative data channels and alternative uses of the DGNSS broadcast channel.

- Investigate the probable need to provide additional messages relating to Galileo, GPS L2C/L5 and Glonass M.

- Investigate the feasibility of providing additional signals giving sub-metre accuracy.

- Explore the advantages of networking stations.

- Investigate the requirement for additional integrity messages or message header flags.

13. References

  - International Maritime Organization. “World-Wide Radionavigation System”, IMO Resolution A.953(23)

  - International Maritime Organization. “Maritime Policy for a Future Global Navigation Satellite System (GNSS) IMO Resolution A.915(22)

  - International Telecommunication Union – Radiocommunication Sector (ITU-R), “Technical Characteristics of Differential Transmissions For Global Navigation Satellite Systems (GNSS) from Maritime Radio DGNSS sites in the frequency Band 283.5-315 kHz in Region 1 and 283.5-325 kHz in Regions 2 and 3”, Recommendation M.823-2, Geneva 1997

  - IALA Recommendation R121 on the performance and monitoring of DGNSS services in the frequency band 283.5-325 kHz

  - U.S. Presidential Decision Directive NSTC- t b completed  (Selective Availability).

  - Ward, N, IALA;  “The Future of IALA DGNSS”  Proceedings of the Institute of Navigation, NTM-2006, Jan 2006.

  - Gutman, Seth; Wagoner, Sher; Mihal Codrescu, Mihal; and Fuller-Rowell, Tim;  National Oceanographic and Atmospheric Administration; The Use of Atmospheric Models to Improve Differential GPS Positioning Accuracy.

  - Last, David; Grant, Alan; Williams Alwyn; Ward, Nick; Enhanced accuracy by regional operation of Europe’s new radiobeacon differential system.  Proceedings of the Institute of Navigation, ION-2002, Sept 2002.

  - Radio Technical Commission for Maritime Services, RTCM Special Committee No. 104, RTCM Recommended Standards for Differential Navstar GPS Service, V2.3, RTCM Paper 136-2001/SC104-STD, August 20, 2001.

  - Radio Technical Commission for Maritime Services, RTCM Special Committee No. 104, RTCM Recommended Standards for Differential NAVSTAR GPS Reference Stations and Integrity Monitors (RSIM), V1.1, RTCM Paper 12-2001/SC104-248, May, 2001.

  - Kee, Changdon; and Kim Jeonghan; Efficient Transmission Technique of Compact RTK Correction Messages for Low-rate RTK Data-link. Proceedings of the Institute of Navigation, ION-NTM 2003, January 2003

  - Hartnett, Richard; Gross, Keith; Bovee, Ben; Nassar, Sam; Johnson, Gregory; Sanders, George; Swaszek, Peter; Wolfe, David; DGPS Accuracy Observations and Potential Data Channel Improvements. Proceedings of the Institute of Navigation, ION-AM 2003, June 2003.

  - Park, B., Kim, J., Jun, S., Kee, C., Kalafus. R.; The Need for Range Rate Corrections in DGPS Correction Messages, , Proceedings of the Institute of Navigation, ION-NTM 2004, Jan 2004.

  - Bartone, C., Barton, I., Zhang, Y., Cleveland, A., Parsons, M.; Antenna Performance Analysis for a High Performance NDGPS Architecture, Proceedings of the Institute of Navigation, ION-NTM 2004, Jan 2004.

  - Ferguson, K., Albright, M., Remondi, B., Cleveland, A., Parsons, M.; NDGPS Reference Station and Integrity Monitor Architecture Modernization, Proceedings of the Institute of Navigation, ION-AM, June 2004.

  - Dr. Remondi, Benjamin, Ferguson, Kendall, Albright, Michael; HA-NDGPS RTK Positioning as a Function of Baud Rate 50 – 1000 bps, Proceedings of the Institute of Navigation, ION-AM, June 2004.