| From: IALA e-Navigation Committee (e-NAV1) | e-NAV1-output-08 |
| --- | --- |
| To: IEC TC 80 WG14and ANM Committee | September 20, 2006 |

Liaison Note

AIS AtoN Stations / Message 21-use of regional bits1

Moved text of foot note up to bottom of fisrt page

The IALA e-NAV Committee WG 4 (AIS Technical Working Group) have considered the paper submitted by IEC WG14-13 and agree with the proposal for paging using the 8 binary bits specified for regional/local applications in Message 21. The use of page 7 (binary 111) as a common international standard for providing the data necessary to display the light and racon status onboard vessels and ashore was agreed, with the proviso that the health bit is only for use by the local competent authority, and should not be used or displayed on board vessels.

The rationale for this statement is that different regional authorities may choose to use the health flag in different ways. Where an authority has implemented the use of  the additional pages in Message 21 , or Addressed Binary Message 6, the health flag may be an indication that there is a fault in one of the local AtoN systems. The health flag may also be set if there is an internal BIIT failure in the AIS AtoN station itself. Both of the above conditions will require further action by the authority before deciding whether the mariner should be alerted.

The important point from IALAs perspective is that there is now, for the first time, a common standard which will allow the display of AtoN status information directly on the vessels bridge.

| From: IALA-IEC WG14-13 | WG14-13-output-03 |
| --- | --- |
| To: IALA e-Navigation Committee (e-NAV1) / RDNAH (use as basis for drafting letter from Chairman) | August 25, 2006 |

Liaison Note

AIS AtoN Stations / Message 21-use of regional bits

# Introduction

In developing the draft testing standard 62320-1, the IALA-IEC WG14 discussed Message 21 and the use of regional bits.  The group also reviewed input provided by the Royal Danish Administration of Navigation and Hydrography (RDNAH), WG14-9/09 (IALA reference AIS17/12/info3), regarding the use of AIS on AtoN.

# Message 21 – Regional Bits

As indicated in IALA Recommendation A-126 in Message 21, 8 bits are reserved for Regional Use.  The diagram below represents the recommendation of WG14 for use of these bits.

Based on the input provided by the RDNAH, WG14 suggests that these bits shall be employed as follows:

- The first three bits shall be use to define a Page ID. The Page ID can range from 0 to 7, allowing 8 pages. The first page was not used for the Regional/International application because it is defined as the default “not used” condition in Recommendation ITU-R M.1371. Page 7 (binary 111) is defined in this liaison note. Pages 0 to 6 are reserved for future use. The future use is envisaged as being for monitoring of AtoN parameters such as voltages, currents, temperatures, etc.

Page 7 shall be implemented in all types of AIS AtoN Stations. The final 5 data bits are defined as in the diagram above.

Explanatory notes are:

  - Manufacturer’s default setting for the Regional Bits of Message 21 shall be all zeros.

  - Health flag alarm shall be set to 1 to indicate a fault in or failure of the AtoN system or AIS AtoN station, at this location. Further indication of the fault or failure detail can be achieved by use of additional pages within the Regional Bits, or addressed binary Message 6.

Note: By using only page 7 we do not need to toggle through the messages, only message id 7 has to be read thus allowing an immediate filtering.

## Justification

Message 21 carries mainly navigation information.

One bit is used for alerting the competent authority that there is a problem at the AIS AtoN Station. This allows a competent authority to avoid using M6, if there is pressure on VDL slots, while still receiving some monitoring information every time Message 21 is sent by the AIS AtoN Station.

WG 14 considers the use of the remaining pages and their data fields should be the subject of consideration and decision by IALA. Keeping the pages not committed allows a Competent Authority to choose their own requirements. For example, a Competent Authority may desire to measure battery voltage and a current and this could require 4 pages. Also some Competent Authorities may not be interested in measuring acceleration at an AtoN site as proposed by the RDNAH paper for Message 21 pages 2 and 3.

Also manufacturers may not wish to implement a complex Regional Bit protocol in their AIS AtoN Stations.

# Requested Action

The e-NAV Committee is invited to review the proposal on the use of Message 21 – Regional Bits.  Specifically, the e-NAV Committee is requested to review the use of the remaining pages, as per the input provided to both IALA(AIS17/12/info3) and IEC (this liaison note) and provide a decision on the implementation of this proposal as input to WG14-14 (Oct 16-20, 2006).

Comments should be forwarded to the Chairman of WG14, A. Stewart at alans@nlb.org.uk and the Convenor for WG14, J. Carson-Jackson at jillian.carson@gmail.com

These 8 bits are called “AtoN status” in ITU-R M.1371-3