| GAP ANALYSIS AND PRACTICAL E-NAVIGATION SOLUTIONS WITH HUMAN ELEMENT CONSIDERING HEAP e-NAV10/7/7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| USER FIELD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CATEGORY OF GAP ANALYSIS (COMSAR 15/11 Section 36) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Sub-category of gap analysis (COMSAR 15/11 Section 36) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | ◦ Related User Needs (NAV 56/WP.5/Rev.1 Annex 2,3,4) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Aspects | Identifier | Identified Gaps (COMSAR 15/11 Annex 1/2/3) (IALA e-NAV9-output-15C) (COMSAR 15/WP.6/Rev.1 Annex 3) (T/E) for Human element (ref. HEAP e-nav table "T" & "E") | Relationship to the strategy |  | Related Functions (NAV 56/WP.5/Rev.1, Annex 1, p. 11-30) | Existing equipment, systems, technologies | Operation Area | Proposed practical e-navigation solutions to address identified gaps |  |  |  |
|  |  |  |  |  |  |  | key element | core objective |  |  |  | Operational ◦ procedural / automation ◦ Human Element (ref. HEAP e-nav table "R") | Technical ◦ H/W , S/W, equipment, links, etc. ◦ Human Element (ref. HEAP e-nav table "R") | Regulatory ◦ regulation, standard ◦ Human Element (ref. HEAP e-nav table "R") | Training ◦ human element (ref. HEAP e-nav table "R") |
| (1)SHIPBOARD USER | {1}INFORMATION / DATA MANAGEMENT | [1]Common Data Structure/Harmonized Data Formats | ◦ User-Selectable Presentation of Information Received via Communication Equipment ◦ Maritime Safety Information(MSI) ◦ Standardized and Automated Reporting ◦ Reduction of Administrative Burden and Increase use of Electronic Documentation ◦ Automated Updating of Base Line Data and Documents ◦ Alert Management |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 111-Gte01 | Lack of harmonized data formats for the transfer of information received via communication equipment (e.g., Maritime Safety Information, MSI) to the navigational systems for presentation. (T) T1-T2-T3-T4-T11-T13-T22-T32 (E) E21-E33 | 1,3,5,7 | 3,5,7,8,10 | A2, A11 | n/a | 1,2,3,4,5 | ◦ (111-Gte01-Sop01) Automatic transfer of information received via communication equipment into the navigational systems for presentation (HE) OPR1-OPR2-OPR4 | ◦ (111-Gte01-Ste01) New IEC 61162 sentence should be developed ◦ (111-Gte01-Ste02) S/W modification of communication and navigation equipment (HE) TCH1 | ◦ (111-Gte01-Sre01) Revision of IEC 61162 standard (HE) REG1 | ◦ (111-Gte01-Str01) Revision of training course to operate comm. and nav. equipment involved within transfer of information (HE) TRN4 |
|  |  |  |  |  | 111-Gte02 | There are no standardized data formats established for ship reporting. (T) T5-T22-T31-T32 (E) E21-E31-E33 | 1,3,5,7 | 3,5,7,8,10 | A16 | n/a | 1,2,3,4,5 | ◦ (111-Gte02-Sop01) Semi-automatic transmission of ship reporting with user input and status of ship (HE) OPR1-OPR2-OPR4-OPR7 | ◦ (111-Gte02-Pte01)New data model for ship reporting based on CMDS (HE) TCH1 | ◦ (111-Gte02-Sre01) Regulation of data structure and model within SOLAS based on CMDS and Data Model ◦ (111-Gte02-Sre02) Technical standard and guidelines for ship reporting data structure recognized internationally (HE) REG1 | ◦ (111-Gte02-Str01) Develop training course for automated procedure of ship reporting (HE) TRN4 |
|  |  |  |  |  | 111-Gte03 | Lack of harmonized data formats for support of data entry for documentation by means of information requested from other systems. (T) T11-T22 (E) E33 | 1,3,5,7 | 3,5,7,8,10 | A17 | n/a | 1,2,3,4,5 | ◦ (111-Gte03-Sop01) Semi-automatic data entry for documentation(need TBD) using structured data format ◦ (111-Gte03-Sop02) Make a required document by selecting any required contents from the structured data format and contents (HE) OPR1-OPR2-OPR4 | ◦ (111-Gte03-Ste01) Definition of kinds of documentation and contents or parameter of each one. ◦ (111-Gte03-Ste02) Develop the method of entry(e.g.. Keyboard or specific button, etc.) ◦ (111-Gte03-Ste03) New s/w and h/w for data entry on new or exist equipment (HE) TCH1 | ◦ (111-Gte03-Sre01) Regulation of the range of data entry activity and item which could be used as a resource of ship reporting or carriage-required publications. ◦ (111-Gte03-Sre02) Technical guidelines or standard for structured data format for entry (HE) REG1 | ◦ (111-Gte03-Str01) Develop training course for procedure of data entry using harmonized data format and related equipment (HE) TRN4 |
|  |  |  |  |  | 111-Gte04 | Lack of standard formats for data exchange with regard to updating of data and documents, chart and voyage planning publications. (T) T11-T22-T23-T24 (E) E4-E14-E33-E34 | 1,3,5,7 | 3,5,7,8,10 | A1.10 | ENC update | 1,2,3,4,5 | ◦ (111-Gte04-Sop01) Automatic update of information using standard formats for data exchange with possible human intervention, if necessary (HE) OPR2 | ◦ (111-Gte04-Ste01) Develop the digital version of paper publications based on S-10X ◦ (111-Gte04-Ste02) Develop the update data structure also based on S-10X ◦ (111-Gte04-Ste03) Communication links should be connected to User computer onboard to receive update data ◦ (111-Gte04-Ste04) New S/W to manage digital publication and update data for it (HE) TCH1 | ◦ (111-Gte04-Sre01) Develop new regulation of digital publication and update for satisfying the carriage requirements ◦ (111-Gte04-Sre02) Develop New standard, S-10X for digital publication onboard (HE) REG1 | ◦ (111-Gte04-Str01) Familiarization for using digital publication on User's computer (HE) TRN4 |
|  |  |  |  |  | 111-Gte05 | Lack of technical solutions for processing, filtering of information exchanged via communication equipment. (T) T22-T31-T32-T33 (E) E21-E22-E23 | 1,5 | 3,8 | n/a | ◦ Inmarsat ◦ VHF, ◦ MF, ◦ HF ◦ Other communications carriers |  | ◦ (111-Gte05-Sop01) Received information to be decoded and adjusted for presentation in “clear text” to the mariner. (HE) OPR1-OPR2-OPR3-OPR4 | ◦ (111-Gte05-Ste01) S-100 : Filtering performed through effective software and the possibility of presentation (monitors) (HE) TCH1 | ◦ (111-Gte05-Sre01) Develop filtering procedures (regulations). (HE) REG1 | ◦ (111-Gte05-Str01) Mariner to be trained For proper filtering of information. ◦ (111-Gte05-Str02) Some information should not be “filtered away” (HE) TRN4 |
|  |  |  |  | Regulatory | 111-Gre01 | Lack of international standards for navigation data formats. Lack of real-time environmental information (current, tide, weather) received automatically. Lack of international standards for navigation data formats and water level and current information as well as a standard for dates. (T) T1-T2-T3-T4-T22 (E) E33-E34 | 1,3,5,7 | 3,5,7,8,10 | n/a | IEC 61162 sentences S-57 | 1,2,3,4,5 | ◦ (111-Gre01-Sop01) Automatically produce and receive data between ship and shore or between systems | ◦ (111-Gre01-Ste01) Develop the definition of navigation data, environmental data and their format ◦ (111-Gre01-Ste02) Automatic gathering the information to produce data ◦ (111-Gre01-Ste03) Automatic communication between systems | ◦ (111-Gre01-Sre01) Develop new S-10X standard to include the navigation data | ◦ (111-Gre01-Str01) Equipment manufacturer should be familiarized to the New standard ◦ (111-Gre01-Sre02) Revise the existing course to be familiarized the new data and its format |
|  |  |  |  |  | 111-Gre02 | Lack of interface standards for status of equipment. (T) T11-T12-T13-T14-T22 (E) E32-E33-E34 | 1,3,5,7 | 3,5,7,8,10 | A1 | IEC 61162 sentences | 1,2,3,4,5 | ◦ (111-Gre02-Sop01) Automatically self-check the status of equipment and distribute it for quality check (HE) OPR2-OPR4 | ◦ (111-Gre02-Ste01) Define the status of each equipment based on ensuring seaworthiness. ◦ (111-Gre02-Ste01) Revise or Develop newly IEC 61162 sentences (HE) TCH1 | ◦ (111-Gre02-Sre01) Revise Performance standard of each equipment for status information ◦ (111-Gre02-Sre02) Revision of IEC 61162 standard (HE) REG1 | ◦ (111-Gre02-Str01) Familiarization to status information of each equipment ◦ (111-Gre02-Str02) Training to respond the equipment status if necessary. (HE) TRN4 |
|  |  |  |  | Operational |  | No identified Gap |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Training | 111-Gtr01 | Those working with information management systems need to be competent and need to stay current (T) T22 (E) E32-E33-E34 | 1,3,5,7 | 3,5,7,8,10 | n/a | n/a | 1,2,3,4,5 | ◦ n/a | ◦ (111-Gtr01-Ste01) Define the information management system onboard (HE) TCH1 | ◦ (111-Gtr01-Sre01) Develop new certification system to approve competency (HE) REG1 | ◦ (111-Gtr01-Str01) Additional training course to be familiarized and competent with the information management system (HE) TRN4-TRN5 |
| (1)SHIPBOARD USER | {1}INFORMATION / DATA MANAGEMENT | [2]Improved Reliability and Indication of Reliability | ◦ Indication of Reliability ◦ Improved Reliability |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 112-Gte01 | Lack of effective and harmonized means for assessment of the accuracy and plausibility of indicated information. Lack of effective ways to indicate levels of reliability. (T) T11-T12-T13-T14-T22 (E) E32-E33-E34 | 1,3,7 | 6 | n/a | INS GNSS(GPS) | 1,2,3,4,5 | ◦ (112-Gte01-Sop01) System assess automatically the accuracy, plausibility and reliability of information and provide the results on user demand or give alerts in case of exceeding threshold. (HE) OPR1-OPR2-OPR3-OPR4-OPR5 | ◦ (112-Gte01-Ste01) Develop the appropriate method to assess in terms of accuracy, quality, plausibility and reliability for each e-navigation information. ◦ (112-Gte01-Ste01) Develop graphical or numerical presentation based on statistics or quantifying algorithms of the assessment way (HE) TCH1 | ◦ (112-Gte01-Sre01) Develop or revise the performance standard or regulation of each equipment for its own information ◦ (112-Gte01-Sre01) INS performance standard should be revised (HE) REG2 | ◦ (112-Gte01-Str01) Revise the training course to be familiarized with the way of indication of assessed results on the display (HE) TRN5 |
|  |  |  |  | Regulatory | 112-Gre01 | Lack of standardized regulations for determination (standardized algorithms) of integrity, quality, reliability assessment, and reliability thresholds. |  |  |  |  |  | ◦ n/a | ◦ n/a | ◦ see (112-Gte01-Sre01) (HE) REG2 | ◦ n/a |
|  |  |  |  | Operational | 112-Gop01 | Lack of assessments to quantify reliability parameters (e.g., specific assessment of electronic position fixing systems). |  |  |  |  |  | ◦ n/a | ◦ see (112-Gte01-Ste01) (HE) TCH1 | ◦ n/a | ◦ n/a |
|  |  |  |  | Training | 112-Gtr01 | Improved competence of installation and repair person for providing better reliability of systems and equipment (T) T12-T13 | 1,2,3,7 | 6,9 | n/a | n/a | n/a | ◦ n/a | ◦ n/a | ◦ (112-Gtr01-Sre01) Develop the certification system for competency of maintenance person such like GMDSS maintenance work (HE) REG2 | ◦ (112-Gtr01-Str01) Installation and repair person should prove their knowledge to be assured their competency for system reliability (HE) TRN5 |
|  |  |  |  |  | 112-Gtr02 | Training for standardized status information is necessary. |  |  |  |  |  | ◦ n/a | ◦ n/a | ◦ n/a | ◦ (112-Gtr02-Str01) Revise the training course to be familiarized with the status information in a view of system reliability check (HE) TRN5 |
| (1)SHIPBOARD USER | {1}INFORMATION / DATA MANAGEMENT | [3]Nautical Charts and Publications | ◦ Automated Updating of Base Line Data and Documents ◦ Effective and Robust Communications |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 113-Gte01 | ENC coverage is inadequate. (T) T22-T23-T24-T25 (E) E14-E32-E33-E34 | 1,3,6 | 1,3,5,8,10 | A1.10, A2.1 | ENC, ECDIS | 1,2,3,4,5 | ◦ (113-Gte01-Sop01) Automatic alarm to recall mariner's recognition ◦ (113-Gte01-Sop02) Automatic bridge operation mode transition into prepared operation without ENC, currently RNC with paper chart (HE) OPR1-OPR2-OPR3 | ◦ (113-Gte01-Ste01) Alarm to recall mariner's recognition for the navigation without ENC ◦ (113-Gte01-Ste01) Newly develop Shore-based service(ex. Remote navigation) to help ship's navigation safety for same level of ENC navigation (HE) TCH1-TCH2 | ◦ (113-Gte01-Sre01) IHO/IMO encourage the competence of ENC availability ◦ (113-Gte01-Sre01) New regulation to assure the navigation safety in no-ENC area in sense of bridge operation mode and shore-based services (HE) REG2-REG4 | ◦ (113-Gte01-Str01) Operation without ENC should not cause excessive need of training ◦ (113-Gte01-Str01) Absence of ENC should not cause more workload and stress of mariner (HE) TRN5 |
|  |  |  |  |  | 113-Gte02 | Lack of timely delivery of ENCs and updates via internet. Lack of real-time tidal data, AIS data and ENC updates. (T) T11-T22-T25 (E) E14-E33-E34 | 1,6,7 | 1,5,8,10 | A1.10, A2.1 | ECDIS, AIS, GMDSS | 1,2,5 | ◦ (113-Gte02-Sop01) Automatic update of ENC for voyage plan on user demand ◦ (113-Gte02-Sop02) Automatically display tidal-data on ECDIS with real-time via communication equipment (HE) OPR1-OPR2-OPR3 | ◦ (113-Gte02-Ste01) AIS message revision for ENC updates ◦ (113-Gte02-Ste02) GMDSS modernization to enhance data transmission ◦ (113-Gte02-Ste03) Implementation and presentation of CMDS for real-time data such like tidal information ◦ (113-Gte02-Ste04) regional or coastal communication channel such like WiMAX, WCDMA etc. (HE) TCH1 | ◦ (113-Gte02-Sre01) Develop the global regulation for ENC update procedure and communication channel (HE) REG2 | ◦ (113-Gte02-Str01) Revise the training course for ECDIS and ENC update ◦ (113-Gte02-Str02) Revise the training course to be familiarized to display of ECDIS including real-time data (HE) TRN4-TRN5 |
|  |  |  |  |  | 113-Gte03 | Lack of digitized nautical publications (Pilots, tide tables, light list, etc.). (T) T11-T22-T25 (E) E33-E34 | 1,2,3,7 | 1,5,8,9,10 | A1.3,A1.10, A2.2 | Nautical publications in paper form | 1,2,3,4,5 | ◦ (113-Gte03-Sop01) Using digital publications on user computer onboard or supplementary display of navigational system (HE) OPR2-OPR3 | ◦ (113-Gte03-Ste01) Develop digital format for the publication based on S-100 ◦ (113-Gte03-Ste02) Develop the efficient way to use digital publication including update mechanism ◦ (113-Gte03-Ste03) Develop the S/W to use digital publication on onboard computer (HE) TCH1 | ◦ (113-Gte03-Sre01) Approve the digital publication as a fulfillment of carriage requirement and revise the Convention or standard ◦ (113-Gte03-Sre01) Develop new data structure for digital publication and Adopt officially (HE) REG2 | ◦ (113-Gte03-Str01) Revise the training course for general skill on user computer to use the S/W for digital publication (HE) TRN4 |
|  |  |  |  | Regulatory | 113-Gre01 | Lack of standardized symbology. (T) T11-T22-T25 (E) E33-E34 | 1,2,3,7 | 1,2,5,7,8,9,10,11 | A6 | ECDIS | 1,2,3,4,5 | ◦ n/a | ◦ (113-Gre01-Ste01) More symbolic presentation for clear and direct perception of mariner ◦ (113-Gre01-Ste02) Symbol design by ergonomics (HE) TCH1 | ◦ (113-Gre01-Sre01) Develop holistic standard for symbol presentation compliant with the information contents based on ergonomics (HE) REG2 | ◦ (113-Gre01-Str01) Develop the training course to be familiarized new symbolic presentation environment (HE) TRN4 |
|  |  |  |  |  | 113-Gre02 | Lack of definition of parameters for ENC liability. (T) T11-T22-T25 (E) E33-E34 | 1,3,6 | 6,10,11 | n/a | ENC, ECS, ECDIS | 1,2,3,4,5 | ◦ n/a | ◦ (113-Gre02-Ste01) Revise ENC structure(S-57 or S-10X) for new parameter ◦ Revise ECDIS S/W and ECS S/W regarding the liability parameter (HE) TCH1 | ◦ (113-Gre02-Sre01) Define the concept of ENC liability for potential value-added services ◦ (113-Gre02-Sre02) IHO revise S-57 and develop S-10X with considering ◦ (113-Gre02-Sre03) Develop the concept of ENC scalability on non-SOLAS vessels ◦ (113-Gre02-Sre04) Revise the performance standard of ECDIS of IMO ◦ (113-Gre02-Sre05) Revise the standard of ECS of ISO ◦ (113-Gre02-Sre06) Revise the standard of ECDIS and ECS of IEC (HE) REG2-REG4 | ◦ (113-Gre02-Str01) Revise the training course of ECDIS to be familiarized the presentation contents representing the liability information (HE) TRN4 |
|  |  |  |  |  | 113-Gre03 | Lack of free available electronic charts. Unnecessary complexity introduced by encryption of electronic charts. (T) T11-T22-T25 (E) E33-E34 | 6 | 1,10 | n/a | ENC, ECDIS | 1,2,3,4,5 | ◦ (113-Gre03-Sop01) User can use free ENCs as a minimum to ensure ship's safety without encryption(S-63) (HE) OPR2-OPR3 | ◦ (113-Gre03-Ste01) Any distribution and updating system should be provided through the publicly open method (HE) TCH1 | ◦ (113-Gre03-Sre01) Newly revise the ENC concept and responsibility of production in the level of Convention (HE) REG4 | ◦ n/a |
|  |  |  |  | Operational | 113-Gop01 | Current design of electronic charts and publications does not provide enough "decisional" support as opposed to "informational" support. (T) T11-T22-T25 (E) E33-E34 | 1,3,5,6 | 8 | n/a | ENC, ECDIS | 1,2,3,4,5 | ◦ (113-Gop01-Sop01) ENC/ECDIS including nautical publications can provide the decisional support to the mariner (HE) OPR1-OPR2 | ◦ (113-Gop01-Ste01) Revise ENC structure(S-57 or S-10X) for new function ◦ (113-Gop01-Ste02) Revise ECDIS S/W regarding the function and presentation of decisional support ◦ (113-Gop01-Ste03) Develop new function on ECDIS for decisional support (HE) TCH1 | ◦ (113-Gop01-Sre01) IHO/IMO develop the concept of decisional support of ENC and publications ◦ (113-Gop01-Sre02) Revise the S-57 and Develop S-10X for new concept ◦ (113-Gop01-Sre03) Revise the related regulation of IMO (HE) REG2 | ◦ (113-Gop01-Str01) Revise the training course of ECDIS to be familiarized the concept and function of decisional support (HE) TRN5 |
|  |  |  |  | Training | 113-Gtr01 | Familiarization to presentation and context of information such as metadata and all ancillary data to charts (T) T11-T22-T25 (E) E33-E34 | 2,3 | 9 | n/a | ENC, ECDIS | 1,2,3,4,5 | ◦ (113-Gtr01-Sop01) User can select on-line help or training material on the systems to be familiarized (HE) OPR1-OPR2 | ◦ (113-Gtr01-Ste01) Develop the on-line help or training function in S/W (HE) TCH1 | ◦ (113-Gtr01-Sre01) Revise the related standards or regulations for on-line help function (HE) REG4 | ◦ (113-Gtr01-Str01) Revise the training course of appropriate system to be familiarized the presentation and context of information ◦ (113-Gtr01-Str02) Revise the training course to help usage of on-line help function (HE) TRN4-TRN5 |
| (1)SHIPBOARD USER | {2}EFFECTIVE AND ROBUST VOICE COMMUNICATION AND DATA TRANSFER | n/a | ◦ Effective and Robust Communications |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 120-Gte01 | Insufficient reliability of data communications | 1,5 | 3,6 | n/a | Inmarsat, Other satellite systems | 1,2,3,4,5 | ◦ (120-Gte01-Sop01) Data communications to be standardized. ◦ (120-Gte01-Sop02) Information to be presented to the mariner in a “standard mode”, independent of communication service (i.e. satellite, terrestrial etc). ◦ (120-Gte01-Sop03) Communications should provide the reliability without human intervention ◦ (120-Gte01-Sop04) Automatic warning for reliability failure of communication system (HE) OPR1-OPR2-OPR4 | ◦ (120-Gte01-Ste01) Near coastal water data communication may be performed by means of VHF-data (to be developed and standardized). In areas beyond the coverage of existing satellite systems, HF-data should be developed covering the most necessary functions ◦ (120-Gte01-Ste02) Automatic check or verifying procedure of reliability in system S/W(should be more than CRC checksum) (HE) TCH1 | ◦ (120-Gte01-Sre01) Requirements to be developed and if necessary, priority access ◦ (120-Gte01-Sre02) Develop the definition and level of reliability of communication ◦ (120-Gte01-Sre03) Revise the regulation and standards of communication systems as the definition and level of reliability ◦ (120-Gte01-Sre04) Develop the general concept and statement to ensure reliability of marine communication channel in Convention(IMO) or any other international standard(ITU, ISO) (HE) REG2 | ◦ (120-Gte01-Str01) Operator must be trained for operation of data systems (HE) TRN1 |
|  |  |  |  |  |  | Lack of reliability standards for communication technology. (T) T13-T22-T32 (E) E21-E22-E23 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 120-Gte02 | Possible lack of bandwidth and unclear assignment of adequate bandwidth for identified e-navigation communication needs. (T) T13-T22-T32 (E) E21-E22-E23 | 1,3,5 | 3 | n/a | ◦ Inmarsat ◦ VHF ◦ MF ◦ HF | 1,2,3,4,5 | ◦ n/a | ◦ (120-Gte02-Ste01) Changing of bandwidth of VHF channels from 25 kHz to 12,5 kHz may make available channels for e-navigation in the VHF-system.. ◦ (120-Gte02-Ste02) Satellite communication to be adjusted. Consider revision of the HF bands. (HE) TCH1 | ◦ (120-Gte02-Sre01) ITU, ◦ (120-Gte02-Sre02) Liaison statement from IMO to ITU ◦ (120-Gte02-Sre03) Identify communication needs in detail in aspects of bandwidth(IMO) ◦ (120-Gte02-Sre04) Consider bandwidth allocation to fulfill the needs(ITU) (HE) REG2 | ◦ n/a |
|  |  |  |  |  | 120-Gte03 | Lack of systems for source and channel management for communication equipment. Lack of seamless and communication mean dependent protocol for exchanging navigation information between ships Insufficient techniques and procedures for exchange of data between ship shore and on board. (T) T13-T22-T32 (E) E21-E22-E23 | 1,3,5,7 | 3,6 | n/a | ◦ Inmarsat ◦ VHF ◦ MF ◦ HF AIS, GMDSS | 1,2,3,4,5 | ◦ (120-Gte03-Sop01) Automatic selection of source and channels. ◦ (120-Gte03-Sop02) Automatic channel selection in seamless 4S(ship to ship, ship to shore) communication system globally (HE) OPR1-OPR2-OPR3-OPR4 | ◦ (120-Gte03-Ste01) In narrow sense, automatic VHF/MF/HF channel management function ◦ (120-Gte03-Ste02) In wide sense, automatic 4S system channel management function (HE) TCH1 | ◦ (120-Gte03-Sre01) ITU, ◦ (120-Gte03-Sre02) IMO ◦ (120-Gte03-Sre03) Revise the performance standard of GMDSS equipment ◦ (120-Gte03-Sre04) New standard for 4S communication system (HE) REG2 | ◦ (120-Gte03-Str01) Develop the training course of new system (HE) TRN4-TRN5 |
|  |  |  |  |  | 120-Gte04 | Signal security, system security, input security as well as management of access/protocols are insufficient. (T) T6 | 1,5,7 | 1,3 | n/a | n/a | 1,2,3,4,5 | ◦ (120-Gte04-Sop01) Identification procedure to input important data(ex. ID/PW) (HE) OPR3 | ◦ (120-Gte04-Ste01) Modernization of GMDSS into the digital communication ◦ (120-Gte04-Ste02) Advanced communication technology(ex. CDMA, WiMAX) to protect the signal and contents of medium (HE) TCH1 | ◦ (120-Gte04-Sre01) Revision of Convention, performance standard and regulation of GMDSS and Communication equipment by IMO, ITU, IEC ◦ (120-Gte04-Sre02) Global regulation to use new technology by IMO and ITU (HE) REG2 | ◦ (120-Gte04-Str01) Revision of training course to operate identification procedure of the system as appropriate. (HE) TRN4-TRN5 |
|  |  |  |  |  | 120-Gte05 | Lack of integrated GMDSS equipment. (T) T11-T22 (E) E21-E22-E23 | 1,3,5,7 | 3,6,8 | n/a | GMDSS | 1,2,3,4,5 | ◦ (120-Gte05-Sop01) User have a man machine interface to operate all GMDSS equipment (HE) OPR1-OPR2-OPR4 | ◦ (120-Gte05-Ste01) Develop one user interface with automatic selection of communication way (HE) TCH1-TCH2 | ◦ (120-Gte05-Sre01) New standard for GMDSS system as integrated one with one user interface ◦ (120-Gte05-Sre02) Revision of Convention and related PS, etc by IMO ◦ (120-Gte05-Sre03) Revision of standard of IEC and ITU (HE) REG2 | ◦ (120-Gte05-Str01) Revise the training course of current GMDSS for integrated GMDSS system (HE) TRN4-TRN5 |
|  |  |  |  |  | 120-Gte06 | Insufficient provision of short range but wider bandwidth communication means such as WiMAX. (T) T22-T32 (E) E21-E22-E23 | 1,3,5,7 | 1,3,5,8,10,11 | n/a | None in maritime | 1,2 | ◦ (120-Gte06-Sop01) Automatic information exchange without human intervention even automatic communication channel transition to wideband channel (HE) OPR2-OPR4 | ◦ (120-Gte06-Ste01) Global pilot project to prove the availability of wideband technology in maritime ◦ (120-Gte06-Ste02) Automatic transition of communication channels between new technologies and current communication technologies of ship (HE) TCH1 | ◦ (120-Gte06-Sre01) Global regulation to use wideband communication channel in maritime ◦ (120-Gte06-Sre02) New regulation or standard to use new technology in ship and maritime by IMO, ITU, IEC (HE) REG4 | ◦ (120-Gte06-Str01) No further training required to use new communication channel in principle |
|  |  |  |  | Regulatory | 120-Gre01 | Lack of regulations for communication equipment and systems addressing the identified e-navigation communication needs. | 1,3,5 | 3,10 | n/a | SOLAS Chapter IV (GMDSS) and Chapter V (Navigation) | 1,2,3,4,5 | ◦ n/a | ◦ n/a | ◦ (120-Gre01-Sre01) SOLAS to be amended into an e-navigation chapter. ◦ (120-Gre01-Sre02) Develop and revise the Convention and regulation of IMO ◦ (120-Gre01-Sre03) Revise and develop the standard of ITU and IEC (HE) REG1 | ◦ n/a |
|  |  |  |  |  | 120-Gre02 | IMO resolutions for navigation and communication are not harmonized. (T) T11 | 1,3,5 | 3,10 | n/a | ◦ SOLAS CH IV & V. ◦ NAV 56/WP.5/Rev.1, Annex 5 | 1,2,3,4,5 | ◦ (120-Gre02-Sop01) Standards for communications and navigation to be presented to the mariner as a whole. (HE) OPR3 | ◦ (120-Gre02-Ste01) Harmonization of instrument requirements ◦ (120-Gre02-Ste02) Harmonization of functional requirements (HE) TCH1-TCH2 | ◦ (120-Gre02-Sre01) New performance standards to be developed or existing standards to be amended. ◦ (120-Gre02-Sre02) Special Working Group need to be addressed and consider harmonization and modification of the Convention and regulation of SOLAS Ch.IV & V (HE) REG2 | ◦ (120-Gre02-Str01) Training should include standards and where to find them. (HE) TRN5 |
|  |  |  |  |  | 120-Gre03 | Absence of structured communication link to notify anomalies and malfunctions of shipboard navigational systems such as ECDIS. (T) T12-T22 (E) E33-E34 | 1,3,5 | 3,10 | n/a | IEC 61162 | 1,2,3,4,5 | ◦ (120-Gre03-Sop01) Automatic exchange for anomaly and malfunction notification between systems (HE) OPR1-OPR2-OPR4 | ◦ (120-Gre03-Ste01) More secure communication way to notify by the method more than status parameter such like duplicated channel ◦ (120-Gre03-Ste02) Status monitoring function for interfaced system not dependent on transferred data (HE) TCH1 | ◦ (120-Gre03-Sre01) Revise IEC 61162 standard by IEC ◦ (120-Gre03-Sre02) Revise IMO instruments for more secure channel to communicate between systems (HE) REG2 | ◦ (120-Gre03-Str01) No further training required in principle |
|  |  |  |  | Operational | 120-Gop01 | Insufficient use of IMO Standard Marine Communication Phrases (SMCP). (T) T31-T32-T34-T35 (E) E21-E22-E23 | 1,2,3,5,8 | 1,3,5,7,8,0,10,11 | A7, A16 | ◦ Res. A.918(22) : IMO Standard Marine Communication Phrases ◦ Res.A.954(23) :Proper use of VHF Channels at Sea. ◦ MSC/Circ.794 : Draft IMO standard Marine Communication Phrases (SMCPs) | 1,2,3,4,5 | ◦ (120-Gop01-Sop01) Available to the mariner ◦ (120-Gop01-Sop02) Verbal conversation may replaced by SMCP in general ◦ (120-Gop01-Sop03) More use of SMCP in AIS message and DSC function in current situation ◦ (120-Gop01-Sop04) To be used regularly (HE) OPR7-OPR8 | ◦ (120-Gop01-Ste01) SMCP to be developed and adjusted to the e-navigation concept ◦ (120-Gop01-Ste02) Develop the S/W & H/W of communication system to implement digital communication by selecting appropriate SMCP instead of verbal conversation ◦ (120-Gop01-Ste03) Message of AIS and DSC functionality should be upgraded to be used conversation by SMCP ◦ (120-Gop01-Ste04) SMCP to be revised and simple to read and understand (HE) TCH1 | ◦ (120-Gop01-Sre01) Require SMCP to be available to the navigator at watch at all times. ◦ (120-Gop01-Sre02) Revise the regulation of SMCP to represent more situation in detail ◦ (120-Gop01-Sre03) Revise the performance standard of AIS and DSC ◦ (120-Gop01-Sre04) Develop the Convention context for liability to use SMCP in official reporting ◦ (120-Gop01-Sre05) Requirements for using SMCP to be included in regulations. ◦ (120-Gop01-Sre06) Program for training on board to be developed (HE) REG2 | ◦ (120-Gop01-Str01) To be integrated in the education as a navigator ◦ (120-Gop01-Str02) Obligatory training when educated as a navigator. ◦ (120-Gop01-Str03) New training course to use SMCP for official conversation with Authorities ◦ (120-Gop01-Str04) On-line help function should be provided for continuous guidance and training for user (HE) TRN1-TRN2-TRN3-TRN4 |
|  |  |  |  | Training | 120-Gtr01 | Insufficient training strategies for the use of IMO Standard Marine Communication Phrases (SMCP). |  |  |  |  |  | ◦ n/a | ◦ n/a | ◦ n/a | ◦ see (120-Gop01-Str03) ◦ see (120-Gop01-Str04) (HE) TRN1-TRN2-TRN3-TRN4 |
| (1)SHIPBOARD USER | {3}NAVIGATIONAL BRIDGE SYSTEMS AND EQUIPMENT | [2]Improved Reliability and Indication of Reliability | ◦ Need of Resilient PNT system ◦ Indication of Reliability |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 132-Gte01 | Insufficient reliability of position fixing systems. (T) T11-T12 (E) E33-E34 | 4 | 1,2,4,5,6,7,8 | A1.5,A5, A6, A7, A8, A9, A10, A11, A16, A17, A18, A19, A20.1, A23, A24, A26, A30, A31, A32, A33 | GPS, GLONASS | 1.2.3.4.5 | ◦ (132-Gte01-Sop01) existing technology meets some of the requirements, but lacks integrity and is vulnerable to disruption. Current operational procedures may not be sufficient to cope with loss of position input. (HE) OPR4 | ◦ (132-Gte01-Ste01) existing technology can be used, with augmentation (differential GNSS) to provide integrity. Alternative systems are needed to overcome vulnerability to disruption (HE) TCH1 | ◦ (132-Gte01-Sre01) existing GNSS (GPS &GLONASS) are recognized as elements of the WWRNS. Performance Standards exist for GNSS and DGNSS receiving equipment. New or updated PS will be needed for alternative systems, or a multi-system PS might be developed to avoid proliferation of standards and the consequent problems of keeping them up to date (HE) REG2 | ◦ (132-Gte01-Str01) users need to be made aware of the vulnerability of GNSS and the lack of integrity.- users also need to be trained in procedures to be followed if GNSS is disrupted, or in the use of alternative systems (HE) TRN5 |
|  |  |  |  |  | 132-Gte02 | Lack of self-checking functionality of the electronic equipment. (T) T3-T11-T12-T22 (E) E32-E33-E34 | 7 | 10 | A1.1 | GMDSS, INS, ECDIS, RADAR, AIS and etc. | 1,2,3,4,5 | ◦ (132-Gte02-Sop01) Equipments should have BIIT(Built In Integrity Test) or self-checking function at least ◦ (132-Gte02-Sop02) Automatic response procedure of equipment to checking result (HE) OPR1-OPR2 | ◦ (132-Gte02-Ste01) Define the concept and level of self-checking for each equipment ◦ (132-Gte02-Ste02) Develop the automatic response of system for each result (HE) TCH1 | ◦ (132-Gte02-Sre01) Revise the performance standard of electronic equipment (HE) REG2 | ◦ (132-Gte02-Str01) Revise the training course to understand the system's automatic action and report to user (HE) TRN5 |
|  |  |  |  | Regulatory |  | No identified Gap |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Operational |  | No identified Gap |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Training |  | No identified Gap |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
| (1)SHIPBOARD USER | {3}NAVIGATIONAL BRIDGE SYSTEMS AND EQUIPMENT | [4]Improved Ergonomics, Standardization and Alert Management | ◦ Improved Ergonomics ◦ Standard Interface ◦ Alert Management |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 134-Gte01 | Gap is that seafarers are not provided with equipment that they think is easy to use: ergonomic problems of navigation equipments exist in a sense that operations of some products are confusing, which sometimes hinders smooth operations. | 1,2 | 9 | n/a | n/a | 1,2,3,4,5 | ◦ (134-Gte01-Sop01) Develop the standard operational procedure of e-navigation function as a guideline for usability assessment (HE) OPR1-OPR2-OPR4 | ◦ (134-Gte01-Ste01) Adoption of more advanced human machine interface (ex, HAPTIC) ◦ (134-Gte01-Ste02) GMDSS Modernization should consider human machine interface of communication equipment (HE) TCH1-TCH2 | ◦ (134-Gte01-Sre01) Define the minimum level of ergonomic and user friendly design required to fulfill user needs and ensure the safety(IMO) ◦ (134-Gte01-Sre02) Develop the standard of usability assessment for systems in view of S/W and H/W(IMO) (HE) REG2 | ◦ (134-Gte01-Str01) On-line help function should be included in the systems, if available (HE) TRN5 |
|  |  |  |  |  |  | Lack of good human machine interface for the communication means. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Bridge layouts, equipment and systems are seldom designed from an ergonomic and user friendly perspective. (T) T11-T22 (E) E33-E34 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 134-Gte02 | Interoperability of systems and sensors is not realized. (T) T11-T22 (E) E33-E34 | 2,7,8 | 10 | n/a | n/a | 1,2,3,4,5 | ◦ n/a | ◦ n/a | ◦ (134-Gte02-Sre01) INS performance standard should be revised to ensure interoperability between systems and sensors (HE) REG2 | ◦ n/a |
|  |  |  |  |  | 134-Gte03 | Lack of unified symbology. (T) T11-T22 (E) E33-E34 | 2,7,8 | 8,10 | n/a | n/a | 1,2,3,4,5 | ◦ (134-Gte03-Sop01) Presentation of information by symbol rather than text, if available (HE) OPR1-OPR2-OPR4 | ◦ (134-Gte03-Ste01) Ergonomic design of symbol (HE) TCH1-TCH2 | ◦ (134-Gte03-Sre01) Holistic review of current symbol in IMO instrument ◦ (134-Gte03-Sre02) Integrate current regulation for symbology in line with the concept of e-navigation information(ex. MSC.191(79), SN/Circ.243, S-52 etc.) (HE) REG1 | ◦ (134-Gte03-Str01) Revise the training course to be familiarized with the symbology (HE) TRN4 |
|  |  |  |  |  | 134-Gte04 | Lack in presentation of maneuvering information/data(engine-room telegraphs) on navigational display. (T) T11-T22 (E) E33-E34 | 2,7 | 8 | A1, A11 | INS | 1,2,3,4,5 | ◦ (134-Gte04-Sop01) Electronic presentation of maneuvering data on display to recognize clear awareness of ship status (HE) OPR1-OPR2-OPR4-OPR5 | ◦ (134-Gte04-Ste01) INS should have a display for this presentation ◦ (134-Gte04-Ste02) New interface message for maneuvering data ◦ (134-Gte04-Ste03) Develop new symbol or specification to present maneuvering data (HE) TCH1 | ◦ (134-Gte04-Sre01) Define the type and context of maneuvering data ◦ (134-Gte04-Sre02) Revise the IEC 61162 to add new feature (HE) REG2 | ◦ (134-Gte04-Str01) Revise the training course to be familiarized with the new feature of maneuvering data presentation (HE) TRN4 |
|  |  |  |  | Regulatory | 134-Gre01 | Equipment and systems are often not self-descriptive, appropriately designed for the task at hand, controllable, compliant with the users' expectations, as well as fault tolerant. (T) T11-T22 (E) E33-E34 | 1,2,3 | 9,10 | n/a | n/a | 1,2,3,4,5 | ◦ n/a | ◦ (134-Gre01-Ste01) Technical procedure and method to verify system's usability (HE) TCH1 | ◦ (134-Gre01-Sre01) IMO should have a procedure (ex. Regulation) to verify systems of e-navigation based on the user needs and related functions and services ◦ (134-Gre01-Sre02) Usability assessment should be used to verify it. (HE) REG2 | ◦ (134-Gre01-Str01) On-line help function should include self-descriptive information of system's characteristic in view of operational, technical, regulatory and training (HE) TRN4 |
|  |  |  |  |  | 134-Gre02 | Control(e.g.. Type approval) of software and hardware updates is not sufficient. | 1,3,5,7 | 10 | A1.1, A1.2 | n/a | 1,2,3,4,5 | ◦ (134-Gre02-Sop01) Automatic onboard verification of system certification in line with the regulation dependent on the region ◦ (134-Gre02-Sop02) Automatically maintain the system s/w(incl. O/S) and h/w as up-to-date ◦ (134-Gre02-Sop03) Shore authority can check onboard certification status remotely (HE) OPR2 | ◦ (134-Gre02-Ste01) Develop the electronic certification system protected with secure algorithm ◦ (134-Gre02-Ste02) Develop new interface message for electronic certification data and protection algorithm with security ◦ (134-Gre02-Ste03) Adoption of on-air update technology of system itself (HE) TCH1 | ◦ (134-Gre02-Sre01) New regulation and standard to use electronic certification system onboard ◦ (134-Gre02-Sre02) Revise related standard of equipment to use on-air update technology in view of s/w and h/w ◦ (134-Gre02-Sre03) Revise the interface standard to ensure the security of certification data ◦ (134-Gre02-Sre04) Revise the carriage requirement of systems to include the up-to-date requirement of its s/w(incl. o/s) and h/w (HE) REG1 | ◦ (134-Gre02-Str01) Control of s/w and h/w update should not require additional training of mariners ◦ (134-Gre02-Str02) Human intervention of this procedure should be minimum but there are certain recognition of the certification status (HE) TRN4 |
|  |  |  |  |  |  | Type approval procedure for navigation equipment should become more flexible and progressive. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | "Lifetime regulatory compliance" should be discontinued for systems that are dependent on software for their key functions. Instead, software dependent systems should only be suitable for regulatory compliance when using software that complies with the most current version(s) of the relevant standard(s). |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Regulation of upgrading of navigation equipment operating systems is missing. (T) T11-T22-T25 (E) E33-E34 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 134-Gre03 | Existing documents (performance standards, guidelines, etc.) with regard to ergonomics are missing harmonization and are seldom applied. | 1,2,3 | 10 | n/a | n/a | 1,2,3,4,5 | ◦ n/a | ◦ (134-Gre03-Ste01) Review existing documents technically to ensure enough ergonomics to fulfill the user needs (HE) TCH1-TCH2 | ◦ (134-Gre03-Sre01) Review and revise these documents to ensure the ergonomics -MSC.252(83) (INS) -MSC/Circ.982 (Ergonomic Criteria for Bridge Equipment and Layout) -SN.1/Circ.288 (Guidelines for bridge equipment and systems, their arrangement and integration) -MSC.191(79) (Pres. of NAV-related Info on NAV-Displays) -MSC.302(87) (Bridge Alert Management) -A.1021(26) (Code on Alerts and Indicators). ◦ (134-Gre03-Sre02) Develop the ergonomic requirement as mandatory it (HE) REG1 | ◦ n/a |
|  |  |  |  |  |  | Existing documents (performance standards, guidelines, etc.) with regard to ergonomics are not applied for communication equipment and systems (incl. GMDSS). |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Existing documents (performance standards, guidelines, etc.) for alert management are not applied. (T) T11-T22 (E) E33-E34 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 134-Gre04 | Currently, there are no guidelines or guidance for usability evaluation. (T) T11-T22 (E) E33-E34 | 1,2,7 | 10 | n/a | n/a | 1,2,3,4,5 | ◦ n/a | ◦ n/a | ◦ (134-Gre04-Sre01) Define the minimum level of ergonomic and user friendly design required to fulfill user needs and ensure the safety(IMO) ◦ (134-Gre04-Sre02) Develop the standard of usability assessment for systems in view of S/W and H/W(IMO) (HE) REG2 | ◦ n/a |
|  |  |  |  |  | 134-Gre05 | Lack of standardization for operation of functions to observe the passage plan. (T) T11-T22 (E) E33-E34 | 2,7 | 1,2 | A3,A4,A6.4, | ECDIS,ENC | 1,2 | ◦ n/a | ◦ (134-Gre05-Ste01) Develop the procedure to monitor and observe passage plan by electronic way (HE) TCH1 | ◦ (134-Gre05-Sre01) Review in detail for ECDIS performance standard (HE) REG2 | ◦ (134-Gre05-Str01) Revise the training course of ECDIS to be familiarized electronic procedure related passage plan (HE) TNR5 |
|  |  |  |  |  | 134-Gre06 | Lack of performance standards for interoperability of systems and sensors (according to the modular concept). (T) T11-T12-T13-T22 (E) E32-E33-E34 | 2,7,8 | 10 | n/a | n/a | 1,2,3,4,5 | ◦ n/a | ◦ n/a | ◦ (134-Gre06-Sre01) INS performance standard should be revised to ensure interoperability between systems and sensors ◦ (134-Gre06-Sre01) SN.1/Circ.274 should be reviewed to develop modular concept further (HE) REG2 | ◦ n/a |
|  |  |  |  | Operational | 134-Gop01 | Seafarers sometimes experience difficulties in accessing necessary information because of ergonomic problems. (T) T11-T22 (E) E34 | 2,7 | 9,10 | A1, A6,A11 | n/a | 1,2,3,4,5 | ◦ (134-Gop01-Sop01) Standardized operational procedure for GMDSS operation should be developed (HE) OPR1-OPR2-OPR4 | ◦ (134-Gop01-Ste01) The GMDSS system should designed based on the standard operational procedure (HE) TCH1-TCH2 | ◦ (134-Gop01-Sre01) Regulation for usability assessment should include the consideration of ergonomics for user to operate correct function easily (HE) REG2 | ◦ (134-Gop01-Str01) Training course of GMDSS should be revised based on the standard operational procedure (HE) TRN4-TRN5 |
|  |  |  |  | Training | 134-Gtr01 | Difficulties exist for personnel to transfer from ship to ship. (T) T11-T22 (E) E32-E33-E34 | 2 | 9 | n/a | n/a | 1,2,3,4,5 | ◦ (134-Gtr01-Sop01) Onboard training possible with the on-line help or site training function of system itself (HE) OPR7-OPR8 | ◦ (134-Gtr01-Ste01) Onboard training and on-line help should be provided with ergonomic way such like symbol , audio guidance or etc. (HE) TCH1 | ◦ (134-Gtr01-Sre01) Develop the guideline or Standard of onboard training and on-line help function (HE) REG2 | ◦ (134-Gtr01-Str01) In different ships with different equipments, user must not need to spend more time to be educated for operation of same function (HE) TRN4 |
| (1)SHIPBOARD USER | {3}NAVIGATIONAL BRIDGE SYSTEMS AND EQUIPMENT | [5]Presentation of Information Received via Communication Equipment (e.g., MSI) on the Navigation display | ◦ User-Selectable Presentation of Information Received via Communication Equipment ◦ Maritime Safety Information(MSI) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 135-Gte01 | Upon receiving real-time Maritime Safety Information (MSI) and other navigational warning/broadcasts relevant for the vessel's navigation, there is no appropriate and suitable interfacing technique that could allow these data/information to be available (visible) in real-time to the mariner. | 1,2,5,7,8 |  | A2, A11 | NAVTEX SafetyNET VHF ECDIS, INS,GMDSS | 1,2,3,4,5 | ◦ (135-Gte01-Sop01) For appropriate MSI to be presented on a navigational display using standard symbology and text support ◦ (135-Gte01-Sop02) Navigation system need to identify appropriate MSI during route planning and voyage planning ◦ (135-Gte01-Sop03) [Human element consideration should be given to the effective use of symbology and prevention of information overload] ◦ (135-Gte01-Sop04) Automatic transfer of information received via communication equipment into the navigational systems for presentation ◦ (135-Gte01-Sop05) MSI or other navigational warning/broadcasts via communication equipment should be presented on navigational display system to be recognized by OOW ◦ (135-Gte01-Sop06) If presented information have a parameter for urgency, the system should provide the alarm ◦ (135-Gte01-Sop07) Secondary information display could be operable as an option ◦ (135-Gte01-Sop08) Shore authority should provide critical information to ship's safety almost in real time and have appropriate system for this purpose (HE) OPR1-OPR2-OPR3-OPF4-OPR5-OPR6 | ◦ (135-Gte01-Ste01) Database and presentation library to display the MSI ◦ (135-Gte01-Ste02) Message based on S-100 and portrayal ◦ (135-Gte01-Ste03) Standard interface btw suitable carriers such as NAVTEX, AIS and Display ◦ (135-Gte01-Ste04) It should be a display with standards. ◦ (135-Gte01-Ste05) Enables consistency of alerts to users ◦ (135-Gte01-Ste06) New IEC 61162 sentence should be developed ◦ (135-Gte01-Ste07) S/W modification of communication and navigation equipment ◦ (135-Gte01-Ste08) Navigational display system such like ECDIS should have revised performance standard(IMO) and related presentation standard(IEC, IHO) (HE) TCH1-TCH2 | ◦ (135-Gte01-Sre01) Standards for presentation and symbology ◦ (135-Gte01-Sre02) SOLAS Ch.V/18 & Ch.IV ◦ (135-Gte01-Sre03) IMO Res.A.705(17) ◦ (135-Gte01-Sre04) IMO Res.A.706(17) ◦ (135-Gte01-Sre05) Joint IMO/IHO/WMO Manual on MSI ◦ (135-Gte01-Sre06) S-100 ◦ (135-Gte01-Sre07) Revision of IEC 61162 standard ◦ (135-Gte01-Sre08) Revision of ECDIS, INS performance standard ◦ (135-Gte01-Sre09) Revision of ECDIS presentation library and symbology (HE) REG1-REG2 | ◦ (135-Gte01-Str01) Recognizing symbols and managing alerts ◦ (135-Gte01-Str02) Relationship with other equipment ◦ (135-Gte01-Str03) Revision of training course to operate comm. and nav. equipment involved within transfer of information (HE) TRN4-TRN5 |
|  |  |  |  |  |  | Lack of technical solutions for processing, routing, and filtering of information received via communication equipment to enable transfer of the information to navigational systems. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Lack of technical solutions for presenting communication information/Maritime Safety Information (MSI) on navigational displays. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Lack of presentation of warning broadcasts on navigation displays. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Insufficient means for sorting and display of Maritime Safety Information (MSI) such as NAVTEX, SafetyNET. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Lack of user-selectable and task oriented presentation of information received via communication equipment (including MSI) on navigational systems. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Insufficient network of storage, sharing and distribution of MSI. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Unless having prior subscription, the current system does not allow for Maritime Safety Information (MSI) and other navigational warnings/broadcast, etc., to be received in real-time mode and be integrated with the navigation display. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Unavailability of information in real-time with possible presentation on the navigational display to support bridge operation. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Lack of integrated secondary screen option for digital publications and MSI. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Lack of interface messages between sender and receiver for monitoring of local/coastal warning broadcasts/watching GMDSS system (NAVTEX, NAVAREA message). (T) T1-T2-T3-T4-T11-T22 (E) E33-E34 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 135-Gte02 | Lack of information about special berthing requirements on navigation systems. (T) T11-T23-T24-T25-T31-T32-T33-T34 (E) E21-E22 | 2,3,6,7 | 1,8,10 | A5 | ECDIS, S-100 | 1 | ◦ (135-Gte02-Sop01) Information for special berthing should be presented on navigational system such like ECDIS (HE) OPR3-OPR6 | ◦ (135-Gte02-Ste01) Revise the presentation library for new feature ◦ (135-Gte02-Ste01) Develop the digital format for information of special berthing (HE) TCH1 | ◦ (135-Gte02-Sre01) Revise IEC 61162 standard by IEC ◦ (135-Gte02-Sre02) Revise S-10X or S-52 by IHO ◦ (135-Gte02-Sre03) Revise ECDIS performance standard or INS (HE) REG2 | ◦ (135-Gte02-Str01) Revise the training course of ECDIS as appropriate (HE) TRN4 |
|  |  |  |  |  | 135-Gte03 | Lack of presentation of calling message of pilot on navigational display. | 1,5,7 | 1,2,3,8 | A5,A25,A26 | ECDIS, INS | 1,2 | ◦ (135-Gte03-Sop01) Pilot and mariner onboard can exchange their information on demand ◦ (135-Gte03-Sop02) Related information should be presented on navigational display system (HE) OPR3-OPR6-OPR7-OPR8 | ◦ (135-Gte03-Ste01) Develop the communication link and message format for exchange of pilotage information (ex. In AIS) ◦ (135-Gte03-Ste02) Revise the ECDIS s/w and presentation library (HE) TCH1 | ◦ (135-Gte03-Sre01) New IEC 61162 sentence ◦ (135-Gte03-Sre02) Define new AIS application specific message (HE) REG1 | ◦ (135-Gte03-Str01) Revise the training course of ECDIS as appropriate (HE) TRN4 |
|  |  |  |  |  |  | (digital) Communication with pilot could be improved. (T) T11-T23-T24-T25-T31-T32-T33-T34 (E) E21-E22 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Regulatory | 135-Gre01 | IMO resolutions for navigation and communication are not harmonized. | 1,3,7 | 1,3,9,10 | n/a | n/a | 1,2,3,4,5 | ◦ n/a | ◦ n/a | ◦ (135-Gre01-Sre01) Performance standard and regulation system of IMO should be reviewed not based on each equipment but on user needs and function required to provide services (HE) REG2 | ◦ n/a |
|  |  |  |  | Operational |  | No identified Gap |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Training |  | No identified Gap |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
| (1)SHIPBOARD USER | {3}NAVIGATIONAL BRIDGE SYSTEMS AND EQUIPMENT | [6]Documents in Electronic Form and Automated Updates of Information | ◦ Reduction of Administrative Burden and Increase of Electronic Documentation ◦ Automated Updating of Base Line Data and Documents ◦ Effective and Robust Communications |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 136-Gte01 | Information should be provided in proper electronic format for data rather than the documents being digital copies of existing paper publications | 1,2,3,7 | 8,10 | A1,A2,A14,A15 | Onboard computer | 1,2,3,4,5 | ◦ (136-Gte01-Sop01) Automatic update of information using standard formats for data exchange with possible human intervention, if necessary ◦ (136-Gte01-Sop02) Using digital publications on user computer onboard or supplementary display of navigational system (HE) OPR2-OPR4 | ◦ (136-Gte01-Sop01) Develop the digital version of paper publications based on S-10X ◦ (136-Gte01-Sop02) Develop the update data structure also based on S-10X ◦ (136-Gte01-Sop03) Communication links should be connected to User computer onboard to receive update data ◦ (136-Gte01-Sop04) New S/W to manage digital publication and update data for it ◦ (136-Gte01-Sop05) Develop the efficient way to use digital publication including update mechanism ◦ (136-Gte01-Sop06) Develop the S/W to use digital publication on onboard computer (HE) TCH1 | ◦ (136-Gte01-Sre01) Develop new regulation of digital publication and update for satisfying the carriage requirements ◦ (136-Gte01-Sre02) Develop New standard, S-10X for digital publication onboard ◦ (136-Gte01-Sre03) Revise and/or develop the presentation library of ECDIS or navigation display system (HE) REG1 | ◦ (136-Gte01-Str01) Familiarization for using digital publication on User's computer ◦ (136-Gte01-Str02) Revise the training course for general skill on user computer to use the S/W for digital publication (HE) TRN4 |
|  |  |  |  |  |  | New equipment/system or task based on INS-task concept MSC.252(83) for management of information formerly available in printed format is necessary. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Information may be difficult to localize in electronic documents.(search function) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Lack of automatic updating of documents. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Lack of automatic update of navigation charts and publications in real-time on demand on ECDIS. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Electronic systems can not automatically determine the status of available data and automatically retrieve the most current and comprehensive data. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Regulations for new navigational display systems should be standardized. (T) T11-T22-T23-T24-T25 (E) E14-E33-E34 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Regulatory | 136-Gre01 | Legal aspects regarding access and usage rights of updating information are not solved. (T) T11-T25 | 1,8 | 10,11 | n/a | ENC,ECDIS | 1,2,3,4,5 | ◦ (136-Gre01-Sop01) User can use free ENC update data as a minimum to ensure ship's safety without encryption(S-63) (HE) OPR4 | ◦ (136-Gre01-Ste01) Any updating system should be provided through the publicly open method (HE) TCH1 | ◦ (136-Gre01-Sre01) Newly revise the ENC responsibility of updating production in the level of Convention (HE) REG4 | ◦ n/a |
|  |  |  |  |  | 136-Gre02 | Documentation requirements possibly not allow for documentation in electronic form. (E) E33-E34 | 1,2,3,7 | 8,10 | A1,A2,A14,A15 | n/a | 1,2,3,4,5 | ◦ n/a | ◦ n/a | ◦ (136-Gre02-Sre01) Revise the Convention to allow electronic document as a carriage requirement (HE) REG1 | ◦ n/a |
|  |  |  |  |  | 136-Gre03 | Too many regulations are adding to the administrative burden of the mariner on board. (E) E09 | - | - | - | - | - | ◦ n/a | ◦ n/a | ◦ (136-Gre03-Sre01) Regulations should be revised not based on each equipment but on user needs and function required to provide services (HE) REG3-REG4 | ◦ n/a |
|  |  |  |  | Operational | 136-Gop01 | Insufficient indication of information updates. (T) T11-T22-T25 (E) E33-E34 | 1,8 | 10,11 | n/a | ENC,ECDIS | 1,2,3,4,5 | ◦ (136-Gop01-Sop01) User should be acknowledged automatically for update information (HE) OPR1-OPR2-OPR4 | ◦ (136-Gop01-Ste01) Revise the scheme or parameter to indicate update information in ENC update standard (HE) TCH1 | ◦ (136-Gop01-Sre01) Revise and/or develop the S-57 or S-10X standard (HE) REG1 | ◦ n/a |
|  |  |  |  |  | 136-Gop02 | Ineffective access to information. (T) T11-T22 (E) E33-E34 | 2,7 | 8,10 | n/a | INS | 1,2,3,4,5 | ◦ (136-Gop02-Sop01) All required information should be accessible at central station in bridge (HE) OPR1-OPR4 | ◦ (136-Gop02-Ste01) INS design should reflect the information access method (HE) TCH1-TCH2 | ◦ (136-Gop02-Sre01) Revise the INS performance standard (HE) REG2 | ◦ (136-Gop02-Str01) Revise the training course of INS (HE) TRN4-TRN5 |
|  |  |  |  | Training |  | No identified Gap |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
| (1)SHIPBOARD USER | {4}SHIP REPORTING | n/a | ◦ Standardized and automated reporting |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 140-Gte01 | With the exception of Polling, current system does not provide provision for automatic ship reporting. (T) T11-T22-T31-T32 (E) E21 | 1,3,7,8 | 7,10,11 | A16 | Manually generated reports except in one or two local areas - AIS. Polling only used for LRIT. LRIT provides insufficient information of ship reporting. | 1,2,3 | (140-Gte01-Sop01) 1) Automate ship reporting sourcing ship information from ship to populate VTS or MDA system without human intervention. (140-Gte01-Sop02) 2) Use ubiquitous system to meet Voluntary, Mandatory and Statuary ship reporting needs. (140-Gte01-Sop03) 3) Source information on Cargo, hull and passengers from origin (e.g. safeseanet) (HE) OPR2-OPR4 | (140-Gte01-Ste01) 1) Dynamic information (Position, Course and speed) provided into system by ship via suitable carriers offering validation of data. (AIS and AIS-S) (140-Gte01-Ste02) 2) Static data to be provided from source and indexed to the ships dynamic data by a data exchange system fit for purpose to provide all static parts. (International data exchange?) (HE) TCH1 | (140-Gte01-Sre01) 1) Change of A.851.(20) (140-Gte01-Ste02) 2) Architecture for sourcing information on static information. (HE) REG1 | ◦ n/a |
|  |  |  |  |  | 140-Gte02 | Lack of automated and standardized ship reporting function. (T) T11-T31-T32 (E) E21 | 1,3,7,8 | 7,10,11 | A16 | radio telephone, telex or email. | 1,2,3,4,5 | (140-Gte02-Sop01) Usage of a vessel technology that includes the most types of vessels ensuring the most compete picture possible. AIS and AIS-S . AIS now has precedence for being used in for reporting. (HE) OPR2-OPR4 | (140-Gte02-Ste01) Follow precedence of using AIS for ship reporting by Sweden. (HE) TCH1 | (140-Gte02-Sre01) Change of A.851.(20) to include the use of automated processes suited to ship reporting such as AIS and AIS-S. (HE) REG1 | ◦ n/a |
|  |  |  |  |  | 140-Gte03 | Single-window and/or automated and single entry for any required reporting information into the system for it to be shared by authorized authorities without further intervention by the ship during navigation, except it has any relevance for navigational purposes (VTS/PILOT/HARBOR/COLREG). (T) T22-T31-T32 (E) E21 | 1,3,7,8 | 7,10,11 | A16 | None used | 1,2,3,4,5 | (140-Gte03-Sop01) Access of information to support Maritime Incidents and casualties. (HE) OPR2 | (140-Gte03-Ste01) International data exchange to enable shore authorities to index AIS report and ship ID to latest information on Cargo, Hull and Passenger. The IDE and SafeSeaNet are examples of relevant existing systems. When located, use e-mail or EDI to retrieve information. (HE) TCH1 | (140-Gte03-Sre01) Investigate the use of a global version of the SafeSeaNet architecture and the IDE solely to find out where the information is held. (HE) REG3 | ◦ n/a |
|  |  |  |  |  | 140-Gte04 | Automated entry of internal ship data for reporting (including updates of information) is not available. (T) T11-T22 | 1,3,7,8 | 7,10,11 | A16 | Manual interface with minimal keyboard display | 1 | (140-Gte04-Sop01) Ensure that accurate information is made available to the AIS and as such to message 5 by automated means. (HE) OPR2 | (140-Gte04-Ste01) 1) Link between Port competent authority to ship for update of M5 automatically into the ships AIS. (140-Gte04-Ste02) 2) Change of ships name and MMSI information to be possible only by a representative of a competent authority. (140-Gte04-Ste03) 3) Automate the process to install static information into ship reporting equipment such as message 5 of AIS. (140-Gte04-Ste04) 4) Owner / operators be provided with AIS-S so that they can inform the ship when static data is not correct (ISM?) (HE) TCH1 | (140-Gte04-Sre01) Evolve 851 (20) 2) recommend procedures for populating the AIS or other automated device with correct information. (HE) REG3 | ◦ n/a |
|  |  |  |  |  | 140-Gte05 | Insufficient means for ship reporting. (T) T13-T22-T31-T32 (E) E21 | 1,3,7,8 | 7,10,11 | A16 | Manually generated reports except in one or two local areas - AIS. | 1,2,3,4,5 | (140-Gte05-Sop01) Remove need for human interface and communication of manually operated systems and provide automated systems based on shipboard AIS that will seamlessly populate VTS and MDA systems, anywhere in the world. (HE) OPR2 | (140-Gte05-Ste01) Stimulate ship reporting through automated means by providing the tools needed. (HE) TCH1 | (140-Gte05-Sre01) Evolve 851 (20) as it presently a barrier automated means by recommended the use of manually generated ship information, even when the ship is not always the best source, and manually integration and assessment by Authorities. (HE) REG3 | ◦ n/a |
|  |  |  |  | Regulatory | 140-Gre01 | Legal aspects regarding access and sharing of reporting information are not solved. | 1,3,7,8 | 7,10,11 | A16 | Up to individual countries | 1,2,3,4,5 | (140-Gre01-Sop01) Legal aspects for the use of data from ships (e.g. AIS and AIS-S) to be used ashore should not interfere with the use of the ships data to support safety of navigation, pollution prevention and SAR in remote areas of the world in the spirit of A.851(20). (HE) OPR3-OPR7 | (140-Gre01-Ste01) AIS fitted to aircraft, Hale, or satellite should be treated the same as terrestrial AIS. The carrier of information should be accepted whether it is of commercial or non commercial origin. Just as in the case of GMDSS. (HE) TCH1 | (140-Gre01-Sre01) To ensure that commercial services providing information critical for ship reporting, guidelines may be required. (HE) REG3 | ◦ n/a |
|  |  |  |  |  | 140-Gre02 | Single-window and/or automated and single entry for any required reporting information into the system for it to be shared by authorized authorities without further intervention by the ship during navigation, except it has any relevance for navigational purposes (VTS/PILOT/HARBOR/COLREG). (T) T22-T32 (E) E21 | 1,3,7,8 | 7,10,11 | A16 | Minimal keyboard display | 1,2 | (140-Gre02-Sop01) Ensure that accurate information is input to the AIS and as such to message 5 by automated means. (HE) OPR2-OPR7 | (140-Gre02-Ste01) 1) Link between Port competent authority to ship for update of M5 automatically into the ships AIS. Change of ID information to be possible only by a representative of a competent authority. (140-Gre02-Ste02) 2) Automate the process to install static information into ship reporting equipment such as message 5 of AIS. (140-Gre02-Ste03) 3) Owner / operators be provided with AIS-S so that they can inform the ship when not correct (ISM code?) (HE) TCH1 | (140-Gre02-Sre01) 1) Evolve 851 (20). (140-Gre02-Sre02) 2) provide a standardized procedures and state sets for collating, communication and installing the information. (HE) REG1 | ◦ n/a |
|  |  |  |  |  | 140-Gre03 | The more a legal protocol that permits a government agency to automatically (and without notice or agreement from the master) pick up the ship in a MDA system and maintain an interest in it | 1,3,7,8 | 7,10,11 | n/a |  | 1,2,3,4,5 | ◦ There is a need to additionally identify the reason for this legal protocol that may be clash with commercial interest (HE) OPR2 | ◦ TBD | ◦ TBD | ◦ TBD |
|  |  |  |  |  | 140-Gre04 | National reporting requirements are not harmonized. (T) T22-T31-T32 (E) E21 | 1,3,7,8 | 7,10,11 | A16 | Manually generated reports - Telex or Mail. Morse is no longer recommended. | 1,2,3,4,5 | (140-Gre04-Sop01) Delivery of ship reporting information concerning the ships its voyage, cargo and dynamic status (position and velocity) (HE) OPR2-OPR4 | (140-Gre04-Ste01) Analyze all information requirements of ship reporting and determine which if any are obsolete, and the best source of the information required. (HE) TCH1 | (140-Gre04-Sre01) Evolve 851 (20) (HE) REG1 | ◦ n/a |
|  |  |  |  |  | 140-Gre05 | Lack of standardized reporting formats. (T) T22-T31-T32 (E) E21 | 1,3,7,8 | 7,10,11 | A16 | Manually generated reports - Telex or Mail. Morse is no longer recommended. | 1,2,3,4,5 | (140-Gre05-Sop01) Simplify procedures and provide appropriate automated processes for accessing high quality information. (HE) OPR2-OPR4 | (140-Gre05-Ste01) 1) Reduce information required from ship, concentrating on that in the control of the ship. (140-Gre05-Ste02) 2)Source other information as and when required from origin of the information through international exchange. (140-Gre05-Ste03) 3) Provide new recommendations for procedures for accessing, and technological solutions for retrieving information. (HE) TCH1 | (140-Gre05-Sre01) Evolve 851 (20) (HE) REG1 | ◦ n/a |
|  |  |  |  | Operational | 140-Gop01 | Reporting procedures are not globally standardized. (T) T22-T31-T32 (E) E21 | 1,3,7,8 | 7,10,11 | A16 | Manually generated reports - Telex or Mail. Morse is no longer recommended. | 1,2,3,4,5 | (140-Gop01-Sop01) Shipboard and radio communication technology should facilitate provision of information from a device that provides automatically the identification position, velocity, draft and other information required for safety of navigation from anywhere in the world. (HE) OPR2-OPR4 | (140-Gop01-Ste01) 1) Use AIS in VTS Port and coastal infrastructure vicinity, and AIS by satellite for remote coastal areas and deep sea. Will provide a globally standardized system for provision of ship information. (140-Gop01-Ste02) 2) By indexing the ID of the ship through a global exchange would enable access for sourcing information on Hull, Cargo, passengers as required. (HE) TCH1 | (140-Gop01-Sre01) Evolve 851 (20) (HE) REG1 | ◦ n/a |
|  |  |  |  |  | 140-Gop02 | The needs to report, for safety, commercial and legislative reasons require time and effort. (T) T22-T31-T32 (E) E21 | 1,3,7,8 | 7,10,11 | A16 | Most components in place but not interfaced to computing and communication devices. | 1,2,3,4,5 | (140-Gop02-Sop01) Shipboard and radio communication technology should facilitate provision of information from a device that provides automatically the identification position, velocity, draft and other information required for safety of navigation from anywhere in the world. (HE) OPR2-OPR4 | (140-Gop02-Ste01) 1) As above but additionally, to provide on board technology to automatically compare real time parameters to known safe envelopes for critical ship attributes that may effect safety of navigation, pollution or seaworthiness, degradation of which requires reporting. (140-Gop02-Ste02) 2) Provide for automatically communicating this status to Flag and or coastal states. Provide interface to sensors, VDR or ship condition monitoring system. (HE) TCH1 | (140-Gop02-Sre01) 1) Evolve 851 (20) (140-Gop02-Sre02) 2) Provide performance requirements for interfacing to critical attributes through interface to sensors, VDR or Ship condition monitoring (HE) REG2 | ◦ n/a |
|  |  |  |  | Training | 140-Gtr01 | When standardized reporting formats are introduced adequate training is necessary. | 1,3,7,8 | 7,10,11 | n/a | n/a | 1,2,3,4,5 | ◦ (140-Gtr01-Sop01) More automated procedure of ship reporting should be developed (HE) OPR2 | ◦ n/a | ◦ (140-Gtr01-Sre01) Revise the Convention to adopt automated ship reporting and its operation (HE) REG3 | ◦ (140-Gtr01-Str01) Develop the training course to be familiarized automated reporting procedure as a standard operation ◦ (140-Gtr01-Str02) Developed procedure for automatic reporting should not require more training load (HE) TRN4 |
|  |  |  |  |  |  | The design of reporting and administrative activities require too much training in their use. (T) T22-T31-T32 (E) E21 |  |  |  |  |  |  |  |  |  |
| (1)SHIPBOARD USER | {5}TRAINING AND FAMILIARIZATION | n/a | ◦ Familiarization Requirements |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 150-Gte01 | Insufficient familiarization material for safety-related equipment. | 2 | 9 | n/a | n/a | 1,2,3,4,5 | ◦ (150-Gte01-Sop01) Familiarization material should be correlated with the standard operational procedure to be operated in e-navigation (HE) OPR4 | ◦ n/a | ◦ (150-Gte01-Sre01) Regulation and/or standard to be revised should include the level and contents of familiarization material, Or there must be a guideline for it (HE) REG2 | ◦ (150-Gte01-Str01) On-line help or on-site training function could be provided as a type of familiarization material (HE) TRN4-TRN5 |
|  |  |  |  |  |  | Lack of specifications of familiarization material for new and existing performance standards. (T) T11 (E) E33-E34 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Regulatory |  | No identified Gap |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Operational | 150-Gop01 | Insufficient familiarization and awareness training of seafarers and relevant personnel in the detection and reporting of anomalies to appropriate channel, feedback and recording of subsequent action/measures. (T) T11 (E) E33-E34 | 2 | 9 | n/a | n/a | 1,2,3,4,5 | ◦ (150-Gop01-Sop01) Identification of anomalies related type-specific functions should be assisted automatically by the system function (HE) OPR1-OPR2-OPR4-OPR5 | ◦ n/a | ◦ n/a | ◦ (150-Gop01-Str01) Training for a system should be focused to standardized operational procedure of e-navigation ◦ (150-Gop01-Str02) Functions dependent on manufacturer or type should be trained and covered by familiarization material and/or on-site training and on-line help function (HE) TRN4-TRN5 |
|  |  |  |  | Training | 150-Gtr01 | Considering the wide-scope and complex nature of e-navigation, the current training regime for shipboard users appears inadequate. The gap in training could be attributed to under three main headings: (i) Ship specific (e.g., type, voyage, location, etc.), (ii) User specific (e.g., operational level, management level) or even, (iii) System specific (e.g., layout, equipment, workstation), (iv) Training in use of all navigational products and symbology. | 2 | 9 | n/a | n/a | 1,2,3,4,5 | ◦ (150-Gtr01-Sop01) Type specific function should be trained and/or assisted by on-line help function of system itself ◦ (150-Gtr01-Sop02) Users should be able to use the type-specific function without excessive training for these if they have trained for standardized operational procedure (HE) OPR3-OPR4-OPR5-OPR6-OPR7 | ◦ (150-Gtr01-Ste01) Develop the standardized operational procedure to fulfill the e-navigation user need (HE) TCH1 | ◦ (150-Gtr01-Sre01) Revise the Convention and related regulation to adopt standardized operational procedure (HE) REG2 | ◦ (150-Gtr01-Str01) Training for a system should be focused to standardized operational procedure of e-navigation ◦ (150-Gtr01-Str02) Functions dependent on manufacturer or type should be trained and covered by familiarization material and/or on-site training and on-line help function (HE) TRN1-TRN2-TRN3-TRN4-TRN5 |
|  |  |  |  |  |  | Current training regime does not go far enough to specify type specific training needs for seafarers. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Lack of auditable and structured training regime for type specific complex systems such as the operation of ECDIS. (T) T11 (E) E33-E34 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 150-Gtr02 | Insufficient training in correct use and activation of priority messages. (T) T22-T32 (E) E21-E33 | 2,3,7 | 5,9 | A37,A38,A39 | GMDSS, AIS | 1,2,3,4,5 | ◦ (150-Gtr02-Sop01) User should have a simple procedure to activate priority message (HE) OPR2-OPR3 | ◦ (150-Gtr02-Ste01) Develop the standardized human machine interface for priority message on GMDSS and AIS ◦ (150-Gtr02-Ste02) Activation for message should be managed on the display of navigation equipment such like ECDIS or INS (HE) TCH1 | ◦ (150-Gtr02-Sre01) Revise the related performance standard of GMDSS, ECDIS, INS, AIS ◦ (150-Gtr02-Sre02) Revise the IEC 611662 standard (HE) REG2 | ◦ (150-Gtr02-Str01) Revise the training course for simple and standardized procedure for priority message (HE) TRN4 |
| (2)SHOREBASED USERS | {1}INFORMATION / DATA MANAGEMENT | [1]Common Data Structure/Harmonized Data Formats | ◦ Effective and Robust Communications ◦ Collection of information ◦ Provision of information to vessels ◦ Shore-to-shore information exchange ◦ Management of information |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 211-Gte01 | Lack of a common maritime information/data structure harmonizing the policies for the security and use of data. | 1,3,5 | 2,8 | A2,A34,A35,A36 | n/a | 1,2,3,4,5 | ◦ (211-Gte01-Sop01) Shorebased user such as VTS or Authorities should have a function to exchange information with ship based on international standard ◦ (211-Gte01-Sop02) Related information and data for shorebased users should be integrated and managed in common maritime data structure based on S-100 (211-Gte01-Sop03) IALA Rec on harmonized functional requirements for networking and info exchange (IVEF). Work on this has been finalized by the IALA e-NAV/VTS Committees (awaiting Council approval - as of March 2011). This work needs to synchronized between VTS and e-NAV (HE) OPR2-OPR4 | ◦ (211-Gte01-Ste01) Develop the common maritime data structure to manage all required information and data ◦ (211-Gte01-Ste02) Revise the software of shorebased system in order to use the developed CMDS for their intended functions ◦ (211-Gte01-Ste03) Information exchange between shorebased users should be handled with the internet based protocol(ex. TCP/IP) including the encryption (211-Gte01-Ste04) systems and software will need to support the new IVEF (HE) TCH1 | ◦ (211-Gte01-Sre01) Revise the Convention and related regulations to adopt the common maritime data structure ◦ (211-Gte01-Sre02) Develop and adopt the CMDS as a international standard ◦ (211-Gte01-Sre03) Develop or adopt current one as an international standard for ensuring the security of information exchange (HE) REG1 | ◦ (211-Gte01-Str01) Develop the training course for shorebased users to be familiarized with the CMDS and its contents of their own work (HE) TRN4 |
|  |  |  |  |  |  | Insufficient identification of harmonization needs for standards, formats and protocols. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Lack of protocols, formats and data structure that enable shore based authorities to exchange information with other authorized shore based users. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | No standardized format for data exchange between VTS centers and other e-Nav stakeholders. (T) T11-T22-T31-T32 (E) E21 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 211-Gte02 | There is a gap between information capability of current information management systems and those that will be required as volumes of information increases. | 1,5 | 8 | n/a | n/a | 1,2,3,4,5 | ◦ (211-Gte02-Sop01) User should be able to identify the priority and recognize easily its contents among huge volume of increased information (HE) OPR1-OPR3-OPR5 | ◦ (211-Gte02-Ste01) Information management system(ex. VTS, CMDS DB) should have enough capacity of s/w and h/w to manage the volumes of information of e-navigation ◦ (211-Gte02-Ste02) CMDS should have a parameter for priority, source and ownership of information ◦ (211-Gte02-Ste03) Information management system should provide the function to support decision making of users based on the processing of information (HE) TCH1 | ◦ (211-Gte02-Sre01) Develop the guideline or standard of capability and function for information management system of e-navigation (IALA) (HE) REG3 | ◦ (211-Gte02-Str01) Develop the training course for shorebased users to be familiarized with the information management system and its decision-making support function (HE) TRN2 |
|  |  |  |  |  |  | Tools that have the capability to manage increased levels/volumes of information are not in use. (T) T11-T22-T31-T32 (E) E21 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Regulatory | 211-Gre01 | Inconsistent rules that require some coastal states to maintain domain awareness. | 1,3 | 1,2,10 | A32 | n/a | 1,2 | ◦ n/a | ◦ (211-Gre01-Ste01) Identify the element of information for global agreement to ensure domain awareness of coastal states (HE) TCH1 | ◦ (211-Gre01-Sre01) Revise the Convention to address the responsibility of the coastal states to keep domain awareness for maritime operation (HE) REG3 | ◦ n/a |
|  |  |  |  |  |  | Insufficient collection of data required to establish accurate marine domain awareness. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Operational |  | No identified Gap |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Training |  | No identified Gap |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
| (2)SHOREBASED USERS | {2}EFFECTIVE AND ROBUST VOICE COMMUNICATION AND DATA TRANSFER | n/a | ◦ Effective and robust communication |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical |  | No identified Gap |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Regulatory | 220-Gre01 | In the Initial GAP Analysis of the Annex of 23/8/10 does not fully extend to the shore to ship communication that is necessary to give the ship the information it needs. (T) T11-T22-T31-T32 (E) E21 | 5 | 3 | A2,A33,A34,A35,A36 | GMDSS, AIS, VSAT | 1,2,3,4,5 | ◦ (220-Gre01-Sop01) Shorebased user should be able to send an information easily both by broadcasting to all and by polling to designated user using ergonomic way rather than MMSI concept (HE) OPR1-OPR2-OPR4 | ◦ (220-Gre01-Ste01) Shipboard communication system should be developed to communicate based on IP technology ◦ (220-Gre01-Ste02) Shorebased system should have a human machine interface to call mobile station with MMSI or deliberate identifier such as IP address (HE) TCH1 | ◦ (220-Gre01-Sre01) Revise the maritime interface standard for an environment of internet based communication (HE) REG2 | ◦ (220-Gre01-Str01) Revise shorebased user training course to be familiarized with new system (HE) TRN4 |
|  |  |  |  | Operational |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Training | 220-Gtr01 | Lack of international guidance on security of data and its sharing (T) T6 | 2,5 | 3,9 | n/a | GMDSS, AIS, INMARSAT, VSAT | 1,2,3,4,5 | (220-Gtr01-Sop01) Develop guidance on the security of data and its sharing. Data security issues will need to be addressed in SOPs (HE) OPR5 | (220-Gtr01-Ste01) Issues may include, AIS data, single window, encryption etc (HE) TCH1 | (220-Gtr01-Sre01) Compliance with guidance will have to be addressed by national law. (HE) REG3 | (220-Gtr01-Str01) Stakeholders will need to be trained to comply with data security requirements (HE) TRN5 |
| (2)SHOREBASED USERS | {3}SYSTEMS AND EQUIPMENT | [5]Presentation of Information | ◦ Management of information ◦ Provision of information to vessels ◦ Efficient provision of information to the mariner in difficult navigational and meteorological circumstances or in case of defects and deficiencies. This service is normally provided at the request of the vessel or by the VTS when deemed necessary. This pertains to VTS operator providing assistance in determining vessel’s position and providing advice to assist in the on board navigational decision making. VTS operator should have confidence that the information collected and sent to the ship is correct. Must be capable of establishing effective communication with the bridge team. 1.0 Provide Navigational Assistance Services (NAS) 1.1 Provide navigation information, warning and advice 1.2 Provide instructions subject to the authority of the VTS. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 235-Gte01 | Insufficient delivery and presentation of maritime information that shore based authorities are required to provide to ships. | 2,3,5 | 2,7,9 | n/a | n/a | 1,2,3,4,5 | (235-Gte01-Sop01) Both ship and shore will need procedures and training in best practice for the efficient use of VTS services. (235-Gte01-Sop02) Human element issues for the presentation of VTS information onboard need to be considered (HE) OPR1-OPR2-OPR4-OPR5-OPR6 | (235-Gte01-Ste01) Both ship and shore systems will need to support the agreed common data structure and format for the exchange of e-navigation information. (HE) TCH1 | (235-Gte01-Sre01) Consideration should be given to the minimum equipment requirement for both shore and ship, to exchange e-navigation information (HE) REG1 | (235-Gte01-Str01) Training should be provided for the procedures and use of the equipment both on board and ashore (HE) TRN4-TRN5 |
|  |  |  |  |  |  | There are no standard data formats for on board capture and presentation that cover the entire scope of information provided by a VTS (T) T22 (E) E21 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Regulatory |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Operational | 235-Gop01 | Lack of harmonized presentation of domain awareness to improve situational awareness for allied and other support services. (T) T22 (E) E21 | 2,3,5 | 2,7,9 | n/a | VTS | 1,2,3,4,5 | ◦ (235-Gop01-Sop01) Shorebased user should have a harmonized presentation of their system to support situational awareness (HE) OPR1-OPR2-OPR4-OPR5-OPR6 | ◦ (235-Gop01-Ste01) Information structure or data format of domain awareness should be defined and developed ◦ (235-Gop01-Ste02) Symbology and presentation library like S-52 should be developed for these information (HE) TCH1-TCH2 | ◦ (235-Gop01-Sre01) Develop CMDS considering data structure for domain awareness ◦ (235-Gop01-Sre02) Develop international standard or guidance for presentation of domain awareness information (HE) REG1 | ◦ (235-Gop01-Str01) Develop training course of shorebased user to be familiarized with new presentation and its contents (HE) TRN4-TRN5 |
|  |  |  |  | Training |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
| (2)SHOREBASED USERS | {4}SHIP REPORTING | n/a | ◦ Collection of information ◦ Management of information ◦ Shore-to-shore information exchange |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Regulatory |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Operational | 240-Gop01 | Management of report caused by the needs to report, for safety, commercial and legislative reasons require time and effort. (T) T22-T23 (E) E21 | 1,2,3,5,7 | 1,2,3,4,8,10 | A16 | n/a | 1,2,3,4,5 | ◦ (240-Gop01-Sop01) Management of report by shorebased user should be supported by the system with automatic way to reduce required time and effort ◦ (240-Gop01-Sop02) Standardized way and procedure of various report should be developed (HE) OPR2 | ◦ (240-Gop01-Ste01) Digital format for report should be developed ◦ (240-Gop01-Ste02) Management system in s/w and h/w should be developed to handle various report from ship in type of digital format (HE) TCH1-TCH2 | ◦ (240-Gop01-Sre01) Make global agreement and adopt international standard for digital format of various report between ship and shore ◦ (240-Gop01-Sre02) Develop the guideline to design the management system for report (HE) REG1 | ◦ (240-Gop01-Str01) Develop the training course to be familiarized with the management system for report handling (HE) TRN2 |
|  |  |  |  | Training |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
| (2)SHOREBASED USERS | {5}TRAINING AND FAMILIARIZATION | n/a | ◦ Training and Familiarization of shorebased users |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Regulatory |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Operational |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Training | 250-Gtr01 | IALA VTS guidance may not be being developed in harmony with the concepts of e-navigation | 1,2 | 9 | n/a | VTS | 1,2 | (250-Gtr01-Sop01) IALA should establish procedures to ensure that guidelines and recommendations are harmonized with the development of e-navigation (HE) OPR4-OPR5-OPR6-OPR7 | n/a | n/a | (250-Gtr01-Str01) Consideration should be given to communicating the scope and impact of e-Navigation to other committees (HE) TRN2-TRN3 |
|  |  |  |  |  |  | VTS Operating procedures and guidelines should be harmonized with e-navigation (T) T31 (E) E21 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 250-Gtr02 | Not only the shipboard users but also shore-based users (e.g., Pilot, VTS operators, etc.) need to be appropriately trained in order to efficiently use and obtain the maximum benefit of e-navigation. (T) T39 (E) E21-E22 | 1,2 | 9 | n/a | n/a | 1,2 | ◦ (250-Gtr02-Sop01) Shore-based user can have enough information by on-line help function consisted of familiarization material for their operation (HE) OPR4-OPR5-OPR6-OPR7 | ◦ (250-Gtr02-Ste01) In software, there should be on-line function to show enough material to be trained continuously and be informed with updated data (HE) TCH1 | ◦ (250-Gtr02-Sre01) Develop the guidance to design shore-based system to be human-oriented way ◦ (250-Gtr02-Sre02) Revise the certification requirement of shore-based users to be more familiarized with computerized function and system (HE) REG3 | ◦ (250-Gtr02-Str01) Revise shorebased user training course to be familiarized with new system (HE) TRN2-TRN3 |
| (2)SHOREBASED USERS | {6}TRAFFIC MONITORING | n/a | ◦ Management of information ◦ Provision of information to vessels, ◦ Quality assurance, ◦ Shore to shore information exchange ◦ (IALA VTS committee)This refers to acquiring information at the VTS from ship and shore to develop accurate domain awareness for the early identification of risk and effective response to support efficiency, safety and security of marine traffic and environment protection. 1. Monitor Traffic Situation, 1.1 Acquire and manage information, 1.2 Monitor individual ship , 1.3 Assess traffic situation, 1.4 Assess environmental conditions ◦ (IALA VTS Committee) Efficient provision of information to the mariner in difficult navigational and meteorological circumstances or in case of defects and deficiencies. This service is normally provided at the request of the vessel or by the VTS when deemed necessary. This pertains to VTS operator providing assistance in determining vessel’s position and providing advice to assist in the on board navigational decision making. VTS operator should have confidence that the information collected and sent to the ship is correct. Must be capable of establishing effective communication with the bridge team. 1.0 Provide Navigational Assistance Services (NAS), 1.1 Provide navigation information, warning and advice, 1.2 Provide instructions subject to the authority of the VTS. ◦ (IALA VTS Committee) The services of the VTS centre should be maintained during an emergency response. Other authorities e.g. SAR, Environmental responses and Security agencies might be responsible for handling external emergencies within the VTS area. If a marine incident occurs or is likely to occur in a VTS area, VTS can be used to support incident management and mitigation. In the context of vessel traffic safety VTS may support responsible authorities involved with Maritime Assistance Services (MAS), places of refuge, security incidents, Search and Rescue (SAR) operations, pollution response and salvage operations. In some VTSs such operations are carried out under the supervision of the VTS authority. - Manage Incidents & Emergency Management by a VTS ◦ (IALA VTS committee) The traffic organization service concerns the operational management of traffic and the forward planning of vessel movements to prevent congestion and dangerous situations, and is particularly relevant in times of high traffic density or when the movement of special transports may affect the flow of other traffic. The service may also include establishing and operating a system of traffic clearances or VTS sailing plans or both in relation to priority of movements, allocation of space, mandatory reporting of movements in the VTS area, routes to be followed, and speed limits to be observed or other appropriate measures which are considered necessary by the VTS authority. 1.1 Provide Traffic Organization Services (TOS), 1.2 Plan traffic organization criteria, 1.3 Plan traffic flow, 1.4 Allocation of time slots, 1.5 Assign and monitor anchoring, 1.6 Issue a clearance and allocate departure time, 1.7 Monitor adherence to applicable rules |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 260-Gte01 | Traffic monitoring Tools that have the capability to manage increased levels/volumes of information are not in use. (T) T22-T31-T32 (E) E21 | 1,5 | 8 | n/a | n/a | 1,2,3,4,5 | ◦ (260-Gte01-Sop01) VTS User should be able to identify the priority and recognize easily its contents among huge volume of increased information from ship (HE) OPR1-OPR2-OPR3-OPR5-OPR6 | ◦ (260-Gte01-Ste01) Information management system(ex. VTS, CMDS DB) should have enough capacity of s/w and h/w to manage the volumes of information of e-navigation ship and other shore-based authorities ◦ (260-Gte01-Ste02) CMDS should have a parameter for priority, source and ownership of information ◦ (260-Gte01-Ste03) Information management system of VTS should provide the function to support decision making of operators based on the processing of information (HE) TCH1 | ◦ (260-Gte01-Sre01) Develop the guideline or standard of capability and function for information management system of e-navigation (IALA) (HE) REG2 | ◦ (260-Gte01-Str01) Develop the training course for shorebased users to be familiarized with the information management system and its decision-making support function (HE) TRN2-TRN3 |
|  |  |  |  |  | 260-Gte02 | Current VTS hardware may not have the capacity for increased collection, integration, exchange, presentation, storage and analysis of data. (T) T22-T31-T32 (E) E21 | 2,5,7 | 2,6,7 | A33 A33.1/2/3/4/5 | - | 1,2 | (260-Gte02-Sop01) To provide guidance to VTS authorities on type and volume expected within an e-Nav environment (HE) OPR1-OPR2-OPR4 | (260-Gte02-Ste01) Migrate to new hardware and software. VTS to consider how it receives and integrates additional information. Address bandwidth and processing capabilities. (HE) TCH1-TCH2 | n/a | (260-Gte02-Str01) identify training required for applying guidance (HE) TRN4-TRN5 |
|  |  |  |  |  | 260-Gte03 | Lack of procedures that enable shore based authorities to monitor quality of navigation systems on board as well as quality of information and effectiveness of communication. | 1,3,7 | 1,2,10 | n/a | n/a | 1,2,3,4,5 | ◦ n/a | ◦ n/a | ◦ (260-Gte03-Sre01) For automated and remote monitoring for onboard system, there should be an international agreement by IMO ◦ (260-Gte03-Sre02) Revise the maritime system interface standard(ex. IEC 61162) to cover this issue (HE) REG2 | ◦ n/a |
|  |  |  |  |  | 260-Gte04 | Current VTS hardware/software may not have the capacity for real time display of vessels’ track to provide a NAS or TOS service (T) T3-T22-T31-T32 (E) E21 | 2,5,7 | 2,8,10 | A35,A36 | VTS | n/a | (260-Gte04-Sop01) VTS authorities should assess the hardware and software requirements for providing NAS and TOS (HE) OPR4 | (260-Gte04-Ste01) Migrate to new hardware and software. VTS to consider how it receives and integrates additional information. Address bandwidth and processing capabilities. (HE) TCH1 | n/a | (260-Gte04-Str01) identify training required for the provision of NAS and TOS (HE) TRN4-TRN5 |
|  |  |  |  |  | 260-Gte05 | Some operating systems and software are no longer supported. In some VTS’, there is a problem of interoperability between applications (T) T3-T22-T31-T32 (E) E21 | 5,7 | 2,8,10 | n/a | VTS | n/a | (260-Gte05-Sop01) Consider IALA guidance on inter-VTS exchange format (IVEF) service. In the review of IALA Rec V 128, address issue of system and software life cycles (HE) OPR2 | (260-Gte05-Ste01) Migrate to new software. VTS to consider how it receives and integrates additional information. Address bandwidth and processing capabilities. (HE) TCH1 | n/a | (260-Gte05-Str01) Provide educational material for addressing and managing system life cycle issues (HE) TRN4 |
|  |  |  |  |  | 260-Gte06 | Bandwidth limitations shore / ship. Shortage of VHF marine frequencies INMARSAT C is a “store and forward” system. Speed of data exchange and capacity varies. There can be unintended interference between ship transmissions (e.g. AIS) and shore based infrastructure. (T) T22-T31-T32 (E) E21 | 5 | 3 | n/a | GMDSS, INMARSAT, AIS | 1,2,3,4,5 | (260-Gte06-Sop01) To provide guidance to VTS authorities on expected volume of data transfer and bandwidth implications within an e-Nav environment. Communication means to be transparent and user friendly to the end user (HE) OPR2-OPR4 | (260-Gte06-Ste01) Evaluate new technologies (e.g. WiMAX, Voice over IP) to meet ship/shore operational requirements (HE) TCH1-TCH2 | (260-Gte06-Sre01) Consider regulations to assure inter-operability between ship/shore (HE) REG2 | (260-Gte06-Str01) Provide adequate training for all communication needs (HE) TRN4 |
|  |  |  |  |  | 260-Gte07 | Inaccurate AIS data. (T) T12-T13-T22-T31-T32 (E) E21-E32 | 1,3,7 | 10 | n/a | AIS | 1,2,3,4,5 | (260-Gte07-Sop01) Vessels to have procedure within the SMS to check for accuracy of AIS data on a regular basis. (260-Gte07-Sop02) Shore authorities to have procedure to notify vessel and report to PSC. (260-Gte07-Sop03) Reduction in errors through improved design and integration (HE) OPR5 | (260-Gte07-Ste01) Technical solutions to address standards of installation and sensor reliability (HE) TCH1 | (260-Gte07-Sre01) Regular inspection by Port State Control and annual inspection (MSC 87 amendments to SOLAS) will minimize inaccurate broadcasts from AIS Class A units. (HE) REG3 | (260-Gte07-Str01) Ensure training identifies AIS errors and their likely causes (HE) TRN5 |
|  |  |  |  | Regulatory |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Operational | 260-Gop01 | Lack of common understanding of the scope and procedures of NAS and TOS internationally (T) T22-T32 (E) E21 | 1,3,5 | 2,3,4 | A35,A36 | n/a | 1,2,3,4,5 | (260-Gop01-Sop01) Implementation of NAS guidance (IALA Guideline No 1068) (260-Gop01-Sop02) Development of TOS guidance (HE) OPR5-OPR7 | n/a | (260-Gop01-Sre01) Consideration should be given to providing procedural guidance for NAS and TOS in IALA V 103 model course and the STCW convention (HE) REG3 | (260-Gop01-Str01) Training for NAS and TOS to be provided to ship and shore personnel. The use of simulation should be considered (HE) TRN3-TRN5 |
|  |  |  |  | Training | 260-Gtr01 | Not all VTS Operators are trained to IALA V-103 model training courses. Not all VTS training organizations have accredited VTS training courses (T) T31 (E) E21 | 2 | 9 | n/a | n/a | n/a | (260-Gtr01-Sop01) IALA to maintain the V 103 suite of model courses to evolving operational needs. Encourage training organizations to become accredited according to IALA guidelines (HE) OPR5-OPR7 | (260-Gtr01-Ste01) Use of simulation for training and assessment should be considered (HE) TCH1-TCH2 | (260-Gtr01-Sre01) The need for mandatory training for VTSOs, including certification / accreditation of training institutes (HE) REG1 | (260-Gtr01-Str01) All VTS training courses to adopt V 103 model training courses (HE) TRN3-TRN5 |
|  |  |  |  |  | 260-Gtr02 | There is a lack of understanding by seafarers as to the type of VTS service being provided in many ports (T) T23 (E) E21 | 2 | 9 | n/a | n/a | n/a | (260-Gtr02-Sop01) IALA needs to develop guidance for VTS authorities as to how they identify their operations and services to mariners (HE) OPR5-OPR7 | (260-Gtr02-Ste01) where data is provided in an e-navigation environment, information from a VTS should clearly indicate the type of service/s provided (HE) TCH1 | ◦ n/a | (260-Gtr02-Str01) V 103 model training courses and STCW need to include training in this regard (HE) TRN3-TRN5 |
| (3)SEARCH ADS RESCUE | {1}INFORMATION / DATA MANAGEMENT | n/a | ◦ SAR should have access to relevant information contained within the e-navigation domain ◦ Need of other data: .1 passenger/crew list;, .2 passenger tracking systems data; .3 hazardous cargo; .4 evacuation plans; .5 ships plans; .6 number of lifeboats/liferafts/life saving appliances (Survival suits); .7 EPIRB/SART and AIS/SART data; .8 SAR- co-operation plan data for passenger ships; and .9 ship's IMO number ◦ Additional Information – Responder .1 receive data as a vessel comes into area of SAR operations that informs shore stations on the ship's communications options available. Not just GMDSS but consider Email/Fax/Broadband; .2 information on a vessel's capabilities for supporting SAR operations; Fast Rescue Craft/Hospitals/Fire fighting; .3 identify vessels that provide Met data – direct access to met data on ships; .4 use vessel data for coverage prediction based on height of eye above sea level; and .5 understand constraints of ship data for example: type/draught/maneuvering capabilities. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 310-Gte01 | Lack of mechanisms to provide SAR (RCC) function with the full range of relevant e-navigation information in digital format. | 1,3,5,7 | 3,5 | A39 | AIS | 1,2,3,4,5 | ◦ (310-Gte01-Sop01) SAR Authority should have a access rights to use any digital information of shipside and/or shoreside e-navigation system to be required for SAR activity through the robust communication channel with high priority (HE) OPR3-OPR4-OPR5-OPR6-OPR7 | ◦ (310-Gte01-Ste01) Identify the digital data required for SAR operation ◦ (310-Gte01-Ste02) Develop the efficient way to use digital data via communication channel between SAR and ship ◦ (310-Gte01-Ste03) Develop the security system only to permit access from SAR authority (HE) TCH1 | ◦ (310-Gte01-Sre01) There should be an international agreement to promote the facility of SAR Authority to handle the digital data from e-navigation system ◦ (310-Gte01-Sre02) In regulations, assign access rights to permit SAR authority can access this data officially without unnecessary grant step (HE) REG1 | ◦ n/a |
|  |  |  |  |  |  | Hardware: Resources and capability available for infrastructure can be lacking and therefore tools needed for accessing digital data may not be available. Lack of data in digital format. (T) T29 (E) E21-E23 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Regulatory |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Operational | 310-Gop01 | Insufficient access to and quality of information from ships in distress. (T) T29-T22 (E) E21-E23 | 1,3,5 | 1,3,5 | n/a | AIS | 1,2,3,4,5 | ◦ (310-Gop01-Sop01) Ship have to usually collect and integrate related data needed for SAR operation ◦ (310-Gop01-Sop02) SAR authority should have a function to request ship's data on-line to recognize the status of ship in emergency (HE) OPR5-OPR6 | ◦ (310-Gop01-Ste01) Identify required information and/or data to be provided for SAR operation in emergency case ◦ (310-Gop01-Ste02) AIS should have a message for exchange of SAR related information between ship and SAR authority (HE) TCH1 | ◦ (310-Gop01-Sre01) Revise the Convention for access to information of ships in distress ◦ (310-Gop01-Sre02) Revise AIS message guidance (HE) REG3 | ◦ n/a |
|  |  |  |  |  | 310-Gop02 | Insufficient support of the use of LRIT data for SAR. (T) T29 (E) E21-E23 | 1,3,5 | 1,3,5 | n/a | LRIT | 1,2,3,4,5 | ◦ (310-Gop02-Sop01) SAR Authority should be able to access LRIT data for SAR operation even onboard of SAR ship (HE) OPR5-OPR6 | ◦ (310-Gop02-Ste01) Consider and revise LRIT scheme to promote more efficient way to access LRIT data by SAR authority (HE) TCH1 | ◦ (310-Gop02-Sre01) IMO should support and promote the efficient access to the LRIT data by SAR authority (HE) REG3 | ◦ n/a |
|  |  |  |  | Training |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
| (3)SEARCH ADS RESCUE | {2}EFFECTIVE AND ROBUST VOICE COMMUNICATION AND DATA TRANSFER | n/a | ◦ Effective Communication and information sharing ◦ Priority for distress communications |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 320-Gte01 | Lack of an automated data network connecting all stakeholders in SAR intervention, including improved communication between RCC and shore-, land-, sea- and air-based entities. | 1,3,5,7 | 1,3,10 | A38,A39 | GMDSS, AIS | 1,2,3,4,5 | ◦ (320-Gte01-Sop01) The management procedure of SAR operation and information to be shared between them should be developed with an automatic way in principle (HE) OPR2-OPR4-OPR7 | ◦ (320-Gte01-Ste01) AIS(or DSC) can be a solution to share SAR information between all stakeholders in SAR place if we have additional AIS message format for it ◦ (320-Gte01-Ste02) Shared information should be presented on navigational display to help situational awareness (HE) TCH1 | ◦ (320-Gte01-Sre01) Revise the AIS message guideline ◦ (320-Gte01-Sre02) Should be an international agreement for SAR management procedure ◦ (320-Gte01-Sre02) Revise the ECDIS standard (HE) REG2 | ◦ (320-Gte01-Str01) SAR person should be familiarized with the shared information and its presentation to recognize whole status of SAR operation (HE) TRN5 |
|  |  |  |  |  |  | Lack of access to the details of all relevant onboard communication and capabilities for SAR authorities. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Limited resources for communication infrastructure in SAR operation (T) T22-T31 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Regulatory | 320-Gre01 | Lack of solutions for maintaining priority for distress communication. | 1,3,5 | 3,5 | n/a | GMDSS, AIS | 1,2,3,4,5 | ◦ (320-Gre01-Sop01) VHF(or GMDSS) system should automatically change VHF channel into assigned it by AIS SAR message (HE) OPR1-OPR2-OPR4 | ◦ (320-Gre01-Ste01) Consider to allocate a VHF channel(not Ch.16) only for SAR operation when the stakeholder engaged in SAR operation ◦ (320-Gre01-Ste02) Potential AIS message for SAR should have a parameter to assign SAR operation channel of VHF in a field of SAR activity ◦ (320-Gre01-Ste03) Develop the new interface sentence(IEC 61162) between AIS and VHF equipment (HE) TCH1-TCH2 | ◦ (320-Gre01-Sre01)Assign high priority to the AIS message for SAR operation to be developed for e-navigation SAR system ◦ (320-Gre01-Sre02) Revise the standard and regulation of AIS and VHF(or GMDSS) (HE) REG2 | ◦ (320-Gre01-Str01) SAR person should only be acknowledged for this situation and not required additional training for this operation (HE) TRN5 |
|  |  |  |  |  |  | Potential loss of priority for distress communication. (E) E21-E23 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Operational | 320-Gop01 | Lack of an effective SAR system in some parts of the world. Lack of adequate number of trained personnel. (T) T11-T22-T29 (E) E29 | 1,3 | 5 | n/a | n/a | 1,2,3,4,5 | ◦ n/a | ◦ (320-Gop01-Ste01) The equipment and systems involved in SAR operation should be designed with considering ergonomics to have effective SAR system (HE) TCH1-TCH2 | ◦ n/a | ◦ (320-Gop01-Str01) On scene operator and coordinator should be advised by the decision support system with enough information for situation awareness (HE) TRN5 |
|  |  |  |  | Training |  | Ships manning does not take account of the potential of SAR, and also of the practice for SAR. | 1,3 | 5 | n/a | n/a | 1,2,3,4,5 | ◦ TBD | ◦ TBD | ◦ TBD | ◦ TBD |
| (3)SEARCH ADS RESCUE | {3}SYSTEMS AND EQUIPMENT | N/A | ◦ SAR Authorities need access to the details of all relevant onboard communication equipment and capabilities ◦ how do we accommodate non-GMDSS alerting and communications technology, including mobile technology and future developments in maritime communications systems? ◦ the current functions of the present GMDSS need to be supported and enhanced: .1 transmission of ship to shore distress alerts by at least two separate and independent means; .2 reception of shore to ship distress alert; .3 transmission and reception of ship-to-ship distress alerts; .4 transmission and reception of Search and Rescue (SAR) coordinating communications; .5 transmission and reception of on-scene communications; .6 transmission and reception of locating signals; .7 transmission and reception of maritime safety information (MSI); .8 transmission and reception of general radio communications to and from shore based radio systems or networks; and .9 transmission and reception of bridge to bridge communications ◦ improved safety, through promotion of standards in safe navigation supported by: .1 improved decision support enabling the mariner and competent authorities ashore to select relevant unambiguous information pertinent to the prevailing circumstances; and .2 a reduction in human error through provision of automatic indicators, warnings and fail-safe methods |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical | 330-Gte01 | Currently, collecting information pertaining to a distress situation consumes valuable time. (T) T22-T29-T32 (E) E19-E21-E23 | 1,2,5 | 1,3,5 | A38 | ◦ Inmarsat, ◦ VHF, ◦ MF, ◦ HF, ◦ cellular – depends on distance | 1,2,3,4,5 | (330-Gte01-Sop01) Simplification of distress and safety instruments and procedures. (HE) OPR1-OPR2-OPR4 | (330-Gte01-Ste01) Integrate a simplified GMDSS into the e-navigation concept (HE) TCH1-TCH2 | ◦ (330-Gte01-Sre01) GMDSS amendments, ◦ (330-Gte01-Sre02) Make it easier to establish voice communications (HE) REG2 | (330-Gte01-Str01) Explicit distress and safety training (HE) TRN3 |
|  |  |  |  | Regulatory |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Operational |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Training |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
| (3)SEARCH ADS RESCUE | {7}Operation | n/a | ◦ how do we accommodate the whole SAR system including non-SOLAS rescue crafts and SAR aircrafts? ◦ support the effective operation of contingency response, and search and rescue services |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Regulatory |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Operational |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Training |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
| (3)SEARCH ADS RESCUE | {7}Operation | [7]on-scene action | ◦ On-Scene Action .1 datum markers identifiable by all on-scene assets, for example: for drift/sea current and weather info; .2 locating devices; .3 monitor execution of search coverage plan; and .4 individual casualty location and tracking. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Technical |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Regulatory |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Operational |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |
|  |  |  |  | Training |  | No identified Gap. |  |  |  |  |  | ◦ | ◦ | ◦ | ◦ |