e-NAV10	Input paper

Agenda item	8

Task Number	6

Author(s)	M.Hoppe et al

Identified gaps for the future integrated PNT-System

# Summary

This paper represents the first draft of a gap analysis with regard to the provision of Position, Navigation & Timing functionality within e-Navigation.

## Purpose of the document

The committee is requested to develop the paper and use it to inform inputs to IMO regarding required PNT functionality.

## Related documents

References relate to separate input paper to e-NAV10 on Maritime Integrated PNT System.

# Background

With PNT defined as one of the core elements of e-Navigation, it is necessary to identify gaps incurrent service provision and functionality, in order that future systems can be defined to better meet these requirements.

# Discussion

The draft document identifies which needs are addressed by current PNT service provision, what is required within the scope of e-Navigation, and what issues require to be included as part of the transition to e-Navigation.

# References

- IMO Resolution A.915(22) Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System (GNSS).

# Action requested of the Committee

The Committee is requested to:

- Develop inputs to MSC 90, requesting that IMO develop Performance Standards for a multi-system receiver. The Performance Standards should be based on this gap analysis, once completed.

- Draft WP - Identified Gaps for the Future Integrated PNT-System

| Annex A - Draft WP - Identified gaps for the future integrated PNT-System - |  |  |  |  |
| --- | --- | --- | --- | --- |
| Aspect | Today | e-Navigation | Issues | Reference to |
| PNT-Functionality (Geodetic Datum) | The output of position information based upon WGS-84. If a sentence uses a datum other than WGS-84 then the IEC 61162 sentence DTM must be used. | The future PNT system should use one harmonised geodetic Datum which provides sufficient accuracy. | Should be taken into account when developing new performance standards for future PNT (Multi GNSS receiver). | Action Item 1.1 |
| PNT-Functionality (Geodetic Datum) | ENC is based on WGS84. The Accuracy is in a range of meters. | To use the future PNT system also for navigation in the harbour area the and for docking the overall accuracy should be in a range of 1 dm. | Improvement of ENC data (including depth information) accuracy is needed for harbour navigation and docking. To make use of exact depth information it is necessary to include tide information. Tide information should be provided from MSI services. | Action Item 1.1 |
| PNT-Functionality (Output data specification) | Performance standards per sensor are the normal case. | Completeness of requirements should be achieved. | Should be taken into account when developing new performance standards for future PNT. Primary PNT output data should be position (latitude, longitude, altitude), SOG, ROT, COG, Heading, and time. STW is derived from SDME; draught and air draught are manual inputs amended by additional factors (eg. squat). Vertical clearance and depth (UKC) are navigation relevant data which can be only derived by applications in combination with further externally provided data (tidal, ENC…). The Consistent Common Reference Point should be taken into account when developing new performance standards for future PNT. | Action Item 1.1 |
| PNT-Integrity | GPS receivers, type approved with respect to IEC 61108-1 Ed. 2, shall provide an indication if the position calculated is likely to be outside of the IMO performance standards; The function is performed by RAIM algorithms. The alarm threshold is preset by the user. | Automatic assessment of the provided accuracy and integrity of all PNT output data (Position, Navigation and Time) [User need] | The integrity functionality should be able to provide alarms according to the current requirements which exist in the different phases of navigation (ocean, harbour approach, harbour and Docking) Standardised methods should be used to evaluate the accuracy and integrity of the primary PNT output data. The provision of the assessment results requires the specification of secondary PNT output data (data and system integrity, alerts and status messages). Different Methods to indicate alarms are discussed: - Automatic detection of predefined service areas representing the different phases of navigation - The appropriate areas representing the different phases of navigation will be provided from responsible administrations by using Maritime Safety Information (MSI) services - Indication of the present status for all phases of navigation at the same time - Manual input of Alarm threshold by the user It should be noted that nowadays no accuracy assessment of navigation and timing data is performed. | Action Item 1.1 and 1.2 |
| PNT-Integrity | Integrity assessment is performed using a sensor specific approach. | Integrity assessment should be performed on functionalities to improve the alarm management. | Should be taken into account by the development of the future PNT system. | Action Item 1.2 |
| PNT-Integrity | Performance Requirements are only available for the position (Latitude and Longitude) | Performance requirements are also necessary for the following PNT output data: - Position (Altitude) - Navigation (Heading, COG, SOG and ROT) - Time (UTC) | Development of appropriate minimum performance standards on IMO level for the different phases of navigation (revision of IMO A.915(22)) with respect to all required PNT output data. | Action Item 1.2 |
| PNT-Integrity | Navigation parameters like STW or UKC are only provided from one sensor on board. Hence integrity assessment is not possible | TBD | STW, air draught and depth (UKC) are navigation relevant data which will be derived by applications in combination with further externally provided data (tidal, ENC…). | Action Item 1.2 |
| PNT-Integrity | Scalability of integrity | Quantification of reliability [User need] | Clarification is necessary, what does integrity really means Definition of different integrity levels (low-, medium and high level) | Action Item 1.2 |
| PNT-Standardisation and harmonisation | Output of onboard navigation data: Every navigation sensor provide a specific output data set - EPFS (position) - THD (Heading) - Etc. It is not allowed to derive a position from a THD. | Integration of different independent sensors within the PNT to improve accuracy and integrity of PNT data. | Revision of SOLAS regulations. Clarify the usage of different output data within the PNT system to derive resilient PNT output data. | Action Item 1.2 |
| PNT-Standardisation and harmonisation | All navigation sensors have different IMO performance standards and IEC test procedures. | One PNT performance standard containing requirements concerning accuracy, integrity, availability and continuity. Homogenous description of requirements, | Development of a new PNT performance standard Product notion (IMO->IALA) | Action Item 1.2 |
| GNSS-Augmentation | Terrestrial GNSS-Augmentation - MF-Radiobeacon - AIS-VDL Message#17 | Terrestrial GNSS-Augmentation - MF-Radiobeacon - AIS-VDL Message #17 - Maritime Ground Based Augmentation System (MGBAS) Space Based Augmentation Systems (SBAS) - EGNOS - WAAS - MSAS - GAGAN | IMO performance standards and IEC test procedures exists for terrestrial GNSS-Augmentation onboard components as MF-radiobeacon and AIS Message 17. MGBAS and SBAS are currently not recognised as maritime GNSS augmentation systems. No performance standards and test procedures are in place for maritime usage. MGBAS is needed for high precision navigation applications like docking. SBAS is integrated in nearly all new receiver equipment. SBAS has the potential to further improve the integrity functionality within the envisaged PNT-System. | Action Item 2.1 |
| GNSS-Augmentation | Shore site GNSS-Augmention services provide integrity information based on local and far field monitoring. Harmonised alarm threshold doesn’t exist. DGNSS services are not certificated. | DGNSS integrity monitoring should base on harmonized alarm thresholds. DGNSS should be certified. | Revise IALA Recommendations. Initiate certification of safety related services: Product notion (IMO-> IALA) | Action Item 2.1 |
| GNSS-Backup *) | Only Loran-C exist in some regions | Provision of Resilient PNT which take into account the vulnerability of GNSS | Initiate development of terrestrial backup systems for harbour approach: - R-Mode for MF radiobeacon and/or AIS) - eLoran | Action Item 2.2 |
| PNT- Communication *) | The GPS receiver equipment (IEC 61108-1 Ed. 2) provide at least one output from which position information can be supplied to other equipment in accordance to IEC 61162. | The new PNT module should use all relevant data from onboard navigation sensors. The PNT output data should provided to all onboard equipment (INS) which make use of PNT data. | The new standard IEC 61162-450 provides a TCP/IP-Ethernet connection which provides the required flexibility and data capacity for data communication. | Action Item 2.2 |
| PNT- Communication *) | Validity of output data (Differentiation against integrity). | PNT system should provide an indication of valid and invalid sensor data. | Traceable criteria to determine valid and invalid data have to developed Revision of appropriate performance standards. | Action Item 2.2 |

*) Gaps from Action Item, 2.2 which are not yet discussed in IALA working group