IEC TC 80 WG 15

074 IEC-AIS-WG 15 MTG 4

25 March 2011

INTERNATIONAL ELECTROTECHNICAL COMMISSION

TECHNICAL COMMITTEE No 80: Maritime navigation and radiocommunication equipment and systems

Working Group 15

# Liaison note to IALA AIS TWG:

# Request for technical clarifications on ITU-R M.1371-4

# 1. Introduction

IEC TC 80 WG 15 thanks IALA e-Nav Committee for their Liaison note to IEC from March 2011 responding a request of WG 15 for technical clarification on the behaviour of Class B SO.

IEC TC80 WG15 held a meeting at BSH in Hamburg, Germany, during the week of March 21st to discuss the IEC62287-2 SOTDMA Class B AIS standard and CDV comments to IEC61993-2 Edition 2 (Class A AIS).

The following points were raised during the meeting:

1. Request for change of the sensor priority list.

The Class A AIS unit uses a priority scheme for selection of internal or external position source. This table should be modified to give priority to external position under all conditions. The internal GPS should only be used as a backup when no external position is available, regardless of being differentially corrected or not.

Today there are conditions where the internal position will have priority over the external source. There shall be only one position source under normal conditions, and this position source shall be used for AIS.

The group noted an incident reported where a ferry and a fishing boat crashed. It was found that incorrect position from the AIS caused by reception of inaccurate correction data was used causing confusion.

2. Regarding NavStatus and reporting intervals

ITU.R M-1371 states in Table 1 that the required reporting interval of three minutes is valid when NavStatus is Moored or At Anchor. This is in conflict with section 4.3.1.3 where three minutes is also required for Aground and Not Under Command.

It is the opinion of the group that only Moored and At Anchor shall cause a three minute reporting interval. Section 4.3.1.3 should be adopted to Table 1.

3. Maximum number of consecutive slots

ITU.R M-1371 Annex 2 allows for a maximum length of five slot transmissions, excluding Class B units. The IMO SN Circ.289 recommends a maximum of three slots for Application Specific Messages. It is the opinion of the group that Class A units should be limited to three slot transmission in general.

4. Group Assignment Message 23

The assigned intervals “Next Longer” and “Next shorter” in Table 75 of ITU.R M-1371 Annex 8 are not clearly defined. These intervals should refer to the autonomous reporting intervals, not the fixed intervals in Table 75.

5. Stored Channel Management Areas

Current areas are sorted by time of reception. There is a limit of eight areas, where the oldest area will be deleted if a ninth is received. This happens even when the oldest area is being used.

It is the opinion of the group that the areas should be sorted by distance, not age. This would cause the most distant area to be deleted if a ninth area is received.

Action requested

WG15 asks IALA to consider these points for their AIS clarifications and in the next draft of the ITU AIS standard.

HÅKAN LINDLEY

Convenor IEC TC80 WG15

Swedish Transport Agency

Maritime Department

Box 653

SE-60115 NORRKÖPING

Mobile  +46 708 156800

E-mail     hakan.lindley@transportstyrelsen.com